3GPP TSG-RAN5 Meeting #101	R5-236589

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0548 | rev | - | Current version: | 17.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PICS for NR mpsPriorityIndication support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Technologies Int |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of mpsPriorityIndication is applicable for both NR and E-UTRA to differentiate the UE support of this feature, updating pc_mpsPriorityIndication_r16 with pc_NR_mpsPriorityIndication_r16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated pc_mpsPriorityIndication_r16 with pc_NR_mpsPriorityIndication_r16 in clause A.4.4 Table A 4.4-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PICS for mpsPriorityIndication for NR and EUTRA can not be differentiated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause A.4.4 |  |  |  |  |  |  |  |  |
| sss |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of modified section>

## A.4.4	Additional information

Table A.4.4-1: Additional information

| Item |  |  | Additional information |  |  | Ref. |  |  | Release |  |  | Mnemonic |  |  | Comments |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | Support of ICMP or ICMP IPv6 |  |  | RFC 792 OR RFC 4443, RFC 4884 |  |  | NA |  |  | pc_IP_Ping |  |  | UE supports ICMP or ICMPv6 protocol to enable IP Ping Operation |  |  |
| 2 |  |  | Support of IMS |  |  | 24.229, Annex U |  |  | Rel-15 |  |  | pc_IMS_5GS |  |  |  |  |  |
| 3 |  |  | Support of rachReport |  |  | 38.306, 4.2.17 |  |  | Rel-16 |  |  | pc_rachReport_r16 |  |  | UE supports delivery of rachReport upon request from the network. |  |  |
| 4 |  |  | Support of GNSS |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_GNSS_location_r16 |  |  | UE is equipped with a GNSS or A-GNSS receiver that may be used to provide detailed location information along with SON or MDT related measurements in RRC_CONNECTED, RRC_IDLE and RRC_INACTIVE. |  |  |
| 5 |  |  | Support of UL PDCP Packet Average Delay |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_PDCP_Delay_r16 |  |  | UE supports UL PDCP Packet Average Delay measurement and reporting in RRC_CONNECTED state |  |  |
| 6 |  |  | Support logged MDT |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_loggedMeasurements_r16 |  |  | UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. |  |  |
| 7 |  |  | Support of uncompensated barometric pressure measurement reporting |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_barometer_r16 |  |  | UE supports uncompensated barometric pressure measurement reporting upon request from the network. |  |  |
| 8 |  |  | Support of orientation information reporting |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_orientation_r16 |  |  | UE supports orientation information reporting upon request from the network. |  |  |
| 9 |  |  | Support of speed information reporting |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_speed_r16 |  |  | UE supports speed information reporting upon request from the network. |  |  |
| 10 |  |  | Support of Bluetooth measurements in RRC_CONNECTED state |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_immMeasBT_r16 |  |  | UE supports Bluetooth measurements in RRC_CONNECTED state. |  |  |
| 11 |  |  | Support of WLAN measurements in RRC_CONNECTED state |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_immMeasWLAN_r16 |  |  | UE supports WLAN measurements in RRC_CONNECTED state. |  |  |
| 12 |  |  | Support of Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_loggedMeasBT_r16 |  |  | UE supports Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state. |  |  |
| 13 |  |  | Support of WLAN measurements in RRC_IDLE and RRC_INACTIVE state |  |  | 38.306, 4.2.18 |  |  | Rel-16 |  |  | pc_loggedMeasWLAN_r16 |  |  | UE supports WLAN measurements in RRC_IDLE and RRC_INACTIVE state. |  |  |
| 14 |  |  | Support of SDT in RRC_INACTIVE state via Random Access Procedure |  |  | 38.306, 4.2.2 |  |  | Rel-17 |  |  | pc_ra_SDT_r17 |  |  | UE supports SDT via Random Access procedure in RRC_INACTIVE state |  |  |
| 15 |  |  | Support of SRB SDT in RRC_INACTIVE state |  |  | 38.306, 4.2.2 |  |  | Rel-17 |  |  | pc_srb_SDT_r17 |  |  | UE supports SRB SDT in RRC_INACTIVE state |  |  |
| 16 |  |  | Support of SDT in RRC_INACTIVE state via Configured Grant Type 1 |  |  | 38.306, 4.2.7.2 |  |  | Rel-17 |  |  | pc_cg_SDT_r17 |  |  | UE supports SDT via Configured Grant Type 1 in RRC_INACTIVE state |  |  |
| 17 |  |  | Support of NR NTN access |  |  | 38.306, 4.2.2 |  |  | Rel-17 |  |  | pc_nonTerrestrialNetwork_r17 |  |  | UE supports NR NTN access. |  |  |
| 18 |  |  | Support of RRC INACTIVE state in NTN |  |  | 38.331, 6.3.3 |  |  | Rel-17 |  |  | pc_inactiveStateNTN_r17 |  |  | UE supports RRC INACTIVE state in NTN |  |  |
| 19 |  |  | Support of RA-SDT in NTN |  |  | 38.331, 6.3.3 |  |  | Rel-17 |  |  | pc_ra_SDT_NTN_r17 |  |  | UE supports RA-SDT in NTN |  |  |
| 20 |  |  | Support of SRB-SDT in NTN |  |  | 38.331, 6.3.3 |  |  | Rel-17 |  |  | pc_srb_SDT_NTN_r17 |  |  | UE supports SRB-SDT in NTN |  |  |
| 21 |  |  | Support of storage and delivery of multiple CEF reports |  |  | 38.306, 4.2.18 |  |  | Rel-17 |  |  | pc_multiple_CEF_Report_r17 |  |  | UE supports the storage and delivery of multiple CEF reports upon request from the network |  |  |
| 22 |  |  | Support of the storage of Early Measurement Logging in logged measurements. |  |  | 38.306, 4.2.18 |  |  | Rel-17 |  |  | pc_ earlyMeasLog_r17 |  |  | UE supports the storage of Early Measurement Logging in logged measurements and the reporting upon request from the network as specified in TS 38.331 |  |  |
| 23 |  |  | Support of IDC problem detection |  |  | 38.331, 6.2.2 |  |  | Rel-17 |  |  | pc_inDeviceCoexDetected_r17 |  |  | UE supports that measurement logging is suspended due to IDC problem detection |  |  |
| 24 |  |  | Support of delivery of on-Demand SI information upon request from the network |  |  | 38.306, 4.2.17 |  |  | Rel-17 |  |  | pc_onDemandSI_Report_r17 |  |  |  |  |  |
| 25 |  |  | Support of the storage and delivery of 2-step RACH related information upon request from the network |  |  | 38.306, 4.2.17 |  |  | Rel-17 |  |  | pc_twoStepRACH_Report_r17 |  |  |  |  |  |
| 26 |  |  | Support of mpsPriorityIndication on RRC release with redirect |  |  | 38.306, 4.2.2 |  |  | Rel-16 |  |  | pc_NR_mpsPriorityIndication_r16 |  |  | UE supports mpsPriorityIndication on RRC release with redirect as specified in TS 38.331 |  |  |
| 27 |  |  | Support of RLF-Report for conditional handover |  |  | 38.306, 4.2.17 |  |  | Rel-17 |  |  | pc_rlfReportCHO_r17 |  |  | UE supports RLF-Report for conditional handover. |  |  |
| 28 |  |  | Support of RLF-Report for DAPS handover. |  |  | 38.306, 4.2.17 |  |  | Rel-17 |  |  | pc_rlfReportDAPS_r17 |  |  | UE supports RLF-Report for DAPS handover. |  |  |
| 29 |  |  | Support of the storage and delivery of Successful Handover Report. |  |  | 38.306, 4.2.17 |  |  | Rel-17 |  |  | pc_success_HO_Report_r17 |  |  | UE supports the storage and delivery of Successful Handover Report. |  |  |

### <End of modified section>