3GPP TSG-RAN5 Meeting #101	R5-236607

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0350 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new TC 6.5.4 User Regroup CT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NIST |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2023-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There was no test case for User Regroup CT |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added TC 6.5.4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MCPTT Clients could fail in the field without conformance verification of User Regroup CT |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### <START OF CHANGES>

### 6.5.4	On-network / Regroup using a preconfigured group / Creating a user regroup using preconfigured group / Removing a user regroup using preconfigured group / Client Terminated (CT)

6.5.4.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { UE (MCPTT Client) receives a request via a SIP MESSAGE message to create an MCPTT user regroup using a preconfigured group }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message }

}

(2)

with { UE (MCPTT Client) having established an MCPTT user regroup using a preconfigured group }

ensure that {

when { UE (MCPTT Client) receives a request to establish an MCPTT On-demand Pre-arranged Group Call using the user regroup }

then { UE (MCPTT Client) sends a SIP INVITE message using the user regroup to establish an MCPTT On-demand Pre-arranged Group Call }

}

(3)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the MCPTT User engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server }

}

(4)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call }

ensure that {

when { MCPTT User requests to terminate the ongoing MCPTT Group Call }

then { UE (MCPTT Client) sends a SIP BYE message and leaves the MCPTT session }

}

(5)

with { UE (MCPTT Client) having an affiliation with an established MCPTT user regroup }

ensure that {

when { UE (MCPTT Client) receives a request to remove the user regroup via a SIP MESSAGE message }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message }            }

6.5.4.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 16.3.1.3, 16.2.1.3, 16.3.1.4, 16.2.1.4. Unless otherwise stated these are Rel-16 requirements.

[TS 24.379, clause 16.3.1.3]

The procedure in clause 16.2.1.3 is used by the MCPTT client when the MCPTT server notifies the MCPTT client of the creation of a user regroup using preconfigured group.

[TS 24.379, clause 16.2.1.3]

Upon receiving a "SIP MESSAGE request to the MCPTT client to request creation of a regroup using preconfigured group", the MCPTT client:

1)	should notify the MCPTT user of the creation of the regroup using preconfigured group;

2)	shall send a 200 (OK) response to the MCPTT server according to 3GPP TS 24.229 [4];

3)	in the application/vnd.3gpp.mcptt-regroup+xml MIME body is contained in the incoming SIP MESSAGE request:

a)	if a <users-for-regroup> element is included in that MIME body, shall store the value of the <mcptt-regroup-uri> element as the temporary group identity and associate that with the group identity received in the <mcptt-regroup-uri> element, along with the information that the created regroup is a user regroup and should store the contents of the <users-for-regroup> element as the list of users that are part of that user regroup: or

b)	if a <groups-for-regroup> element is included in that MIME body, shall store the value of the <mcptt-regroup-uri> element as the temporary group identity and associate that with the group identity received in the <mcptt-regroup-uri> element, along with the information that the created regroup is a group regroup and should store the contents of the <groups-for-regroup> element as the list of groups that are part of that group regroup:

4)	shall consider that the MCPTT Client is affiliated with the regroup;

5)	should not initiate calls targeting any of the constituent groups but instead target the regroup for the duration of a group regroup; and

6)	if the regroup is a chat group, the MCPTT client should join the regroup when this notification of creation is received.

[TS 24.379, clause 16.3.1.4]

The procedure in clause 16.2.1.4 is used by the MCPTT client when the MCPTT server notifies the MCPTT client of the removal of a user regroup using preconfigured group.

[TS 24.379, clause 16.2.1.4]

Upon receiving a "SIP MESSAGE request to the MCPTT client to request removal of a regroup using preconfigured group", the MCPTT client:

1)	should notify the MCPTT user of the removal of the regroup using preconfigured group;

2)	shall send a 200 (OK) response to the MCPTT server according to 3GPP TS 24.229 [4]; and

3)	shall consider that the MCPTT client is de-affiliated from the regroup.

6.5.4.3	Test description

6.5.4.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2] clause 5.5.10 is inserted.

Preamble:

-	The UE has performed procedure 'MCPTT UE registration' as specified in TS 36.579-1 [2] clause 5.4.2.

-	The UE has performed procedure 'MCX Authorization/Configuration and Key Generation' as specified in TS 36.579-1 [2] clause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.5.4.3.2	Test procedure sequence

Table 6.5.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: The E-UTRA/EPC actions which are related to the MCPTT call establishment are described in TS 36.579-1 [2], clause 5.4.3 'MCX CO communication in E-UTRA'. The test sequence below shows only the MCPTT relevant messages exchanged. | - | - | - | - |
| 1 | Make the UE (MCPTT client) request creation of a new group (group B). (NOTE 1) | - | - | - | - |
| 2 | The UE (MCPTT client) correctly performs procedure 'MCX CO Group Creation' as described in TS 36.579-1 [2] Table 5.3.26.3-1 to create group B. | - | - | - | - |
| 3 | The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 2) | - | - | - | - |
| 4 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to establish an MCPTT user regroup using a preconfigured group? | - | - | 1 | P |
| 5 | Make the UE (MCPTT client) request the establishment of a group call with automatic commencement mode and implicit floor request using the user regroup. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO session establishment/modification without provisional responses other than 100 Trying' as described in TS 36.579-1 [2] Table 5.3A.1.3-1 to establish a group call with automatic commencement mode and implicit floor control according to option b.i of NOTE 1 in TS 36.579.1 [2] Table 5.3A.1.3-1 using the user regroup? | - | - | 2 | P |
| 7 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 3 | P |
| 8 | Make the UE (MCPTT client) release the floor. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT Floor release – Floor Idle' as described in TS 36.579-1 [2] Table 5.3A.15.3-1? | - | - | 3 | P |
| 10 | Make the UE (MCPTT client) release the call. (NOTE 1) | - | - | - | - |
| 11 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO call release' as described in TS 36.579-1 [2] Table 5.3.10.3-1 to release the call? | - | - | 4 | P |
| 12 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CT' as described in TS 36.579-1 [2] Table 5.3.33.3-1 to remove the user regroup? | - | - | 5 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

6.5.4.3.3	Specific message contents

Table 6.5.4.3.3-1: HTTP PUT (step 2, Table 6.5.4.3.2-1;
step 1, TS 36.579-1 [2], Table 5.3.26.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.4.4-1, condition GROUPCREATE

Table 6.5.4.3.3-2: SIP MESSAGE from the SS (Step 4, Table 6.5.4.3.2-1;
Step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_B |  | TS 24.379 [9] clause 16.3.2.4 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.4.3.3-3 |  | TS 24.379 [9] clause 16.3.2.4 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.4.3.3-4 |  | TS 24.379 [9] clause 16.3.2.4 |  |

Table 6.5.4.3.3-3: MCPTT-Info in SIP MESSAGE (Table 6.5.4.3.3-2)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | not present |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |
| mcptt-client-id | Encrypted <mcptt-client-id> with mcpttString set to px_MCPTT_ID_User_B | Encryption according to NOTE 1 in TS 36.579-1 [2] Table 5.5.3.2.2-1 | TS 24.379 [9] clause 16.3.2.4 |  |

Table 6.5.4.3.3-4: MCPTT-Regroup in SIP MESSAGE (Table 6.5.4.3.3-2)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.16-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttregroup |  |  |  |  |
| mcpttregroup-Params |  |  |  |  |
| groups-for-regroup | not present |  |  |  |
| users-for-regroup |  |  |  |  |
| users [1] | px_MCPTT_ID_User_A |  |  |  |
| users [2] | px_MCPTT_ID_User_B |  |  |  |
| users [3] | px_MCPTT_ID_User_D |  |  |  |

Table 6.5.4.3.3-5: SIP INVITE from the UE (step 6, Table 6.5.4.3.2-1;
step 2, TS 36.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP Message as described in Table 6.5.4.3.3-6 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.4.3.3-7 |  |  |  |

Table 6.5.4.3.3-6: SDP in SIP INVITE (Table 6.5.4.3.3-5)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.1-1, condition INITIAL_SDP_OFFER, IMPLICIT_GRANT_REQUESTED

Table 6.5.4.3.3-7: MCPTT-Info in SIP INVITE (Table 6.5.4.3.3-5)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1, condition GROUP-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | Encrypted <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_T_ID | Encryption according to NOTE 1 and NOTE 2 in TS 36.579-1 [2] Table 5.5.3.2.1-1 |  |  |

Table 6.5.4.3.3-8: SIP 200 (OK) from the SS (step 6, Table 6.5.4.3.2-1; 
step 4, TS 36.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.5.4.3.3-9 |  |  |  |

Table 6.5.4.3.3-9: SDP in SIP 200 (OK) (Table 6.5.4.3.3-8)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.1.2-1, condition SDP_ANSWER, IMPLICIT_GRANT_REQUESTED, IMPLICIT_FLOOR_GRANTED

Table 6.5.4.3.3-10: SIP MESSAGE from the SS (Step 12, Table 6.5.4.3.2-1;
Step 2, TS 36.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  |  |  |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_B |  | TS 24.379 [9] clause 16.3.2.5 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.4.3.3-3 |  | TS 24.379 [9] clause 16.3.2.5 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.4.3.3-11 |  | TS 24.379 [9] clause 16.3.2.5 |  |

Table 6.5.4.3.3-11: MCPTT-Regroup in SIP MESSAGE (Table 6.5.4.3.3-10)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.16-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttregroup |  |  |  |  |
| mcpttregroup-Params |  |  |  |  |
| preconfig-group-id | not present |  |  |  |
| groups-for-regroup | not present |  |  |  |
| regroup-action | "remove" |  |  |  |

#### <END OF CHANGES>