3GPP TSG-RAN5 Meeting #101	R5-236624

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0559 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to eCall Setup and MSD Update procedure A.23 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight technologies UK, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure A.23: eCall Setup and MSD Update / 5GS currently refers to “annex A.2.1 of TS 34.229-1 [2] applying condition A7” which indicates “INVITE for creating an emergency session within an emergency registration”. For some test cases in which this procedure is invoked, it is appropriate to use condition A6 “INVITE for creating an emergency session in case of no registration” instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the option to use condition A6 instead of A7, to be specified by the calling test case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail this testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 CR 4089 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s220679 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# A.23	eCall Setup and MSD Update / 5GS

Expected sequence

| Step | Direction |  | Message/Procedure | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | UE sends INVITE with initial MSD |
| 2 |  |  | 200 OK | SS responds INVITE with 200 OK |
| 3 |  |  | ACK | UE responds ACK for 200 OK |
| 4 |  |  | SIP INFO | SS sends SIP INFO to request MSD update |
| 5 |  |  | 200 OK | UE responds SIP INFO with 200 OK |
| 6 |  |  | SIP INFO | UE sends SIP INFO with MSD update |
| 7 |  |  | 200 OK | SS responds SIP INFO with 200 OK. |

Specific message contents:

INVITE (Step 1)

Use the default message "INVITE for MO Call Setup" in Annex A.2.1 of TS 34.229-1 [2] applying condition A6 or A7 and condition A20 or A21 as indicated by the test case and the following inclusion along with the applicable message body of default message:

| Header/param | Value/remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: - - v=0 - o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) - s=(session name) - c=IN (addrtype) (connection-address for UE) [Note 1] Time description: - t= (start-time) (stop-time) Media description: - m=audio (transport port) [Note 2] - c=IN (addrtype) (connection-address for UE) [Note 1] - b=AS: (bandwidth-value) Note 1: At least one "c=" field shall be present. Note 2: EVS codec shall be present in the media attributes, optionally including channel number "/1". |

200 OK for INVITE (Step 2)

Use the default message “200 OK for other requests than REGISTER or SUBSCRIBE” in Annex A.3.1 of TS 34.229-1 [2] with conditions A6, A12, (A13 OR A14) and A22 and following inclusion along with the applicable message body of default message:

| Header/param | Value/remark |
| --- | --- |
| Content-Type media-type | application/sdp |
| Content-Length value | length of message-body |
| Message-body | The following SDP types and values. Session description: v=0 o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) s=- c=IN (addrtype) (connection-address for SS) b=AS:37 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 1] b=AS:37 b=RS:0 b=RR:0 Attributes for media: a=rtpmap: (payload type) EVS/16000/1 [Note 1] a=fmtp: (format) mode-change-capability=2; max-red=220 a=ptime:20 a=maxptime:240 Note 1: The value for fmt, payload type and format is copied from step 1. |

INFO (Step 4)

Use the default message “INFO for MT” in Annex A.2.20 of TS 34.229-1 [2].

INFO (Step 6)

Use the default message “INFO for MO” in Annex A.2.19 of TS 34.229-1 [2].