3GPP TSG-RAN5 Meeting #101	R5-236626

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2964 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for Test Procedure for eCall over IMS establishment in 5GS: eCall Only Support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_EIEI-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current procedure for for eCall over IMS establishment in 5GS for a device in “eCall Only” mode expects that following the triggering of eCall, the device will perform 5GS registration and then PDU session establishment will happen according to “pc_noOf_PDUsSameConnection”, followed by then emergency PDU session establishment for the eCall. In actuality the PDU session establishment for emergency and non-emergency IMS PDU sessions can happen in parallel. This means the UE can send 2 PDU SESSION ESTABLISHMENT REQUEST messages before receiving any PDU SESSION ESTABLISHMENT ACCEPT, which will cause test case failure with the current procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Bring the text of step 19a1 of 4.5.2.2-2 into the test case and add clarification that the excpected number of non-emergency PDU sessions should be 1 but the sequence 4.9.11 (including establishment of 1 emergency PDU session) can execute in parallel with this. Also one editorial correction for SIB1 content. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS eCall over 5GS test cases which use this procedure may fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.29 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR R5s230685 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.9.29	Test Procedure for eCall over IMS establishment in 5GS: eCall Only Support

4.9.29.1	Scope

The purpose of this test procedure is to allow successful completion of eCall over IMS in 5GS when UE is in 5GMM-DEREGISTERED.eCALL-INACTIVE.

The test procedure is applicable for UEs with eCall over IMS support.

Note: The trigger to initiate MO call will be part of test case from where the generic procedure is called.

4.9.29.2	Procedure description

4.9.29.2.1	Initial conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

User Equipment:

-	The Test UICC shall be inserted. It shall provide Emergency Numbers.

The test procedure assumes that the UE is in test state 0-A, subclause 4.4A.2 on the NR Cell. All necessary details required shall be explicitly specified in the TC which calls the procedure in its entirety or refers to parts of it.

4.9.29.2.2	Procedure sequence

The establishment of eCall over IMS in 5GS is assumed to always be mobile originated.

Table 4.9.29.2.2-1: Test procedure sequence for eCall over IMS Emergency establishment in 5GS (eCall Only Support)

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Unless otherwise stated all the messages below are transmitted on the cell specified in the test case. | - | - | - | - |
| 1 | The UE is switched on and SS waits 15 sec to allow the UE to camp on the serving cell and enter and remain in substate 5GMM-DEREGISTERED.eCALL-INACTIVE | - | - | - | - |
| 2 | Make the UE attempt an eCall (Note) | - | - | - | - |
| 3-20 | Steps 2 to 15 of generic procedure 4.5.2.2-2 takes place. | - | - | - | - |
|  | EXCEPTION: Step 20A can occur in parallel with steps 21-26 |  |  |  |  |
| 20A | The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = 1 |  |  |  |  |
| 21-26 | Steps 8 to 13 of test procedure 4.9.11 takes place with condition ‘eCall’. | - | - | - | - |
| Note: The request to originate a manual or Automatic eCall may be performed by MMI or AT command |  |  |  |  |  |

4.9.29.2.3	Specific Message content

All specific message contents shall be according to subclause 4.6 and 4.7 with the exceptions below.

Table 4.9.29.2.3-1: SIB1 (at any time prior and during the procedure, Table 4.9.29.2.2-1)

| Derivation Path: Table 4.6.1-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| ims-EmergencySupport |  | true |  |  |
| } |  |  |  |  |