3GPP TSG WG RAN5 Meeting #101	R5-236634

# Chicago, USA, 13 – 17 Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Clause 6.2B for IoT NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The lower power limit of power calss5 for NS_24 is incorrect in Table 6.2B.3.5-2 ,need to correct it The test environment of A-MPR was incorrect set as 5 conditions, to follow the test environment specified for legacy A-MPR test configuration in 36.521-1, set the test environment as normal condition only |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction the test environment in Table 6.2B.3.4.1-1 Correction the test environment Table 6.2B.3.4.1-2 Correction to PCMAX,c , T(PCMAX_L,c) and lower limit of PC5 in Table 6.2B.3.5-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test requirement is incorrect |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of changes >>

Table 6.2B.3.4.1-1: Test Configuration Table (network signalled value “NS_02N”)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

Table 6.2B.3.4.1-2: Test Configuration Table (network signalled value "NS_24")

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [12] subclause 8.1.1 |  | NC |  |  |
| Test Frequencies as specified in TS 36.508 [12] subclause 8.1.3.1 |  | Frequency ranges defined in Annex K.1.2 |  |  |
| Test Parameters |  |  |  |  |
| Configuration ID | Downlink Configuration | Uplink Configuration |  |  |
|  | N/A for Maximum Power Reduction (MPR) test case | Modulation | Ntones | Sub-carrier spacing (kHz) |
| 1 (Note 1) |  | QPSK | 3@0 | 15 |
| 2 (Note 1) |  | QPSK | 3@3 | 15 |
| 3 (Note 1) |  | QPSK | 3@9 | 15 |
| 4 (Note 1) |  | QPSK | 6@0 | 15 |
| 5 (Note 1) |  | QPSK | 6@6 | 15 |
| 6 (Note 1) |  | QPSK | 12@0 | 15 |
| Note 1: Applicable to UE supporting UL multi-tone transmissions |  |  |  |  |

<<END of changes >>

<<Skip Unchanged >>

<<Start of changes >>

6.2B.3.5	Test requirement

The maximum output power derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2B.3.5-1 to Table 6.2B.3.5-2. The allowed A-MPR values specified in Table 6.2B.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2B.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in TS.36.521-1[14] Table 6.2.5 F.3-1 and 6.2.5F.3-2 apply.

Table 6.2B.3.5-1: UE Power Class test requirements (network signalled value “NS_02N”)

| Test ID | MPR (dB) |  | A_MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 |  | N/A | 22.5 | 2.0 | 25.7 | 19.8 | 19.5 | 2.0 | 22.7 | 16.8 |
| 2 | 0 |  | N/A | 23 | 2.0 | 25.7 | 20.3 | 20 | 2.0 | 22.7 | 17.3 |
| 3 | 0.5 |  | N/A | 22.5 | 2.0 | 25.7 | 19.8 | 19.5 | 2.0 | 22.7 | 16.8 |
| 4 | 1 |  | N/A | 22 | 2.0 | 25.7 | 19.3 | 19 | 2.0 | 22.7 | 16.3 |
| 5 | 1 |  | N/A | 22 | 2.0 | 25.7 | 19.3 | 19 | 2.0 | 22.7 | 16.3 |
| 6 | 2 |  | N/A | 21 | 2.0 | 25.7 | 18.3 | 18 | 2.0 | 22.7 | 15.3 |
|  |  | NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5F |  |  |  |  |  |  |  |  |  |

Table 6.2B.3.5-2: UE Power Class test requirements (network signalled value "NS_24")

| Test ID | MPR (dB) |  | A_MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0.5 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 16.5 | 5.0 | 22.7 | 10.8 |
| 2 | 0 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 16.5 | 5.0 | 22.7 | 10.8 |
| 3 | 0.5 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 16.5 | 5.0 | 22.7 | 10.8 |
| 4 | 1 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 16.5 | 5.0 | 22.7 | 10.8 |
| 5 | 1 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 16.5 | 5.0 | 22.7 | 10.8 |
| 6 | 2 |  | 3.5 | 19.5 | 3.5 | 25.7 | 15.3 | 16.5 | 5.0 | 22.7 | 10.8 |
|  |  | NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [7] clause 6.2.5F |  |  |  |  |  |  |  |  |  |

<<END of changes >>