3GPP TSG-RAN5 Meeting #101	R5-236678

# Chicago, US, Novmember 13th - 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0551 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for RedCap RRM TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add PICS for following UE capabilities which are used in RedCap RRM TCs: - nr-AutonomousGaps-r16 - eutra-AutonomousGaps-r16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - PICS of UE capabilities mention above are added: - nr-AutonomousGaps-r16 - eutra-AutonomousGaps-r16 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

Table A.4.3.6-1: UE Measurement Capabilities

| Item |  | UE Measurement Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support NR measurements and events A triggered reporting |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_eventA_MeasAndReport |  | Yes |  |  |  |  |  |
| 2 |  | Support two independent measurement gap configurations for FR1 and FR2 |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_independentGapConfig |  | No |  |  |  |  |  |
| 3 |  | Support NR intra-frequency and inter-frequency measurements and at least periodical reporting |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_intraAndInterF_MeasAndReport |  | Yes |  |  |  |  |  |
| 4 |  | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured with an associated SS/PBCH |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |  | No |  |  |  |  |  |
| 5 |  | Support inter-RAT E-UTRA measurements and events B triggered reporting |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_eventB_MeasAndReport |  | Yes |  |  |  |  |  |
| 6 |  | Support SS-SINR measurents |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_ss_SINR_Meas |  | No |  |  |  |  |  |
| 7 |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_eutra_CGI_Reporting |  | Yes |  |  |  |  |  |
| 7a |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell using autonomous gap and reporting the acquired information to the network as specified in TS 38.331 [9] when MR-DC is not configured |  | 38.306,4.2.9 |  | Rel-16 |  | pc_eutra_AutonomousGaps_r16 |  | No |  |  |  |  |  |
| 8 |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_nr_CGI_Reporting |  | Yes |  |  |  |  |  |
| 8a |  | Support acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell using autonomous gap and reporting the acquired information to the network as specified in TS 38.331 [9] when MR-DC is not configured. |  | 38.306, 4.2.9 |  | Rel-16 |  | pc_nr_AutonomousGaps_r16 |  | No |  |  |  |  |  |
| 9 |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is configured. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_nr_CGI_Reporting_ENDC |  | Yes |  |  |  |  |  |
| <Unchanged contents are omitted> |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 80 |  | Indicates the support of simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. |  | 38.306, 4.2.7 |  | Rel-17 |  | pc_simultaneous_RxTx_InterBandCA |  | No |  |  |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |  |

<End of modified section 1>