3GPP TSG-RAN5 Meeting #101	R5-236680

# Chicago, US, Novmember 13th - 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2707 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addtion of RedCap RRM TC 16.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To add minimum requirements for NR RedCap CG-SDT test cases. - To add NR Redcap RRM TC - 16.2.1.1 NR SA FR1 CG-SDT for 1Rx UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Minimum requirements for NR RedCap CG-SDT test cases are added. - NR Redcap RRM TC - 16.2.1.1 NR SA FR1 CG-SDT for 1Rx UE is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.2.1.0 (new), 16.2.1.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0347 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

16.2	RRC_INACTIVE state mobility for RedCap

16.2.1	Configured Grant based Small Data Transmissions for RedCap

16.2.1.0	Minimum conformance requirements

16.2.1.0.1	Minimum conformance requirements for TA validation using CG-SDT

The requirements in this clause are applicable when the UE is configured with timing alignment (TA) validation using cg-SDT-RSRP-ChangeThreshold for transmitting in uplink using CG-SDT as specified in TS 38.331 [13].

The requirements in this clause are applicable for the UE performing small data tranmissions using configured resources as TS 38.331 [13].

The UE is allowed to transmit using the configured uplink resources provided that the UE is synchronized towards (i.e. using the timing derived using the latest available NTA value as specified in subclause 7.1.2) the serving cell prior to transmission. If the UE is not able to obtain the synchronization towards the serving cell then the UE shall drop the small data transmission. The UE determines the small data transmission occasion according to the received CG-SDT configuration as specified in TS 38.331 [13].

When cg-SDT-RSRP-ChangeThreshold [13] is configured for TA validation based on the RSRP change criterion according to clause 5.8.2 in [12], the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA value as specified in TS 38.133 [6] subclause 7.1 provided that

-	the first RSRP (RSRP1) measurement and the second RSRP (RSRP2) measurements used in the TA validation are valid measurements and,

-	timing alignment validation for transmission using CG-SDT is valid according to the validation criteria in clause 5.8.2 in [12].

RSRP1 and RSRP2 are considered valid provided that the conditions in Table 16.2.1.0.1-1 are met for FR1.

Table 16.2.1.0.1-1 Valid measurement for FR1

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(640ms, M1*TDRX)) |
| RSRP2 | (T2 – min(640ms, M1*TDRX)) ≤ T2’ ≤ T2 |

If at least one of RSRP1 and RSRP2 is considered to be invalid based on the above conditions, then the UE shall not validate the CG-SDT using RSRP1 and RSRP2 and shall not transmit using CG-SDT. Additionally, for the initial CG-SDT transmission, the UE shall not transmit in an CG-SDT occasion that occurs more than 640 ms after T2.

Where:

-	T1 is the time when

-	RRCRelease with CG-SDT configuration (TS 38.331 [2]) is received, or

-	the latest TA is received if TA is received while in RRC_INACITVE state.

-	T1’ is the time when the UE has completed RSRP1.

-	T2 is the time when the UE performs TA validation as defined in TS 38.321 [12] clause 5.27.2 for transmission using CG-SDT.

-	T2’ is the time when the UE has completed RSRP2.

-	TDRX is the DRX cycle length in ms.

-	M1 the scaling factor as defined in TS 38.133 [6] clause 4.2.2.2.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement

-	The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration.

For UE which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to SS-RSRP/RSRQ measurement

-	If deriveSSB_IndexFromCell is enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration.

-	If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH on all symbols within SMTC window duration.

For UEs which do not support simultaneousRxDataSSB-DiffNumerology [11] the following restrictions apply due to measurement of serving cell.

-	The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/ on SSB symbols of the serving cell.

The UE is not required to meet the following measurement requirements during subsequent SDT transmissions:

-	Measurements of inter-frequency NR cells in TS 38.133 [6] clause 5.1.2.4

-	Measurements of inter-RAT E-UTRAN cells in TS 38.133 [6] clause 5.1.2.5

-	Idle Mode CA/DC Measurements in TS 38.133 [6] clause 5.4

The UE is allowed to delay the reception of PRS resources on the positioning frequency layer until the SDT session is completed if the measurement using PRS resource overlaps with the SDT resources.

The normative reference for this requirement is TS 38.133 [6] clauses 5.2B.

16.2.1.1	NR SA FR1 CG-SDT for 1Rx UE

Editor’s Note: This test case is incomplete in following aspects:

-	Test parameters are still in brackets;

-	Test procedure needs further updates;

-	TT analysis is still missing.

16.2.1.1.1	Test purpose

To verify that the UE properly perform TA validation for CG-SDT transmission as specified in TS 38.133 [6] clause 5.2B.2.1.

16.2.1.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards and supporting CG-SDT.

16.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.2.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.2.1.1.

16.2.1.1.4	Test description

Same as the test description given in clause 6.2.1.4.

16.2.1.1.4.1	Initial conditions

Same as the initial conditions given in clause 6.2.1.4.1 with following exceptions:

-	Table 6.2.1.4.1-1 is replaced by Table 16.2.1.1.4.1-1;

-	Table 6.2.1.4.1-2 is replaced by Table 16.2.1.1.4.1-2;

-	Table 6.2.1.4.1-3 is replaced by Table 16.2.1.1.4.1-3;

Table 16.2.1.1.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 16.2.1.1-1 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR FDD |
| 16.2.1.1-2 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR TDD |
| 16.2.1.1-3 | SSB SCS 30 kHz, data SCS 30 kHz, BW 20 MHz, NR TDD |
| 16.2.1.1-4 | SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz, NR HD-FDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 16.2.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.2.1.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 16.2.1.1.4.1-3: General test parameters

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Duplex mode | Config 1 |  | FDD |  |
|  | Config 2,3 |  | TDD |  |
|  | Config 4 |  | HD-FDD |  |
| TDD Configuration | Config 1,4 |  | N/A |  |
|  | Config 2 |  | TDDConf.1.1 |  |
|  | Config 3 |  | TDDConf.2.1 |  |
| BWchannel | Config 1,2,4 | MHz | 10: NRB,c = 52 |  |
|  | Config 3 |  | 20: NRB,c = 51 |  |
| PDSCH Reference measurement channel | Config 1,4 |  | SR.1.1 FDD |  |
|  | Config 2 |  | SR.1.1 TDD |  |
|  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel | Config 1,4 |  | CR.1.1 FDD |  |
|  | Config 2 |  | CR.1.1 TDD |  |
|  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel | Config 1,4 |  | CCR.1.3 FDD |  |
|  | Config 2 |  | CCR.1.3 TDD |  |
|  | Config 3 |  | CCR.2.2 TDD |  |
| SSB configuration | Config 1,2,4 |  | SSB.1 FR1 |  |
|  | Config 3 |  | SSB. 1 RedCap FR1 |  |
| OCNG Patterns |  |  | OP.1 |  |
| Initial BWP Configuration |  |  | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  |  | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  |  | SMTC.1 |  |
| DRX configuration |  |  | DRX.7 |  |
| T_delay_modeB |  | s | 4 |  |
| T1 |  | s | 0.4 |  |
| T2 |  | s | 1.28 | 2 x W1 |
| T3 |  | s | 2.72 | T_timer_modeB - W1 -W2 |
| T4 |  | s | 2.24 | w1+w2+w3+100 |
| T5 |  | s | 4.22 | T3+W2+W3 |
| cg-SDT-RSRP-ChangeThreshold |  | dB | 8 |  |
| cg-SDT-RSRP-ThresholdSSB |  | dBm | -110 |  |
| cg-SDT-TimeAlignmentTime |  |  | Infinity |  |
| CG-SDT resource period |  | ms | 320 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  | dB | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |
| Propagation condition |  |  | AWGN |  |

16.2.1.1.4.2	Test procedure

Same as the test procedure given in clause 6.2.1.4.2.

16.2.1.1.4.3	Message contents

Same as the message contents given in 6.2.1.4.3 with following exceptions:

-	Table 6.2.1.4.3-2 is replaced by Table 16.2.1.1.4.3-1.

Table 16.2.1.1.4.3-1: SDT-MAC-PHY-CG-Config-r17

| Derivation Path: TS 38.508-1 [6], Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SDT-MAC-PHY-CG-Config-r17::= SEQUENCE { |  |  |  |
| cg-SDT-ConfigLCH-RestrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF CG-SDT-Config-LCH-restriction-r17 { | 1 entry |  |  |
| logicalChannelIdentity-r17 | LogicalChannelIdentity with condition DRB1 |  |  |
| configuredGrantType1Allowed-r17 | true |  |  |
| allowedCG-List-r17 | Not present |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-NUL-r17 CHOICE { |  |  |  |
| setup | BWP-UplinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-ConfigInitialBWP-SUL-r17 | Not present |  |  |
| cg-SDT-ConfigInitialBWP-DL-r17 |  |  |  |
| setup | BWP-DownlinkDedicatedSDT-r17 |  |  |
| } |  |  |  |
| cg-SDT-TimeAlignmentTimer-r17 | infinity |  |  |
| cg-SDT-RSRP-ThresholdSSB-r17 | 46 | -110dBm |  |
| cg-SDT-TA-ValiditationConfig-r17 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| cg-SDT-RSRP-ChangeThreshold-r17 | dB8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

16.2.1.1.5	Test requirements

Same as the test requirements given in clause 6.2.1.5 with following exceptions:

-	Table 6.2.1.5-1 is replaced by Table 16.2.1.1.5-1.

Table 16.2.1.1.5-1: Cell-specific test parameters (NR Cell 1)

| Parameter | Config | Unit | SSB#0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Note1 |  | dBm/15kHz | -98 |  |  |  |  |
| Note1 | 1,2,4 | dBm/SSB SCS | -98 |  |  |  |  |
|  | 3 |  | -95 |  |  |  |  |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
| SS RSRP Note2 | 1,2,4 | dBm/SSB SCS | -98 | -98 | -98 | -98 | -98 |
|  | 3 |  | -95 | -95 | -95 | -95 | -95 |
| Io Note2 | 1,2,4 | dBm/ 9.36 MHz | -67.04 | -67.04 | -67.04 | -67.04 | -67.04 |
|  | 3 | dBm/ 18.36 MHz | -67.12 | -67.12 | -67.12 | -67.12 | -67.12 |
|  |  | dB | 0 | 0 | 0 | 0 | 0 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

<End of modified section 2>