3GPP TSG-RAN5 Meeting #101	R5-236724

# Chicago, US, Novmember 13th - 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2751 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RedCap RRM TC 16.3.2.2.1 with TT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - To update RedCap RRM TC 16.3.2.2.1 according to TT analysis. - To correct wrong reference in Initial Condition - Unlike Rel-15 legacy RACH TCs, the test configuration of TDD+15 kHz is also included in RedCap RACH TCs. Although the prachConfigurationIndex = 102 used in the RACH test case is the same, Same prachConfigurationIndex still may corresponds to different PRACH formats for FDD and TDD respectively. To be specific, PRACH format A1 and A2 shall be used for FDD and TDD respectively when prachConfigurationIndex = 102 according to 38.211 Table 6.3.3.2-2/3 In addition, format A1/A2 correspond to different DELTA_PREAMBLE defined in 38.321. So we have: - For Config 1(FDD +15kHz): DELTA_PREAMBLE = 8+3*0 = 8dB; - For Config 2(TDD+15kHz): DELTA_PREAMBLE = 5+3*0 = 5dB; - For Config 3(TDD+30kHz): DELTA_PREAMBLE = 5+3*1 = 8dB; - For Config 4(HD-FDD +15kHz): DELTA_PREAMBLE = 8+3*0 = 8dB; And the test requirement for first PRACH preamble transmit power shall be: - For Config 1/3/4: first PRACH power = TargetPower (-120) - Pathloss (-90dB) + DELTA_PREAMBLE (8dB) = -22dBm; - For Config 2: first PRACH power = TargetPower (-120) - Pathloss (-90dB) + DELTA_PREAMBLE (5dB) = -25dBm, rather than -22dBm We suggest update test requirements accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - RedCap RRM TC 16.3.2.2.1 is updated based on TT analysis - Refererence is corrected. - Test requirements is updated. - Brackets are removed from test parameters. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RRM TT, RAN4 dependency |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

##### 16.3.2.2.1	NR SA FR1 4-step RA type contention based random access test in FR1 for NR standalone for 1 Rx UE

16.3.2.2.1.1	Test purpose

To verify that the behaviour of the random access procedure is according to the requirements and that the PRACH power settings and timing are within specified limits. This test will verify the requirements in 38.133 [6] clause 6.2.2B.2 and Clause 7.1A.2 in an AWGN model.

16.3.2.2.1.2	Test applicability

This test applies to all types of NR RedCap UE with 1Rx from Release 17 onwards.

16.3.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.3.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.3.2.2.1.

16.3.2.2.1.4	Test description

16.3.2.2.1.4.1	Initial conditions

Same as the initial conditions given in sub-clause 6.3.2.2.1.4.1 with following exceptions:

-	Table 6.3.2.2.1.4.1-1 is replaced by Table 16.3.2.2.1.4.1-1.

-	Table 6.3.2.2.1.4.1-2 is replaced by Table 16.3.2.2.1.4.1-2.

Table 16.3.2.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.3.2.2.1-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.3.2.2.1-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.1-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.3.2.2.1-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table 16.3.2.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.3.2.2.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

16.3.2.2.1.4.2	Test procedure

Same as the test procedure given in sub-clause 6.3.2.2.1.4.2 with following exceptions:

-	Step 4 (Test 1) is replaced by following steps:

4.	Test 1: Correct behaviour when transmitting Random Access Preamble

4.1. The UE shall send a preamble to the System Simulator. The System Simulator shall check that the Random Access Preamble belongs to one of the Random Access Preambles associated with the SSB with index 0, which has SS-RSRP above the configured rsrp-ThresholdSSB+1dB.

16.3.2.2.1.4.3	Message contents

Same as the message contents given in 6.3.2.2.1.4.3.

16.3.2.2.1.5	Test requirement

Same as the test requirements given in sub-clause 6.3.2.2.1.5 with following exceptions:

-	Table 6.3.2.2.1.5-1 is replaced by Table 16.3.2.2.1.5-1.

-	The check on the power of the first preamble in Test 2, Test 3, Test 5 and Test 7 is replaced as follows:

-	The power of the first preamble shall be X dBm within the accuracy specified in Table 16.3.2.2.1.5-2, where

-	X = -23.55 for test configuration 16.3.2.2.1-1, 16.3.2.2.1-3 and 16.3.2.2.1-4,

-	X = -26.55 for test configuration 16.3.2.2.1-2.

Table 16.3.2.2.1.5-1: General test parameters

| Parameter |  |  | Unit | Test-1 | Comments |
| --- | --- | --- | --- | --- | --- |
| SSB Configuration |  | Config 1,2,4 |  | SSB.1 FR1 | As defined in A.3 and A.3A, except for number of SSBs per SS-burst and SS/PBCH block index as below |
|  |  | Config 3 |  | SSB.1 RedCap FR1 |  |
| Number of SSBs per SS-burst |  |  |  | 2 | Different from the definition in A.3 and A.3A |
| SS/PBCH block index |  |  |  | 0,1 | Different from the definition in A.3 and A.3A |
| Duplex Mode for Cell 1 |  | Config 1 |  | FDD |  |
|  |  | Config 2,3 |  | TDD |  |
|  |  | Config 4 |  | HD-FDD |  |
| TDD Configuration |  | Config 2 |  | TDDConf.1.1 |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |
| CSI-RS for tracking |  | Config 1,4 |  | TRS.1.1 FDD |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |
| OCNG Pattern Note 1 |  |  |  | OP.1 | As defined in A.2.1 |
| PDSCH parameters Note 4 |  | Config 1,4 |  | SR.1.1 FDD | As defined in A.1.1 |
|  |  | Config 2 |  | SR.1.1 TDD |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | CR.1.1 FDD |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | CCR.1.1 FDD |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |
| NR RF Channel Number |  |  |  | 1 |  |
| EPRE ratio of PSS to SSS |  |  |  | 0 |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  | dB |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |
| SSB with index 0 |  |  | dB | 4.55 | Power of SSB with index 0 is set to be above configured rsrp-ThresholdSSB+1dB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | 4.55 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -93.45 |  |
| SSB with index 1 |  |  | dB | -17 | Power of SSB with index 1 is set to be below configured rsrp-ThresholdSSB+1dB |
|  |  | Config 1,2,4 | dBm/15kHz | -98 |  |
|  |  | Config 3 |  | -101 |  |
|  |  |  | dB | -17 |  |
|  | SS-RSRP Note 3 |  | dBm/ SCS | -115 |  |
| Io Note 2 |  | Config 1,2,4 | dBm | -64.19/9.36MHz | For symbols without SSB index 1 |
|  |  | Config 3 |  | -64.27/18.36 MHz |  |
| ss-PBCH-BlockPower |  |  | dBm/ SCS | -5 | As defined in clause 6.3.2 in TS 38.331 [13]. |
| Configured UE transmitted power () |  |  | dBm | 23 | As defined in clause 6.2.4 in TS 38.101-1 [2]. |
| PRACH Configuration |  |  |  | PRACH.1 FR1 | As defined in A.7.1 |
| Propagation Condition |  |  | - | AWGN |  |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. The OCNG pattern is chosen during the test according to the presence of a DL reference measurement channel. Note 2: SS-RSRP, Es/Iot and Io levels have been derived from other parameters for information purpose. They are not settable parameters. Note 3: The DL PDSCH reference measurement channel is used in the test only when a downlink transmission dedicated to the UE under test is required. |  |  |  |  |  |

Table 16.3.2.2.1.5-2 Absolute power tolerance Test requirements

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 20.1 dB |

<End of modified section 1>