# Title:	Test Tolerances for NR SA FR1 RRC connection release with redirection

Source:	Huawei, HiSilicon

# 1.	Introduction

This document relates to the NR SA FR1 RRC connection release with redirection test case in TS 38.533.

The test case in question are:

- 6.3.2.3.1, NR SA FR1 RRC connection release with redirection

- 16.3.2.3.1, NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 1 Rx UE

- 16.3.2.3.2, NR SA FR1-FR1 Redirection from NR in FR1 to NR in FR1 for 2 Rx UE

- 18.2.2.1, Redirection from E-UTRA to NR SA FR1 for redcap UE

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v16.1.0 Annex A.

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1

A.6.3.2.3.1.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to NR requirements specified in clause 6.2.3.2.1.

A.6.3.2.3.1.2	Test Parameters

Supported test configurations are shown in table A.6.3.2.3.1.2-1. The time delay is tested by using the parameters in table A.6.3.2.3.1.2-2, and A.6.3.2.3.1.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The “RRCConnectionRelease” message containing the relevant system information of Cell 2 shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.6.3.2.3.1.2-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.2.3.1.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 1 |  |

Table A.6.3.2.3.1.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 2 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC configuration |  | Config 1,2 |  | SMTC.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SMTC.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
|  |  |  | dB | 4 | 4 |  | -infinity |  | 4 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -64.59 | -64.59 |  | -70.05 |  | -64.59 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -58.49 | -58.49 |  | -63.94 |  | -58.49 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

A.6.3.2.3.1.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 960 ms from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90%.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = 110 ms and is specified in clause 12 in TS 38.331 [2].

Tidentify-NR = 680 ms in the test.

TSI-NR = 0 ms is assumed, since the UE is provided with the SI (including MIB and all relevant SIBs) of the target NR cell before the RRC connection is released by the old NR cell.

TRACH = 170 ms in the test.

This gives a total of 960 ms.

# 3. Discussion

The test case has 2 time periods: T1 and T2. To analyse the test case it is helpful to show the value of SS-RSRP for each cell versus time, as shown below:

During T1 NR Cell 1 is present and the nominal Ês/Iot value will be 4 dB, easily meeting the core requirement of Ês/Iot ≥ -6 dB.

NR Cell 2 does not exist during T1, and only appears during T2.

During T1 and T2 for NR Cell 1, the SS-RSRP stays the same at -94 dBm/15kHz.

During T2 for NR Cell 2, Ês/Iot is 4 dB which easily meets the core requirement of Ês/Iot ≥ -4 dB and the SS-RSRP stays at -94 dBm/15kHz.

The design of the test case requires that cell 1 to be detectable, and the cell 2 is detectable during T2.

The effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

# 4. Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.6.3.2.3.1.2-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

The key parameters appear in section a) of the accompanying spreadsheet. The layout for NR Cell1 and NR Cell 2 is similar to Table A.6.3.2.3.1.2-3 in TS 38.133, but the AWGN is given a separate set of column for each frequency. This allows the spreadsheet calculations to be done in a consistent way.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

In general all the values are listed for each time interval T1 and T2 but as cell 2 is not present during T1 those entries are greyed out.

c) Uncertainties

The SS provides 2 cells on different frequencies, each with AWGN. We propose to control the following parameters:

- AWGN absolute power on cell 1 frequency, Noc1 ±1.5 dB

- Ratio of NR Cell 1 signal / noise, Ês1 / Noc1 ±0.3 dB

- AWGN absolute power on cell 2 frequency, Noc2 ±1.5 dB

- Ratio of NR Cell 2 signal / noise, Ês2 / Noc2 ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary. For these test cases the signals are not faded, and the values for Ês / Noc are chosen to be similar to other inter-frequency test cases. The absolute level uncertainties of Noc are chosen to ensure the analysis and Test Requirements are valid for frequency bands up to 7.125 GHz.

During T1, the UE is in RRC_CONNECTED state and NR Cell 1 is present, and must be detectable. The relevant conditions are defined in the following extract from TS 38.133:

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements

10.1.2.1.1	Absolute SS-RSRP Accuracy

Unless otherwise specified, the requirements for absolute accuracy of SS-RSRP in this clause apply to a cell on the same frequency as that of the serving cell in FR1.

The accuracy requirements in Table 10.1.2.1.1-1 are valid under the following conditions:

-	Conditions defined in 38.101-1 [18] Clause 7.3 for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.1-1: SS-RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| 4.5 | 9 | -6 dB | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
|  |  |  | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 8 | 11 | -6 dB | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in Section 3.5.2. |  |  |  |  |  |  |  |

To ensure this absolute SS-RSRP accuracy, the conditions of Io, SSB Es/Iot in Table 10.1.2.1.1-1, and conditions of SSB_RP and SSB Es/Iot in section Annex B.2.2 need to be fulfilled.

B.2.2	Conditions for NR intra-frequency measurements

This section defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

To fulfil these conditions, NR Cell 1 Es/Iot shall be above -6dB, the tightest requirement for Io shall be between -117.5dBm/15kHz (-89.5dBm/9.36MHz) or -114.5dBm/30kHz (-83.5dBm/38.16MHz) and -50dBm/BWchannel, and the tightest requirement for SS-RSRP power shall be above -123.5dBm/15 kHz or -120.5dBm/30 kHz for Band NR_FDD_FR1_H.

During T2, the NR Cell 2 shall be considered detectable when for each relevant SSB, the side conditions should be met that,

- SSB_RP and SSB Ês/Iot according to Annex B.2.5 for a corresponding NR Band.

The relevant conditions are defined in the following extract from TS 38.133:

B.2.5	Conditions for RRC connection release with redirection to NR

This section defines the following conditions for RRC connection release with redirection to NR: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.5-1 for FR1 NR cells.

The conditions are defined in Table B.2.5-2 for FR2 NR cells.

Table B.2.5-1: Conditions for for RRC connection release with redirection to NR in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_FR1_C | -124 | -121 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -123 | -120 |  |
|  | NR_FDD_FR1_G | -122 | -119 |  |
|  | NR_FDD_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in Section 3.5.2. |  |  |  |  |

The SSB Es/Iot of NR Cell 2 shall be above -4dB, and the tightest requirement for SSB_RP shall be above -121.5dBm/15kHz or -118.5dBm/30kHz for band group NR_FDD_FR1_H.

d) Controlled parameters critical to verdict

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Source and Reference” column of section g) in the spreadsheet.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. The sensitivity factors are the ratio (effect on a critical parameter y / a test equipment uncertainty x), and are usually in dB/dB. In this RRM test case there is a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict. The sensitivity factors are therefore derived by inspection as one or zero.

For example, an error of 1dB in the absolute AWGN level Noc1 would cause 1dB error in the Cell 1 SS-RSRP, so the sensitivity factor is 1.000 during T1 and T2. However the same error of 1dB in the absolute AWGN level Noc1 would cause no change to Cell 1 Es/Iot, because all other powers are specified relative to Noc1, so the sensitivity factor is zero.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, taking into account the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

The normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

During T1 and T2 for NR Cell 1, the uncertainties do not take Io, Ês/Iot or SSB_RP power outside the allowed range, therefore no offsets are required.

During T2 for NR Cell 2, the uncertainties do not take Ês/Iot or SSB_RP power outside the allowed range, therefore no offsets are required.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

As no offsets have been applied, all values are the same as in section b).

In general all the values are listed for each time interval T1 and T2 but as cell 2 is not present during T1 those entries are greyed out.

g) Check controlled parameters Min/Max

It is not necessary to calculate all parameters during each time interval T1 and T2, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

It can be seen that with the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.3.2.3.1 also contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- NR Cell 1 and NR Cell 2 SSB_RP minimum requirement, section g) and associated Source/Reference

- Io minimum requirement, section g) and associated Source/Reference

# 6. Treatment of other test cases

TS 38.533 also contains similar intra-NR redirection RedCap test cases 16.3.2.3.2 for 2 Rx RedCap UEs in clause 16.3.2. Same level of uncertainties applies to these test cases since Rel-15 legacy UE and Rel-17 RedCap UE with 2 Rx share the same requirements according to 38.133. For these 2 Rx RedCap test cases:

- Test parameters for config 1/2/4 are identical with that of 6.3.2.3.1 config 1/2 with the exception that HD-FDD duplex mode is used in config 4. Then the same test tolerances for 6.3.2.3.1 config 1/2 apply considering duplex mode bringing no any additional measurement uncertainties.

- Test parameters for config 3 are identical with that of 6.3.2.3.1 config 3 with the exception that the BW is only 20MHz (51 RBs). As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed comparing with 6.3.2.3.1 config 3:

  - Channel bandwidth and RBs in Channel bandwidth, section a) and associated comment

From the analysis one can observe that the only difference is that Io value is 10*(log106/51) = 3.18dB lower. Then the same test tolerances for 6.3.2.3.1 config 3 apply considering that minimum Io requirement is satisfied with a margin much larger than 3.18dB.

TS 38.533 also contains similar E-UTRA to NR RedCap test cases 18.2.2.1 for both 1Rx and 2 Rx RedCap UEs in clause 18.2.2. Same level of uncertainties applies to these test cases since Rel-15 legacy UE and Rel-17 RedCap UE with 2 Rx share the same requirements according to 38.133.

For 2 Rx RedCap:

- Test parameters related to NR Cell 2 for config 1/2/4/5/7/8 are identical with that of 16.3.2.3.2 config 1/2/4. Then the same test tolerances for 6.3.2.3.1 config 1/2/4 apply.

- Test parameters related to NR Cell 2 for config 3/6 are identical with that of 16.3.2.3.2 config 3. Then the same test tolerances for 6.3.2.3.1 config 3 apply.

TS 38.533 also contains similar test case 16.3.2.3.1 for 1 Rx RedCap UEs in clause 16.3.2. Although the inter-frequency measurement accuracy requirements for 1Rx RedCap, specified in 38.133 clause 10.1A.4.1.1, are different with Rel-15 legacy UE, the side conditions to apply accuracy requirements are the same.

### 9.3B.2	Requirements applicability

The requirements in clause 9.3B apply, provided:

-	The cell being identified or measured is detectable.

An inter-frequency cell shall be considered detectable when for each relevant SSB:

-	For 2 Rx RedCap UE:

-	SS-RSRP related side conditions given in clauses 10.1.4 and 10.1.5 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.9 and 10.1.10 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.14 and 10.1.15 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.3 for a corresponding Band.

-	For 1 Rx RedCap UE:

-	SS-RSRP related side conditions given in clauses 10.1A.4 for FR1 for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1A.4 for FR1 for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1A.4 for FR1, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.16 for a corresponding Band.

#### 10.1A.4.1	Inter-frequency SS-RSRP accuracy requirements

##### 10.1A.4.1.1	Absolute Accuracy of SS-RSRP in FR1

The accuracy requirements in clause 10.1.4.1.1 shall apply when RedCap UE is capable of 2Rx. When UE is only required to support 1RX, the absolute accuracy requirements in Table 10.1A.4.1.1-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for inter-frequency measurements are fulfilled according to Annex B.2.3 for a corresponding Band for each relevant SSB.

Table 10.1A.4.1.1-1: SS-RSRP Inter frequency Absolute accuracy for 1Rx RedCap UE in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 3 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -70 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -70 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -70 |
| 5.5 | 10 | -6 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -70 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -70 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -70 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -70 |
| 9 | 12 | -6 | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A, NR_FDD_FR1_B, NR_TDD_FR1_C, NR_FDD_FR1_D, NR_TDD_FR1_D, NR_FDD_FR1_E, NR_TDD_FR1_E, NR_FDD_FR1_F, NR_FDD_FR1_G, NR_FDD_FR1_H | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: Void NOTE 3: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

## B.2.16	Conditions for NR inter-frequency measurements

This clause defines the following conditions for RedCap NR inter-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.16-1 and Table B.2.16-2 for 1 Rx and 2 Rx RedCap for FR1 NR cells.

The conditions are defined in Table B.2.16-3 for FR2 NR cells.

Table B.2.16-1: Conditions for inter-frequency measurements in FR1 for 1 Rx RedCap

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_RC_FR1_A, NR_TDD_RC_FR1_A | -125 | -122 | -4 |
|  | NR_FDD_RC_FR1_B | -124.5 | -121.5 |  |
|  | NR_TDD_RC_FR1_C | -124 | -121 |  |
|  | NR_FDD_RC_FR1_D, NR_TDD_RC_FR1_D | -124.5 | -120.5 |  |
|  | NR_FDD_RC_FR1_E, NR_TDD_RC_FR1_E | -123 | -120 |  |
|  | NR_FDD_RC_FR1_F | -122.5 | -119.5 |  |
|  | NR_FDD_RC_FR1_G | -122 | -119 |  |
|  | NR_FDD_RC_FR1_H | -121.5 | -118.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

However, in RRC Re-direction TCs only side conditions are checked. So, the same level of uncertainties also applies to these test cases.