3GPP TSG-RAN5 Meeting #101	R5-236754

# Chicago, US, Novmember 13th - 17th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2966 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PCI of simulated cells with mod-3 problem |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In RAN5#99 we had discussed the mod-3 problem in TC 8.1.3.1.18.2 (R5-232510). After further check we realized that there are also other simulated Cells having mod-3 problem. To eliminate potential problems in SIG testing, as well as similar problems in future, we suggest change PCI of simulated cell 11 and 31 as follows: - PCI of Cell 11: 11->9, - PCI of Cell 31: 31->35. For better understanding, here is a brief introduction of mod-3 problem: Simulated NR Cells and their default parameters are defined in 38.508-1 Table 4.4.2-1 and Table 4.4.2-2 respectively. PCIs of these cells and the SSB indices they transmitted can be summarized as follows (Cell 33 is omitted since it’s SUL only). Test freq.; Cells transmit SSB#0; Cells transmit SSB#1. NRf1; Cell 2 (PCI 2), Cell 11 (PCI 11), Cell 489 (PCI 489); Cell 1 (PCI 0), Cell 4 (PCI 4). NRf2; Cell 12 (PCI 12); Cell 3 (PCI 3), Cell 23 (PCI 23). NRf3; Cell 13 (PCI 13); Cell 6 (PCI 6). NRf4; N/A; Cell 14 (PCI 14). NRf5; Cell 30 (PCI 30); Cell 10 (PCI 10), Cell 31 (PCI 31). NRf6; Cell 29 (PCI 29); Cell 28 (PCI 28). NRf8; N/A; Cell 32 (PCI 32). NRf10; N/A; Cell 500 (PCI 500). Among these cells, the cells highlighted in yellow transmit SSBs at the same time-frequency domain location, more seriously, the PCIs of these cells have the same remainder after mod 3. This means that these cells will send exactly the same PSS sequences on the same time-frequency locations according to 38.211 cl.7.4.2. #### 7.4.2.1 Physical-layer cell identities There are 1008 unique physical-layer cell identities given by where NID(1)∈0,1,…,335 and NID(2)∈0,1,2. ##### 7.4.2.2.1 Sequence generation The sequence for the primary synchronization signal is defined by. In NR’s design, PSS is mainly used for cell identification. UE is assumed to perform matched filtering on the received signal using PSS sequence. It considers that there exists a PSS if the filter output is higher than certain threshold, and then, further detects corresponding SSS to perform DL synchronization, SSB index reading, RSRP/RSRQ measurement, etc.. So, if two cells send same PSS in the same time-frequency location. The UE may mistakenly consider that there is only one cell, resulting in a series of negative consequences, such as poorer measurement accuracy, poorer DL sync accuracy, faling to identify a cell, etc.. In practical networks, operators usually ensure that adjacent intra-frequency cells have different NID(2) to avoid this situation. Therefore, any two cells with the same NID(2) are separated by at least one cell. Although it’s impossible to avoid mod-3 probelm, UE still can identify the existence of different cells since these cells have receiving timing difference (RTD) as depicted in figure below. However, such an RTD does not exist in testing. According to 38.523-3 Table 7.1.5.2-1, the radio frame boundaries of all intra-frequency cells are completely aligned. The UE may fail to identify the existence of the two cells depicted in figure below. |
| Summary of change: |  | - PCI of simulated cell 11 and 31 are changed as follows: - PCI of Cell 11: 11->9, - PCI of Cell 31: 31->35. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Workplan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.4.2	Simulated cells

NOTE 1: For NAS test cases see clause 6.3.2.

NOTE 2: Test frequency and range defined in table 4.4.2-1 do not apply to TS 38.521-1, TS 38.521-2 and TS 38.521-3 test cases.

Test frequencies and simulated NR cells are defined in table 4.4.2-1. Test frequencies and simulated E-UTRA and UTRA cells are defined in TS 36.508 [2] table 4.4.2-1.

For NR cells, NRf1 is the default test frequency. For UTRA cells, f8 as specified in TS 36.508 [2] table 4.2.2-1 is the default test frequency. For E-UTRA cells, f1 as specified in TS 36.508 [2] table 4.2.2-1 is the default test frequency.

Default parameters for simulated NR cells are specified in table 4.4.2-2 and table 4.4.2-3.

Default parameters for simulated E-UTRA cells are specified in TS 36.508 [2] table 4.4.2-1A and table 4.4.2-2.

Default parameters for simulated UTRA cells are specified in TS 36.508 [2] clause 4.4.4.

Common parameters for NR simulated cells are specified in clauses 4.4.3.

Other cell specific parameters are specified in TS 36.508 [2] clause 4.4.3 to 4.4.7.

Table 4.4.2-1: Definition of test frequencies and simulated NR cells

| Test frequency | RAT | Operating band | Range | Simulated NR cells |
| --- | --- | --- | --- | --- |
| NRf1 | NR | Operating band under test | Mid (Note 1, Note 3 | NR Cell 1, NR Cell 2, NR Cell 4, NR Cell 11, NR Cell 489 |
| NRf2 | NR | Operating band under test | High (Note 1, Note 3) | NR Cell 3, NR Cell 12, NR Cell 23 |
| NRf3 | NR | Operating band under test | Low (Note 1, Note 3) | NR Cell 6, NR Cell 13 |
| NRf4 | NR | Operating band under test | (Note 1) | NR Cell 14 |
| NRf5 | NR | Operating band for inter-band cells | Mid (Note 1) | NR Cell 10, NR Cell 30, NR Cell 31 |
| NRf6 | NR | Operating band for inter-band cells | High (Note 1) | NR Cell 28, NR Cell 29 |
| NRf7 | NR | Operating band for inter-band cells | Low (Note 1) |  |
| NRf8 | NR | Operating band for SDL cell | Mid (Note 1) | NR Cell 32 |
| NRf9 | NR | Operating band for SUL cell | Mid (Note 1) | NR Cell 33 |
| NRf10 | NR | Operating band for inter-band cells | Mid (Note bb) | NR Cell 500 |
| Note 1: For signalling test, see clause 6.2.3. Note 2: Void. Note 3: For RRM test with NR intra-band non-contiguous CA, the test frequencies for the set of non-contiguous component carriers are specified in clauses 4.3.1.1.4 for FR1 and in clause 4.3.1.2.4 for FR2 without any regard to range. Thus “Low”, “Mid” and “High” information in this table does not apply. Unless otherwise stated, test point with maximum Wgap is chosen. Note 4: For signalling test with NR inter-band CA three bands only, the test band is different from the Operating band for NRf1 and the Operating band for NRf5. |  |  |  |  |

Table 4.4.2-2: Default NR parameters for simulated NR cells

| cell ID |  | NR Cell Identifier |  |  | Physical layer cell identity |  | PRACH-rootSequenceIndex FDD |  | PRACH-rootSequenceIndex TDD | SSB-Index2,3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | gNB Identifier |  | Cell Identity |  |  | LRA=139 Note 1 |  | LRA=139 Note 1 |  |  |
| NR Cell 1 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '00 0000 0000'B | 0 |  | 0 |  | 0 |  | 1 |
| NR Cell 2 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '00 0000 0010'B | 2 |  | 32 |  | 32 |  | 0 |
| NR Cell 3 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0000 0011'B | 3 |  | 0 |  | 0 |  | 1 |
| NR Cell 4 |  | '00 0000 0000 0000 0000 0000 0011'B |  | '00 0000 0100'B | 4 |  | 64 |  | 64 |  | 1 |
| NR Cell 6 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0000 0110'B | 6 |  | 0 |  | 0 |  | 1 |
| NR Cell 10 |  | '00 0000 0000 0000 0000 0000 0101'B |  | '00 0000 1010'B | 10 |  | 0 |  | 0 |  | 1 |
| NR Cell 11 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0000 1001'B | 9 |  | 96 |  | 96 |  | 0 |
| NR Cell 12 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0000 1100'B | 12 |  | 32 |  | 32 |  | 0 |
| NR Cell 13 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0000 1101'B | 13 |  | 32 |  | 32 |  | 0 |
| NR Cell 14 |  | '00 0000 0000 0000 0000 0000 0111'B |  | '00 0000 1110'B | 14 |  | 0 |  | 0 |  | 1 |
| NR Cell 23 |  | '00 0000 0000 0000 0000 0000 0110'B |  | '00 0001 0111'B | 23 |  | 64 |  | 64 |  | 1 |
| NR Cell 28 |  | '00 0000 0000 0000 0000 0000 0010'B |  | '00 0001 1100'B | 28 |  | 0 |  | 0 |  | 1 |
| NR Cell 29 |  | '00 0000 0000 0000 0000 0000 0100'B |  | '00 0001 1101'B | 29 |  | 32 |  | 32 |  | 0 |
| NR Cell 30 |  | '00 0000 0000 0000 0000 0000 0111'B |  | '00 0001 1110'B | 30 |  | 32 |  | 32 |  | 0 |
| NR Cell 31 |  | '00 0000 0000 0000 0000 0000 0110'B |  | ’00 00100011’B | 35 |  | 64 |  | 64 |  | 1 |
| NR Cell 32 |  | '00 0000 0000 0000 0000 0001'B |  | '00 0010 0000'B | 32 |  | - |  | - |  | 1 |
| NR Cell 33 |  | '00 0000 0000 0000 0000 0001'B |  | '00 0010 0001'B | 33 |  | - |  | - |  | - |
| NR Cell 489 |  | '00 0000 0000 0000 0000 0000 0001'B |  | '01 1110 1001'B | 489 |  | 128 |  | 128 |  | 0 |
| NR Cell 500 |  | '00 0000 0000 0000 0000 0000 1000'B |  | '00 0000 0111'B | 500 |  | 0 |  | 0 |  | 1 |
| Note 1: To avoid collision of the preambles between intra-frequency cells, with the default zeroCorrelationZoneConfig value set to 15, the PRACH-rootSequenceIndex values have been separated by 32 root sequences per intra-frequency cell. Note 2: This SSB-Index does not apply for RRM test cases in TS 38.533 [18]. RRM test cases shall use the SSB index defined in A.3 of TS 38.533 [18]. Note 3: For signalling test, simultaneous co-existence of active NR cells defined on the same frequency and same SSB-Index is not recommended due to the same timing used for intra-frequency NR cells in TS 38.523-3 [23]. |  |  |  |  |  |  |  |  |  |  |  |

Table 4.4.2-3: Default NAS parameters for simulated NR cells

| cell ID | Tracking Area |  |  |  | TA# list (Note 1) | 5G-GUTI (Note 2) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  | TAC |  | AMF Identifier |  |  | 5G-TMSI |
|  |  | MCC | MNC |  |  | AMF region ID | AMF Set ID | AMF Pointer |  |
| NR Cell 1 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 | Arbitrarily selected according to TS 23.003 [26] clause 2.10 |
| NR Cell 2 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 3 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 4 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 6 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 10 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 11 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 23 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 12, NR Cell 28 | TAI-3 | 002 | 11 | 1 | TAI-3 | 253 | 1 | 1 |  |
| NR Cell 13, NR Cell 29 | TAI-4 | 003 | 21 | 1 | TAI-4 | 252 | 1 | 1 |  |
| NR Cell 14, NR Cell 30 | TAI-5 | 004 | 31 | 1 | TAI-5 | 251 | 1 | 1 |  |
| NR Cell 31 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 254 | 1 | 1 |  |
| NR Cell 489 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| NR Cell 500 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 254 | 1 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column 5G-GUTI indicates 5G-GUTI included in the response messages of the registration procedure (REGISTRATION ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |  |  |  |  |  |

<End of modified section 1>