3GPP TSG-RAN5 Meeting #101	R5-236774

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2968 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default settings of UICC and USIM for 5G ProSe |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Default settings of UICC and USIM for 5G ProSe were not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding default settings of UICC and USIM for 5G ProSe according to TS 31.102. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for NR sidelink relay will be not applicable. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.3.3.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.8.3.3.4	Default settings of UICC and USIM for 5G ProSe

EFAD (Administrative Data)

-	Additional information:

Byte 3 (second byte of additional information):

b5 is used to indicate whether the USIM enables the UE to use "operator-managed" radio resources, in the cases described in 3GPP TS 23.304[60].

- b5=1: the UE is authorized to use "operator-managed" radio resources.

EFUST (USIM Service Table):

| Services |  | Activated | Version |
| --- | --- | --- | --- |
| Service n°139 | 5G ProSe | Optional |  |
| Note: Only 5G ProSe related services indicated. |  |  |  |

EF5G_PROSE_ST (5G ProSe Service Table)

If service n°139 is "available" in the USIM Service Table, this file shall be present. This EF indicates which 5G ProSe services are available.

File size:	1 Bytes

Default values:	Bytes 1 (HEX):	0F

Service n°1 5G ProSe configuration data for direct discovery is supported.

Service n°2 5G ProSe configuration data for direct communication is supported.

Service n°3 5G ProSe configuration data for UE-to-network relay UE is supported.

Service n°4 5G ProSe configuration data for remote UE is supported.

EF5G_PROSE_DD (5G ProSe configuration data for direct discovery)

If service n°139 is "available" in the USIM Service Table and service n°1 is "available" in EF5G_PROSE_ST, this file shall be present. This EF contains 5G ProSe policy for direct discovery. The format of the 5G ProSe policy for direct discovery are specified in 3GPP TS 24.555[59].

The 5G ProSe configuration data for direct discovery contents:

Table 4.8.3.3.4-1: 5G ProSe policy for direct discovery

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| 5G ProSe configuration data for direct discovery Tag | 'A0' | M | 1 |
| Length | Note 1 | M | Note 2 |
| Validity timer Tag | '85' | M | 1 |
| Length | 5 | M | Note 2 |
| Validity timer information | 'FF FF FF FF FF'H | M | 5 |
| Served by NG-RAN Tag | '80' | M | 1 |
| Length | X1 | M | Note 2 |
| Served by NG-RAN information | See Table 4.7.8.2-2 | M | X1 |
| Not served by NG-RAN Tag | '81' | M | 1 |
| Length | X2 | M | Note 2 |
| Not served by NG-RAN information | See Table 4.7.8.2-6 | M | X2 |
| ProSe direct discovery UE ID Tag | '86' | M | 1 |
| Length | 3 | M | Note 2 |
| ProSe direct discovery UE ID information | FFS | M | 3 |
| ProSe identifiers Tag | '82' | M | 1 |
| Length | X3 | M | Note 2 |
| ProSe identifiers information | See Table 4.7.8.2-14 | M | X3 |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules Tag | '83' | M | 1 |
| Length | X4 | M | Note 2 |
| ProSe identifier to default destination layer-2 ID for initial discovery signalling mapping rules information | See Table 4.7.8.2-15 | M | X4 |
| Group member discovery parameters Tag | '84' | O | 1 |
| Length | X5 | O | Note 2 |
| Group member discovery parameters information | See Table 4.7.8.2-12 | O | X5 |
| HPLMN 5G DDNMF address information Tag | '98' | O | 1 |
| Length | X6+1 | O | Note 2 |
| HPLMN 5G DDNMF address information type | FFS | O | 1 |
| HPLMN 5G DDNMF address information information | See Table 4.7.8.2-17 | O | X6 |
| Note 1: This is the total size of the constructed TLV object. Note 2: The length is coded according to ISO/IEC 8825-1 [35]. |  |  |  |

EF5G_PROSE_DC (5G ProSe configuration data for direct communication)

If service n°139 is "available" in the USIM Service Table and service n°2 is "available" in EF5G_PROSE_ST, this file shall be present. This EF contains 5G ProSe policy for direct communication. The format of the 5G ProSe policy for direct communication are specified in 3GPP TS 24.555[59].

The 5G ProSe configuration data for direct communication contents:

Table 4.8.3.3.4-2: 5G ProSe policy for direct communication

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| 5G ProSe configuration data for direct communication Tag | 'A0' | M | 1 |
| Length | Note 1 | M | Note 2 |
| Validity timer Tag | '85' | M | 1 |
| Length | 5 | M | Note 2 |
| Validity timer information | 'FF FF FF FF FF'H | M | 5 |
| Served by NG-RAN Tag | '80' | M | 1 |
| Length | X1 | M | Note 2 |
| Served by NG-RAN information | See Table 4.7.8.3-2 | M | X1 |
| Not served by NG-RAN Tag | '81' | O | 1 |
| Length | X2 | O | Note 2 |
| Not served by NG-RAN information | See Table 4.7.8.3-5 | O | X2 |
| Privacy config Tag | '87' | O | 1 |
| Length | X3 | O | Note 2 |
| Privacy config information | See Table 4.7.8.3-11 | O | X3 |
| 5G ProSe direct communication in NR-PC5 Tag | '88' | O | 1 |
| Length | X4 | O | Note 2 |
| 5G ProSe direct communication in NR-PC5 information | See Table 4.7.8.3-16 | O | X4 |
| ProSe application to path preference mapping rules Tag | '89' | O | 1 |
| Length | X5 | O | Note 2 |
| ProSe application to path preference mapping rules information | See Table 4.7.8.3-41 | O | X5 |
| ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules Tag | '91' | O | 1 |
| Length | X6 | O | Note 2 |
| ProSe identifiers to NR Tx profile for broadcast and groupcast mapping rules information | See Table 4.7.8.3-43 | O | X6 |
| Note 1: This is the total size of the constructed TLV object. Note 2: The length is coded according to ISO/IEC 8825-1 [35]. |  |  |  |

EF5G_PROSE_U2NRU (5G ProSe configuration data for UE-to-network relay UE)

If service n°139 is "available" in the USIM Service Table and service n°3 is "available" in EF5G_PROSE_ST, this file shall be present. This EF contains 5G ProSe policy for UE-to-network relay UE. The format of the 5G ProSe policy for UE-to-network relay UE are specified in 3GPP TS 24.555[59].

The 5G ProSe configuration data for UE-to-network relay UE contents:

Table 4.8.3.3.4-3: 5G ProSe policy for UE-to-network relay UE

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| 5G ProSe configuration data for UE-to-network relay UE Tag | 'A0' | M | 1 |
| Length | Note 1 | M | Note 2 |
| Validity timer Tag | '85' | M | 1 |
| Length | 5 | M | Note 2 |
| Validity timer information | 'FF FF FF FF FF'H | M | 5 |
| Served by NG-RAN Tag | '80' | M | 1 |
| Length | X1 | M | Note 2 |
| Served by NG-RAN information | See Table 4.7.8.4-2 | M | X1 |
| Not served by NG-RAN Tag | '81' | M | 1 |
| Length | X2 | M | Note 2 |
| Not served by NG-RAN information | See Table 4.7.8.4-5 | M | X2 |
| Default destination layer-2 IDs for sending the discovery signalling for announcement and additional information and for receiving the discovery signalling for solicitation Tag | '8A' | M | 1 |
| Length | X3 | M | Note 2 |
| Default destination layer-2 IDs for sending the discovery signalling for announcement and additional information and for receiving the discovery signalling for solicitation information | See Table 4.7.8.4-11b | M | X3 |
| User info ID for discovery Tag | '8E' | M | 1 |
| Length | 6 | M | Note 2 |
| User info ID for discovery information | FFS | M | 6 |
| RSC info list Tag | '8B' | M | 1 |
| Length | X4 | M | Note 2 |
| RSC info list information | See Table 4.7.8.4-12 | M | X4 |
| 5QI to PC5 QoS parameters mapping rules Tag | '8C' | M | 1 |
| Length | X5 | M | Note 2 |
| 5QI to PC5 QoS parameters mapping rules information | See Table 4.7.8.4-17 | M | X5 |
| ProSe identifier to ProSe application server address mapping rules Tag | '8D' | O | 1 |
| Length | X6 | O | Note 2 |
| ProSe identifier to ProSe application server address mapping rules information | See Table 4.7.8.4-19 | O | X6 |
| Privacy timer Tag | '92' | O | 1 |
| Length | X7 | O | Note 2 |
| Privacy timer information | FFS | O | X7 |
| 5G PKMF address information Tag | '93' | O | 1 |
| Length | X8 | O | Note 2 |
| 5G PKMF address information information | See Table 4.7.8.4-21 | O | X8 |
| Note 1: This is the total size of the constructed TLV object. Note 2: The length is coded according to ISO/IEC 8825-1 [35]. |  |  |  |

EF5G_PROSE_RU (5G ProSe configuration data for remote UE)

If service n°139 is "available" in the USIM Service Table and service n°4 is "available" in EF5G_PROSE_ST, this file shall be present. This EF contains 5G ProSe policy for remote UE. The format of the 5G ProSe policy for remote UE are specified in 3GPP TS 24.555[59].

The 5G ProSe configuration data for remote UE contents:

Table 4.8.3.3.4-4: 5G ProSe policy for remote UE

| Description | Value | M/O | Length (bytes) |
| --- | --- | --- | --- |
| 5G ProSe configuration data for remote UE Tag | 'A0' | M | 1 |
| Length | Note 1 | M | Note 2 |
| Validity timer Tag | '85' | M | 1 |
| Length | 5 | M | Note 2 |
| Validity timer information | 'FF FF FF FF FF'H | M | 5 |
| Served by NG-RAN Tag | '80' | M | 1 |
| Length | X1 | M | Note 2 |
| Served by NG-RAN information | FFS | M | X1 |
| Not served by NG-RAN Tag | '81' | M | 1 |
| Length | X2 | M | Note 2 |
| Not served by NG-RAN information | FFS | M | X2 |
| Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information Tag | '8F' | M | 1 |
| Length | X3 | M | Note 2 |
| Default destination layer-2 IDs for sending the discovery signalling for solicitation and for receiving the discovery signalling for announcement and additional information information | FFS | M | X3 |
| User info ID for discovery Tag | '8E' | M | 1 |
| Length | 6 | M | Note 2 |
| User info ID for discovery information | FFS | M | 6 |
| RSC info list Tag | '8B' | M | 1 |
| Length | X4 | M | Note 2 |
| RSC info list information | FFS | M | X4 |
| N3IWF selection information for 5G ProSe layer-3 remote UE Tag | '90' | O | 1 |
| Length | X5 | O | Note 2 |
| N3IWF selection information for 5G ProSe layer-3 remote UE information | FFS | O | X5 |
| Privacy timer Tag | '92' | O | 1 |
| Length | 2 | O | Note 2 |
| Privacy timer information | FFS | O | 2 |
| 5G PKMF address information Tag | '93' | O | 1 |
| Length | X6 | O | Note 2 |
| 5G PKMF address information information | FFS | O | X6 |
| Note 1: This is the total size of the constructed TLV object. Note 2: The length is coded according to ISO/IEC 8825-1 [35]. |  |  |  |

<End of Changed Section>