3GPP TSG-WG5 Meeting #101	R5-236785

# Chicago, USA, 13th – 17th November 2023

| Text Proposal |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.551 | CR | n/a | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR for TS 38.551, Annex C |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Switzerland AG |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_OTA-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | n/a |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TS 38.551 alignment with deffinitions on TS 38.151 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial alignment with TS 38.151 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of requirements alignment between RAN4 and RAN5 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex C.3.3, C.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change #1 >

## C.3.3	Doppler/Temporal correlation

This measurement checks the Doppler/temporal correlation. For Doppler/Temporal correlation validation measurement, only Vertical validation is required.

The Doppler spectrum is measured with a spectrum analyser as shown in Figure C.3.3-1. In this case a signal generator transmits CW signal through the NR MIMO OTA test system. The signal is received by a test antenna within the test area. Finally, the signal is analysed by a spectrum analyser and the measured spectrum is compared to the target spectrum. This setup can be used to measure Doppler Spectrum of the Channel models defined in Annex C.1.

Method of measurement:

Figure C.3.3-1: Setup for Doppler measurements

Sine wave (CW, carrier wave) signal is transmitted from the signal generator. The signal is connected from the signal generator to fading emulator via cables. The fading emulator output signals are connected to power amplifier boxes via cables. The amplified signals are then transferred via cables to the probe antennas. The probe antennas radiate the signals over the air to the test antenna. The Doppler spectrum is measured by the spectrum analyser and the trace is saved.

Signal generator settings for Doppler/Temporal correlation measurements are presented in Table C.3.3-1.

Table C.3.3-1: Signal generator settings for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Modulation |  | OFF |

Spectrum analyser settings for Doppler/Temporal correlation measurements are presented in Table C.3.3-2.

Table C.3.3-2: Spectrum analyser settings for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Minimum Span | Hz | 4 kHz |
| RBW | Hz | 1 |
| VBW | Hz | 1 |
| Number of points |  | 16002 |
| Averaging |  | 100 |

Channel model specification for Doppler/Temporal correlation measurements is presented in Table C.3.3-3.

Table C.3.3-3: Channel model specification for Doppler/Temporal correlation measurements

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Channel model |  | As specified in Annex C.1 |
| Mobile speed | km/h | 100 |

Method of measurement result analysis: Measurement data file (Doppler power spectrum) is saved into hard drive. The data is read into, e.g., Matlab. The analysis is performed by taking the Fourier transformation of the Doppler spectrum. The resulting temporal correlation function Rt(∆t)  is normalized such that max(abs(Rt(∆t)))=1. Then the function values left from the maximum i.e., the negative lags are cut out. Further on the function values after five periods are cut out.

Time Domain Alternate Method

Time domain techniques can also be used to validate the tempoal correlation. The temporal correlation validation measurement setup is illustrated in Figure C.3.3-2. In this case a Signal generator transmits a CW signal through the MIMO test system. The signal is received by a test antenna within the test area. Finally, the signal is collected by a signal analyser and the measured signal is stored as IQ data format for postprocessing.

Figure C.3.3-2: Setup for Doppler measurements based on time domain technique

The time domain doppler spectrum is measured by the signal analyzer and the trace in IQ format is saved. Follow the same procedure to post process the data and calculate the temporal correlation curve. Data recording is synchronized with the channel emulator trigger.

The settings for the signal analyzer are in Table C.3.3-4:

Table C.3.3-4: Signal Analyser Settings

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Sampling | Hz | At least 15 times bigger than the max Doppler spread (fd=v/λ) |
| Observation time | s | At least 16s. Channel Model length should be the same or greater than the observation time. |

Beam-Specific Block Diagram

It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

Figure C.3.3-3: Setup for Beam-Specific Doppler measurements (Beam 1)

Figure C.3.3-4: Setup for Beam-Specific Doppler measurements (Beam 2 CDL-C UMa only)

The detailed Temporal correlation reference value for CDL-C UMa and CDL-C UMi channel model validation is defined is table C.3.3-5.

Table C.3.3-5: Autocorrelation Targets

| Lambda Separation | CDL-C UMa beam 1 at ≤ 2.5 GHz | CDL-C UMa beam 2 at ≤ 2.5 GHz | CDL-C UMa beam 1 at > 2.5 GHz | CDL-C UMa beam 2 at > 2.5 GHz | CDL-C UMi beam 1 at ≤ 2.5 GHz | CDL-C UMi beam 1 at > 2.5 GHz |
| --- | --- | --- | --- | --- | --- | --- |
| 0.0 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 | 1.000 |
| 0.1 | 0.986 | 0.974 | 0.985 | 0.973 | 0.995 | 0.995 |
| 0.2 | 0.945 | 0.907 | 0.942 | 0.904 | 0.982 | 0.982 |
| 0.3 | 0.882 | 0.832 | 0.874 | 0.825 | 0.962 | 0.961 |
| 0.4 | 0.801 | 0.776 | 0.787 | 0.765 | 0.936 | 0.935 |
| 0.5 | 0.709 | 0.738 | 0.689 | 0.723 | 0.906 | 0.905 |
| 0.6 | 0.613 | 0.695 | 0.586 | 0.675 | 0.872 | 0.871 |
| 0.7 | 0.518 | 0.623 | 0.486 | 0.599 | 0.834 | 0.834 |
| 0.8 | 0.430 | 0.525 | 0.394 | 0.496 | 0.793 | 0.793 |
| 0.9 | 0.353 | 0.426 | 0.315 | 0.391 | 0.750 | 0.749 |
| 1.0 | 0.289 | 0.360 | 0.252 | 0.319 | 0.705 | 0.704 |
| 1.1 | 0.240 | 0.335 | 0.206 | 0.290 | 0.659 | 0.658 |
| 1.2 | 0.204 | 0.320 | 0.174 | 0.273 | 0.614 | 0.612 |
| 1.3 | 0.181 | 0.287 | 0.154 | 0.239 | 0.569 | 0.568 |
| 1.4 | 0.167 | 0.233 | 0.143 | 0.185 | 0.527 | 0.525 |
| 1.5 | 0.159 | 0.176 | 0.137 | 0.129 | 0.487 | 0.485 |
| 1.6 | 0.155 | 0.141 | 0.135 | 0.096 | 0.450 | 0.448 |
| 1.7 | 0.153 | 0.135 | 0.134 | 0.092 | 0.417 | 0.415 |
| 1.8 | 0.150 | 0.137 | 0.134 | 0.095 | 0.387 | 0.385 |
| 1.9 | 0.144 | 0.132 | 0.130 | 0.093 | 0.361 | 0.358 |
| 2.0 | 0.135 | 0.117 | 0.122 | 0.089 | 0.337 | 0.335 |
| 2.1 | 0.121 | 0.097 | 0.109 | 0.086 | 0.316 | 0.313 |
| 2.2 | 0.105 | 0.076 | 0.090 | 0.076 | 0.296 | 0.293 |
| 2.3 | 0.085 | 0.062 | 0.069 | 0.064 | 0.277 | 0.274 |
| 2.4 | 0.065 | 0.071 | 0.047 | 0.067 | 0.258 | 0.255 |
| 2.5 | 0.048 | 0.090 | 0.031 | 0.088 | 0.239 | 0.236 |
| 2.6 | 0.039 | 0.099 | 0.033 | 0.103 | 0.219 | 0.216 |
| 2.7 | 0.038 | 0.088 | 0.046 | 0.099 | 0.198 | 0.195 |
| 2.8 | 0.042 | 0.058 | 0.057 | 0.073 | 0.178 | 0.175 |
| 2.9 | 0.043 | 0.037 | 0.062 | 0.038 | 0.158 | 0.154 |
| 3.0 | 0.041 | 0.067 | 0.060 | 0.045 | 0.138 | 0.135 |
| 3.1 | 0.037 | 0.103 | 0.050 | 0.080 | 0.120 | 0.116 |
| 3.2 | 0.036 | 0.120 | 0.036 | 0.100 | 0.103 | 0.100 |
| 3.3 | 0.044 | 0.115 | 0.019 | 0.099 | 0.089 | 0.085 |
| 3.4 | 0.056 | 0.097 | 0.010 | 0.081 | 0.076 | 0.073 |
| 3.5 | 0.068 | 0.082 | 0.019 | 0.061 | 0.066 | 0.063 |
| 3.6 | 0.075 | 0.083 | 0.029 | 0.053 | 0.057 | 0.055 |
| 3.7 | 0.076 | 0.090 | 0.034 | 0.060 | 0.051 | 0.049 |
| 3.8 | 0.068 | 0.089 | 0.036 | 0.073 | 0.046 | 0.044 |
| 3.9 | 0.051 | 0.079 | 0.044 | 0.091 | 0.042 | 0.041 |
| 4.0 | 0.027 | 0.068 | 0.062 | 0.111 | 0.039 | 0.038 |
| 4.1 | 0.007 | 0.063 | 0.090 | 0.127 | 0.037 | 0.035 |
| 4.2 | 0.036 | 0.062 | 0.123 | 0.133 | 0.036 | 0.034 |
| 4.3 | 0.067 | 0.057 | 0.155 | 0.129 | 0.038 | 0.036 |
| 4.4 | 0.093 | 0.052 | 0.182 | 0.126 | 0.043 | 0.040 |
| 4.5 | 0.111 | 0.055 | 0.200 | 0.131 | 0.051 | 0.048 |
| 4.6 | 0.119 | 0.063 | 0.207 | 0.139 | 0.061 | 0.058 |
| 4.7 | 0.116 | 0.066 | 0.200 | 0.138 | 0.073 | 0.070 |
| 4.8 | 0.101 | 0.058 | 0.180 | 0.117 | 0.085 | 0.082 |
| 4.9 | 0.078 | 0.047 | 0.149 | 0.079 | 0.096 | 0.093 |
| 5.0 | 0.051 | 0.048 | 0.110 | 0.034 | 0.107 | 0.104 |

<End of Change #1 >

<Start of Change #2 >

## C.3.5	Cross-polarization

This measurement checks how well the measured vertically or horizontally polarized power levels follow expected values. The test setup for cross-polarization is the same as PDP validation in Figure C.3.2-1.

Method of measurement: 		Step the emulation and store traces from VNA.

VNA settings for cross-polarization measurements are presented in Table C.3.5-1.

Table C.3.5-1: VNA settings for cross-polarization

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink Centre Frequency in Table C.3.1-1 |
| Span | MHz | 40 |
| Number of traces |  | 1000 |
| Number of points |  | 802 |
| Averaging |  | 1 |

Channel model specification for cross-polarization measurements is presented in Table C.3.5-2.

Table C.3.5-2: Channel model specification for cross-polarization

| Item | Unit | Value |
| --- | --- | --- |
| Centre frequency | MHz | Downlink centre frequency in Table C.3.1-1 |
| Distance between traces in channel model | wavelength (Note) | > 2 |
| Channel model |  | As specified in Annex C.1 |
| Mobile speed (** | km/h | 30 |
| NOTE: Time [s] = distance [] / MS speed [/s] MS speed [/s] = MS speed [m /s] / Speed of light [m/s] * Centre frequency [Hz] (** The mobile speed is valid for the Time Domain Alternative method only |  |  |

Measurement Procedure:

Step the emulation and store traces from VNA. i.e., run the emulation to CIR number 1, pause, measure VNA trace, run the emulation to CIR number 10, pause, measure VNA trace. Continue until 1000 VNA traces are measured.

a.	Use a vertically polarized sleeve dipole to measure the V component.

b.	Use a horizontally polarized (vertically oriented) magnetic loop dipole, or a horizontally polarized sleeve dipole measured in four orthogonal horizontal positions and summed to measure the H component.

Method of measurement result analysis:

Measured VNA traces (frequency responses HV(t,f) and HHt,f are saved into a hard drive. The data is read into, e.g., Matlab. The frequency responses are averaged in power over time and frequency and the V/H ratio calculated as follows:

PV=tfHV(t,f)2

PH=tfHH(t,f)2

PV/H[dB]=10log10PVPH

Beam-Specific Block Diagram

It is assumed that the beams are mapped to the inputs of the channel emulator as follows:

-	Beam 1: Input 1 and Input 2

-	Beam 2: Input 3 and Input 4 (CDL-C UMa only)

Figure C.3.5-1: Setup for Beam-Specific V/H measurements (Beam 1)

Figure C.3.5-2: Setup for Beam-Specific V/H measurements (Beam 2 CDL-C UMa only)

Time Domain Alternative Method:

The power in the Vertical and Horizontal polarizations can also be measured in time domain. The measurement setup for Beam-Specific are presented in Figures C.3.5-3, and C.3.5-4.

Figure C.3.5-3: Setup for Beam-Specific V/H measurements (Beam 1)

Figure C.3.5-4: Setup for Beam-Specific V/H measurements (Beam 2 CDL-C UMa only)

The instruments settings are the same as those in C.3.4-6 and C.3.4-7. The measurement analysis is the same as that of the frequency domain method setting the summation over f to a single point.

The reference V/H-ratios for CDL-C UMa and CDL-C UMi channel model validation are defined in table C.3.5-3 and C.3.5-4, respectively.

Table C.3.5-3: Reference V/H-ratios for CDL-C UMa

| UMa C, fc ≤ 2.5 GHz | Beam 1 | Input 1+2: V/H = 0 dB |
| --- | --- | --- |
|  | Beam 2 | Input 3+4: V/H = 0 dB |
| UMa C, fc > 2.5 GHz | Beam 1 | Input 1+2: V/H = 0 dB |
|  | Beam 2 | Input 3+4: V/H = 0 dB |

Table C.3.5-4: Reference V/H-ratios for CDL-C UMi

| UMi C, fc ≤ 2.5 GHz | Beam 1 | Inputs 1+2: V/H = 0 dB |
| --- | --- | --- |
| UMi C, fc > 2.5 GHz | Beam 1 | Inputs 1+2: V/H = 0 dB |

<End of Change #2 >