3GPP TSG-RAN5 Meeting #101 	R5-236801

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2973 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to PDCCH-ConfigCommon for performance tests for TDD patterns other than SCS 30 kHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are conflicts of SSB and Type0PDCCH CSS for TDD frame patterns other than SCS 30 kHz. Note that the conflict for TDD SCS 30 kHz and FDD has been solved in R5-204053. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 5.4.2.0-4, searchSpaceZero was configured for eah frame patterns of TDD 15 kHz, 60 kHz, 120 kHz to solve the conflicts of SSB and CSS. Editorial correction for text style in Table 5.4.2.0-3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The conflicts will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

5.4.2.0	Parameters common to all Demod and CSI tests

Physical layer parameters

Table 5.4.2.0-1: Physical layer parameters for DCI format 1_1 and DCI format 1_0

| Derivation Path: Table 4.3.6.1.2.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PUCCH resource indicator | PUCCH-ResourceId[1] = 0 in pucch-ResourceSetID[1] or PUCCH-ResourceId[1] = 8 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘000’B | FR1 |
|  | PUCCH-ResourceId[5] = 12 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘100’B | FR2_SCS60kHz |
|  | PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | ‘110’B | FR2_SCS120kHz |
| PDSCH-to-HARQ_feedback timing indicator | K1 slots as defined in 9.2.3 in TS 38.213. K1 value used for the test is specified in the test description of each test case in TS 38.521-4. |  |  |

Common Serving Parameters

ServingCellConfigCommon

Table 5.4.2.0-2: ServingCellConfigCommon

| Derivation Path: Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| downlinkConfigCommon | DownlinkConfigCommon |  |  |
| ssb-PositionsInBurst CHOICE { |  |  |  |
| shortBitmap | 1000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (FDD OR (TDD AND SCS15)) OR FREQ<=1.88GHz) |
| mediumBitmap | 10000000 |  | FR1 AND SSB#0 AND (1.88GHz<FREQ<=3GHz AND (TDD AND SCS30) OR FREQ>3GHz) |
| } |  |  |  |
| ssb-periodicityServingCell | ms20 |  |  |
| dmrs-TypeA-Position | pos2 |  |  |
| subcarrierSpacing | SubcarrierSpacing according to test case id |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  |  |
|  | Not present |  | FR1.30-1A FR2.120-1A FR1.FDD |
| ss-PBCH-BlockPower | 0 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FREQ<=1.88GHz | Frequency range <= 1.88GHz |
| 1.88GHz<FREQ<=3GHz | Frequency range > 1.88GHz and <= 3GHz |
| FREQ>3GHz | Frequency range > 3GHz |
| FR1.30-1A | TDD UL-DL pattern FR1.30-1A is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR2.120-1A | TDD UL-DL pattern FR2.120-1A is used. Ref Annex A.1.3 of TS 38.521-4 |
| SSB#N | Cell configured with SSB-Index set to N as defined in Table 4.4.2-2 |

TDD-UL-DL-Config

Table 5.4.2.0-3: TDD-UL-DL-Config

| Derivation Path: Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms5 |  | FR1.15-1, FR1.30-1 |
|  | ms2p5 |  | FR1.30-2, FR1.30-3 |
|  | ms3 |  | FR1.30-4 |
|  | ms2 |  | FR1.30-5 |
|  | ms1 |  | FR1.30-6 FR2.60-1 |
|  | ms0p625 |  | FR2.120.1 |
|  | ms0p5 |  | FR2.120.2 |
| nrofDownlinkSlots | 7 |  | FR1.30-1 |
|  | 3 |  | FR1.15-1 FR1.30-2 FR1.30-3 FR1.30-4 FR2.120.1 |
|  | 1 |  | FR1.30-5 FR1.30-6 |
|  | 2 |  | FR2.60-1 FR2.120.2 |
| nrofDownlinkSymbols | 6 |  | FR1.30-1 FR1.30-4 |
|  | 10 |  | FR1.15-1 FR1.30-2 FR1.30-3 FR1.30-6 FR2.120.1 |
|  | 12 |  | FR1.30-5 |
|  | 11 |  | FR2.60-1 FR2.120.2 |
| nrofUplinkSlots | 2 |  | FR1.30-1 FR1.30-4 FR1.30-5 |
|  | 1 |  | FR1.15-1 FR1.30-2 FR1.30-3 FR2.60-1 FR2.120.1 FR2.120.2 |
|  | 0 |  | FR1.30-6 |
| nrofUplinkSymbols | 4 |  | FR1.30-1 FR1.30-4 FR1.30-5 |
|  | 0 |  | FR1.30-6 FR2.60-1 FR2.120.2 |
|  | 2 |  | FR1.15-1 FR1.30-2 FR1.30-3 FR2.60-1 FR2.120.1 FR2.120.2 |
| } |  |  |  |
| pattern2 | Not present |  | FR1.15-1, FR1.30-1 FR1.30-2 FR1.30-5 FR2.60-1 FR2.120.1 FR2.120.2 |
| pattern2 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms2p5 |  | FR1.30-3 |
|  | ms2 |  | FR1.30-4 |
|  | ms1 |  | FR1.30-6 |
| nrofDownlinkSlots | 2 |  | FR1.30-3 |
|  | 4 |  | FR1.30-4 |
|  | 0 |  | FR1.30-6 |
| nrofDownlinkSymbols | 10 |  | FR1.30-3 |
|  | 0 |  | FR1.30-4 |
|  | 12 |  | FR1.30-6 |
| nrofUplinkSlots | 2 |  | FR1.30-3 |
|  | 0 |  | FR1.30-4 |
|  | 1 |  | FR1.30-6 |
| nrofUplinkSymbols | 2 |  | FR1.30-3 |
|  | 0 |  | FR1.30-4 FR1.30-6 |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1.15-1 | TDD UL-DL pattern FR1.15-1 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-1 | TDD UL-DL pattern FR1.30-1 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-2 | TDD UL-DL pattern FR1.30-2 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-3 | TDD UL-DL pattern FR1.30-3 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-4 | TDD UL-DL pattern FR1.30-4 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-5 | TDD UL-DL pattern FR1.30-5 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR1.30-6 | TDD UL-DL pattern FR1.30-6 is used. Ref Annex A.1.2 of TS 38.521-4 |
| FR2.60-1 | TDD UL-DL pattern FR2.60-1 is used. Ref Annex A.1.3 of TS 38.521-4 |
| FR2.120-1 | TDD UL-DL pattern FR2.120-1 is used. Ref Annex A.1.3 of TS 38.521-4 |
| FR2.120-2 | TDD UL-DL pattern FR2.120-2 is used. Ref Annex A.1.3 of TS 38.521-4 |

PDCCH Configuration

PDCCH-ConfigCommon

Table 5.4.2.0-4: PDCCH-ConfigCommon

| Derivation Path: Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon::= SEQUENCE { |  |  |  |
| searchSpaceZero | 4 |  | FDD FR1.15-1 FR1.30-1 FR1.30-2 FR1.30-3 FR1.30-4 FR2.60-1 FR2.120-1 FR2.120-2 |
|  | 2 |  | FR1.30-5 FR1.30-6 |
| commonSearchSpaceList SEQUENCE (SIZE (1..4)) OF SearchSpace { | 1 entry | 1 search space for both SA and NSA |  |
| SearchSpace [1] | SearchSpace with condition CSS | entry 1 |  |
| } |  |  |  |
| searchSpaceOtherSystemInformation | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |

<<End of change>>