3GPP TSG-RAN5 Meeting #101 	R5-236806

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2512 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to UL assignment for different SCS in 6.3A.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, UL assignment of ON/OFF time mask for CA is defined for each SCS. If SCS is different for each CC, UL of both CCs is not assigned at the same time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UL assignment was updated so that UL is transmitted at the same time regardless SCS in 6.3A.3.1.4.2 and Table 6.3A.3.1.4.3-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UL for different SCS will not be transmitted at the same time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.3A.3.1	Transmit ON/OFF time mask for CA (2UL CA)

6.3A.3.1.1	Test purpose

To verify that the general ON/OFF time mask for CA (2UL CA) meets the requirements given in 6.3A.3.1.5

The transmit power time mask for transmit ON/OFF defines the transient period(s) allowed between transmit OFF power and transmit ON power symbols for CA.

Transmission of the wrong power increases interference to other channels, or increases transmission errors in the uplink channel.

6.3A.3.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that support 2UL CA.

6.3A.3.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.3A.3.0.

6.3A.3.1.4	Test description

6.3A.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in table 6.3A.3.1.4.1-1 or 6.3A.3.1.4.1-2. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3A.3.1.4.1-1: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band.. |  |  |  |  |

Table 6.3A.3.1.4.1-2: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.1.4 for intra band contiguous CA in FR1 |  | Low range High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | Inner Full | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in 6.1A-1a NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. |  |  |  |  |

Table 6.3A.3.1.4.1-3: Test Configuration Table for intra-band non-contiguous CA when UE supporting IE dualPA-Architecture

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1.1.5 for intra band non-contiguous CA in FR1 |  | Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration for PCC & SCC | Uplink Configuration |  |  |
|  |  | Modulation for all CCs | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 1 | N/A for this test | DFT-s-OFDM QPSK | SCS 15kHz: 45@0 SCS 30kHz: 24@0 SCS 60kHz: 12@0 | SCS 15kHz: 45@0 SCS 30kHz: 24@0 SCS 60kHz: 12@0 |
| NOTE 1: The specific configuration of each RB allocation is defined in 6.1A-1a NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths and SCS are specified in Table 5.5A.3-1. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Tables 6.3A.3.1.4.1-1 to 6.3A.3.1.4.1-3a as appropriate.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.3A.3.1.4.3.

6.3A.3.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.3A.3.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133 [19], clause 9.3).

4.	SS sends uplink scheduling information via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.3A.3.1.4.1-1 or 6.3A.3.1.4.1-2 on both PCC and SCC. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC. The UL assignment is such that the UE transmits on slots 8 for 15kHz SCS, on slots 16 and 17 for 30kHz SCS and on slots 32 through 35 for 60kHz SCS.

5.	Send continuously uplink power control "up" commands for both carriers in every uplink scheduling information to the UE; allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	On power sub test:

6.1.	Measure the sum of mean output power over all component carriers in the CA configuration of the UE PUSCH transmission during one slot of the radio access mode. For FDD band in inter-band CA with both TDD band and FDD band, only slots overlapping with only UL symbols in TDD are under test.

7.	OFF power sub test:

7.1.	Measure the UE transmission OFF power  for each component carrier during the slot prior to the PUSCH transmission, excluding a transient period of 10 µs in the end of the slot.

7.2.	Measure the UE transmission OFF power of each component carrier during the slot following the PUSCH transmission, excluding a transient period of 10 µs at the beginning of the slot.

6.3A.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 with following exceptions.

Table 6.3A.3.1.4.3-1: PUSCH-ConfigCommon

| Derivation Path: TS 38.508-1[5], Table 4.6.3-90 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| p0-NominalWithGrant | -100 |  |  |
| } |  |  |  |

Table 6.3A.3.1.4.3-2: TDD-UL-DL-Config

| Derivation Path: TS 38.508-1[5], Table 4.6.3-192 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-UL-DL-ConfigCommon ::= SEQUENCE { |  |  |  |
| referenceSubcarrierSpacing | SubcarrierSpacing |  |  |
| pattern1 SEQUENCE { |  |  |  |
| dl-UL-TransmissionPeriodicity | ms10 |  | FR1 |
| nrofDownlinkSlots | 6 |  | FR1_15kHz |
|  | 13 |  | FR1_30kHz |
|  | 27 |  | FR1_60kHz |
| nrofDownlinkSymbols | 10 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 12 |  | FR1_60kHz |
| nrofUplinkSlots | 3 |  | FR1_15kHz |
|  | 6 |  | FR1_30kHz |
|  | 12 |  | FR1_60kHz |
| nrofUplinkSymbols | 4 |  | FR1_30kHz |
|  | 2 |  | FR1_15kHz, |
|  | 8 |  | FR1_60kHz |
| } |  |  |  |
| pattern2 | Not present |  |  |
| } |  |  |  |

Table 6.3A.3.1.4.3-3: PUSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1[5], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| k2 | 4 |  | FR1_15kHz,FR1_30kHz |
|  | 6 |  | FR1_60kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | 2 | K2+ Δ=4 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_15kHz |
|  | 6 | K2+ Δ=9 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 (NOTE 1) | FR1_30kHz |
| mappingType | typeA |  |  |
| startSymbolAndLength | 27 | Start symbol(S)=0, Length(L)=14 |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Values are chosen so that first slot of a TDD-UL-DL slot configuration period can be used for the Random Access Response and the last slot (of the same or another period) for the corresponding Msg3. |  |  |  |

| Condition | Explanation |
| --- | --- |
| FR1_15kHz | FR1 is used under the test. SCS is set to 15kHz. |
| FR1_30kHz | FR1 is used under the test. SCS is set to 30kHz. |
| FR1_60kHz | FR1 is used under the test. SCS is set to 60kHz. |

Table 6.3A.3.1.4.3-4: ServingCellConfigCommon

| Derivation Path: 38.508-1[5], Table 4.6.3-168 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| ss-PBCH-BlockPower | 18 |  | SCS_15kHz |
|  | 21 |  | SCS_30kHz |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCS_15kHz | SCS=15kHz for SS/PBCH block |
| SCS_30kHz | SCS=30kHz for SS/PBCH block |

Table 6.3A.3.1.4.3-5: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

Table 6.3A.3.1.4.3-6: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-62 FrequencyInfoUL-SIB |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-Max | 20 |  | Inter-band CA |
|  | 19 |  | intra-band contiguous for CA_n48B |
|  | 18 |  | intra-band contiguous for CA_n41C |
|  | 17 |  | intra-band contiguous for CA_n77C |
|  | [16] |  | intra-band non-contiguous 2 UL CA PC3 (MPRIM3 to meet -13dBm/MHz) |
|  | [8] |  | intra-band non-contiguous 2 UL CA PC3 (MPRIM3 to meet -30dBm/MHz) |

6.3A.3.1.5	Test requirement

The requirement for the transmit ON power and transmit OFF power for CA measured in steps 5, 6 and 7 of the test procedure shall not exceed the values specified in Table 6.3A.3.1.5-1.

Table 6.3A.3.1.5-1: General ON/OFF time mask

|  | Channel bandwidth / minimum output power / measurement bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| Transmit OFF power | ≤ -50+TT dBm |  |  |  |  |  |  |  |  |  |  |  |  |
| Transmission OFF Measurement bandwidth | 4.515 | 9.375 | 14.235 | 19.095 | 23.955 | 28.815 | 38.895 | 48.615 | 58.35 | 68.07 | 78.15 | 88.23 | 98.31 |
| Transmit ON power | For inter-band CA configurations, The test requirement of transmit on power of 2UL CA is the same as Test ID 9of Table 6.2A.2.1.5-1 as appropriate. For intra-band contiguous CA configurations, The test requirement of transmit on power of 2UL CA is the same as Test ID 3 of Table 6.2A.2.1.5-1a, Table 6.2A.2.1.5-1b and 6.2A.2.1.5-1c as appropriate. For intra-band non-contiguous CA configurations, The test requirement of transmit on power of 2UL CA is the same as Test ID 4 of Table 6.2A.2.1.5-1e, Test ID 3 of 6.2A.2.1.5-1f as appropriate. |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: TT for each frequency and channel bandwidth of OFF power is specified in Table 6.3A.3.1.5-2. NOTE 2: TT or each frequency and channel bandwidth of Transmit ON power is specified in Table 6.2A.2.1.5-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.3A.3.1.5-2: Test Tolerance for OFF power

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 6GHz |
| --- | --- | --- |
| BW ≤ 40MHz | 1.5 | 1.8 |
| 40MHz < BW ≤ 100MHz | 1.7 | 1.8 |

<<End of change>>