3GPP TSG-RAN5 Meeting #101 	R5-236809

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2515 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RB allocation of A-MPR for NS_07 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some test points for A-MPR (NS_07) have following issues. - LCRB for DFT-s OFDM does not fulfill the following definition in TS 38.211 6.3.1.4. - RBstart for A4 does not fulfill the condition of Table 6.2.3.3.29-1. Incorrect message exception in Table 6.2.3.4.3.35-1 (NS_07) |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 6.2.3.4.1-31, RB allocation was corrected as below. - Test ID 2, 10, 18, 26, 34, SCS 15 kHz: 7@6 -> 8@6 - Test ID 7, 15, 23, 31, 39, 47, 55, 63, 71, SCS 15 kHz: 2@19 -> 2@20 Test ID 8, 16, 24, 32, 40, 48, 56, 64, 72, SCS 15 kHz: 2@36 -> 2@37 Test ID 8, 16, 24, 32, 40, 48, 56, 64, 72, SCS 30 kHz: 1@18 -> 1@19 IE value in Table 6.2.3.4.3.35-1 was corrected to 2 according to Table 6.2.3.3.1-1A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance test will not be performed correctly due to incorrect RB allocations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. Unless stated otherwise, Edge RB allocations get the same A-MPR as Outer RB allocations. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, NS_07, NS_27 and NS_35 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity and NR Power Class 1 UE release 15 and forward.

The requirements of this test apply in test case 6.5.2.4.2 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 3 UE release 15 and forward that don’t support Tx diversity.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_07, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_24, NS_27, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and Power Class 3 UE release 15 and forward that don’t support Tx diversity.

NOTE:	Test execution is not necessary if 6.5.2.3, 6.5.2.4.2 and 6.5.3.3 are executed.

6.2.3.3	Minimum conformance requirements

<<Unchanged sections skipped>>

6.2.3.3.29	A-MPR for NS_07

Table 6.2.3.3.29-1: A-MPR regions for NS_07

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 | >0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26, ≤ 3.42 | ≥ 1.8 | A3 |
|  |  | >3.42 | ≤ 0.36 | A4 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 6.48 | ≥ 3.24 | A3 |
|  |  | > 6.48 | ≤ 0.36 | A4 |

Table 6.2.3.3.29-2: A-MPR for NS_07

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM QPSK | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 16 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 64 QAM | 12 | 9 | 6 | 3 |
| DFT-s-OFDM 256 QAM | 12 | 9 | 6 | 3 |
| CP-OFDM QPSK | 14 | 10 | 7 | 3 |
| CP-OFDM 16 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 64 QAM | 14 | 10 | 7 | 3 |
| CP-OFDM 256 QAM | 14 | 10 | 7 | 3 |

<<Unchanged sections skipped>>

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

<<Unchanged tables skipped>>

Table 6.2.3.4.1-31: Test Configuration table for NS_07

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |  |  |
| A-MPR test parameters for NS_07 |  |  |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz |
| 1 | Low | Default | Default | N/A for A-MPR test cases | DFT-s OFDM | PI/2 BPSK |  | Edge_1RB_Left |  |
| 2 | Low | 5 MHz | Default |  |  | PI/2 BPSK |  | 8@6 | 4@3 |
| 3 | Low | 10 MHz | Default |  |  | PI/2 BPSK |  | 8@14 | 4@7 |
| 4 | Low | 5 MHz | Default |  |  | PI/2 BPSK |  | 10@8 | 5@4 |
| 5 | Low | 10 MHz | Default |  |  | PI/2 BPSK |  | 18@19 | 10@10 |
| 6 | High | 5 MHz | Default |  |  | PI/2 BPSK |  | 10@0 | 5@0 |
| 7 | Low | 5 MHz | Default |  |  | PI/2 BPSK |  | 2@20 | 1@10 |
| 8 | Low | 10 MHz | Default |  |  | PI/2 BPSK |  | 2@37 | 1@19 |
| 9 | Low | Default | Default |  |  | QPSK |  | Edge_1RB_Left |  |
| 10 | Low | 5 MHz | Default |  |  | QPSK |  | 8@6 | 4@3 |
| 11 | Low | 10 MHz | Default |  |  | QPSK |  | 8@14 | 4@7 |
| 12 | Low | 5 MHz | Default |  |  | QPSK |  | 10@8 | 5@4 |
| 13 | Low | 10 MHz | Default |  |  | QPSK |  | 18@19 | 10@10 |
| 14 | High | 5 MHz | Default |  |  | QPSK |  | 10@0 | 5@0 |
| 15 | Low | 5 MHz | Default |  |  | QPSK |  | 2@20 | 1@10 |
| 16 | Low | 10 MHz | Default |  |  | QPSK |  | 2@37 | 1@19 |
| 17 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |
| 18 | Low | 5 MHz | Default |  |  | 16 QAM |  | 8@6 | 4@3 |
| 19 | Low | 10 MHz | Default |  |  | 16 QAM |  | 8@14 | 4@7 |
| 20 | Low | 5 MHz | Default |  |  | 16 QAM |  | 10@8 | 5@4 |
| 21 | Low | 10 MHz | Default |  |  | 16 QAM |  | 18@19 | 10@10 |
| 22 | High | 5 MHz | Default |  |  | 16 QAM |  | 10@0 | 5@0 |
| 23 | Low | 5 MHz | Default |  |  | 16 QAM |  | 2@20 | 1@10 |
| 24 | Low | 10 MHz | Default |  |  | 16 QAM |  | 2@37 | 1@19 |
| 25 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |
| 26 | Low | 5 MHz | Default |  |  | 64 QAM |  | 8@6 | 4@3 |
| 27 | Low | 10 MHz | Default |  |  | 64 QAM |  | 8@14 | 4@7 |
| 28 | Low | 5 MHz | Default |  |  | 64 QAM |  | 10@8 | 5@4 |
| 29 | Low | 10 MHz | Default |  |  | 64 QAM |  | 18@19 | 10@10 |
| 30 | High | 5 MHz | Default |  |  | 64 QAM |  | 10@0 | 5@0 |
| 31 | Low | 5 MHz | Default |  |  | 64 QAM |  | 2@20 | 1@10 |
| 32 | Low | 10 MHz | Default |  |  | 64 QAM |  | 2@37 | 1@19 |
| 33 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |
| 34 | Low | 5 MHz | Default |  |  | 256 QAM |  | 8@6 | 4@3 |
| 35 | Low | 10 MHz | Default |  |  | 256 QAM |  | 8@14 | 4@7 |
| 36 | Low | 5 MHz | Default |  |  | 256 QAM |  | 10@8 | 5@4 |
| 37 | Low | 10 MHz | Default |  |  | 256 QAM |  | 18@19 | 10@10 |
| 38 | High | 5 MHz | Default |  |  | 256 QAM |  | 10@0 | 5@0 |
| 39 | Low | 5 MHz | Default |  |  | 256 QAM |  | 2@20 | 1@10 |
| 40 | Low | 10 MHz | Default |  |  | 256 QAM |  | 2@37 | 1@19 |
| 41 | Low | Default | Default |  | CP-s OFDM | QPSK |  | Edge_1RB_Left |  |
| 42 | Low | 5 MHz | Default |  |  | QPSK |  | 7@6 | 4@3 |
| 43 | Low | 10 MHz | Default |  |  | QPSK |  | 8@14 | 4@7 |
| 44 | Low | 5 MHz | Default |  |  | QPSK |  | 10@8 | 5@4 |
| 45 | Low | 10 MHz | Default |  |  | QPSK |  | 18@19 | 10@10 |
| 46 | High | 5 MHz | Default |  |  | QPSK |  | 10@0 | 5@0 |
| 47 | Low | 5 MHz | Default |  |  | QPSK |  | 2@20 | 1@10 |
| 48 | Low | 10 MHz | Default |  |  | QPSK |  | 2@37 | 1@19 |
| 49 | Low | Default | Default |  |  | 16 QAM |  | Edge_1RB_Left |  |
| 50 | Low | 5 MHz | Default |  |  | 16 QAM |  | 7@6 | 4@3 |
| 51 | Low | 10 MHz | Default |  |  | 16 QAM |  | 8@14 | 4@7 |
| 52 | Low | 5 MHz | Default |  |  | 16 QAM |  | 10@8 | 5@4 |
| 53 | Low | 10 MHz | Default |  |  | 16 QAM |  | 18@19 | 10@10 |
| 54 | High | 5 MHz | Default |  |  | 16 QAM |  | 10@0 | 5@0 |
| 55 | Low | 5 MHz | Default |  |  | 16 QAM |  | 2@20 | 1@10 |
| 56 | Low | 10 MHz | Default |  |  | 16 QAM |  | 2@37 | 1@19 |
| 57 | Low | Default | Default |  |  | 64 QAM |  | Edge_1RB_Left |  |
| 58 | Low | 5 MHz | Default |  |  | 64 QAM |  | 7@6 | 4@3 |
| 59 | Low | 10 MHz | Default |  |  | 64 QAM |  | 8@14 | 4@7 |
| 60 | Low | 5 MHz | Default |  |  | 64 QAM |  | 10@8 | 5@4 |
| 61 | Low | 10 MHz | Default |  |  | 64 QAM |  | 18@19 | 10@10 |
| 62 | High | 5 MHz | Default |  |  | 64 QAM |  | 10@0 | 5@0 |
| 63 | Low | 5 MHz | Default |  |  | 64 QAM |  | 2@20 | 1@10 |
| 64 | Low | 10 MHz | Default |  |  | 64 QAM |  | 2@37 | 1@19 |
| 65 | Low | Default | Default |  |  | 256 QAM |  | Edge_1RB_Left |  |
| 66 | Low | 5 MHz | Default |  |  | 256 QAM |  | 7@6 | 4@3 |
| 67 | Low | 10 MHz | Default |  |  | 256 QAM |  | 8@14 | 4@7 |
| 68 | Low | 5 MHz | Default |  |  | 256 QAM |  | 10@8 | 5@4 |
| 69 | Low | 10 MHz | Default |  |  | 256 QAM |  | 18@19 | 10@10 |
| 70 | High | 5 MHz | Default |  |  | 256 QAM |  | 10@0 | 5@0 |
| 71 | Low | 5 MHz | Default |  |  | 256 QAM |  | 2@20 | 1@10 |
| 72 | Low | 10 MHz | Default |  |  | 256 QAM |  | 2@37 | 1@19 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2.3.4.3.35	Message contents exceptions for network signalled value "NS_07"

1.	Information element additionalSpectrumEmission is set to NS_07. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.3.4.3.35-1: AdditionalSpectrumEmission: Additional spurious emissions test requirement for "NS_07"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 2 (NS_07) |  |  |

<<Unchanged sections skipped>>

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-38. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

<<Unchanged tables skipped>>

Table 6.2.3.5-37: UE Power Class 3 test requirements (NS_07) for band n13

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 2 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 3 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 4 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 5 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 6 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 7 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 8 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 9 | 23 | 1 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 10 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 11 | 23 | 0 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 12 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 13 | 23 | 0 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 14 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 15 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 16 | 23 | 0 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 17 | 23 | 2 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 18 | 23 | 1 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 19 | 23 | 1 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 20 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 21 | 23 | 1 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 22 | 23 | 2 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 23 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 24 | 23 | 1 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 25 | 23 | 2.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 26 | 23 | 2.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 27 | 23 | 2.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 19-TT |
| 28 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 29 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 30 | 23 | 2.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 31 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 32 | 23 | 2.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 33 | 23 | 4.5 | 12 | 0 | 11 | 6 | 3 | 25+TT | 5-TT |
| 34 | 23 | 4.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 35 | 23 | 4.5 | 9 | 0 | 14 | 5 | 3 | 25+TT | 9-TT |
| 36 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 37 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 38 | 23 | 4.5 | 6 | 0 | 17 | 5 | 3 | 25+TT | 12-TT |
| 39 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 40 | 23 | 4.5 | 3 | 0 | 18.5 | 4 | 3 | 25+TT | 14.5-TT |
| 41 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 42 | 23 | 1.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 43 | 23 | 1.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 44 | 23 | 1.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 45 | 23 | 1.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 46 | 23 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 47 | 23 | 1.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 48 | 23 | 1.5 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 49 | 23 | 3 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 50 | 23 | 2 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 51 | 23 | 2 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 52 | 23 | 2 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 53 | 23 | 2 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 54 | 23 | 3 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 55 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 56 | 23 | 2 | 3 | 0 | 20 | 2.5 | 3 | 25+TT | 17-TT |
| 57 | 23 | 3.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 58 | 23 | 3.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 59 | 23 | 3.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 60 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 61 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 62 | 23 | 3.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 63 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 64 | 23 | 3.5 | 3 | 0 | 19.5 | 3.5 | 3 | 25+TT | 16-TT |
| 65 | 23 | 6.5 | 14 | 0 | 9 | 6 | 3 | 25+TT | 3-TT |
| 66 | 23 | 6.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 67 | 23 | 6.5 | 10 | 0 | 13 | 5 | 3 | 25+TT | 8-TT |
| 68 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 69 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 70 | 23 | 6.5 | 7 | 0 | 16 | 5 | 3 | 25+TT | 11-TT |
| 71 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| 72 | 23 | 6.5 | 3 | 0 | 16.5 | 5 | 3 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

<<End of change>>