3GPP TSG-RAN5 Meeting #101 	R5-236810

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2516 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RB allocation of A-MPR for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test, NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Table 6.2.3.4.1-13, the RB allocation of the following Test ID for SCS 30 kHz and 60 kHz is incorrect because it exceeds NRB in Table 5.3.2-1. - Test ID 219, 259, 299, 339 |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Table 6.2.3.4.1-13, the RB allocation was corrected as below. - Test ID 219, 259, 299, 339 (SCS 30 kHz): 38@54 -> 24@54 - Test ID 219, 259, 299, 339 (SCS 60 kHz): 18@27 -> 11@27 Editorial correction: In Table 6.2.3.5-18, A-MPR = 0 was clarified in Test ID 179, 183, 185, 187. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR for NS_27 will not be tested correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_40, NS_09 not complete.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_07, NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

<<Unchanged sections skipped>>

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-30. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

<<Unchanged tables skipped>>

Table 6.2.3.4.1-13: Test Configuration table for NS_27 (contiguous allocation)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies |  |  |  |  |  |  | Refer to uplink carrier centre frequency (Fc) in test parameters. Associated parameters defined in Table 6.2.3.4.1-13a and 6.2.3.4.1-13b. |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Refer to test parameters (15, 20, 30, 40 MHz) |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest (Note 3) |  |  |  |
| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |
| 217 | 3565.02 | 30 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 218 | 3565.02 | 30 | Default |  |  | QPSK |  | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 219 | 3565.02 | 30 | Default |  |  | QPSK |  | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 220 | 3565.02 | 30 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 221 | 3684.99 | 30 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 222 | 3684.99 | 30 | Default |  |  | QPSK |  | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 223 | 3684.99 | 30 | Default |  |  | QPSK |  | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 224 | 3684.99 | 30 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 225 | 3585 | 30 | Default |  |  | QPSK |  | Edge_1RB_Left (A8) |  |  |
| 226 | 3585 | 30 | Default |  |  | QPSK |  | Outer_Full (4) |  |  |
| 227 | 3664.98 | 30 | Default |  |  | QPSK |  | Edge_1RB_Right (A8) |  |  |
| 228 | 3664.98 | 30 | Default |  |  | QPSK |  | Outer_Full (4) |  |  |
| 229 | 3570 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 230 | 3570 | 40 | Default |  |  | QPSK |  | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 231 | 3570 | 40 | Default |  |  | QPSK |  | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 232 | 3570 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A7) |  |  |
| 233 | 3679.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A7) |  |  |
| 234 | 3679.98 | 40 | Default |  |  | QPSK |  | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 235 | 3679.98 | 40 | Default |  |  | QPSK |  | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 236 | 3679.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A7) |  |  |
| 237 | 3600 | 40 | Default |  |  | QPSK |  | Edge_1RB_Left (A8) |  |  |
| 238 | 3600 | 40 | Default |  |  | QPSK |  | Outer_Full (4.5) |  |  |
| 239 | 3649.98 | 40 | Default |  |  | QPSK |  | Edge_1RB_Right (A8) |  |  |
| 240 | 3649.98 | 40 | Default |  |  | QPSK |  | Outer_Full (4.5) |  |  |
| 241 | 3557.52 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A3) |  |  |
| 242 | 3557.52 | 15 | Default |  |  | 16 QAM |  | Outer_Full (A3) |  |  |
| 243 | 3692.49 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A3) |  |  |
| 244 | 3692.49 | 15 | Default |  |  | 16 QAM |  | Outer_Full (A3) |  |  |
| 245 | 3562.5 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A4) |  |  |
| 246 | 3562.5 | 15 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 247 | 3687.48 | 15 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A4) |  |  |
| 248 | 3687.48 | 15 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |
| 249 | 3560.01 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A5) |  |  |
| 250 | 3560.01 | 20 | Default |  |  | 16 QAM |  | Outer_Full (A5) |  |  |
| 251 | 3690 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Right (A5) |  |  |
| 252 | 3690 | 20 | Default |  |  | 16 QAM |  | Outer_Full (A5) |  |  |
| 253 | 3570 | 20 | Default |  |  | 16 QAM |  | Edge_1RB_Left (A6) |  |  |
| 254 | 3570 | 20 | Default |  |  | 16 QAM |  | Outer_Full (2) |  |  |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 255 | 3679.98 | 20 | Default |  |  | 16 QAM | Edge_1RB_Right (A6) |  |  |
| 256 | 3679.98 | 20 | Default |  |  | 16 QAM | Outer_Full (2) |  |  |
| 257 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 258 | 3565.02 | 30 | Default |  |  | 16 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 259 | 3565.02 | 30 | Default |  |  | 16 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 260 | 3565.02 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 261 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 262 | 3684.99 | 30 | Default |  |  | 16 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 263 | 3684.99 | 30 | Default |  |  | 16 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 264 | 3684.99 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 265 | 3585 | 30 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 266 | 3585 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 267 | 3664.98 | 30 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 268 | 3664.98 | 30 | Default |  |  | 16 QAM | Outer_Full (4) |  |  |
| 269 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 270 | 3570 | 40 | Default |  |  | 16 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 271 | 3570 | 40 | Default |  |  | 16 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 272 | 3570 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 273 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A7) |  |  |
| 274 | 3679.98 | 40 | Default |  |  | 16 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 275 | 3679.98 | 40 | Default |  |  | 16 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 276 | 3679.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A7) |  |  |
| 277 | 3600 | 40 | Default |  |  | 16 QAM | Edge_1RB_Left (A8) |  |  |
| 278 | 3600 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 279 | 3649.98 | 40 | Default |  |  | 16 QAM | Edge_1RB_Right (A8) |  |  |
| 280 | 3649.98 | 40 | Default |  |  | 16 QAM | Outer_Full (4.5) |  |  |
| 281 | 3557.52 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A3) |  |  |
| 282 | 3557.52 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 283 | 3692.49 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A3) |  |  |
| 284 | 3692.49 | 15 | Default |  |  | 64 QAM | Outer_Full (A3) |  |  |
| 285 | 3562.5 | 15 | Default |  |  | 64 QAM | Edge_1RB_Left (A4) |  |  |
| 286 | 3562.5 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 287 | 3687.48 | 15 | Default |  |  | 64 QAM | Edge_1RB_Right (A4) |  |  |
| 288 | 3687.48 | 15 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 289 | 3560.01 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A5) |  |  |
| 290 | 3560.01 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 291 | 3690 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A5) |  |  |
| 292 | 3690 | 20 | Default |  |  | 64 QAM | Outer_Full (A5) |  |  |
| 293 | 3570 | 20 | Default |  |  | 64 QAM | Edge_1RB_Left (A6) |  |  |
| 294 | 3570 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 295 | 3679.98 | 20 | Default |  |  | 64 QAM | Edge_1RB_Right (A6) |  |  |
| 296 | 3679.98 | 20 | Default |  |  | 64 QAM | Outer_Full (2) |  |  |
| 297 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 298 | 3565.02 | 30 | Default |  |  | 64 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 299 | 3565.02 | 30 | Default |  |  | 64 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 300 | 3565.02 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 301 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 302 | 3684.99 | 30 | Default |  |  | 64 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 303 | 3684.99 | 30 | Default |  |  | 64 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 304 | 3684.99 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 305 | 3585 | 30 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 306 | 3585 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 307 | 3664.98 | 30 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 308 | 3664.98 | 30 | Default |  |  | 64 QAM | Outer_Full (4) |  |  |
| 309 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 310 | 3570 | 40 | Default |  |  | 64 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 311 | 3570 | 40 | Default |  |  | 64 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 312 | 3570 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 313 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A7) |  |  |
| 314 | 3679.98 | 40 | Default |  |  | 64 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 315 | 3679.98 | 40 | Default |  |  | 64 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |

| A-MPR test parameters for NS_27 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test ID | Fc (MHz) | Ch BW (MHz) | SCS (kHz) | Downlink Configuration | Uplink Configuration |  |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |  |  |
|  |  |  |  |  |  |  | SCS 15 kHz | SCS 30 kHz | SCS 60 kHz |
| 316 | 3679.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A7) |  |  |
| 317 | 3600 | 40 | Default |  |  | 64 QAM | Edge_1RB_Left (A8) |  |  |
| 318 | 3600 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 319 | 3649.98 | 40 | Default |  |  | 64 QAM | Edge_1RB_Right (A8) |  |  |
| 320 | 3649.98 | 40 | Default |  |  | 64 QAM | Outer_Full (4.5) |  |  |
| 321 | 3557.52 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 322 | 3557.52 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 323 | 3692.49 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 324 | 3692.49 | 15 | Default |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 325 | 3562.5 | 15 | Default |  |  | 256 QAM | Edge_1RB_Left (A4) |  |  |
| 326 | 3562.5 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 327 | 3687.48 | 15 | Default |  |  | 256 QAM | Edge_1RB_Right (A4) |  |  |
| 328 | 3687.48 | 15 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 329 | 3560.01 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A5) |  |  |
| 330 | 3560.01 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 331 | 3690 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 332 | 3690 | 20 | Default |  |  | 256 QAM | Outer_Full (A5) |  |  |
| 333 | 3570 | 20 | Default |  |  | 256 QAM | Edge_1RB_Left (A6) |  |  |
| 334 | 3570 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 335 | 3679.98 | 20 | Default |  |  | 256 QAM | Edge_1RB_Right (A6) |  |  |
| 336 | 3679.98 | 20 | Default |  |  | 256 QAM | Outer_Full (2) |  |  |
| 337 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 338 | 3565.02 | 30 | Default |  |  | 256 QAM | 108@41 (A2) | 54@21 (A2) | 25@11 (A2) |
| 339 | 3565.02 | 30 | Default |  |  | 256 QAM | 74@41 (A1) | 24@54 (A1) | 11@27 (A1) |
| 340 | 3565.02 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 341 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 342 | 3684.99 | 30 | Default |  |  | 256 QAM | 108@0 (A2) | 54@0 (A2) | 27@0 (A2) |
| 343 | 3684.99 | 30 | Default |  |  | 256 QAM | 81@0 (A1) | 40@0 (A1) | 20@0 (A1) |
| 344 | 3684.99 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 345 | 3585 | 30 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 346 | 3585 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 347 | 3664.98 | 30 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 348 | 3664.98 | 30 | Default |  |  | 256 QAM | Outer_Full (4) |  |  |
| 349 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 350 | 3570 | 40 | Default |  |  | 256 QAM | 153@63 (A2) | 72@32 (A2) | 32@16 (A2) |
| 351 | 3570 | 40 | Default |  |  | 256 QAM | 99@69 (A1) | 49@34 (A1) | 24@16 (A1) |
| 352 | 3570 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 353 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A7) |  |  |
| 354 | 3679.98 | 40 | Default |  |  | 256 QAM | 137@0 (A2) | 68@0 (A2) | 35@0 (A2) |
| 355 | 3679.98 | 40 | Default |  |  | 256 QAM | 99@38 (A1) | 49@18 (A1) | 24@9 (A1) |
| 356 | 3679.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A7) |  |  |
| 357 | 3600 | 40 | Default |  |  | 256 QAM | Edge_1RB_Left (A8) |  |  |
| 358 | 3600 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| 359 | 3649.98 | 40 | Default |  |  | 256 QAM | Edge_1RB_Right (A8) |  |  |
| 360 | 3649.98 | 40 | Default |  |  | 256 QAM | Outer_Full (4.5) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: For FR1 bands where highest supported SCS is 60 kHz the highest tested SCS is limited to 30 kHz as carrier with SCS=60 kHz cannot be used as PCell. |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable Table from table 6.2.3.5-1 to Table 6.2.3.5-38. The allowed A-MPR values specified in Table 6.2.3.3.1-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.1.3-1 apply.

<<Unchanged tables skipped>>

Table 6.2.3.5-18: UE Power Class 3 test requirements (NS_27)

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |
| 179 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 180 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 181 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 182 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5.0 | 3 | 25+TT | 12-TT |
| 183 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 184 | 23 | 0 | 4.5 | 11 | 0 | 12 | 6.0 | 3 | 25+TT | 6-TT |
| 185 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 186 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| 187 | 23 | 0 | 4.5 | 0 | 0 | 18.5 | 4.0 | 3 | 25+TT | 14.5-TT |
| <<Unchanged rows skipped>> |  |  |  |  |  |  |  |  |  |  |

<<End of change>>