3GPP TSG-RAN5 Meeting #101 	R5-236815

# Chicago, USA, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-3 | CR | 1701 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to message contents for p-Max in Out-of-band blocking and Spurious response for intra-band EN-DC with FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu, Eurofins KCTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TS 38.101-3, Out-of-band blocking and Spurious response test cases for FR1 EN-DC, the UL gap between LTE and NR is defined as 25 dB (29 dB below PCMAX_L and 4 dB below PCMAX_L) to solve the testability issue due to the ADC dynamic range limitation of TE. However, according to the current test specifications that refer to common RF test in TS 36.508, LTE p-Max is defined as 10 dBm, and it causes the UL gap to be 38 dB (NR: 19 dBm = 23 dBm – 4 dB, LTE: -19 dBm = 10 dBm – 29 dB). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added p-Max configurations to message contents |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testability will not be certified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6B.3.3, 7.6B.3.3_1.1, 7.6B.3.3_1.2, 7.7B.3, 7.7B.3_1.1, 7.7B.3_1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

7.6B.3.3	Out-of-band blocking for inter-band EN-DC within FR1 (2 CCs)

<<Unchanged sections skipped>>

7.6B.3.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 with the following exceptions.

Table 7.6B.3.3.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.6B.3.3.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.6B.3.3.5	Test Requirement

<<Unchanged sections skipped>>

7.6B.3.3_1	Out-of-band blocking for EN-DC within FR1 (>2 CCs)

7.6B.3.3_1.1	Out-of-band blocking for EN-DC within FR1 (3 CCs)

<<Unchanged sections skipped>>

7.6B.3.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.6B.3.3_1.1.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.6B.3.3_1.1.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.6B.3.3_1.1.5	Test Requirement

<<Unchanged sections skipped>>

7.6B.3.3_1.2	Out-of-band blocking for EN-DC within FR1 (4 CCs)

<<Unchanged sections skipped>>

7.6B.3.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.6B.3.3_1.2.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.6B.3.3_1.2.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.6B.3.3_1.2.5	Test Requirement

<<Unchanged sections skipped>>

7.7B.3	Spurious Response for inter-band EN-DC within FR1 (2 CCs)

<<Unchanged sections skipped>>

7.7B.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 with the following exceptions.

Table 7.7B.3.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.7B.3.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.7B.3.5	Test Requirement

<<Unchanged sections skipped>>

7.7B.3_1	Spurious Response for EN-DC within FR1 (>2 CCs)

7.7B.3_1.1	Spurious Response for EN-DC within FR1 (3 CCs)

<<Unchanged sections skipped>>

7.7B.3_1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.7B.3_1.1.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.7B.3_1.1.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.7B.3_1.1.5	Test Requirement

<<Unchanged sections skipped>>

7.7B.3_1.2	Spurious Response for EN-DC within FR1 (4 CCs)

<<Unchanged sections skipped>>

7.7B.3_1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6 Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED with the following exceptions.

Table 7.7B.3_1.2.4.3-1: PhysicalCellGroupConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| p-NR-FR1 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |

Table 7.7B.3_1.2.4.3-2: RRCConnectionReconfiguration

| Derivation Path: TS 36.508 [11], Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionReconfiguration-v1530-IEs |  |
| p-MaxUE-FR1-r15 | 23 |  | Power Class 3 UE AND pc_dynamicPowerSharing |
|  | 20 |  | Power Class 3 UE AND NOT pc_dynamicPowerSharing |
|  | 26 |  | Power Class 2 UE AND pc_dynamicPowerSharing |
|  | 23 |  | Power Class 2 UE AND NOT pc_dynamicPowerSharing |
| } |  |  |  |

7.7B.3_1.2.5	Test Requirement

<<End of change>>