3GPP TSG RAN WG5 Meeting #101 	R5-236833

Chicago, USA, 13th – 17th November 2023

Source:				Apple Inc

Title:					TP to TR 38.870 Annex B on changes to MU tables

Agenda Item:		5.3.22.2

Document for:	Discussion and Endorsement

# Introduction

The work item led by RAN4 for NR FR1 TRP-TRS enhancements was approved [1] during the September 2022 plenary meeting, revised [2] in December 2022, and includes a secondary responsibility for RAN5 to develop the Measurement Uncertainty (MU) assessment.

The aim of this contribution is to introduce changes to the preliminary Measurement Uncertainty tables for Annex B of TR 38.870 [3] to reflect the updates based on the analysis made for TS 38.561 [4] and in [5][6].

It is understood that RAN5 will finalize the MU estimation following the performance requirements definition in a future update of TS 38.561 [4].

Proposal 1: Endorse the text proposal below.

# Conclusion

The following proposal is made in this contribution:

Proposal 1: Endorse the text proposal below.

# References

- RP-222669, “New WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies”, vivo, 3GPP TSG RAN Meeting #97-e, September 2022

- RP-223112, “Revised WID: Enhancement of UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)”, vivo, 3GPP TSG RAN Meeting #98-e, December 2022

- 3GPP TR 38.870, v0.4.0 (2023-05)

- 3GPP TS 38.561, v0.3.0 (2023-06)

- R5-234555,” AC MU Analysis for NR FR1 TRP-TRS (Rel.17)”, Rohde & Schwarz, 3GPP TSG RAN5 Meeting #100, August 2023

- R5-234556, “TP to TS 38.561 on MU contents”, Rohde & Schwarz, 3GPP TSG RAN5 Meeting #100, August 2023

# Appendix – Text Proposal to TR 38.870 v0.6.0

< Unchanged Text Skipped>

< Beginning of Changes >

## B.2.12	Coarse sampling grid

This contributor describes the uncertainty of the measured TRP/TRS value due to the finite number of measurement grid points. Decreasing of sampling density to finite number of samples affects the measurement uncertainty by two different errors. First is due to inadequate number of samples and second is a systematic discrimination approximation error in TRP and TRS equations. Different TRP quadratures also have an effect on the MU.

Until further analysis of standard deviation based on adequate measurement results samples considering  radiation patterns with high directivity, e.g.: > 6dBi. The adoption of coarse sampling grid is FFS.

The legacy TRP grid of 15 / and TRS grid of 30 /, remains the reference test grid for TRP and TRS measurements.

< Unchanged Text skipped >

# B.4	MU Assessment for TRP

## B.4.1	MU Assessment for TRP in Anechoic Chamber

The uncertainty contributions related to TRP are listed in Table B.4.1-1. A preliminary example uncertainty budgets are presented in Table B.4.1-2 and Table B.4.1-3.

Table B.4.1-1 Uncertainty contributions in TRP measurement for anechoic chamber method

| UID | Description of uncertainty contribution | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of receiver chain | B.2.1 |
| 2 | Insertion loss of receiver chain | B.2.2 |
| 3 | Influence of the measurement antenna cable | B.2.3 |
| 4 | Measurement Receiver: uncertainty of the absolute level | B.2.4 |
| 5 | Measurement distance | B.2.7 |
| 6 | Quality of quiet zone | B.2.8 |
| 7 | DUT Tx-power drift | B.2.9 |
| 8 | Uncertainty related to the use of phantoms | B.2.11 |
| 9 | Coarse sampling grid | B.2.12 |
| 10 | Random uncertainty | B.2.13 |
| 11 | Frequency Response | B.2.14 |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 12 | Uncertainty of network analyzer | B.2.15 |
| 13 | Mismatch of receiver chain | B.2.1 |
| 14 | Insertion loss of receiver chain | B.2.2 |
| 15 | Mismatch in the connection of calibration antenna | B.2.1 |
| 16 | Influence of the calibration antenna feed cable | B.2.3 |
| 17 | Influence of the measurement antenna cable | B.2.3 |
| 18 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | B.2.16 |
| 19 | Measurement distance | B.2.7 |
| 20 |  | Quality of the Quiet Zone |  | B.2.8 |  |
| Systematic Errors |  |  |  |  |  |
| 22 |  | Systematic Error related to TRP grids |  | B.2.12 |  |

Table B.4.1-2 Preliminary example of uncertainty budget for TRP hand only (browsing mode) measurement for anechoic chamber method for NR FR1 bands

| UID | Uncertainty Source | Comment | Uncertainty Value [dB] |  | Prob Distr | Div | ci | Standard Uncertainty [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Below 3GHz |  |  |  | Above 3GHz |  |  |  |  |  |  |  | Below 3GHz |  |  | Above 3GHz |  |  |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | Mismatch of receiver chain |  | Гreceiver < 0.33 Гmeasurement antenna < 0.5 Cable attenuation > 3dB | 0.26 |  | 0.26 |  |  |  | U-shaped |  | 1.4 |  | 1 |  | 0.18 |  |  | 0.18 |  |  |
| 2 |  | Insertion loss of receiver chain |  | Systematic with Stage 1 (=> cancels) | 0 |  | 0 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| 3 |  | Influence of the measurement antenna cable |  | Systematic with Stage 1 (=> cancels) | 0 |  | 0 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| 4 |  | Measurement Receiver: uncertainty of the absolute level |  | Spectrum Analyzer | 0.42 |  | 0.54 |  |  |  | Normal |  | 2 |  | 1 |  | 0.21 |  |  | 0.27 |  |  |
| 5 |  | Measurement distance |  | DUT is not offset for hand-only phantom testing | 0 |  | 0 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| 6 |  | Quality of quiet zone |  | Surface standard deviation of power measurements in ripple test | 0.5 |  | 0.5 |  |  |  | Actual |  | 1 |  | 1 |  | 0.50 |  |  | 0.50 |  |  |
| 7 |  | DUT Tx-power drift |  | Drift | 0.2 |  | 0.2 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.12 |  |  | 0.12 |  |  |
| 8 |  | Uncertainty related to the use of phantoms |  | Material Dielectric Constant, Material Conductivity, Geometry/Shape (incl. spacer), Data Mode Fixture | 0.64 |  | 0.64 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.37 |  |  | 0.37 |  |  |
| 9 |  | Coarse sampling grid |  | Sampling grids per Table Table B.2.12-1 | 0 |  | 0.11 |  |  |  | Actual |  | 1 |  | 1 |  | 0.00 |  |  | 0.11 |  |  |
| 10 |  | Random Uncertainty |  | Fixed MU to account for all the unknown, unquantifiable, etc. uncertainties | 0.25 |  | 0.25 |  |  |  | Normal |  | 2 |  | 1 |  | 0.13 |  |  | 0.13 |  |  |
| 11 |  | Frequency Response |  | Average path loss corrected | 0 |  | 0 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |  |  |  |  |  |  |
| 12 |  | Uncertainty of network analyzer |  | From datasheet of VNA with assessed transmission coefficients | [0.2] |  | [0.5] |  |  |  | Normal |  | 2 |  | 1 |  | [0.10] |  |  | [0.25] |  |  |
| 13 |  | Mismatch of receiver chain |  | Taken into account in VNA uncertainty term | 0 |  | 0 |  |  |  | U-shaped |  | 1.4 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| 14 |  | Insertion loss of receiver chain |  | Systematic with Stage 2 (=> cancels) | 0 |  | 0 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| 15 |  | Mismatch in the connection of calibration antenna |  | Taken in to account in VNA setup uncertainty | 0 |  | 0 |  |  |  | U-shaped |  | 1.4 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| 16 |  | Influence of the calibration antenna feed cable |  | Gain calibration with a dipole | 0.3 |  | 0.3 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.17 |  |  | 0.17 |  |  |
| 17 |  | Influence of the measurement antenna cable |  | Systematic with Stage 2 (=> cancels) | 0 |  | 0 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| 18 |  | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna |  | Calibration report with traceability to a National Metrology Institute | [0.58] |  | [0.58] |  |  |  | Normal |  | 2 |  | 1 |  | [0.29] |  |  | [0.29] |  |  |
| 19 |  | Measurement distance |  | Dipole: aligned with phase center | 0 |  | 0 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.00 |  |  | 0.00 |  |  |
| 20 |  | Quality of the Quiet Zone |  | Peak-to-null ripple | 0.5 |  | 0.5 |  |  |  | Rectangular |  | 1.7 |  | 1 |  | 0.29 |  |  | 0.29 |  |  |
| Combined standard uncertainty [dB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [0.84] |  |  | [0.88] |  |  |  |
| Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [1.64] |  |  | [1.73] |  |  |  |
| 21 | Systematic Error related to TRP grids | mean error | 0 | 0 | Actual | 1 | 1 | FFS1 | FFS1 |
| Total Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [1.64] |  |  | [1.73] |  |  |  |
| NOTE 1: The values may be updated after further analysis as detailed in Annex A.4.2.12. TRP grid of 15 / and TRS grid of 30 /, remains the reference test grid for TRP and TRS measurements. |  |  |  |  |  |  |  |  |  |

Table B.4.1-3 Preliminary example of uncertainty budget for TRP Beside Head and Hand (speech mode) measurement for anechoic chamber method for NR FR1 bands

| UID | Uncertainty Source | Comment | Uncertainty Value [dB] |  | Prob Distr | Div | ci | Standard Uncertainty [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Below 3GHz | Above 3GHz |  |  |  | Below 3GHz | Above 3GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |
| 1 | Mismatch of receiver chain | Гreceiver < 0.33 Гmeasurement antenna < 0.5 Cable attenuation > 3dB | 0.26 | 0.26 | U-shaped | 1.4 | 1 | 0.18 | 0.18 |
| 2 | Insertion loss of receiver chain | Systematic with Stage 1 (=> cancels) | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 3 | Influence of the measurement antenna cable | Systematic with Stage 1 (=> cancels) | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 4 | Measurement Receiver: uncertainty of the absolute level | Spectrum Analyzer | 0.42 | 0.54 | Normal | 2 | 1 | 0.21 | 0.27 |
| 5 | Measurement distance | d=1.6m, Δd=0.05m | 0.27 | 0.27 | Rectangular | 1.7 | 1 | 0.16 | 0.16 |
| 6 | Quality of quiet zone | Surface standard deviation of power measurements in ripple test | 0.5 | 0.5 | Actual | 1 | 1 | 0.50 | 0.50 |
| 7 | DUT Tx-power drift | Drift | 0.2 | 0.2 | Rectangular | 1.7 | 1 | 0.12 | 0.12 |
| 8 | Uncertainty related to the use of phantoms | Material Dielectric Constant, Material Conductivity, Geometry/Shape (incl. spacer), Beside Head and Hand | 0.99 | 0.99 | Rectangular | 1.7 | 1 | 0.57 | 0.57 |
| 9 | Coarse sampling grid | Sampling grids per Table Table B.2.12-1 | FFS1 | FFS1 | Actual | 1 | 1 | FFS1 | FFS1 |
| 10 | Random Uncertainty | Fixed MU to account for all the unknown, unquantifiable, etc. uncertainties | 0.25 | 0.25 | Normal | 2 | 1 | 0.13 | 0.13 |
| 11 | Frequency Response | Average path loss corrected | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |  |  |  |  |  |  |
| 12 | Uncertainty of network analyzer | From datasheet of VNA with assessed transmission coefficients | [0.2] | [0.5] | Normal | 2 | 1 | [0.10] | [0.25] |
| 13 | Mismatch of receiver chain | Taken into account in VNA uncertainty term | 0 | 0 | U-shaped | 1.4 | 1 | 0.00 | 0.00 |
| 14 | Insertion loss of receiver chain | Systematic with Stage 2 (=> cancels) | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 15 | Mismatch in the connection of calibration antenna | Taken in to account in VNA setup uncertainty | 0 | 0 | U-shaped | 1.4 | 1 | 0.00 | 0.00 |
| 16 | Influence of the calibration antenna feed cable | Gain calibration with a dipole | 0.3 | 0.3 | Rectangular | 1.7 | 1 | 0.17 | 0.17 |
| 17 | Influence of the measurement antenna cable | Systematic with Stage 2 (=> cancels) | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 18 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | Calibration report with traceability to a National Metrology Institute | [0.58] | [0.58] | Normal | 2 | 1 | [0.29] | [0.29] |
| 19 | Measurement distance | Dipole: aligned with phase center | 0 | 0 | Rectangular | 1.7 | 1 | 0.00 | 0.00 |
| 20 | Quality of the Quiet Zone | Peak-to-null ripple | 0.5 | 0.5 | Rectangular | 1.7 | 1 | 0.29 | 0.29 |
| Combined standard uncertainty [dB] |  |  |  |  |  |  |  | [0.96] | [1.00] |
| Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  | [1.87] | [1.96] |
| 21 | Systematic Error related to TRP grids | mean error | FFS1 | FFS1 | Actual | 1 | 1 | FFS1 | FFS1 |
| Total Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  | [1.87] | [1.97] |
| NOTE 1: The values may be updated after further analysis as detailed in Annex A.4.2.12. TRP grid of 15 / and TRS grid of 30 /, remains the reference test grid for TRP and TRS measurements. |  |  |  |  |  |  |  |  |  |

< Unchanged Text skipped >

# B.5	MU Assessment for TRS

## B.5.1	MU Assessment for TRS in Anechoic Chamber

The uncertainty contributions related to TRS are listed in Table B.5.1-1. A preliminary example uncertainty budgets are presented in Table B.5.1-2 and Table B.5.1-3.

Table B.5.1-1 Uncertainty contributions in TRS measurement for anechoic chamber method

| UID | Description of uncertainty contribution | Details in clause |
| --- | --- | --- |
| Stage 2: DUT measurement (Figure 7.2-1, Figure 7.2-2) |  |  |
| 1 | Mismatch of transmitter chain | B.2.1 |
| 2 | Insertion loss of transmitter chain | B.2.2 |
| 3 | Influence of the measurement antenna cable | B.2.3 |
| 4 | Communication Tester: uncertainty of the absolute output level | B.2.5 |
| 5 | Sensitivity measurement: output level step resolution | B.2.6 |
| 6 | Measurement distance | B.2.7 |
| 7 | Quality of quiet zone | B.2.8 |
| 8 | DUT sensitivity drift | B.2.10 |
| 9 | Uncertainty related to the use of phantoms | B.2.11 |
| 10 | Coarse sampling grid | B.2.12 |
| 11 | Random uncertainty | B.2.13 |
| 12 | Frequency Response | B.2.14 |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 13 | Uncertainty of network analyzer | B.2.15 |
| 14 | Mismatch of transmitter chain | B.2.1 |
| 15 | Insertion loss of transmitter chain | B.2.2 |
| 16 | Mismatch in the connection of calibration antenna | B.2.1 |
| 17 | Influence of the calibration antenna feed cable | B.2.3 |
| 18 | Influence of the measurement antenna cable | B.2.3 |
| 19 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | B.2.16 |
| 20 | Measurement distance | B.2.7 |
| 21 |  | Quality of quiet zone |  | B.2.8 |  |
| Systematic Errors |  |  |  |  |  |
| 22 |  | Systematic Error related to TRS grids |  | B.2.12 |  |

Table B.5.1-2: Preliminary example of uncertainty budget for TRS hand only (browsing mode) measurement for anechoic chamber method for NR FR1 bands

| UID | Uncertainty Source | Comment | Uncertainty Value [dB] |  | Prob Distr |  | Div |  | ci |  | Standard Uncertainty [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Below 3GHz | Above 3GHz |  |  |  |  |  |  | Below 3GHz | Above 3GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Mismatch of receiver chain | Гcomm tester < 0.29 Гmeasurement antenna < 0.5 Cable attenuation > 3dB | 0.22 | 0.22 | U-shaped |  | 1.4 |  | 1 |  | 0.16 | 0.16 |
| 2 | Insertion loss of receiver chain | Systematic with Stage 1 (=> cancels) | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 3 | Influence of the measurement antenna cable | Systematic with Stage 1 (=> cancels) | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 4 | Communication Tester: uncertainty of the absolute output level | Manufacturer’s data sheet | [1.16] | [1.16] | Normal |  | 2 |  | 1 |  | [0.58] | [0.58] |
| 5 | Sensitivity measurement: output level step resolution | Systematic error that can be corrected | 0 | 0 | Rectangular |  | 1.73 |  | 1 |  | 0.00 | 0.00 |
| 6 | Measurement distance | DUT is not offset for hand-only phantom testing | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 7 | Quality of quiet zone | Surface standard deviation of power measurements in ripple test | 0.5 | 0.5 | Actual |  | 1 |  | 1 |  | 0.50 | 0.50 |
| 8 | DUT sensitivity drift | Drift | 0.2 | 0.2 | Rectangular |  | 1.7 |  | 1 |  | 0.12 | 0.12 |
| 9 | Uncertainty related to the use of phantoms | Material Dielectric Constant, Material Conductivity, Geometry/Shape (incl. spacer), Data Mode Fixture | 0.64 | 0.64 | Rectangular |  | 1.7 |  | 1 |  | 0.37 | 0.37 |
| 10 | Coarse sampling grid | Sampling grids per Table Table B.2.12-1 | FFS1 | FFS1 | Actual |  | 1 |  | 1 |  | FFS1 | FFS1 |
| 11 | Random Uncertainty | Fixed MU to account for all the unknown, unquantifiable, etc. uncertainties including digital error rate | 0.4 | 0.4 | Normal |  | 2 |  | 1 |  | 0.20 | 0.20 |
| 12 | Frequency Response | Included in the output level step resolution | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Uncertainty of network analyzer | From datasheet of VNA with assessed transmission coefficients | [0.2] | [0.5] | Normal |  | 2 |  | 1 |  | [0.10] | [0.25] |
| 14 | Mismatch of receiver chain | Taken into account in VNA uncertainty term | 0 | 0 | U-shaped |  | 1.4 |  | 1 |  | 0.00 | 0.00 |
| 15 | Insertion loss of receiver chain | Systematic with Stage 2 (=> cancels) | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 16 | Mismatch in the connection of calibration antenna | Taken in to account in VNA setup uncertainty | 0 | 0 | U-shaped |  | 1.4 |  | 1 |  | 0.00 | 0.00 |
| 17 | Influence of the calibration antenna feed cable | Gain calibration with a dipole | 0.3 | 0.3 | Rectangular |  | 1.7 |  | 1 |  | 0.17 | 0.17 |
| 18 | Influence of the measurement antenna cable | Systematic with Stage 2 (=> cancels) | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 19 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | Calibration report with traceability to a National Metrology Institute | [0.58] | [0.58] | Normal |  | 2 |  | 1 |  | [0.29] | [0.29] |
| 20 | Measurement distance | Dipole: aligned with phase center | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 21 | Quality of the Quiet Zone | Peak-to-null ripple | 0.5 | 0.5 | Rectangular |  | 1.7 |  | 1 |  | 0.29 | 0.29 |
| Combined standard uncertainty [dB] |  |  |  |  |  |  |  |  |  |  | [0.98] | [1.03] |
| Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  |  |  |  | [1.92] | [2.02] |
| 22 | Systematic Error related to TRP grids | mean error | FFS1 | FFS1 |  | Actual |  | 1 |  | 1 | FFS1 | FFS1 |
| Total Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  |  |  |  | [1.92] | [2.02] |
| NOTE 1: The values may be updated after further analysis as detailed in Annex A.4.2.12. TRP grid of 15 / and TRS grid of 30 /, remains the reference test grid for TRP and TRS measurements. |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.5.1-3: Preliminary example of uncertainty budget for TRS Beside Head and Hand (Talk mode) measurement for anechoic chamber method for NR FR1 bands

| UID | Uncertainty Source | Comment | Uncertainty Value [dB] |  | Prob Distr |  | Div |  | ci |  | Standard Uncertainty [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Below 3GHz | Above 3GHz |  |  |  |  |  |  | Below 3GHz | Above 3GHz |
| Stage 2: DUT measurement |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Mismatch of receiver chain | Гcomm tester < 0.29 Гmeasurement antenna < 0.5 Cable attenuation > 3dB | 0.22 | 0.22 | U-shaped |  | 1.4 |  | 1 |  | 0.16 | 0.16 |
| 2 | Insertion loss of receiver chain | Systematic with Stage 1 (=> cancels) | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 3 | Influence of the measurement antenna cable | Systematic with Stage 1 (=> cancels) | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 4 | Communication Tester: uncertainty of the absolute output level | Manufacturer’s data sheet | [1.16] | [1.16] | Normal |  | 2 |  | 1 |  | [0.58] | [0.58] |
| 5 | Sensitivity measurement: output level step resolution | Systematic error that can be corrected | 0 | 0 | Rectangular |  | 1.73 |  | 1 |  | 0.00 | 0.00 |
| 6 | Measurement distance | d=1.6m, Δd=0.05m | 0.27 | 0.27 | Rectangular |  | 1.7 |  | 1 |  | 0.16 | 0.16 |
| 7 | Quality of quiet zone | Surface standard deviation of power measurements in ripple test | 0.5 | 0.5 | Actual |  | 1 |  | 1 |  | 0.50 | 0.50 |
| 8 | DUT sensitivity drift | Drift | 0.2 | 0.2 | Rectangular |  | 1.7 |  | 1 |  | 0.12 | 0.12 |
| 9 | Uncertainty related to the use of phantoms | Material Dielectric Constant, Material Conductivity, Geometry/Shape (incl. spacer), Data Mode Fixture | 0.99 | 0.99 | Rectangular |  | 1.7 |  | 1 |  | 0.57 | 0.57 |
| 10 | Coarse sampling grid | Sampling grids per Table Table B.2.12-1 | FFS1 | FFS1 | Actual |  | 1 |  | 1 |  | FFS1 | FFS1 |
| 11 | Random Uncertainty | Fixed MU to account for all the unknown, unquantifiable, etc. uncertainties including digital error rate | 0.4 | 0.4 | Normal |  | 2 |  | 1 |  | 0.20 | 0.20 |
| 12 | Frequency Response | Included in the output level step resolution | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| Stage 1: Calibration measurement, network analyzer method |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Uncertainty of network analyzer | From datasheet of VNA with assessed transmission coefficients | [0.2] | [0.5] | Normal |  | 2 |  | 1 |  | [0.10] | [0.25] |
| 14 | Mismatch of receiver chain | Taken into account in VNA uncertainty term | 0 | 0 | U-shaped |  | 1.4 |  | 1 |  | 0.00 | 0.00 |
| 15 | Insertion loss of receiver chain | Systematic with Stage 2 (=> cancels) | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 16 | Mismatch in the connection of calibration antenna | Taken in to account in VNA setup uncertainty | 0 | 0 | U-shaped |  | 1.4 |  | 1 |  | 0.00 | 0.00 |
| 17 | Influence of the calibration antenna feed cable | Gain calibration with a dipole | 0.3 | 0.3 | Rectangular |  | 1.7 |  | 1 |  | 0.17 | 0.17 |
| 18 | Influence of the measurement antenna cable | Systematic with Stage 2 (=> cancels) | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 19 | Uncertainty of the absolute gain/ radiation efficiency of the calibration antenna | Calibration report with traceability to a National Metrology Institute | [0.58] | [0.58] | Normal |  | 2 |  | 1 |  | [0.29] | [0.29] |
| 20 | Measurement distance | Dipole: aligned with phase center | 0 | 0 | Rectangular |  | 1.7 |  | 1 |  | 0.00 | 0.00 |
| 21 | Quality of the Quiet Zone | Peak-to-null ripple | 0.5 | 0.5 | Rectangular |  | 1.7 |  | 1 |  | 0.29 | 0.29 |
| Combined standard uncertainty [dB] |  |  |  |  |  |  |  |  |  |  | [1.11] | [1.13] |
| Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  |  |  |  | [2.17] | [2.21] |
| 22 | Systematic Error related to TRP grids | mean error | FFS1 | FFS1 |  | Actual |  | 1 |  | 1 | FFS1 | FFS1 |
| Total Expanded uncertainty [dB] (Confidence interval of 95 %) |  |  |  |  |  |  |  |  |  |  |  |  | [2.17] |  | [2.21] |
| NOTE 1: The values may be updated after further analysis as detailed in Annex A.4.2.12. TRP grid of 15 / and TRS grid of 30 /, remains the reference test grid for TRP and TRS measurements. |  |  |  |  |  |  |  |  |  |  |  |  |

< End of Changes >