3GPP TSG-RAN5 Meeting #101	R5-236839

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 4119 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test case 8.1.6.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-UEConTest, TEI17_Test |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TP for 8.1.6.4.3 is “UE transmits UEInformationResponse message with RA-Report including msgA_RO-FrequencyStart and msgA-RO-FDM and msgA-SubcarrierSpacing set to values used”,but the values were not specified in Table 8.1.6.4.3.3.3-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating Table 8.1.6.4.3.3.3-3: UEInformationResponse to check the TP :‘UE transmits UEInformationResponse message with RA-Report including msgA_RO-FrequencyStart and msgA-RO-FDM and msgA-SubcarrierSpacing set to values used’. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 8.1.6.4.3	SON / RACH logging and reporting / 2-step RACH report

8.1.6.4.3.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supports delivery of rachReport and no contention resolution failure occurred for the last successfully completed 2 step random access procedure }

ensure that {
  when { UE receives UEInformationRequest message with ra-ReportReq set to true }

then { UE transmits UEInformationResponse message with RA-Report including msgA_RO-FrequencyStart and msgA-RO-FDM and msgA-SubcarrierSpacing set to values used}

}

8.1.6.4.3.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.7.10.3, 5.7.10.4, 5.7.10.5. Unless otherwise stated these are Rel-17 requirements.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the RPLMN is included in plmn-IdentityList stored in VarRA-Report:

2>	set the ra-ReportList in the UEInformationResponse message to the value of ra-ReportList in VarRA-Report;

2>	discard the ra-ReportList from VarRA-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

[TS 38.331, clause 5.7.10.4]

Upon successfully performing random-access procedure initialized with 4-step or 2-step RA type, or upon failed or successfully completed on-demand system information acquisition procedure in RRC_IDLE or RRC_INACTIVE state, the UE shall:

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report:

2>	clear the information included in VarRA-Report;

1>	if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of PLMN entries in plmn-IdentityList stored in VarRA-Report is less than maxPLMN; or

2>	if the number of PLMN entries in plmn-IdentityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the successfully completed random-access procedure or the failed or successfully completed on-demand system information acquisition procedure as a new entry in the VarRA-Report:

4>	if the list of EPLMNs has been stored by the UE:

5>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN) without exceeding the limit of maxPLMN;

4>	else:

5>	set the plmn-Identity, in plmn-IdentityList, to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

4>	set the cellId to the global cell identity and the tracking area code, if available, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

4>	if the corresponding random-access procedure was performed on an SCell of MCG:

5>	set the spCellId to the global cell identity of the PCell;

4>	if the corresponding random-access procedure was performed on an SCell of SCG; or

4>	if the corresponding random-access procedure was performed on PSCell:

5>	set the spCellId to the global cell identity of the PSCell, if available, otherwise, set the spCellId to the global cell identity of the PCell;

4>	set the raPurpose to include the purpose of triggering the random-access procedure;

4>	set the ra-InformationCommon as specified in clause 5.7.10.5.

The UE may discard the random access report information, i.e. release the UE variable VarRA-Report, 48 hours after the last successful random access procedure or the failed or successfully completed on-demand system information acquisition procedure related information is added to the VarRA-Report.

NOTE 1:	The UE does not log the RA information in the RA report if the triggering event of the random access is consistent UL LBT on SpCell as specified in TS 38.321 [6].

[TS 38.331, clause 5.7.10.5]

The UE shall set the content in ra-InformationCommon as follows:

…

1>	if contention based random-access resources are used in the random-access procedure:

2>	set the msgA_RO-FrequencyStart and msgA-RO-FDM and msgA-SubcarrierSpacing associated to the 2 step random- access resources if used in the random-access procedure;

2>	if msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure is available:

3>	set the msgA-SubcarrierSpacing associated to the 2 step random-access resources used in the random-access procedure;

2>	else if only 2 step random-access resources are available in the UL BWP used in the random-access procedure:

3>	set the msgA-SCS-From-prach-ConfigurationIndex to the subcarrier spacing as derived from the msgA-PRACH-ConfigurationIndex used in the 2-step random-access procedure;

8.1.6.4.3.3	Test description

8.1.6.4.3.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

Preamble:

-	The UE is in NR RRC_Idle mode (state 1N-A) on NR Cell 1 according to 38.508-1 [4] Table 4.4A.2-1.

8.1.6.4.3.3.2	Test procedure sequence

Table 8.1.6.4.3.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-8 | Steps 1 to 8 of procedure described in TS 38.508-1[4] table 4.5.4.2-3: NR RRC_CONNECTED are performed. | - | - | - | - |
| 9 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 10 | Check: Does the UE transmit a UEInformationResponse message with RA-Report? | --> | NR RRC: UEInformationResponse | 1 | P |

8.1.6.4.3.3.3	Specific message contents

Table 8.1.6.4.3.3.3-1: BWP-UplinkCommon (Preamble)

| Derivation Path: 38.508-1 [4], Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BWP-UplinkCommon ::= SEQUENCE { |  |  |  |
| genericParameters | BWP |  |  |
| msgA-ConfigCommon-r16 CHOICE { |  |  |  |
| setup | MSGA-ConfigCommon |  |  |
| } |  |  |  |

Table 8.1.6.4.3.3.3-2: MSGA-ConfigCommon (Table 8.1.6.4.3.3.3-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.3-81A

Table 8.1.6.4.3.3.3-3: UEInformationResponse (Step 10, Table 8.1.6.4.3.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| ra-ReportList-r16 SEQUENCE (SIZE (1..maxRAReport-r16)) OF RA-Report-r16{ | 1 entry |  |  |
| RA-Report-r16[1] SEQUENCE { |  | Entry 1 |  |
| cellId-r16 CHOICE { |  |  |  |
| cellGlobalId-r16 SEQUENCE { |  |  |  |
| plmn-Identity-r16 | PLMN identity of Cell 1 |  |  |
| cellIdentity-r16 | Cell identity of Cell1 |  |  |
| } |  |  |  |
| } |  |  |  |
| ra-InformationCommon-r16 SEQUENCE { |  |  |  |
| absoluteFrequencyPointA-r16 | Not checked |  |  |
| locationAndBandwidth-r16 | Not checked |  |  |
| subcarrierSpacing-r16 | Not checked |  |  |
| msg1-FrequencyStart-r16 | Not checked |  |  |
| msg1-FrequencyStartCFRA-r16 | Not checked |  |  |
| msg1-SubcarrierSpacing-r16 | Not checked |  |  |
| msg1-SubcarrierSpacingCFRA-r16 | Not checked |  |  |
| msg1-FDM-r16 | Not checked |  |  |
| msg1-FDMCFRA-r16 | Not checked |  |  |
| perRAInfoList-r16 | Not checked |  |  |
| perRAInfoList-v1660 | Not checked |  |  |
| msg1-SCS-From-prach-ConfigurationIndex-r16 | Not checked |  |  |
| msg1-SCS-From-prach-ConfigurationIndexCFRA-r16 | Not checked |  |  |
| msgA-RO-FrequencyStart-r17 | 0 |  |  |
| msgA-RO-FrequencyStartCFRA-r17 | Not checked |  |  |
| msgA-SubcarrierSpacing-r17 | Present |  |  |
| msgA-RO-FDM-r17 | four |  |  |
| msgA-RO-FDMCFRA-r17 | Not checked |  |  |
| msgA-SCS-From-prach-ConfigurationIndex-r17 | Not checked |  |  |
| msgA-TransMax-r17 | Not checked |  |  |
| msgA-MCS-r17 | Not checked |  |  |
| nrofPRBs-PerMsgA-PO-r17 | Not checked |  |  |
| msgA-PUSCH-TimeDomainAllocation-r17 | Not checked |  |  |
| frequencyStartMsgA-PUSCH-r17 | Not checked |  |  |
| nrofMsgA-PO-FDM-r17 | Not checked |  |  |
| dlPathlossRSRP-r17 | Not checked |  |  |
| intendedSIBs-r17 | Not checked |  |  |
| ssbsForSI-Acquisition-r17 | Not checked |  |  |
| msgA-PUSCH-PayloadSize-r17 | Not checked |  |  |
| onDemandSISuccess-r17 | Not checked |  |  |
| } |  |  |  |
| raPurpose-r16 | accessRelated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |