3GPP TSG-RAN5 Meeting #101	R5-236869

# Chicago, USA, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2784 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to H.3.1 NR measurement message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For RRM test cases 5.5.13.1 and 7.5.12.1, the SSB.2 FR2 and SMTC.2 are applied based on the parameters defined by TS 38.133, but the specific configuration is missing in Annex H. Besides, there are other editorial mistakes in H.3.1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the configuration of SSB.2 FR2 and SMTC.2. - Correcting some editorial mistakes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases will not be tested properly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR 2781 2782 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

H.3.1	RRC messages and information elements contents exceptions for NR measurement configuration

RRCReconfiguration

To setup NR Measurement Configuration.

Table H.3.1-1: RRCReconfiguration: NR measurement Configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-13 with condition NR_MEAS |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration ::= SEQUENCE { |  |  |  |  |
| measConfig |  | MeasConfig-DEFAULT | Measurement configuration |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

MeasConfig-DEFAULT

Configuration for NR measurement.

Table H.3.1-2: MeasConfig-DEFAULT: Configuration of NR measurement

| Derivation path: TS 38.508-1 [14], Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| measConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | n entries | n=1 without condition n=2 for INTER-FREQ OR INTER-RAT OR Deactivated SCell |  |
| measObjectId[1] | 1 |  |  |
| measObject[1] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTRA-FREQ MO |  |  |
| } |  |  |  |
| measObjectId[2] | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| measObject[2] CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-DEFAULT with Condition INTER-FREQ MO |  | INTER-FREQ |
|  | MeasObjectNR-DEFAULT with Condition Deactivated SCell |  | Deactivated SCell |
| measObjectEUTRA | MeasObjectEUTRA-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF SEQUENCE { | 1 entry |  |  |
| reportConfigId[1] | ReportConfigId |  |  |
| reportConfig[1] CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-DEFAULT(a4-threshold) | Table H.3.1-4AA |  |
|  | ReportConfigNR-DEFAULT(a3-threshold) | Table H.3.1-4 | INTER-FREQ OR Deactivated SCell |
| reportConfigInterRAT | ReportConfigInterRAT-DEFAULT |  | INTER-RAT |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF SEQUENCE { | 1 entry |  |  |
| measId[1] | MeasId |  |  |
| measObjectId[1] | 1 |  |  |
|  | 2 |  | INTER-FREQ OR INTER-RAT OR Deactivated SCell |
| reportConfigId[1] | ReportConfigId |  |  |
| } |  |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
|  | QuantityConfig | TS 38.508-1 Table 4.6.3-127 | L3 FILTERING NEEDED |
| measGapConfig | MeasGapConfig-DEFAULT |  | GAP NEEDED |
|  | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GAP NEEDED | Measurement gap on the NR Cell is needed for measurement |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |
| Deactivated SCell | Configuration for measurement on deactivated SCell tests |
| L3 FILTERING NEEDED | L3 filtering is needed for measurement |

MeasObjectNR-DEFAULT

NR measurement object configuration.

Table H.3.1-3: MeasObjectNR-DEFAULT: NR measurement object configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-76 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| MeasObjectNR::= SEQUENCE { |  |  |  |  |  |  |  |
| ssbFrequency |  | ARFCN-ValueNR of the SSB associated to serving cell |  | frequency of the serving cell |  | INTRA-FREQ MO |  |
|  |  | ARFCN-ValueNR of the SSB associated to SCell |  |  |  | Deactivated SCell |  |
|  |  | ARFCN-ValueNR of the SSB associated to inter-frequency neighbour cell |  |  |  | INTER-FREQ MO |  |
| ssbSubcarrierSpacing |  | Not present |  |  |  |  |  |
|  |  | SubcarrierSpacing specified in 38.508-1 [14] Table 7.3.1-3a with the specified condition |  |  |  | SSB.1 FR1 SSB.2 FR1 SSB.3 FR1 SSB.4 FR1 SSB.5 FR1 SSB.6 FR1 SSB.1 FR2 SSB.2 FR2 SSB.3 FR2 SSB.4 FR2 SSB.5 FR2 SSB.6 FR2 SSB.7 FR2 SSB.8 FR2 |  |
| smtc1 |  | Not present |  |  |  |  |  |
|  |  | SSB-MTC specified in 38.508-1 [14] Table 7.3.1-3 with the specified condition |  |  |  | SMTC.1 SMTC.2 |  |
| referenceSignalConfig SEQUENCE { |  |  |  |  |  |  |  |
| ssb-ConfigMobility SEQUENCE { |  |  |  |  |  |  |  |
| ssb-ToMeasure |  | Not present |  |  |  |  |  |
|  |  | SSB-ToMeasure specified in 38.508-1 [14] Table 7.3.1-31 with the specified condition |  |  |  | SSB.1 FR2 SSB.2 FR2 SSB.3 FR2 SSB.4 FR2 SSB.5 FR2 SSB.6 FR2 SSB.7 FR2 SSB.8 FR2 |  |
| deriveSSB-IndexFromCell |  | false |  |  |  |  |  |
|  |  | true |  |  |  | Synchronous cells OR RLM |  |
| ssb-PositionQCL-Common-r16 |  | n1 |  |  |  | SSB.1 CCA SSB.2 CCA |  |
|  |  | n2 |  |  |  | SSB.3 CCA SSB.4 CCA |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE { |  |  |  |  |  |  |  |
| thresholdRSRP |  | 0 |  | SS-RSRP < -156dB |  | NOT SS-SINR |  |
| thresholdSINR |  | 0 |  | SS-SINR<-23 |  | SS-SINR |  |
| } |  |  |  |  |  |  |  |
| measCycleSCell-v1530 |  | sf640 |  |  |  | Deactivated SCell |  |
| rmtc-Config-r16 |  | FFS |  |  |  | RSSI OR Channel Occupancy |  |
| associatedMeasGapSSB-r17 |  | Not present |  |  |  |  |  |
|  |  | MeasGapId-r17 |  |  |  | MGe |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTRA-FREQ MO | Configuration for NR MO associated to intra-frequency carrier |
| INTER-FREQ MO | Configuration for NR MO associated to inter-frequency carrier |
| Synchronous cells | SSB indices of neighbourcells can be derived from timing of serving cell |
| RLM | Configuration for RLM tests |
| Deactivated SCell | Configuration for SCell activation and deactivation tests |
| SS-SINR | Configuration for SS-SINR tests |
| MGe | Indicates measurement gap enhancements configuration |  |
| SSB.X CCA | SSB pattern SSB.X CCA configured as per section A.3A (X = {1,2,3,4}) |  |
| RSSI | Configuration associated to RSSI measurements |  |
| Channel Occupancy | Configuration associated to Channel Occupancy measurements |  |

MeasObjectEUTRA-DEFAULT

EUTRA measurement object configuration

Table H.3.1-3A: MeasObjectEUTRA-DEFAULT: InterRAT EUTRA measurement object configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-74 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| MeasObjectEUTRA ::= SEQUENCE { |  |  |  |  |  |  |  |
| cellsToAddModListEUTRAN SEQUENCE (SIZE (1..maxCellMeasEUTRA)) OF SEQUENCE{ |  |  |  |  |  |  |  |
| cellIndexEUTRA |  | 1 |  |  |  |  |  |
| physCellId |  | Physical Cell ID of the E-UTRA cell |  | Annex E |  |  |  |
| cellIndividualOffset |  | dB0 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| eutra-PresenceAntennaPort1 |  | false |  |  |  |  |  |
| associatedMeasGap-r17 |  | Not present |  |  |  |  |  |
|  |  | MeasGapId-r17 |  |  |  | MGe |  |
| } |  |  |  |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| MGe | Indicates measurement gap enhancements configuration |

ReportConfigNR-DEFAULT

NR Report Configuration

Table H.3.1-4: ReportConfigNR-DEFAULT(a3-offset): NR report configuration for event A3

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | A3-offset*2 | The actual value is field value * 0.5 dB |  |
| rsrq | A3-offset*2 | The actual value is field value * 0.5 dB | RSRQ |
| sinr | A3-offset*2 | The actual value is field value * 0.5 dB | SINR |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity-RsIndexes SEQUENCE { |  |  | SSB Index |
| rsrp | true |  |  |
| rsrq | false |  |  |
|  | true |  | RSRQ |
| sinr | false |  |  |
|  | true |  | SINR |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| maxNrofRS-IndexesToReport | 2 |  | SSB Index |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |
| RSRQ | RSRQ trigger quantity |
| SINR | SINR trigger quantity |

Table H.3.1-4AA: ReportConfigNR-DEFAULT(a4-threshold): NR report configuration for event A4

| Derivation Path: 38.508-1 [14], Table 4.6.3-142 with condition EVENT_A4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA4 SEQUENCE { |  |  |  |
| a4-Threshold CHOICE { |  |  |  |
| rsrp | a4-threshold+156 | For thresholds, the actual value is (IE value – 156) dBm |  |
| } |  |  |  |
| hysteresis | 0 |  |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantity-RsIndexes SEQUENCE { |  |  | SSB Index |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| maxReportCells | 2 |  |  |
| maxNrofRS-IndexesToReport | 2 |  | SSB Index |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |

ReportConfigInterRAT-DEFAULT

InterRAT NR Report Configuration.

Table H.3.1-4A: ReportConfigInterRAT- DEFAULT (b2-Thres1, b2-Thres2): InterRAT NR report configuration for FR1 to E-UTRAN handover with b2-Threshold1 = b2-Thres1 and b2-Threshold2EUTRA  = b2-Thres2 dBm

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-141 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigInterRAT ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventB2 SEQUENCE { |  |  |  |
| b2-Threshold1 CHOICE { |  |  |  |
| rsrp | b2-Thres1 | b2-Thres1 – 156 dBm |  |
| } |  |  |  |
| b2-Threshold2EUTRA CHOICE { |  |  |  |
| rsrp | b2-Thres2 | b2-Thres2 – 140 dBm |  |
| } |  |  |  |
| hysteresis | 0 | 0 dB |  |
| timeToTrigger | ms0 |  |  |
| } |  |  |  |
| } |  |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

QuantityConfig-DEFAULT

NR quantity configuration when L3 filtering is not used.

Table H.3.1-5: QuantityConfig-DEFAULT: NR quantity configuration when L3 filtering is not used

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-127 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| QuantityConfig::= SEQUENCE { |  |  |  |
| quantityConfigNR-List SEQUENCE (SIZE (1..maxNrofQuantityConfig)) OF SEQUENCE { | 1 entry |  |  |
| quantityConfigCell[1] SEQUENCE { |  |  |  |
| ssb-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| csi-RS-FilterConfig SEQUENCE { |  |  |  |
| filterCoefficientRSRP | fc0 | No L3 filtering |  |
| filterCoefficientRSRQ | fc0 | No L3 filtering |  |
| filterCoefficientRS-SINR | fc0 | No L3 filtering |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| quantityConfigEUTRA | Not present |  |  |
| quantityConfigEUTRA SEQUENCE { |  |  | INTER-RAT |
| filterCoefficientRSRP | fc0 |  |  |
| filterCoefficientRSRQ | fc0 |  |  |
| filterCoefficientRS-SINR | fc0 |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |

MeasGapConfig-DEFAULT

Measurement gap configuration.

Table H.3.1-6: MeasGapConfig: measurement gap configuration

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-70 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| MeasGapConfig ::= SEQUENCE { |  |  |  |  |  |  |  |
| gapFR2 CHOICE { |  |  |  |  |  | gapFR2 |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| gapOffset |  | 39 |  |  |  | Pattern #13 |  |
| mgl |  | ms5dot5 |  |  |  | Pattern #13 |  |
| mgrp |  | ms40 |  |  |  | Pattern #13 |  |
| mgta |  | ms0 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| gapFR1 CHOICE { |  |  |  |  |  | gapFR1 |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| gapOffset |  | 39 |  |  |  | Pattern #2 |  |
|  |  | 19 |  |  |  | Pattern #4 |  |
| mgl |  | ms3 |  |  |  | Pattern #2 |  |
|  |  | ms6 |  |  |  | Pattern #4 |  |
| mgrp |  | ms40 |  |  |  | Pattern #2 |  |
|  |  | ms20 |  |  |  | Pattern #4 |  |
| mgta |  | ms0 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| gapUE CHOICE { |  |  |  |  |  | gapUE |  |
| setup SEQUENCE { |  |  |  |  |  |  |  |
| gapOffset |  | 39 |  |  |  | Pattern #0 OR Pattern #2 OR Pattern #13 |  |
|  |  | 0 |  |  |  | RLM OR BFD |  |
| mgl |  | ms6 |  |  |  | Pattern #0 OR RLM OR BFD |  |
|  |  | ms3 |  |  |  | Pattern #2 |  |
|  |  | ms5dot5 |  |  |  | Pattern #13 |  |
| mgrp |  | ms40 |  |  |  | Pattern #0 OR Pattern #2 OR Pattern #13 OR RLM OR BFD |  |
| mgta |  | ms0 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| gapToAddModList-r17 SEQUENCE (SIZE (1..maxNrofGapId-r17)) OF GapConfig-r17 SEQUENCE{ |  |  |  |  |  | MGe |  |
| GapConfig-r17[1] SEQUENCE{ |  |  |  |  |  |  |  |
| measGapId-r17 |  | 0 |  |  |  | Pattern #0 |  |
|  |  | 1 |  |  |  | Pattern #1 |  |
|  |  | 2 |  |  |  | Pattern #24 |  |
| gapType-r17 |  | perUE |  |  |  |  |  |
| gapOffset-r17 |  | 39 |  |  |  | Pattern #0 |  |
|  |  | 19 |  |  |  | Pattern #1 |  |
|  |  | 11 |  |  |  | Pattern #24 |  |
| mgl-r17 |  | ms6 |  |  |  | Pattern #0 OR Pattern #1 |  |
|  |  | ms10 |  |  |  | Pattern #24 |  |
| mgrp-r17 |  | ms40 |  |  |  | Pattern #0 |  |
|  |  | ms80 |  |  |  | Pattern #1 OR Pattern #24 |  |
| mgta-r17 |  | ms0 |  |  |  |  |  |
| refServCellIndicator-r17 |  | Not present |  |  |  |  |  |
| refFR2-ServCellAsyncCA-r17 |  | Not present |  |  |  |  |  |
| preConfigInd-r17 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | Pre-CFG |  |
| ncsgInd-r17 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | NCSG |  |
| gapAssociationPRS-r17 |  | Not present |  |  |  |  |  |
|  |  | true |  |  |  | PRS |  |
| gapSharing-r17 |  | Not present |  |  |  |  |  |
| gapPriority-r17 |  | 2 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

| Condition |  | Explanation |  |
| --- | --- | --- | --- |
| Pattern #0 |  | Measurement gap pattern #0 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #1 |  | Measurement gap pattern #1 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #2 |  | Measurement gap pattern #2 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #4 |  | Measurement gap pattern #4 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #13 |  | Measurement gap pattern #13 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| Pattern #24 |  | Measurement gap pattern #24 defined in TS 38.133 [6] Table 9.1.2-1 is used for measurement tests |  |
| RLM |  | Measurement gap pattern for RLM tests |  |
| BFD |  | Measurement gap pattern for Beam Failure Dectection tests |  |
| gapFR2 |  | Indicates measurement gap configuration that applies to FR2 only. gapFR2 cannot be configured together with gapUE. In (NG)EN-DC or NE-DC, gapFR2 can only be set up by NR RRC |  |
| gapFR1 |  | Indicates measurement gap configuration that applies to FR1 only. gapFR1 cannot be configured together with gapUE. In (NG)EN-DC, gapFR1 cannot be set up by NR RRC |  |
| gapUE |  | Indicates measurement gap configuration that applies to all frequencies (FR1 and FR2). If gapUE is configured, then neither gapFR1 nor gapFR2 can be configured. In (NG)EN-DC, gapUE cannot be set up by NR RRC. |  |
| MGe |  | Indicates measurement gap enhancements configuration |  |
| Pre-CFG |  | Pre-configured measurement gap |  |
| NCSG |  | Measurement gap pattern for NCSG (network controlled small gap) tests |  |
| PRS |  | Measurement gap associated with PRS measurement |  |

MeasResults-DEFAULT

measurement result for NR measurements.

Table H.3.1-7: MeasResults: measurement result for NR measurements

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-79 with condition A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| measResults SEQUENCE { |  |  |  |  |
| measId |  | MeasId |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF SEQUENCE { |  | 2 entries |  |  |
| servCellId[1] |  | ServCellIndex of NR SpCell |  |  |
| measResultServingCell[1] SEQUENCE { |  |  |  |  |
| physCellId |  | PhysCellId of NR SpCell |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | (0..127) |  | SINR |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| servCellId[2] |  | ServCellIndex of NR SCell |  | Deactivated SCell |
| measResultServingCell[2] SEQUENCE { |  |  |  | Deactivated SCell |
| physCellId |  | PhysCellId of NR SCell |  |  |
| measResult SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  |  |
| sinr |  | (0..127) |  | SINR |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |  |
| measResultListNR SEQUENCE (SIZE(1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  | INTRA-FREQ OR INTER-FREQ |
| physCellId[1] |  | PhysCellId of NR neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| cellResults SEQUENCE { |  |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |  |
| rsrp |  | (0..127) |  |  |
| rsrq |  | (0..127) |  | RSRQ |
| sinr |  | (0..127) |  | SINR |
| } |  |  |  |  |
| } |  |  |  |  |
| rsIndexResults SEQUENCE { |  | n entires of ResultsPerSSB-Index | ResultsPerSSB-IndexList | SSB Index |
| resultsPerSSB-Index SEQUENCE { |  | entry [1] |  |  |
| ssb-Index |  | SSB-Index | an SS-Block within an SS-Burst |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measResultListEUTRA SEQUENCE (SIZE (1..maxCellReport)) OF SEQUENCE { |  | 1 entry |  | INTER-RAT |
| eutra-PhysCellId [1] |  | PhysCellId of E-UTRA neighbour Cell |  |  |
| measResult[1] SEQUENCE { |  |  |  |  |
| rsrp |  | (0..97) |  |  |
| rsrq |  | (0..34) |  |  |
| } |  |  |  |  |
| cgi-Info |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB Index | To include SSB Index |
| INTRA-FREQ | Configuration for intra-frequency NR measurement tests |
| INTER-FREQ | Configuration for inter-frequency NR measurement tests |
| INTER-RAT | Configuration for inter-RAT EUTRA measurement tests |
| Deactivated SCell | Configuration for measurement on Deactivated SCell tests |
| RSRQ | RSRQ measurement reporting |
| SINR | SINR measurement reporting |

<End of Changes>