3GPP TSG-RAN5 Meeting #101	R5-236874

# Chicago, USA, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0553 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of physical layer baseline capabilities for MR-DC enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To complete the test applicability for the following test cases: - 5.5.13.1 - 7.5.3.14 - 7.5.12.1 - 7.5.14 the related UE capabilities should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding the UE capability for conditional PSCell addition and SCG activation. - Correcting the PICS mnemonic of fast SCell activation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mentioned test cases will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS 38.522 CR 0349 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities |  | Ref. | Release | Mnemonic |  | M |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling |  | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS |  | No |  |  |  |  |
| 2 | Support 256QAM for PDSCH for FR1 |  | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 |  | CY |  |  |  | Mandatory for non-RedCap UEs and optional for RedCap UEs. |
| 3 | Support 256QAM for PDSCH for at least one NR FR2 band |  | 38.306, 4.2.7.2 | Rel-15 | pc_pdsch_256QAM_FR2 |  | No |  |  |  |  |
| 4 | Support 256QAM for PUSCH for at least one NR FR1 band |  | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR1 |  | No |  |  |  |  |
| 4a | Support 256QAM for PUSCH for at least one NR FR2 band |  | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR2 |  | No |  |  |  |  |
| 5 | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols |  | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeA |  | Yes |  | Yes |  |  |
| 6 | Support receiving PDSCH using PDSCH mapping type B |  | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeB |  | Yes |  |  |  |  |
| 7 | Support resource allocation Type 0 for PUSCH |  | 38.306, 4.2.7.10 | Rel-15 | pc_ra_Type0_PUSCH |  | No |  |  |  |  |
| 8 | Support scaling factor 0.75 is applied to the band in the max data rate calculation |  | 38.306, 4.2.7 | Rel-15 | pc_scalingFactor0dot75 |  |  |  |  |  |  |
| <<Unchanged Text Skipped>> |  |  |  |  |  |  |  |  |  |  |  |
| 127 |  | Support of dynamic 2Tx-2Tx UL Tx switching | 38.306, 4.2.7.1 | Rel-17 | pc_2Tx_ULTxSwitchingBandPair_r17 | No |  |  |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |  |
| 128 |  | Indicates whether the UE supports aperiodic CSI-RS for tracking for fast SCell activation, i.e., 1) Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE; 2) Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell. | 38.306, 4.2.7.2 | Rel-17 | pc_aperiodicCSIRS_FastScellActivation_r17 | No |  |  |  | Includes maxNumberAperiodicCSI-RS-PerCC-r17 and maxNumberAperiodicCSI-RS-AcrossCCs-r17 to indicate the number of RS configurations for fast SCell activation that can be indicated by the MAC CE |  |
| 129 |  | Indicates whether the UE supports conditional PSCell addition in EN-DC. The UE supporting this feature shall also support 2 trigger events for same execution condition in conditional PSCell addition in EN-DC. | 38.306, 4.2.7.9 | Rel-17 | pc_condPSCellAdditionENDC_r17 | No |  |  |  |  |  |
| XX1 |  | Support of conditional PSCell addition in NR-DC | 38.306, 4.2.7.12 | Rel-17 | pc_condPSCellAdditionNRDC_r17 | No |  |  |  |  |  |
| XX2 |  | Support of activation (with or without RACH) and deactivation on SCG in EN-DC | 38.306, 4.2.7.9 | Rel-17 | pc_scg_ActivationDeactivationENDC_r17 | No |  |  |  |  |  |
| XX3 |  | Support of activation (with or without RACH) and deactivation on SCG in NR-DC | 38.306, 4.2.7.12 | Rel-17 | pc_scg_ActivationDeactivationNRDC_r17 | No |  |  |  |  |  |

<Unchanged Skipped>

<End of Changes>