3GPP TSG-RAN5 Meeting #101	R5-236894

# Chicago, USA, 13th – 17th November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0560 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New generic procedure A.4.2a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To add a generic procedure for default MO Voice Call setup with preconditions disabled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new generic procedure, A.4.2a MTSI MO Voice Call / Default Configuration with preconditions disabled / 5GS has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Same sequence will need to be repeated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.2a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

200 OK for INVITE (Step 7)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A1, A10, and A19.

ACK (Step 8)

Use the default message "ACK" in Annex A.2.7 of TS 34.229-1 [2] applying conditions A1 and A3.

## A.4.2a	MTSI MO Voice Call / Default Configuration with preconditions disabled / 5GS

Expected sequence

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1 |  |  | INVITE | UE sends INVITE with the first SDP offer. |
| 2 |  |  | 100 Trying | SS sends a 100 Trying provisional response. |
| 3 |  |  | 183 Session Progress | SS sends an SDP answer. |
| 4 |  |  | PRACK | UE acknowledges reception of 183 Session Progress. |
| 5 |  |  | 200 OK | SS responds to PRACK. |
| 6 |  |  | 180 Ringing | SS sends 180 Ringing reliably. |
| 7 |  |  | 200 OK | SS responds to INVITE. |
| 8 |  |  | ACK | UE acknowledges. |

Specific Message Contents

INVITE (Step 1)

Use the default message "INVITE for MO Call Setup" in Annex A.2.1 of TS 34.229-1 [2] applying conditions A1, A3, A4, A28, A29, A30, and A31, and with the following exceptions:

| Header/param | Value/Remark |  |
| --- | --- | --- |
| Supported |  |
| option-tag | precondition – option tag not present Note: this does not affect any other option tags in the Supported header as for instance prescribed in TS 34.229-1 [2] Annex A.2.1 |
| Message-body | The following SDP types and values. Session description: v=0 o=(username) (sess-id) (sess-version) IN (addrtype) (unicast-address for UE) s=(session name) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) Time description: t= (start-time) (stop-time) Media description: m=audio (transport port) RTP/AVP (fmt) c=IN (addrtype) (connection-address for UE) [Note 1] b=AS: (bandwidth-value) b=RS: (bandwidth-value) [Note 2] b=RR: (bandwidth-value) [Note 2] Attributes for media: a=rtpmap: (payload type) EVS/16000 [Note 3, 9] a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red= (att-field) [Note 4, 5] a=rtpmap: (payload type) AMR-WB/16000 [Note 3, 9] a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 4, 6] a=rtpmap: (payload type) telephone-event/16000 a=fmtp: (format) a=rtpmap: (payload type) AMR/8000 [Note 3, 9] a=fmtp: (format) mode-change-capability=2; max-red= (att-field) [Note 4, 6] a=rtpmap: (payload type) telephone-event/8000 a=fmtp: (format) a=ecn-capable-rtp: leap ect=0 [Note 7] a=rtcp-fb:* nack ecn [Note 7] a=rtcp-xr:ecn-sum [Note 7] a=rtcp-rsize [Note 7] a=ptime:20 a=maxptime:240 Attributes for media security mechanism: a=3ge2ae: requested [Note 8] a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:WVNfX19zZW1jdGwgKCkgewkyMjA7fQp9CnVubGVz¦2^20¦ 1:4FEC_ORDER=FEC_SRTP" [Note 8] Attributes for preconditions: not present Note 1: At least one "c=" field shall be present. Note 2: The RR value shall be greater than 0. The RS value can be any value. Note 3: The channel number shall be "/1" or omitted. Note 4: The max-red values from 0 to 220 are allowed. Note 5: The parameters dtx, dtx-recv and evs-mode-switch shall not be present. Note 6: The parameters mode-set, mode-change-period, mode-change-neighbor, crc, robust-sorting and interleaving shall not be included. Note 7: Attributes for ECN Capability may be present if the UE supports Explicit Congestion Notification. Note 8: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. Note 9: The ordering of payload types shall be as listed, i.e., EVS before AMR-WB before AMR according to NG.114 [31]. |  |

100 Trying (Step 2)

Use the default message "100 Trying for INVITE" in Annex A.2.2 of TS 34.229-1 [2] applying condition A1.

183 Session Progress (Step 3)

Use the default message "183 Session Progress" in Annex A.2.3 of TS 34.229-1 [2] applying condition A1, and with the following exceptions:

| Header/param | Value/Remark |
| --- | --- |
| Message-body | The following SDP types and values. Session description: v=0 o=- 1111111111 1111111111 IN (addrtype) (unicast-address for SS) s=- c=IN (addrtype) (connection-address for SS) b=AS:65 Time description: t=0 0 Media description: m=audio (transport port) RTP/AVP (fmt) [Note 1, 2] b=AS:65 b=RS: (bandwidth-value) [Note 3] b=RR: (bandwidth-value) [Note 3] Attributes for media: a=rtpmap: (payload type) EVS/16000/1 [Note 1] a=fmtp: (format) br=5.9-24.4; bw=nb-swb; max-red=220 a=ecn-capable-rtp: leap ect=0 [Note 4] a=rtcp-fb:* nack ecn [Note 4] a=rtcp-xr:ecn-sum [Note 4] a=ptime:20 a=maxptime:240 Attributes for media security mechanism: a=3ge2ae: requested [Note 7] a=crypto:1 AES_CM_128_HMAC_SHA1_80inline:PS1uQCVeeCFCanVmcjkpPywjNWhcYD0mXXtxaVBR¦2^20¦1:4 [Note 5] Note 1: The values for fmt, payload type and format are copied from step 1. Note 2: Transport port is the port number of the SS (see RFC 3264 clause 6). Note 3: The bandwidth-value is copied from step 1. Note 4: Attributes for ECN Capability are present if the UE supports Explicit Congestion Notification. Note 5: Attributes for media plane security are present if the use of end-to-access-edge security is supported by UE. |

PRACK (Step 4)

Use the default message "PRACK" in Annex A.2.4 of TS 34.229-1 [2] applying conditions A1 and A7.

200 OK for PRACK (Step 5)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A10 and A22.

180 Ringing (Step 6)

Use the default message "180 Ringing for INVITE" in Annex A.2.6 of TS 34.229-1 [2] applying conditions A1, A3 and A14.

200 OK for INVITE (Step 7)

Use the default message "200 OK for other requests than REGISTER or SUBSCRIBE" in Annex A.3.1 of TS 34.229-1 [2] applying conditions A1, A10, and A19.

ACK (Step 8)

Use the default message "ACK" in Annex A.2.7 of TS 34.229-1 [2] applying conditions A1 and A3.