3GPP TSG-RAN5 Meeting #101	R5-236895

# Chicago, USA, 13th – 17th November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0561 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restore test case 7.6a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test case 7.6a was incorrectly deleted in version 16.7.0 and has to be restored. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test case 7.6.a is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 7.6.a will incorrectly be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.6	Void

## 7.6a	MTSI MT Voice Call with preconditions at both originating UE and terminating UE / Default Configuration / 5GS

7.6a.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions }

ensure that {

when { UE receives INVITE for voice call with preconditions and to use the EVS default configuration }

then { UE responds with 183 Session Progress including SDP accepting the EVS default configuration}

}

(2)

with { UE having sent 183 Session Progress }

ensure that {

when { UE receives PRACK for 183 Session Progress }

then { UE sends 200 OK for PRACK }

}

(3)

with { UE having sent 200 OK for PRACK }

ensure that {

when { UE receives UPDATE including SDP }

then { UE sends 200 OK for UPDATE including SDP and 180 Ringing }

}

(4)

with { UE having sent 180 Ringing }

ensure that {

when { UE receives PRACK for 180 Ringing }

then { UE sends 200 OK for PRACK }

}

(5)

with { UE having sent 180 Ringing  }

ensure that {

when { User accepts the incoming voice call request }

then { UE sends 200 OK for INVITE }

}

(6)

with { UE having sent 200 OK for INVITE }

ensure that {

when { UE receives ACK followed by BYE }

then { UE sends 200 OK for BYE }

}

7.6a.2	Conformance Requirements

Same as TC 7.6.

7.6a.2A Profile requirements (Informative)

Same as TC 7.6 with the following additions:

[NG.114 Version 1.0 and 2.0, clause C.3]:

Table C.3-1 contains the configuration parameters with their default values, that must be supported by the UE and the network. The UE must use the default value for each parameter in Table C.3-1 unless configured differently as described in Annex C.2.

…

| Reliable 18x policy (Sending SIP 18x reliably) | 1 – Indicates that the SIP 18x responses (other than SIP 183 response) are to be sent reliably | Section 5.56 of 3GPP TS 24.167 [67] (/<X>/ Reliable_18x_policy /<X>/ Send_18x_Reliably) and 3GPP TS 24.229 [8] | 2.2.4 |
| --- | --- | --- | --- |

7.6a.3	Test description

7.6a.3.1	Pre-test conditions

Same as TC 7.6.

7.6a.3.2	Test procedure sequence

Table 7.6a.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0A-0H | Steps 1-8 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 1 | SS sends INVITE for voice call (Step 1 of Annex A.5.1a) happens | <-- | INVITE | - | - |
| 2 | UE may send 100 Trying response (Step 2 of Annex A.5.1a) happens | --> | 100 Trying | - | - |
| 3 | UE sends 183 Session Progress response (Step 3 of Annex A.5.1a) | --> | 183 Session Progress | 1 | P |
| 4 | SS sends PRACK (Step 4 of Annex A.5.1a) | <-- | PRACK | - | - |
| 5 | UE sends 200 OK for PRACK (Step 5 of Annex A.5.1a) | --> | 200 OK | 2 | P |
| 5A-5C | Steps 10-12 of generic procedure specified in Table 4.9.16.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 6 | SS sends UPDATE (Step 6 of Annex A.5.1a) | <-- | UPDATE | - | - |
| 7 | UE sends 200 OK for UPDATE (Step 7 of Annex A.5.1a) | --> | 200 OK | 3 | P |
| 8 | UE sends 180 Ringing (Step 8 of Annex A.5.1a) | --> | 180 Ringing | - | - |
| 9 | SS sends PRACK if 180 Ringing was sent reliably (Step 9 of Annex A.5.1a) | <-- | PRACK | - | - |
| 10 | UE sends 200 OK for PRACK if SS sent PRACK (Step 10 of Annex A.5.1a) | --> | 200 OK | 4 | P |
| 10A | Make UE accept the voice call | - | - | - | - |
| 11 | UE accepts the call (Step 11 of Annex A.5.1a) | --> | 200 OK | 5 | P |
| 12 | SS sends ACK (Step 12 of Annex A.5.1a) | <-- | ACK | - | - |
| 13 | SS releases the call (Step 1 of Annex A.8) | <-- | BYE | - | - |
| 14 | UE sends 200 OK for BYE (Step 2 of Annex A.8) | --> | 200 OK | 6 | P |

7.6a.3.3	Specific message contents

None.