3GPP TSG-RAN5 Meeting #101	R5-236898

# Chicago, USA, 13th – 17th November, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0564 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to test case 7.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new generic procedure for default MO Voice Call setup with preconditions disabled has been added. This test case needs to reference the new procedure Align with the outcome of the Tdoc R5-225274 to update IMS TCs with NG.114 default EVS configuration A2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The new generic procedure as well as generic procedure A.4.1a are referenced in Table 7.12.3.2-1: Main Behaviour. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will use the incorrect generic procedures for MO Voice Call setup. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 7.12	MTSI MO Voice Call with preconditions at originating UE and without preconditions at terminating UE / 5GS

7.12.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and configured to use preconditions }

ensure that {

when { UE is being made to initiate a voice call }

then { UE sends INVITE for voice call with preconditions }

}

(2)

with { UE having sent INVITE with preconditions }

ensure that {

when { UE receives 183 Session Progress without preconditions }

then { UE sends PRACK for 183 Session Progress }

}

(3)

with { UE having sent PRACK }

ensure that {

when { UE receives 200 OK for PRACK followed by 180 Ringing followed by 200 OK for INVITE }

then { UE sends ACK }

}

7.12.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.229, clause 5.1.3.1]:

During the session initiation, if the originating UE indicated the support for the precondition mechanism in the initial INVITE request and:

a)	the received response with an SDP body includes a Require header field with "precondition" option-tag, the originating UE shall include a Require header field with the "precondition" option-tag:

-	in subsequent requests that include an SDP body, that the originating UE sends in the same dialog as the response is received from; and

-	in responses with an SDP body to subsequent requests that include an SDP body and include "precondition" option-tag in Supported header field or Require header field received in-dialog; or

b)	the received response with an SDP body does not include the "precondition" option-tag in the Require header field,

-	in subsequent requests that include an SDP body, the originating UE shall not include a Require or Supported header field with "precondition" option-tag in the same dialog;

-	in responses with an SDP body to subsequent requests with an SDP body but without "precondition" option-tag in the Require or Supported header field, the originating UE shall not include a Require or Supported header field with "precondition" option-tag in the same dialog; and

-	in responses with an SDP body to subsequent requests with an SDP body and with "precondition" option-tag in the Require or Supported header field, the originating UE shall include a Require header field with "precondition" option-tag in the same dialog.

7.12.3	Test description

7.12.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

Preamble:

-	UE is in state 1N-A and registered to IMS

7.12.3.2	Test procedure sequence

Table 7.12.3.2-1: Main Behaviour

| St |  | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |  |  |  |  |
| 1 |  | UE is made to attempt an IMS voice call. |  | - |  | - |  | - |  | - |  |
| 2 |  | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. |  | - |  | - |  | - |  | - |  |
| - |  | EXCEPTION: In parallel with Step 3, parallel behaviour defined in table 7.12.3.2-2 takes place |  | - |  | - |  | - |  | - |  |
| 3 |  | Step 1 of Annex A.4.1a happens (Check: does the UE send INVITE for voice call with preconditions?) |  | --> |  | INVITE |  | 1 |  | P |  |
| 4 |  | Step 2 of Annex A.4.1a happens |  | <-- |  | 100 Trying |  | - |  | - |  |
| 5 |  | Step 3 of Annex A.4.2a happens (Note: the SS sends 183 Session Progress without attributes for preconditions in the SDP body.) |  | <-- |  | 183 Session Progress |  | - |  | - |  |
| 6 |  | Step 4 of Annex A.4.2a happens (Check: does the UE send PRACK?) |  | --> |  | PRACK |  | 2 |  | P |  |
| 7 |  | Step 5 of Annex A.4.2a happens |  | <-- |  | 200 OK |  | - |  | - |  |
| 7A-7C |  | Steps 10-12 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. |  | - |  | - |  | - |  | - |  |
| 8 |  | Step 6 of Annex A.4.2a happens |  | <-- |  | 180 Ringing |  | - |  | - |  |
| 9 |  | Step 7 of Annex A.4.2a happens |  | <-- |  | 200 OK |  | - |  | - |  |
| 10 |  | Step 8 of Annex A.4.2a happens (Check: does the UE send ACK?) |  | --> |  | ACK |  | 3 |  | P |  |

Table 7.12.3.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCReconfigurationComplete message. | --> | NR RRC: RRCReconfigurationComplete | - | - |

7.12.3.3	Specific message contents

None as fully described in Annex A.4.1a and A.4.2a.