3GPP TSG-RAN5 Meeting #101	R5-236918

# Chicago, USA, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1448 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of default message contents for EPS UAS test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ID_UAS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Default contents of ESM messages needs update to cater for test cases in UAS WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated default message contents of MODIFY EPS BEARER CONTEXT REQUEST and PDN CONNECTIVITY REQUEST messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UAS EPS WP will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.3 |  |  |  |  |  |  |  |  |
| sss |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

#### -	MODIFY EPS BEARER CONTEXT REQUEST

This message is sent by the SS to the UE.

Table 4.7.3-18: MODIFY EPS BEARER CONTEXT REQUEST

| Derivation Path: 24.301 clause 8.3.16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | Set according to specific message content. |  |  |
| Procedure transaction identity |  | '0000 0000'B | No procedure transaction identity assigned | NETWORK-INITIATED |
|  |  | The same value as the value set in BEARER RESOURCE MODIFICATION REQUEST message or BEARER RESOURCE ALLOCATION REQUEST message. |  | UE-INITIATED |
| Modify EPS bearer context request message identity |  | '1100 1001'B | Modify EPS bearer context request |  |
| New EPS QoS |  | Use the same value as used in Activate EPS Bearer Context Request message |  |  |
| TFT |  | According to reference dedicated EPS bearer context #2 except for TFT operation code which is set to ‘100’B and TFT identifier is set to ‘00110000’B.. |  |  |
| New QoS |  | Not Present. |  |  |
| Negotiated LLC SAPI |  | Not Present. |  |  |
| Radio priority |  | Not Present. |  |  |
| Packet flow Identifier |  | Not Present. |  |  |
|  |  | '0000 0000'B | Best Effort | pc_GERAN |
| APN-AMBR |  | Not Present. |  |  |
| Protocol configuration options |  | Not present |  |  |
| WLAN offload container |  | Not present |  |  |
| NBIFOM container |  | Not present |  |  |
| Header compression configuration |  | Not present |  |  |
| Extended protocol configuration options |  | Not present |  |  |
| Extended protocol configuration options |  | If present including at least the following container |  | UAS-UUAA |
| Container ID n |  | ‘0041’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 2 octets |  |
| Container ID n contents |  | Service-level-AA container | Service-level-AA container with service-level-AA response parameter with ‘SLAR’ bits set to 01 |  |
| Extended protocol configuration options |  | If present including at least the following container |  | UAS-C2 |
| Container ID n |  | ‘0041’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 2 octets |  |
| Container ID n contents |  | Service-level-AA container | Service-level-AA container with service-level-AA response parameter with ‘C2AR’ bits set to 01 |  |

| Condition | Explanation |
| --- | --- |
| NETWORK-INITIATED | Network initiated ESM procedures |
| UE-INITIATED | UE initiated ESM procedures |
| UAS-UAAA | Used in UAS test cases (TS 36.523-1 [18]) If pc_EPC_UAS, for USS UAV Authorization/Authentication |
| UAS-C2 | Used in UAS test cases (TS 36.523-1 [18]) If pc_EPC_UAS, for UAV and UAV-C pairing |

NOTE:	This message is always sent within SECURITY PROTECTED NAS MESSAGE message.

<End of modified section>

<Start of modified section>

#### -	PDN CONNECTIVITY REQUEST

This message is sent by the UE to the SS.

Table 4.7.3-20: PDN CONNECTIVITY REQUEST

| Derivation Path: 24.301 clause 8.3.20 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Protocol discriminator |  | ESM |  |  |
| EPS bearer identity |  | '0000'B | No EPS bearer identity assigned |  |
| Procedure transaction identity |  | Any value from 1 to 254 |  |  |
| PDN connectivity request message identity |  | '1101 0000'B | PDN connectivity request |  |
| Request type |  | '0001'B | initial request |  |
| PDN type |  | Any value between '001'B, '010'B, '011'B and '100'B | The allowed values are respectively IPv4, IPv6, IPv4v6 and "unused but interpreted as IPv6 by the network" |  |
| PDN type |  | ‘101’B | Non-IP | CIoT_CP_NON_IP |
| PDN type |  | ‘110’B | Ethernet | Ethernet |
| ESM information transfer flag |  | Not present or any allowed value |  |  |
| Access point name |  | Not present | This IE shall not be included when the PDN CONNECTIVITY REQUEST message is sent together with an ATTACH REQUEST message. |  |
| Protocol configuration options |  | Not present or any allowed value | The value received from the UE does not affect the possible verdict associated with the message when received by the SS. The SS shall remember if this IE is present and its contents because this affects subsequent SS behaviour, e.g. coding of ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST. | NOT (pc_NB or pc_NonIP_PDN) |
| Protocol configuration options |  | Not present |  | pc_NB or pc_NonIP_PDN |
| Protocol configuration options |  | Not present |  | Ethernet |
| Protocol configuration options |  | If present including at least the following container |  | Interworking_with_5GS |
| Container ID n |  | ‘001A’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |
| NBIFOM container |  | Not present or any allowed value |  |  |
| Header compression configuration |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | Not present or any allowed value |  |  |
| Extended protocol configuration options |  | If present including at least the following containers |  |  |
| Container ID n |  | ‘001A’H | n assigned to next available number | Interworking_with_5GS |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | PDU session ID | PDU session ID assigned by the UE |  |
| Container ID n+1 |  | ‘0041’H | n assigned to next available number | UAS-UUAA |
| Length of container ID n+1 contents |  |  | 2 octets |  |
| Container ID n+1 contents |  | Service-level-AA container | Service-level-AA container including atleast the following Service-level-AA parameters: Service-level device ID |  |
| Container ID n+1 |  | ‘0041’H | n assigned to next available number | UAS-C2 |
| Length of container ID n+1 contents |  |  | 2 octets |  |
| Container ID n+1 contents |  | Service-level-AA container | Service-level-AA container including atleast the following Service-level-AA parameters: Service-level device ID, Service-level-AA payload type bits set to 10, Service-level-AA payload |  |

| Condition | Explanation |
| --- | --- |
| CIoT_CP_NON_IP | If the UE requests a Non-IP type PDN |
| Interworking_with_5GS | If the UE has indicated support of N1, then the UE shall include this IE to provide details for the interworking with 5GS. Applicable to 5G test cases defined in TS 38.523-1 [71]. Note: The PDU session ID container shall be sent in either the Protocol configuration options or the Extended protocol configuration options IE. |
| Ethernet | Applicable to ethernet testcases. |
| UAS-UAAA | Used in UAS test cases (TS 36.523-1 [18]) If pc_EPC_UAS, for USS UAV Authorization/Authentication |
| UAS-C2 | Used in UAS test cases (TS 36.523-1 [18]) If pc_EPC_UAS, for UAV and UAV-C pairing |

NOTE:	This message is sent without integrity protection before NAS security mode control procedure has been successfully completed and sent within SECURITY PROTECTED NAS MESSAGE message after NAS security mode control procedure has been successfully completed.

<End of modified section>