3GPP TSG-RAN5 Meeting #101	R5-236920

# Chicago, US, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5483 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Missing MU for certain IoT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, CMCC, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | While defining MU and TT for NTN IoT test cases using the general approach of reusing, whenever possible, the MU and TT definition for the equivalent non-NTN NB-IoT test, it has been observed that for a couple of test cases, MU were undefined in annex F despite their TT where indeed defined. In other cases, incorrect test case title has been observed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For test cases 6.2.3F, 6.6.3F.1 and 7.9EA, defined MU in annex F. For 6.7F, updated test case title to include NB2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.2, F.1.3, F.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

## F.1.2	Measurement of transmitter

Table F.1.2-1: Maximum Test System Uncertainty for transmitter tests

| Subclause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2.2 UE Maximum Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.2_1 Maximum Output Power for HPUE | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2 for sum of powers of all CCs with f as the higher one among centre frequencies of 2 CCs. | Overall system uncertainty is maximum of UL measurement uncertainty of 2 CCs. |
| 6.2.2A.4 UE Maximum Output Power for CA (3UL CA) | For UL CA_XD: For BWagg ≤ 40MHz Same as 6.2.2 for sum of powers of all CCs For 40MHz < BWagg ≤ 60MHz ±1.4 dB for f ≤ 4.2GHz ±1.6 dB for 4.2GHz < f ≤ 6.0GHz For UL CA_XC_YA: Same as 6.2.2 for sum of powers of all CCs with f as the higher one among centre frequencies of XC or YA. For UL CA_XA_YA_ZA: FFS | Overall system uncertainty is maximum of UL measurement uncertainty of interband or non-contiguous CCs |
| 6.2.2B UE Maximum Output Power for UL-MIMO | Same as 6.2.2 for each antenna |  |
| 6.2.2B_1 HPUE Maximum Output Power for UL-MIMO | Same as 6.2.2_1 for each antenna |  |
| 6.2.2E UE Maximum Output Power for UE category 0 | Same as 6.2.2 |  |
| 6.2.2EA UE Maximum Output Power for UE category M1 | Same as 6.2.2 |  |
| 6.2.2EB UE Maximum Output Power for UE category 1bis | Same as 6.2.2 |  |
| 6.2.2EC UE Maximum Output Power for UE category M2 | Same as 6.2.2 |  |
| 6.2.2F UE Maximum Output Power for UE category NB1 and NB2 | Same as 6.2.2_1 |  |
| 6.2.2FA UE Maximum Output Power for category NB1 and NB2/Power Class 6 | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.2G.1 UE maximum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.2 |  |
| 6.2.2G.2 UE maximum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2G.3 UE maximum output power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.2.2 for sum of powers of all CCs |  |
| 6.2.2G.4.1 UE Maximum Output Power for V2X Communication / Power class 2 / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.2 |  |
| 6.2.2_s UE Maximum Output Power for subslot/slot TTI | Same as 6.2.2 |  |
| 6.2.3 Maximum Power Reduction | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.3_1 Maximum Power Reduction (MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.1_1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.1_2 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.3A.2_1 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.2_2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.3A.3_1 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.3_2 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.2A.2 |  |
| 6.2.3A.4 Maximum Power Reduction (MPR) for CA (3UL CA) | Same as 6.2.3 for sum of powers of all CCs |  |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO | Same as 6.2.3 for each antenna |  |
| 6.2.3_2 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH | Same as 6.2.3 |  |
| 6.2.3E Maximum Power Reduction for UE category 0 | Same as 6.2.3 |  |
| 6.2.3EA Maximum Power Reduction for UE category M1 | Same as 6.2.3 |  |
| 6.2.3EB Maximum Power Reduction for UE category 1bis | Same as 6.2.3 |  |
| 6.2.3_3 Maximum Power Reduction (MPR) for UL 64QAM | Same as 6.2.3 |  |
| 6.2.3_4 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 64QAM | Same as 6.2.3 |  |
| 6.2.3F Maximum Power Reduction (MPR) for category NB1 and NB2 | Same as 6.2.3 |  |
| 6.2.3G.1.1 Maximum Power Reduction (MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.1.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.1.3 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.2.3 for each CC |  |
| 6.2.3G.1.4 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Intra-band contiguous multi-carrier operation | Same as 6.2.3 for sum of power |  |
| 6.2.3G.2.1 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3G.2.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions | Same as 6.2.3 |  |
| 6.2.3_5 Maximum Power Reduction (MPR) for UL 256QAM | Same as 6.2.3 |  |
| 6.2.3_6 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 256QAM | Same as 6.2.3 |  |
| 6.2.3_s Maximum Power Reduction (MPR) for subslot/slot TTI | Same as 6.2.3 |  |
| 6.2.4 UE Maximum Output Power with additional requirements | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE | ±0.7 dB, f ≤ 3.0GHz |  |
| 6.2.4_2 Additional Maximum Power Reduction (A-MPR) for UL 64QAM | Same as 6.2.4 |  |
| 6.2.4_3 Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping | Same as 6.2.4 |  |
| 6.2.4_4 Additional Maximum Power Reduction (A-MPR) for UL 256QAM | Same as 6.2.4 |  |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.1_1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.1_2 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.4A.2_1 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.2.2A.2 |  |
| 6.2.4A.2_2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.2.2A.2 |  |
| 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) | Same as 6.2.4 for sum of powers of all CCs |  |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO | Same as 6.2.4 for each antenna |  |
| 6.2.4E Additional Maximum Power Reduction (A-MPR) for UE category 0 | Same as 6.2.4 |  |
| 6.2.4EA Additional Maximum Power Reduction (A-MPR) for UE category M1 | Same as 6.2.4 |  |
| 6.2.4EB Additional Maximum Power Reduction (A-MPR) for UE category 1bis | Same as 6.2.4 |  |
| 6.2.4EC Additional Maximum Power Reduction (A-MPR) for UE category M2 | Same as 6.2.4EA |  |
| 6.2.4G.1 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.2.4G.2 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±1.3 dB for sum of power | Maximum MU within both carriers. |
| 6.2.4_s Additional Maximum Power Reduction (A-MPR) for subslot/slot TTI | Same as 6.2.4 |  |
| 6.2.5 Configured UE transmitted Output Power | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.5_s Configured UE Transmitted Output Power for subslot/slot TTI | Same as 6.2.5 |  |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE | Same as 6.2.5 |  |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.2.5 for sum of powers of all CCs |  |
| 6.2.5A.1_3 Configured UE transmitted Output Power for CA and HPUE (intra-band contiguous DL CA and UL CA) for HPUE | Same as 6.2.5 for sum of powers of all CCs |  |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.5A.4 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.2.2A.2 |  |
| 6.2.5B Configured UE transmitted output power for UL-MIMO | Same as 6.2.5 for each antenna |  |
| 6.2.5E Configured Transmitted Power for UE category 0 | Same as 6.2.5 |  |
| 6.2.5EA Configured UE transmitted Power for UE category M1 | Same as 6.2.5 |  |
| 6.2.5EB Configured UE transmitted Power for UE category 1bis | Same as 6.2.5 |  |
| 6.2.5F Configured UE transmitted Output Power for UE category NB1 and NB2 | ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.2.5G.1 Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission | ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for -40 dBm <= UE powers <0dBm |  |
| 6.2.5G.2 Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for -40 dBm <= UE powers <0dBm |  |
| 6.2.5G.3 Configured UE transmitted Output Power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.2.5G.1 for each CC |  |
| 6.3.2 Minimum Output Power | ±1.0 dB, f ≤ 3.0GHz ±1.3 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.2 for each CC |  |
| 6.3.2A.4 Minimum Output Power for CA (3UL CA) | Same as 6.3.2 for each CC |  |
| 6.3.2B Minimum Output Power for UL-MIMO | Same as 6.3.2 for each antenna |  |
| 6.3.2E Minimum Output Power for UE category 0 | Same as 6.3.2 |  |
| 6.3.2EA Minimum Output Power for UE category M1 | Same as 6.3.2 |  |
| 6.3.2EB Minimum Output Power for UE category 1bis | Same as 6.3.2 |  |
| 6.3.2F Minimum Output Power for UE category NB1 and NB2 | ±1.0 dB, f ≤ 3.0GHz ±1.3 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.2G.1 Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.3.2 |  |
| 6.3.2G.2 Minimum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.3.2 |  |
| 6.3.2G.3 Minimum output power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.2 |  |
| 6.3.3 Transmission ON/OFF Power | Transmission OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3A.2 UE Transmit OFF power for CA (inter-band DL CA and UL CA) | Same as 6.3.3A.1 |  |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3A.4 UE Transmit OFF power for CA (3UL CA) | Same as 6.3.3 for each CC |  |
| 6.3.3B UE Transmit OFF power for UL-MIMO | Same as 6.3.3 for each antenna |  |
| 6.3.3D UE Transmit OFF power for ProSe | Same as 6.3.3 |  |
| 6.3.3E UE Transmit OFF Power for UE category 0 | Same as 6.3.3 |  |
| 6.3.3EA UE Transmit OFF power for UE category M1 | Same as 6.3.3 |  |
| 6.3.3EB UE Transmit OFF power for UE category 1bis | Same as 6.3.3 |  |
| 6.3.3F Transmit OFF power for category NB1 and NB2 | Same as 6.3.3 |  |
| 6.3.3G.1 UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.3.3 |  |
| 6.3.3G.2 UE Transmit OFF power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.3.3 for each CC |  |
| 6.3.3G.3 UE Transmit OFF power for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.3 for each CC |  |
| 6.3.4.1 General ON/OFF time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4.1_s General ON/OFF time mask for subslot/slot TTI | Same as 6.3.4.1 |  |
| 6.3.4.2 PRACH and SRS time mask | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) | Same as 6.3.4A.1.1 |  |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.4 General ON/OFF time mask for CA (3UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.5 General ON/OFF time mask for CA (4UL CA) | Same as 6.3.4.1 for each CC |  |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO | Same as 6.3.4.1 for each antenna |  |
| 6.3.4C.1 General ON/OFF time mask for Dual Connectivity | Same as 6.3.4A.1.1 |  |
| 6.3.4E.1 General ON/OFF time mask for UE category 0 | Same as 6.3.4.1 |  |
| 6.3.4E.2 PRACH and SRS time mask for UE category 0 | Same as 6.3.4.2 |  |
| 6.3.4EA.1 General ON/OFF time mask for UE category M1 | Same as 6.3.4.1 |  |
| 6.3.4EA.2 PRACH and SRS ON/OFF time mask for UE category M1 | Same as 6.3.4.2 |  |
| 6.3.4EB.1 General ON/OFF time mask for UE category 1bis | Same as 6.3.4.1 |  |
| 6.3.4EB.2 PRACH and SRS ON/OFF time mask for UE category 1bis | Same as 6.3.4.2 |  |
| 6.3.4EC.2 PRACH and SRS ON/OFF time mask for UE category M2 | Same as 6.3.4.2 |  |
| 6.3.4F.1 General ON/OFF time mask for category NB1 and NB2 | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4F.2 NPRACH time mask for category NB1 and NB2 | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.4G.1 General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.4G.2 General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Transmission ON/OFF Power: ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.3.4G.3 General ON/OFF time mask for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.3.4G.1 for each CC |  |
| 6.3.4G.4 PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.3.5.1 Power Control Absolute power tolerance | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5.2 Power Control Relative power tolerance | ±0.7 dB |  |
| 6.3.5.3 Aggregate power control tolerance | ±0.7 dB |  |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE | ±0.7 dB |  |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE | ±0.7 dB |  |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.2 Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.4 Power Control Absolute power tolerance for CA (3UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.1.5 Power Control Absolute power tolerance for CA(SRS carrier based switching) | Same as 6.3.5.1 for SCC |  |
| 6.3.5A.1.6 Power Control Absolute power tolerance for CA (4UL CA) | Same as 6.3.5.1 for each CC |  |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) | Absolute power ±1dB, f ≤ 4.2GHz Relative power ±0.7 dB Both requirements apply for each CC | Absolute power is used to check when power is near lower and upper limits of the range. Relative power is used to check step sizes. |
| 6.3.5A.2.4.1 Power Control Relative power tolerance for CA (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.2 Power Control Relative power tolerance for CA (inter-band 3DL CA and 3UL CA) | Same as 6.3.5A.2.2 |  |
| 6.3.5A.2.4.3 Power Control Relative power tolerance for CA (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.3.5A.2.3 |  |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.3 Aggregate power control tolerance for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.4 Aggregate Power Control Tolerance for CA (3UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5A.3.5 Aggregate Power Control Tolerance for CA (4UL CA) | Same as 6.3.5.3 for each CC |  |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO | Same as 6.3.5.1 for each antenna |  |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO | Same as 6.3.5.2 for each antenna |  |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO | Same as 6.3.5.3 for each antenna |  |
| 6.3.5C.2 Power Control Relative power tolerance for Dual Connectivity | Same as 6.3.5A.2.1 |  |
| 6.3.5C.2_1 Power Control Relative power tolerance for asynchronous Dual Connectivity | Same as 6.3.5A.2.1 |  |
| 6.3.5E.1 Power Control Absolute power tolerance for UE category 0 | Same as 6.3.5.1 |  |
| 6.3.5E.2 Power Control Relative power tolerance for UE category 0 | Same as 6.3.5.2 |  |
| 6.3.5E.3 Aggregate power control tolerance for UE category 0 | Same as 6.3.5.3 |  |
| 6.3.5EA.1 Power Control Absolute power tolerance for UE category M1 | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB |
| 6.3.5EA.2 Power Control Relative power tolerance for UE category M1 | ±0.7 dB |  |
| 6.3.5EA.3 Aggregate power control tolerance for UE category M1 | Same as 6.3.5.3 |  |
| 6.3.5EA.3_1 Aggregate power control tolerance for UE category M1 (CE Mode B) | Same as 6.3.5.3 |  |
| 6.3.5EB.1 Power Control Absolute power tolerance for UE category 1bis | Same as 6.3.5.1 |  |
| 6.3.5EC.3_1 Aggregate power control tolerance for UE category M2 (CE Mode B) | Same as 6.3.5.1 |  |
| 6.3.5F.1 Power Control Absolute power tolerance for category NB1 and NB2 | ±1.0 dB, f ≤ 3.0GHz ±1.4 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 3.0GHz < f ≤ 4.2GHz DL signal level uncert ± 1.0dB UL meas’t uncert ± 1.0dB |
| 6.3.5F.2 Power Control Relative power tolerance for category NB1 and NB2 | ±0.7 dB |  |
| 6.3.5F.3 Aggregate power control tolerance for category NB1 and NB2 | ±0.7 dB |  |
| 6.3.5G.1 Power Control Absolute power tolerance for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±1.0 dB, f ≤ 3.0GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] f ≤ 3.0GHz DL signal level uncert ± 0.7dB UL meas’t uncert ± 0.7dB 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.3.5G.3 Power Control Absolute power tolerance for V2X Communication / Intra-band contiguous multi-carrier operation | ±2.0 dB, 4.2GHz < f ≤ 6GHz | Overall system uncertainty comprises two quantities: 1. Downlink signal level uncertainty 2. Uplink level measurement uncertainty Items 1 and 2 are assumed to be uncorrelated so can be root sum squared. Test System uncertainty = [SQRT (DL level uncert 2 + UL measurement uncert 2)] 4.2GHz < f ≤ 6GHz DL signal level uncert ± 1.5dB UL meas’t uncert ± 1.3dB |
| 6.5.1 Frequency Error | ±15 Hz DL Signal level: ±0.7 dB, f ≤ 3.0GHz DL Signal level: ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.2 Frequency error for CA (inter-band DL CA and UL CA) | Same as 6.5.1 |  |
| 6.5.1A.3 Frequency error for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.4 Frequency error for CA (3UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1A.5 Frequency error for CA (4UL CA) | Same as 6.5.1 for each CC |  |
| 6.5.1B Frequency Error for UL-MIMO | Same as 6.5.1 for each antenna DL signal level same as 6.5.1 |  |
| 6.5.1D.1 Frequency Error for ProSe Direct Discovery | ±15 Hz Sidelink Signal level: ±0.7 dB, f ≤ 3.0GHz |  |
| 6.5.1D.2 Frequency Error for ProSe Direct Communication | Same as 6.5.1D.1 |  |
| 6.5.1E Frequency Error for UE category 0 | Same as 6.5.1 |  |
| 6.5.1EA Frequency Error for UE category M1 | Same as 6.5.1 |  |
| 6.5.1EB Frequency Error for UE category 1bis | Same as 6.5.1 |  |
| 6.5.1EC Frequency Error for UE category M2 | Same as 6.5.1 |  |
| 6.5.1EC_1 Frequency Error for UE category M2 (CE Mode B) | Same as 6.5.1 |  |
| 6.5.1F Frequency Error for category NB1 | ±15 Hz DL Signal level: ±0.7 dB, f ≤ 3.0GHz |  |
| 6.5.1G.1 Frequency Error for V2X Communication / Non-concurrent with E-UTRA uplink transmission | ±36 Hz, f > 3.0GHz |  |
| 6.5.1G.2 Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ±15 Hz, f ≤ 3.0GHz ±36 Hz, 4.2GHz <f ≤ 6.0GHz |  |
| 6.5.2.1 Error Vector Magnitude | PUSCH: ± 2.5% PUCCH: ± 2.5% PRACH: ± 2.5% |  |
| 6.5.2.1_1 Error Vector Magnitude (EVM) for UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2.1_2 Error Vector Magnitude (EVM) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_2 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_1 EVM for CA (inter-band DL CA and UL CA) with UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_1 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_1 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_2 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.5.2.1 |  |
| 6.5.2A.1.4 Error Vector Magnitude (EVM) for CA (3UL CA) | Same as 6.5.2.1 |  |
| 6.5.2A.1.5 Error Vector Magnitude (EVM) for CA (4UL CA) | Same as 6.5.2.1 |  |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO | Same as 6.5.2.1 for each antenna |  |
| 6.5.2.1A PUSCH-EVM with exclusion period | PUSCH : ± 2.5% |  |
| 6.5.2.1E.1 Error Vector Magnitude for UE category 0 | Same as 6.5.2.1 |  |
| 6.5.2.1E.2 PUSCH-EVM with exclusion period for UE category 0 | Same as 6.5.2.1A |  |
| 6.5.2.1EA.1 Error Vector Magnitude for UE category M1 | Same as 6.5.2.1 |  |
| 6.5.2.1EA.2 PUSCH-EVM with exclusion period for UE category M1 | Same as 6.5.2.1A |  |
| 6.5.2.1EB Error Vector Magnitude (EVM) for UE category 1bis | Same as 6.5.2.1 |  |
| 6.5.2.1EC.1 Error Vector Magnitude for UE category M2 | Same as 6.5.2.1 |  |
| 6.5.2.1EC.2 PUSCH-EVM with exclusion period for UE category M1 | Same as 6.5.2.1A |  |
| 6.5.2.1F.1 Error Vector Magnitude (EVM) for UE category NB1 | Same as 6.5.2.1 |  |
| 6.5.2.1G.1 Error Vector Magnitude (EVM) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.1 |  |
| 6.5.2.1G.2 Error Vector Magnitude (EVM) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | ± 2.5% |  |
| 6.5.2.1G.3 Error Vector Magnitude (EVM) for V2X Communication / Intra-band contiguous multi-carrier operation | ± 2.5% |  |
| 6.5.2.1_s Error Vector Magnitude (EVM) for subslot/slot TTI | Same as 6.5.2 |  |
| 6.5.2.2 Carrier leakage | ±0.8dB |  |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.2 |  |
| 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) | ±0.8dB |  |
| 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) | ±0.8dB |  |
| 6.5.2A.2.4 Carrier leakage for CA (3UL CA) | ±0.8dB |  |
| 6.5.2A.2.5 Carrier leakage for CA (4UL CA) | ±0.8dB |  |
| 6.5.2.3 In-band emissions for non allocated RB | ±0.8dB |  |
| 6.5.2.4 EVM equalizer Spectrum flatness | ±1.4dB |  |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) | Same as 6.5.2.3 |  |
| 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.4 In-band emissions for non allocated RB for CA (3UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2A.3.5 In-band emissions for non allocated RB for CA (4UL CA) | Same as 6.5.2.3 for each CC |  |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO | TBD |  |
| 6.5.2B.2 Carrier leakage for UL-MIMO | Same as 6.5.2.2 for each antenna |  |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO | Same as 6.5.2.3 for each antenna |  |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO | Same as 6.5.2.4 for each antenna |  |
| 6.5.2.2E Carrier Leakage for UE category 0 | Same as 6.5.2.2 |  |
| 6.5.2.2EA Carrier Leakage for UE category M1 | Same as 6.5.2.2 |  |
| 6.5.2.2EB Carrier leakage for UE category 1bis | Same as 6.5.2.2 |  |
| 6.5.2.2EC Carrier leakage for UE category M2 | Same as 6.5.2.2 |  |
| 6.5.2.2F Carrier leakage for UE category NB1 | Same as 6.5.2.2 |  |
| 6.5.2.2G.1 Carrier leakage for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.2 |  |
| 6.5.2.2G.2 Carrier leakage for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission | Same as 6.5.2.2 |  |
| 6.5.2.2G.3 Carrier leakage for V2X Communication / Intra-band contiguous MCC operation | Same as 6.5.2.2 |  |
| 6.5.2.3E In-band emissions for non allocated RB for UE category 0 | Same as 6.5.2.3 |  |
| 6.5.2.3EA In-band emissions for non allocated RB for UE category M1 | Same as 6.5.2.3 |  |
| 6.5.2.3EB In-band emissions for non allocated RB for UE category 1bis | Same as 6.5.2.3 |  |
| 6.5.2.3EC In-band emissions for non allocated RB for UE category M2 | Same as 6.5.2.3 |  |
| 6.5.2.3F In-band emissions for non allocated RB for UE category NB1 | Same as 6.5.2.3 |  |
| 6.5.2.3G.1 In-band emissions for non-allocated RB for V2X Communication | Emission: ±0.8dB UE transmitted power ±1.3 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm |  |
| 6.5.2.3G.2 Introduction of In-band emissions for non-allocated RB for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | same as 6.5.2.3G.1 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  |
| 6.5.2.3G.3 In-band emissions for non-allocated RB for V2X Communication / Intra-band contiguous MCC operation | same as 6.5.2.3G.1 for cc of E-UTRA V2X sidelink |  |
| 6.5.2.4E EVM equalizer spectrum flatness for UE category 0 | Same as 6.5.2.4 |  |
| 6.5.2.4EA EVM equalizer spectrum flatness for UE category M1 | Same as 6.5.2.4 |  |
| 6.5.2.4EB EVM equalizer spectrum flatness for UE category 1bis | Same as 6.5.2.4 |  |
| 6.5.2.4EC EVM equalizer spectrum flatness for UE category M2 | Same as 6.5.2.4 |  |
| 6.5.2.4G.1 EVM equalizer spectrum flatness for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.5.2.4 |  |
| 6.5.2.4G.3 EVM equalizer spectrum flatness for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.5.2.4 for each cc |  |
| 6.6.1 Occupied bandwidth | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz 20MHz < f ≤ 40MHz: 500kHz |  |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) | Same as 6.6.1 for each CC |  |
| 6.6.1A.3 Occupied bandwidth for CA (intra-band non-contiguous DL CA and UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz |  |
| 6.6.1A.5 Occupied bandwidth for CA (4UL CA) | 1.4MHz, 3MHz: 30kHz 5MHz, 10MHz: 100kHz 15MHz, 20MHz: 300kHz 20MHz < f ≤ 40MHz: 500kHz 40MHz < f ≤ 60MHz: [800kHz] 60MHz < f ≤ 80MHz: [1000kHz] for intra-band contiguous CA Same as 6.6.1 for each CC for inter-band CA |  |
| 6.6.1B Occupied bandwidth for UL-MIMO | Same as 6.6.1 for each antenna |  |
| 6.6.1E Occupied bandwidth for UE category 0 | Same as 6.6.1 |  |
| 6.6.1EA Occupied bandwidth for UE category M1 | Same as 6.6.1 |  |
| 6.6.1EB Occupied bandwidth for UE category 1bis | Same as 6.6.1 |  |
| 6.6.1EC Occupied bandwidth for UE category M2 | Same as 6.6.1 |  |
| 6.6.1G.1 Occupied bandwidth for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.6.1 |  |
| 6.6.1G.2 Occupied bandwidth for V2X Communication / Non-concurrent with E-UTRA uplink transmission | Same as 6.6.1 |  |
| 6.6.1G.3 Occupied bandwidth for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.6.1 |  |
| 6.6.1F Occupied bandwidth for UE category NB1 | 200kHz: FFS |  |
| 6.6.2.1 Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH | Same as 6.6.2.1 |  |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1A.2 Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.1 and 6.6.2.2 |  |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.1 |  |
| 6.6.2.1A.5 Additional Spectrum Emission Mask for CA (4UL) | Same as 6.6.2.1 |  |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO | Same as 6.6.2.1 for each antenna |  |
| 6.6.2.1E Spectrum Emission Mask for UE category 0 | Same as 6.6.2.1 |  |
| 6.6.2.1EA Spectrum Emission Mask for UE category M1 | Same as 6.6.2.1 |  |
| 6.6.2.1EB Spectrum Emission Mask for UE category 1bis | Same as 6.6.2.1 |  |
| 6.6.2.1EC Spectrum Emission Mask for UE category M2 | Same as 6.6.2.1 |  |
| 6.6.2.1F Spectrum Emission Mask for UE category NB1 | Same as 6.6.2.1 |  |
| 6.6.2.1G.1 Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.6.2.1G.2 Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions | same as 6.6.2.1G.1 for V2X Sidelink same as 6.6.2.1G.1 for E-UTRA Uplink |  |
| 6.6.2.1G.3 Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.2.1G.1 |  |
| 6.6.2.2 Additional Spectrum Emission Mask | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.0 dB, 4.2GHz < f ≤ 6.0GHz |  |
| 6.6.2.2_1 Additional Spectrum Emission Mask for UL 64QAM | Same as 6.6.2.2 |  |
| 6.6.2.2_2 Additional Spectrum Emission Mask for UL 256QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) | ±1.5 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.2.2A.1_1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.2A |  |
| 6.6.2.2A.1_2 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.2A |  |
| 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_1 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.6.2.2 |  |
| 6.6.2.2A.3 Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2A.5 Additional Spectrum Emission Mask for CA (4UL CA) | Same as 6.6.2.2 |  |
| 6.6.2.2E Additional Spectrum Emission Mask for UE category 0 | Same as 6.6.2.2 |  |
| 6.6.2.2EA Additional Spectrum Emission Mask for UE category M1 | Same as 6.6.2.2 |  |
| 6.6.2.2EB Additional Spectrum Emission Mask for UE category 1bis | Same as 6.6.2.2 |  |
| 6.6.2.2EC Additional Spectrum Emission Mask for UE category M2 | Same as 6.6.2.2 |  |
| 6.6.2.2G.1 Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.6.2.2 |  |
| 6.6.2.2G.2 Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions | Same as 6.6.2.2 for V2X Sidelink Same as 6.6.2.2 E-UTRA uplink |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 |  |
| 6.6.2.3 Adjacent Channel Leakage power Ratio | ±0.8 dB |  |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE | ±0.8 dB |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_5 Adjacent Channel Leakage power Ratio for UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_6 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) | ±0.8 dB |  |
| 6.6.2.3A.1_1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.1_2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) | ±0.8 dB ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |
| 6.6.2.3A.2_1 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.2_2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_1 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_2 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.2 Adjacent Channel Leakage power Ratio for CA (inter-band 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3A.5 Adjacent Channel Leakage power Ratio for CA (4UL CA) | Same as 6.6.2.3 |  |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO | Same as 6.6.2.3 for each antenna |  |
| 6.6.2.3E Adjacent Channel Leakage power Ratio for UE category 0 | Same as 6.6.2.3 | Same as 6.6.2.3 |
| 6.6.2.3EA Adjacent Channel Leakage power Ratio for UE category M1 | Same as 6.6.2.3 |  |
| 6.6.2.3EB Adjacent Channel Leakage power Ratio for UE category 1bis | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM | Same as 6.6.2.3 |  |
| 6.6.2.3F Adjacent Channel Leakage power Ratio for UE category NB1 | ±0.8 dB |  |
| 6.6.2.3G.1 Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | ±0.8 dB |  |
| 6.6.2.3G. 2 Adjacent Channel Leakage power Ratio for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.6.2.3G.1 |  |
| 6.6.2.3G.3 Adjacent Channel Leakage power Ratio for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.6.2.3G.1 |  |
| 6.6.2.4 Void | Void |  |
| 6.6.3.1 Transmitter Spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |
| 6.6.3.1_1 Transmitter Spurious emissions for Multi-Cluster PUSCH | Same as 6.6.3.1 |  |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.1 Transmitter Spurious emissions for CA (3UL CA) (intra-band contiguous 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.2 Transmitter Spurious emissions for CA (3UL CA) (inter-band 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.4.3 Transmitter Spurious emissions for CA (3UL CA) (intra-band non-contiguous 3DL CA and 3UL CA) | Same as 6.6.3.1 |  |
| 6.6.3.1A.5 Transmitter Spurious emissions for CA (4UL CA) | Same as 6.6.3.1 |  |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO | Same as 6.6.3.1, at each antenna used for transmission | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |
| 6.6.3E.1 Transmitter Spurious emissions for UE category 0 | Same as 6.6.3.1 |  |
| 6.6.3EA.1 Transmitter Spurious emissions for UE category M1 | Same as 6.6.3.1 |  |
| 6.6.3F.1 Transmitter Spurious emissions for category NB1 and NB2 | Same as 6.6.3.1 |  |
| 6.6.3G.1 Transmitter Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |
| 6.6.3G.1_1 Spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.6.3G.1 |  |
| 6.6.3G.2 Spurious emission for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.3G.1 |  |
| 6.6.3G.2_1 Spurious emission band UE co-existence for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions | Same as 6.6.3G.1 |  |
| 6.6.3G.3 Spurious emission for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.3G.1 |  |
| 6.6.3G.3_1 Spurious emission band UE co-existence for V2X Communication / Intra-band contiguous MCC operation | Same as 6.6.3G.1 |  |
| 6.6.3.2 Spurious emission band UE co-existence | ± 2.0 dB for results > -60 dBm, f ≤ 3.0GHz ±2.5 dB, 3.0GHz < f ≤ 4.2GHz ± 3.0 dB for results ≤ -60 dBm, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz |  |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.2 Spurious emission band UE co-existence for CA (inter-band DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.3 Spurious emission band UE co-existence for CA (intra-band non-contiguous DL CA and UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.4 Spurious emission band UE co-existence for CA (3UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.2A.5 Spurious emission band UE co-existence for CA (4UL CA) | Same as 6.6.3.2 |  |
| 6.6.3.3 Additional spurious emissions | 9kHz < f ≤ 4 GHz: ± 2.0 dB NS-07 769 ≤ f ≤ 775 MHz: ± 1.5 dB |  |
| 6.6.3.3_1 Additional spurious emissions for UL 64QAM | Same as 6.6.3.3 |  |
| 6.6.3.3_2 Additional spurious emissions for UL 256QAM | Same as 6.6.3.3 |  |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) | 9kHz < f ≤ 4 GHz: ± 2.0 dB |  |
| 6.6.3.3A.1_1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.1_2 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.2 Additional spurious emissions for CA (inter-band DL CA and UL CA) | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_1 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 64QAM | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_2 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 256QAM | Same as 6.6.3.3 |  |
| 6.6.3.3A.4 Additional spurious emissions for CA (3UL CA) | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.5 Additional spurious emissions for CA (4UL CA) | Same as 6.6.3.3A.1 |  |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO | Same as 6.6.3.2 at each antenna used for transmission |  |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO | Same as 6.6.3.3, at each antenna used for transmission |  |
| 6.6.3E.2 Spurious emission band UE co-existence for UE category 0 | Same as 6.6.3.2 |  |
| 6.6.3E.3 Additional spurious emissions for UE category 0 | 6.6.3.3 |  |
| 6.6.3EA.2 Spurious emission band UE co-existence for UE category M1 | Same as 6.6.3.2 |  |
| 6.6.3EA.3 Additional spurious emissions for UE category M1 | Same as 6.6.3.3 |  |
| 6.6.3EB.1 Transmitter Spurious emissions for UE category 1bis | Same as 6.6.3.2 |  |
| 6.6.3EB.2 Spurious emission band UE co-existence for UE category 1bis | Same as 6.6.3.2 |  |
| 6.6.3EB.3 Additional spurious emissions for UE category 1bis | Same as 6.6.3.3 |  |
| 6.6.3EC.1 Transmitter Spurious emissions for UE category M2 | Same as 6.6.3.1 |  |
| 6.6.3EC.2 Spurious emission band UE co-existence for UE category M2 | Same as 6.6.3.2 |  |
| 6.6.3EC.3 Additional spurious emissions for UE category M2 | Same as 6.6.3.3 |  |
| 6.6.3F.2 Spurious emission band UE co-existence for category NB1 UE | ± 2.0 dB for results > -60 dBm, f ≤ 3.0GHz ± 3.0 dB for results ≤ -60 dBm, f ≤ 3.0GHz |  |
| 6.7 Transmit intermodulation | ± 2.6 dB, f ≤ 3.0GHz ±3.6 dB, 3.0GHz < f ≤ 4.2GHz | Overall system uncertainty comprises four quantities: 1. Wanted signal setting error 2. CW Interferer level error 3. Wanted signal meas. error 4. Intermodulation product measurement error The relative level of the wanted signal and the CW interferer has 2 x effect on the intermodulation product. Items 1, 2, 3 and 4 are assumed to be uncorrelated so can be root sum squared to provide the combined effect. Test System uncertainty = SQRT [(2 x SQRT (Wanted setting_error2 + CW_level_error2)) 2 + Wanted_level_meas error2 + Intermodulation product measurement error2] f ≤ 3.0GHz Wanted signal setting ± 0.7dB CW Interferer level ± 1.0dB Wanted signal meas ± 0.7dB Intermodulation product measurement error ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal setting ± 1.0dB CW Interferer level ± 1.3dB Wanted signal meas ± 1.0dB Intermodulation product measurement error ± 1.0dB |
| 6.7A.1 Transmit intermodulation for CA (intra-band contiguous DL CA and UL CA) | Same as 6.7, at each antenna used for transmission |  |
| 6.7A.2 Transmit intermodulation for CA (inter-band DL CA and UL CA) | Same as 6.7, at each antenna used for transmission |  |
| 6.7A.2 Transmit intermodulation for CA (4UL CA) | Same as 6.7, at each antenna used for transmission |  |
| 6.7E Transmit intermodulation for UE category 0 | Same as 6.7 |  |
| 6.7EA Transmit intermodulation for UE category M1 | Same as 6.7 |  |
| 6.7EB Transmit intermodulation for UE category 1bis | Same as 6.7 |  |
| 6.7EC Transmit intermodulation for UE category M2 | Same as 6.7 |  |
| 6.7F Transmit intermodulation for UE category NB1 and NB2 | Same as 6.7 |  |
| 6.7G.1 Transmit intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions | Same as 6.7 |  |
| 6.7G.2 Transmit intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions | Same as 6.7 |  |
| 6.7G.3 Transmit intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation | Same as 6.7 |  |
| 6.8B Time alignment error for UL-MIMO | ±25 ns |  |

## F.1.3	Measurement of receiver

Table F.1.3-1: Maximum Test System Uncertainty for receiver tests

| Subclause |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.3 Reference sensitivity power level |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| 7.3_1 Reference sensitivity level with 4 Rx antenna ports |  | Same as 7.3 |  |  |  |
| 7.3A.1 Reference sensitivity level for CA (intra-band contiguous DL CA and UL CA) |  | Same as 7.3 for each CC |  |  |  |
| 7.3A.2 Reference sensitivity level for CA (intra-band contiguous DL CA without UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.3 Reference sensitivity level for CA (inter-band DL CA without UL CA) |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.3A.4 Reference sensitivity level for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.5 Reference sensitivity level for 3DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3A.6 Reference sensitivity level for CA (inter-band DL CA and UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.7 Reference sensitivity level for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 7.3A.1 |  |  |  |
| 7.3A.9 Reference sensitivity level for 4DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3A.10 Reference sensitivity level for 5DL CA |  | Same as 7.3A.3 |  |  |  |
| 7.3A.11 Reference sensitivity level for 6DL CA |  | Same as 7.3A.1 |  |  |  |
| 7.3B Reference Sensitivity Level for UL-MIMO |  | Same as 7.3 |  |  |  |
| 7.3D.1 Reference sensitivity level for ProSe Direct Discovery |  | Sidelink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3D.2 Reference sensitivity level for ProSe Direct Communication |  | Same as 7.3D.1 |  |  |  |
| 7.3E Reference Sensitivity Level for UE category 0 |  | Same as 7.3 |  |  |  |
| 7.3EA Reference Sensitivity Level for UE category M1 |  | Same as 7.3 |  |  |  |
| 7.3EB Reference sensitivity level for UE category 1bis |  | Same as 7.3 |  |  |  |
| 7.3EC Reference Sensitivity Level for UE category M2 |  | Same as 7.3 |  |  |  |
| 7.3F.1 Reference sensitivity level without repetitions for UE category NB1 and NB2 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3F.2 Reference sensitivity level with repetitions for category NB1 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.3G.1 Reference sensitivity level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.3G.2 Reference sensitivity level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA |  |  |  |
| 7.3G.3 Reference sensitivity level for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.3G.1 |  |  |  |
| 7.4 Maximum input level |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| 7.4_1 Maximum input level with 4 Rx antenna ports |  | Same as 7.4 |  |  |  |
| 7.4A.1 Maximum input level for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.4 for each CC |  | Uncertainties apply for each CC |  |
| 7.4A.1 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA and UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.2 Maximum input level for CA (intra band contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.2 H Maximum input level for SCE-L1 for CA (intra-band contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.3 Maximum input level for CA (inter-band DL CA without UL CA) |  | Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.5 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.3_H Maximum input level for CA (inter-band DL CA without UL CA) for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.4 Maximum input level for CA (intra band non-contiguous DL CA without UL CA) |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.5 Maximum input level for 3DL CA |  | Downlink power same as 7.4A.3 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.5_H Maximum input level for SCE-L1 for 3DL CA |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.7 Maximum input level for 4DL CA |  | Downlink power same as 7.4A.3 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.7_H Maximum input level for 4DL CA for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.8 Maximum input level for 5DL CA |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.8_H Maximum input level for 5DL CA for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.9 Maximum input level for 6DL CA |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4A.9_H Maximum input level for 6DL CA for 256QAM in DL |  | Downlink power same as 7.4 for each CC Uplink power measurement same as 7.4 |  | Uncertainties apply for each CC |  |
| 7.4B Maximum Input Level for UL-MIMO |  | Downlink power same as 7.4 Uplink power measurement same as 7.4, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.4D.1 Maximum input level for ProSe Direct Discovery |  | Sidelink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.4D.2 Maximum input level for ProSe Direct Communication |  | Same as 7.4D.1 |  |  |  |
| 7.4E Maximum input level for UE category 0 |  | Same as 7.4 |  |  |  |
| 7.4EA Maximum input level for UE category M1 |  | Same as 7.4 |  |  |  |
| 7.4EB Maximum input level for UE Category 1bis |  | Same as 7.4 |  |  |  |
| 7.4EC Maximum input level for UE category M2 |  | Same as 7.4 |  |  |  |
| 7.4F Maximum input level for UE category NB1 |  | Downlink power ±0.7 dB, f ≤ 3.0GHz |  |  |  |
| 7.4G.1 Maximum input level for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz |  |  |  |
| 7.4G.2 Maximum input level for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Downlink power ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink Downlink power ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA |  |  |  |
| 7.4G.3 Maximum input level for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.4G.1 |  |  |  |
| 7.4_H Maximum input level for SCE-L1 |  | Same as 7.4 |  |  |  |
| 7.5 Adjacent Channel Selectivity (ACS) |  | ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB |  |
| 7.5_1 Adjacent Channel Selectivity (ACS) with 4 Rx antenna ports |  | Same as 7.5 |  |  |  |
| 7.5A.1 Adjacent Channel Selectivity (ACS) for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.5 for each CC |  | Same as 7.5 The wanted signal level uncertainty applies for each CC. Overall ACS uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.5A.2 Adjacent Channel Selectivity (ACS) for CA (intra band contiguous DL CA without UL CA) |  | Downlink power same as 7.5 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.3 Adjacent Channel Selectivity (ACS) for CA (inter band DL CA without UL CA) |  | ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz ±2.2 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | The wanted signal level uncertainty applies for each CC Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB f ≤ 6GHz Impact of interferer ACLR 0.1dB |  |
| 7.5A.4 Adjacent Channel Selectivity (ACS) for CA (intra-band non-contiguous DL CA without UL CA) |  | Downlink power same as 7.5 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.5 Adjacent Channel Selectivity (ACS) for 3DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.7 Adjacent Channel Selectivity (ACS) for 4DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.8 Adjacent Channel Selectivity (ACS) for 5DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5A.9 Adjacent Channel Selectivity (ACS) for 6DL CA |  | Downlink power same as 7.5A.3 Uplink power measurement same as 7.5 |  | The wanted signal level uncertainty applies for each CC |  |
| 7.5B Adjacent Channel Selectivity (ACS) for UL-MIMO |  | ACS value same as 7.5 Uplink power measurement same as 7.5, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.5D.1 Adjacent channel selectivity (ACS) for ProSe Direct Discovery |  | ACS value ±1.1 dB, f ≤ 3.0GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB |  |
| 7.5D.2 Adjacent channel selectivity (ACS) for ProSe Direct Communication |  | Same as 7.5D.1 |  | Same as 7.5D.1 |  |
| 7.5E Adjacent Channel Selectivity (ACS) for UE category 0 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EA Adjacent Channel Selectivity (ACS) for UE category M1 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EB Adjacent Channel Selectivity (ACS) for UE Category 1bis |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5EC Adjacent Channel Selectivity (ACS) for UE category M2 |  | Same as 7.5 |  | Same as 7.5 |  |
| 7.5F Adjacent Channel Selectivity (ACS) for UE category NB1 |  | ACS value ±1.1 dB, f ≤ 3.0GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.1 Adjacent channel selectivity (ACS) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | ACS ±2.2 dB, 4.2GHz < f ≤ 6GHz |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.2 Adjacent channel selectivity (ACS) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | ACS ±2.2 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink ACS value ±1.1 dB, f ≤ 3.0GHz ±1.5 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink |  | Overall ACS uncertainty comprises three quantities: 1. Wanted signal level error 2. Interferer signal level error 3. Additional impact of interferer ACLR Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The interferer ACLR effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect. f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Impact of interferer ACLR 0.1dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB Impact of interferer ACLR 0.1dB |  |
| 7.5G.3 Adjacent channel selectivity (ACS) for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.5G.1 |  | Same as 7.5G.1 |  |
| 7.6.1 In-band blocking |  | Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB f ≤ 4.2GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1_1 In-band blocking with 4 Rx antenna ports |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1A.1 In-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.1 for each CC |  | Same as 7.6.1 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.1A.2 In-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.3 In-band blocking for CA (inter band DL CA without UL CA) |  | Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz ±2.5 dB, 4.2GHz < f ≤ 6.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level ± 1.5dB f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1A.4 In-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.1A.1 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.5 In-band blocking for 3DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.7 In-band blocking for 4DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.8 In-band blocking for 5DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.9 In-band blocking for 6DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1A.10 In-band blocking for 7DL CA |  | Same as 7.6.1A.3 |  | Same as 7.6.1A.1 |  |
| 7.6.1B In-band blocking for UL-MIMO |  | Blocking same as 7.6.1 Uplink power measurement same as 7.6.1, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.1D.1 In-band blocking for ProSe Direct Discovery |  | Blocking ±1.4 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB f ≤ 4.2GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1D.2 In-band blocking for ProSe Direct Communication |  | Same as 7.6.1D.1 |  | Same as 7.6.1D.1 |  |
| 7.6.1E In-band blocking for UE category 0 |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1EB In-band blocking for UE Category 1bis |  | Same as 7.6.1 |  | Same as 7.6.1 |  |
| 7.6.1F In-band blocking for category NB1 and NB2 |  | Blocking ±1.4 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB |  |
| 7.6.1G.1 In-band blocking for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Blocking ±2.5 dB, 4.2GHz < f ≤ 6.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.5dB 4.2GHz < f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.1G.2 In-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Blocking ±2.5 dB, 4.2GHz < f ≤ 6.0GHz for V2X sidelink Blocking ±1.4 dB, f ≤ 3.0GHz ±1.8 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz for E-UTRA ±1.5 dB, 4.2GHz < f ≤ 6GHz for V2X sidelink |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.5dB f ≤ 4.2GHz, 4.2GHz < f ≤ 6.0GHz Interferer ACLR 0.4dB Broadband noise not applicable |  |
| 7.6.1G.3 In-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.6.1G.1 |  | Same as 7.6.1G.1 |  |
| 7.6.1EA In-band blocking for UE category M1 |  | Blocking ±1.4 dB, f ≤ 3.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz |  | Overall blocking uncertainty can have these contributions: 1. Wanted signal level error 2. Interferer signal level error 3. Interferer ACLR 4. Interferer broadband noise Items 1 and 2 are assumed to be uncorrelated so can be root sum squared to provide the ratio error of the two signals. The Interferer ACLR or Broadband noise effect is systematic, and is added arithmetically. Test System uncertainty = [SQRT (wanted_level_error2 + interferer_level_error2)] + ACLR effect + Broadband noise effect. In-band blocking, using modulated interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB |  |
| 7.6.1EC In-band blocking for UE category M2 |  | Same as 7.6.1EA |  | Same as 7.6.1EA |  |
| 7.6.2 Out of-band blocking |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB Wanted signal 3.0GHz < f ≤ 4.2GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.5 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.3 dB Uplink power measurement ±1.0 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2_1 Out of-band blocking with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2A.1 Out-of-band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.2 for each CC |  | Same as 7.6.2 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.2A.2 Out-of-band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.3 Out-of-band blocking for CA (inter band DL CA without UL CA) |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB Wanted signal 3.0GHz < f ≤ 4.2GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.5 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.3 dB Uplink power measurement ±1.0 dB Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2A.4 Out-of-band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.5 Out-of-band blocking for 3DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.7 Out-of-band blocking for 4DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.8 Out-of-band blocking for 5DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.9 Out-of-band blocking for 6DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2A.10 Out-of-band blocking for 7DL CA |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.6.2B Out-of-band blocking for UL-MIMO |  | Blocking same as 7.6.2 Uplink power measurement same as 7.6.2, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.2D.1 Out-of-band blocking for ProSe Direct Discovery |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB ±0.7 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2D.2 Out-of-band blocking for ProSe Direct Communication |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.6.2E Out of-band blocking for UE category 0 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2F Out of-band blocking for category NB1 and NB2 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.1 Out-of-band blocking for V2X Communication/ Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.2 Out-of-band blocking for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2G.3 Out-of-band blocking for V2X Communication / Intra-band contiguous multi-carrier operation |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2EA Out-of-band blocking for UE category M1 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB Uplink power measurement ±0.7 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.2EB Out-of-band blocking for UE category 1bis |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.2EC Out-of-band blocking for UE category M2 |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.6.3 Narrow band blocking |  | Blocking ±1. 1 dB, f ≤ 3.0GHz ±1. 5 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB Interferer signal level ± 1.0dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3_1 Narrow band blocking with 4 Rx antenna ports |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3A.1 Narrow band blocking for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.3 for each CC |  | Same as 7.6.3 The wanted signal level uncertainty applies for each CC. Overall blocking uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.6.3A.2 Narrow band blocking for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.3 Narrow band blocking for CA (inter band DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.4 Narrow band blocking for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.5 Narrow band blocking for 3DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.7 Narrow band blocking for 4DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.8 Narrow band blocking for 5DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.9 Narrow band blocking for 6DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3A.10 Narrow band blocking for 7DL CA |  | Same as 7.6.3A.1 |  | Same as 7.6.3A.1 |  |
| 7.6.3B Narrow band blocking for UL-MIMO |  | Blocking same as 7.6.3 Uplink power measurement same as 7.6.3, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.6.3D.1 Narrow band blocking for ProSe Direct Discovery |  | Blocking ±1.1 dB, f ≤ 3.0GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3D.2 Narrow band blocking for ProSe Direct Communication |  | Same as 7.6.3D.1 |  | Same as 7.6.3D.1 |  |
| 7.6.3E Narrow band blocking for UE category 0 |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EA Narrow band blocking for UE category M1 |  | Blocking ±1. 1 dB, f ≤ 3.0GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz |  | Narrow band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 0.7dB Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.6.3EB Narrow band blocking for UE category 1bis |  | Same as 7.6.3 |  | Same as 7.6.3 |  |
| 7.6.3EC Narrow band blocking for UE category M2 |  | Same as 7.6.3EA |  | Same as 7.6.3EA |  |
| 7.7 Spurious response |  | Same as 7.6.2 |  | Same as 7.6.2. |  |
| 7.7_1 Spurious response with 4 Rx antenna ports |  | Same as 7.6.2 |  | Same as 7.6.2 |  |
| 7.7A.1 Spurious response for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.2 Spurious response for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.3 Spurious response for CA (inter band DL CA without UL CA) |  | Same as 7.6.2A.3 |  | Same as 7.6.2A.1 |  |
| 7.7A.4 Spurious response for CA (intra-band non-contiguous DL CA without UL CA) |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.5 Spurious response for 3DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.7 Spurious response for 4DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.8 Spurious response for 5DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7A.9 Spurious response for 6DL CA |  | Same as 7.6.2A.1 |  | Same as 7.6.2A.1 |  |
| 7.7B Spurious response for UL-MIMO |  | Same as 7.7 Uplink power measurement same as 7.6.2, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.7D.1 Spurious response for ProSe Direct Discovery |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.7D.2 Spurious response for ProSe Direct Communication |  | Same as 7.6.2D.1 |  | Same as 7.6.2D.1 |  |
| 7.7E Spurious response for UE category 0 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EA Spurious response for UE category M1 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EB Spurious response for UE Category 1bis |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7EC Spurious response for UE category M2 |  | Same as 7.7 |  | Same as 7.7 |  |
| 7.7F Spurious response for category NB1 |  | Wanted signal f ≤ 3.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.3 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.2 dB |  | Out of band blocking, using CW interferer: f ≤ 3.0GHz Wanted signal level ± 0.7dB Interferer signal level: ± 1.0dB up to 3GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.7G.1 Spurious response for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB |  | Out of band blocking, using CW interferer: 4.2GHz < f ≤ 6.0GHz Wanted signal level ± 1.5dB Interferer signal level: ± 1.0dB up to 3GHz ± 3.0dB up to 12.75GHz Interferer ACLR not applicable Impact of interferer Broadband noise 0.1dB Figures are combined to give Test System uncertainty, using formula given for 7.6.1 |  |
| 7.7G.2 Spurious response for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Wanted signal 4.2GHz < f ≤ 6.0GHz for V2X sidelink Blocking, 1MHz < finterferer ≤ 3 GHz: ±1.9 dB Blocking, 3 GHz < finterferer ≤ 12.75 GHz: ±3.5 dB Same as 7.6.2 for E-UTRA |  | Same as 7.7G.1 Same as 7.6.2 |  |
| 7.7G.3 Spurious response for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 7.7G.1 |  | Same as 7.7G.1 |  |
| 7.8.1 Wide band intermodulation |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2.6 dB, 3.0GHz < f ≤ 4.2GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB CW Interferer level ± 0.8dB Mod Interferer level ± 1.0dB |  |
| 7.8.1_1 Wide band Intermodulation with 4 Rx antenna ports |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1A.1 Wideband intermodulation for CA (intra band contiguous DL CA and UL CA) |  | Same as 7.8.1 for each CC |  | Same as 7.8.1 The wanted signal level uncertainty applies for each CC. Overall intermodulation uncertainty calculation includes the uncertainty for wanted level error only once, as the uncertainty of other CCs is not expected to have any significant effect. |  |
| 7.8.1A.2 Wideband intermodulation for CA (intra band contiguous DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.3 Wideband intermodulation for CA (inter band DL CA without UL CA) |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2.6 dB, 3.0GHz < f ≤ 4.2GHz ±3.8 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB 3.0GHz < f ≤ 4.2GHz Wanted signal level ± 1.0dB CW Interferer level ± 0.8dB Mod Interferer level ± 1.0dB 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.3dB Mod Interferer level ± 1.5dB |  |
| 7.8.1A.4 Wideband intermodulation for CA (intra band non-contiguous DL CA without UL CA) |  | Same as 7.8.1A.1 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.5 Wideband intermodulation for 3DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.7 Wideband intermodulation for 4DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.8 Wideband intermodulation for 5DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1A.9 Wideband intermodulation for 6DL CA |  | Same as 7.8.1A.3 |  | Same as 7.8.1A.1 |  |
| 7.8.1B Wide band intermodulation for UL-MIMO |  | Intermodulation same as 7.8.1 Uplink power measurement same as 7.8.1, at each antenna connector used for transmission |  | The overall UL power is the linear sum of the output powers over all Tx antenna connectors |  |
| 7.8.1D.1 Wide band Intermodulation for ProSe Direct Discovery |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2 |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB |  |
| 7.8.1D.2 Wide band Intermodulation for ProSe Direct Communication |  | Same as 7.8.1D.2 |  | Same as 7.8.1D.2 |  |
| 7.8.1E Wide band intermodulation for UE category 0 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EA Wide band Intermodulation for UE category M1 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EB Wide band Intermodulation for UE category 1bis |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1EC Wide band Intermodulation for UE category M2 |  | Same as 7.8.1 |  | Same as 7.8.1 |  |
| 7.8.1F Wide band intermodulation for category NB1 |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] f ≤ 3.0GHz Wanted signal level ± 0.7dB CW Interferer level ± 0.5dB Mod Interferer level ± 0.7dB |  |
| 7.8.1G.1 Wide band Intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Intermodulation ±3.8 dB, 4.2GHz < f ≤ 6GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.3dB Mod Interferer level ± 1.5dB |  |
| 7.8.1G.2 Wide band Intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Intermodulation ±1.4 dB, f ≤ 3.0GHz ±2.6 dB, 3.0GHz < f ≤ 4.2GHz ±3.8 dB, 4.2GHz < f ≤ 6GHz Uplink power measurement ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz |  |  |  |
| 7.8.1G.3 Wide band Intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation |  | Intermodulation ±3.8 dB, 4.2GHz < f ≤ 6GHz |  | Overall intermodulation uncertainty comprises three quantities: 1. Wanted signal level error 2. CW Interferer level error 3. Modulated Interferer level error Effect of interferer ACLR has not been included as modulated interferer has larger frequency offset The effect of the closer CW signal has twice the effect. Items 1, 2 and 3 are assumed to be uncorrelated so can be root sum squared to provide the combined effect of the three signals. Test System uncertainty = SQRT [(2 x CW_level_error)2 +(mod interferer_level_error)2 +(wanted signal_level_error)2] 4.2GHz < f ≤ 6GHz Wanted signal level ± 1.5dB CW Interferer level ± 1.3dB Mod Interferer level ± 1.5dB |  |
| 7.9 Spurious emissions |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB |  |  |  |
| 7.9_1 Spurious emissions with 4 Rx antenna ports |  | Same as 7.9 |  |  |  |
| 7.9A Spurious emissions for CA |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |  |  |
| 7.9E Spurious emissions for UE category 0 |  | Same as 7.9 |  |  |  |
| 7.9EA Spurious emissions for UE category M1 |  | Same as 7.9 |  |  |  |
| 7.9EB Spurious emissions for UE Category 1bis |  | Same as 7.9 |  |  |  |
| 7.9EC Spurious emissions UE category M2 |  | Same as 7.9 |  |  |  |
| 7.9F Spurious emissions for category NB1 |  | Same as 7.9 |  |  |  |
| 7.9G.1 Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 30MHz ≤ f ≤ 4.0GHz: ± 2.0 dB 4 GHz < f ≤ 19 GHz: ± 4.0 dB 19GHz < f ≤ 26 GHz: ± 6.0 dB |  |  |  |
| Note 1: Unless otherwise noted, only the Test System stimulus error is considered here. The effect of errors in the throughput measurements due to finite test duration is not considered. |  |  |  |  |  |

## <<< Skip unchanged sections >>>

## F.3.2	Measurement of transmitter

Table F.3.2-1: Derivation of Test Requirements (Transmitter tests)

| Test |  | Minimum Requirement in TS 36.101 |  |  |  | Test Tolerance (TT) | Test Requirement in TS 36.521-1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.2.2 UE Maximum Output Power |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-3 dB 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-4.0 dB Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2_1 UE Maximum Output Power for HPUE |  | f ≤ 3.0GHz Power class 1: 31dBm +2/-3dB Power class 2: 26dBm ±2dB 3.0GHz < f ≤ 4.2GHz Power class 2: 26 dBm +2/-3 dB |  |  |  | 0.7dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: 31dBm +2.7/-3.7dB Power class 2: 26dBm ±2.7dB Power class 2: 26 dBm +3.0/-4.0 dB |  |
| 6.2.2A.1 UE Maximum Output Power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.2.2 |  |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.2A.2 UE Maximum Output Power for CA (inter-band DL CA and UL CA) |  | Same as 6.2.2 |  |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.2B UE Maximum Output Power for UL-MIMO |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2/-3 dB Power class 4: [FFS] 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm +2/-4 dB |  |  |  | Same as 6.2.2 | Formula: Upper limit +TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm +2.7/-3.7 dB Power class 4: [FFS] Power class 3: 23dBm +3.0/-5.0 dB Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.2B_1 HPUE Maximum Output Power for UL-MIMO |  | f ≤ 3.0GHz Power class 1: 31dBm +2/-3 dB Power class 2: 26dBm +2/-3 dB 3.0GHz < f ≤ 4.2GHz Power class 2: 26dBm +2/-4 dB |  |  |  | Same as 6.2.2 | Formula: Upper limit + TT, Lower limit - TT Power class 1: 31dBm +2.7/-3.7dB Power class 2: 26dBm +2.7/-3.7dB Power class 2: 26 dBm +3.0/-5.0 dB |  |
| 6.2.2E UE Maximum Output Power UE category 0 |  | Same as 6.2.2 |  |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.2EA UE Maximum Output Power UE category M1 |  | f ≤ 3.0GHz Power class 3: 23dBm ±2 dB Power class 5: 20dBm ±2 dB Power class 6: 14dBm ±2.5 dB 3.0GHz < f ≤ 4.2GHz Power class 3: 23dBm ±2 dB Power class 5: 20dBm ±2 dB Power class 6: 14dBm ±2.5 dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±2.7 dB Power class 5: 20dBm ±2.7 dB Power class 6: 14dBm ±3.2 dB Power class 3: 23dBm ±3.0 dB Power class 5: 20dBm ±3.0 dB Power class 6: 14dBm ±3.5 dB |  |
| 6.2.2EB UE Maximum Output Power UE category 1bis |  | Same as 6.2.2 |  |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.2EC UE Maximum Output Power for UE category M2 |  | Same as 6.2.2EA |  |  |  | Same as 6.2.2EA | Same as 6.2.2EA |  |
| 6.2.2F UE Maximum Output Power for UE category NB1 and NB2 |  | f ≤ 3.0GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] Power class 5: 20dBm ±2 dB 3.0GHz < f ≤ 4.2GHz Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2 dB Power class 4: [FFS] Power class 5: 20dBm ±2 dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±2.7 dB Power class 4: [FFS] Power class 5: 20dBm ±2.7 dB Power class 1: [FFS] Power class 2: [FFS] Power class 3: 23dBm ±3.0 dB Power class 4: [FFS] Power class 5: 20dBm ±3.0 dB |  |
| 6.2.2FA UE Maximum Output Power for category NB1 and NB2/Power Class 6 |  | f ≤ 3.0GHz Power class 6: 14dBm ±2.5dB 3.0GHz < f ≤ 4.2GHz Power class 6: 14dBm ±2.5dB |  |  |  | 0.7dB 1.0 dB | Formula: Upper limit + TT, Lower limit - TT Power class 6: 14dBm ±3.2dB Power class 6: 14dBm ±3.5dB |  |
| 6.2.2G.1 UE maximum output power for V2X Commincation / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  |  |  | 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2G.2 UE maximum output power for V2X Commincation / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm +2/-3 dB |  |  |  | 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm +3.3/-4.3 dB |  |
| 6.2.2G.3 UE maximum output power for V2X Commincation / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz Power class 3: 23dBm ±2 dB |  |  |  | 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 3: 23dBm ±3.3 dB |  |
| 6.2.2G.4.1 UE maximum output power for V2X Commincation / Power class 2 / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Power class 2: 26dBm ±2 dB |  |  |  | 1.3 dB | Formula: Upper limit + TT, Lower limit - TT Power class 2: 26dBm ±3.3 dB |  |
| 6.2.2_s UE Maximum Output Power for subslot/slot TTI |  | Same as 6.2.2 |  |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.3 Maximum Power Reduction |  | Power class 3 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 3.7dB 16QAM: 23dBm +2.7 / - 4.7dB QPSK: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 5.0dB 16QAM: 23dBm +3.0 / - 6.0dB |  |
| 6.2.3_1 Maximum Power Reduction for HPUE |  | Power class 1 and 2 f ≤ 3.0GHz QPSK: MPR ≤ 1dB 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB Power class 2: 3.0GHz < f ≤ 4.2GHz QPSK: MPR ≤ 1 dB 16QAM: MPR ≤ 1 dB 16QAM: MPR ≤ 2 dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 1: QPSK: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 4.7dB 16QAM: 31dBm +2.7 / - 5.7dB Power class 2: QPSK: 26dBm +2.7 / - 3.7dB 16QAM: 26dBm +2.7 / - 3.7dB 16QAM: 26dBm +2.7 / - 4.7dB QPSK: 26dBm +3.0 / -5.0 dB 16QAM: 26dBm +3.0 / -5.0 dB 16QAM: 26dBm +3.0 / -6.0 dB |  |
| 6.2.3_2 UE Maximum Output Power for Multi-Cluster PUSCH |  | Same as 6.2.2 |  |  |  | Same as 6.2.2 | Same as 6.2.2 |  |
| 6.2.3_3 Maximum Power Reduction (MPR) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz 64QAM: MPR ≤ 2dB 64QAM: MPR ≤ 3dB 3.0GHz < f ≤ 4.2GHz 64QAM: MPR ≤ 2dB 64QAM: MPR ≤ 3dB |  |  |  | 0.7 dB 0.7 dB 1.0 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: 64QAM: 23dBm +2.7 / - 4.7dB 64QAM: 23dBm +2.7 / - 6.2dB 64QAM: 23dBm +3.0 / - 5.0dB 64QAM: 23dBm +3.0 / - 6.5dB |  |
| 6.2.3_4 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 64QAM |  | Same as 6.2.3_3 |  |  |  | Same as 6.2.3_3 | Same as 6.2.3_3 |  |
| 6.2.3_5 Maximum Power Reduction (MPR) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz 256QAM: MPR ≤ 5dB 3.0GHz < f ≤ 4.2GHz 256QAM: MPR ≤ 5dB |  |  |  | 0.7 dB 1.0 dB | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: 256QAM: 23dBm +2.7 / - 7.7dB 256QAM: 23dBm +3.0 / - 8.0dB |  |
| 6.2.3_6 Maximum Power Reduction (MPR) for Multi-Cluster PUSCH with UL 256QAM |  | Power class 3 f ≤ 3.0GHz 256 QAM: MPR = 5.5dB 256 QAM: MPR = 6 dB 256 QAM: MPR = 7 dB 256 QAM: MPR = 7.5dB 3.0GHz < f ≤ 4.2GHz 256 QAM: MPR = 5.5dB 256 QAM: MPR = 6 dB 256 QAM: MPR = 7 dB 256 QAM: MPR = 7.5dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H_C) + TT, Lower limit: -T(PCMAX_L_C) – MPR – TT PCMAX_H_C = PPowerClass PCMAX_L_C = PPowerClass-MPR Power class 3: 256QAM: 23dBm +2.7 / - 11.2dB 256QAM: 23dBm +2.7 / - 11.7dB 256QAM: 23dBm +2.7 / - 12.7dB 256QAM: 23dBm +2.7 / - 13.2dB 256QAM: 23dBm +2.7 / - 11.5dB 256QAM: 23dBm +2.7 / - 12.0dB 256QAM: 23dBm +2.7 / - 13.0dB 256QAM: 23dBm +2.7 / - 13.5dB |  |
| 6.2.3A.1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB MPR = 10dB MPR = 10.5dB 3.0GHz < f ≤ 4.2GHz MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 8.5dB MPR = 10.0dB MPR = 10.0dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -max(TL,c, T(PCMAX_L )) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: TL,c=2dB 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +2.7 / - 14.2dB 23dBm +2.7 / - 15.7dB 23dBm +2.7 / - 17.2dB TL,c=3dB 23dBm +3.0 / - 4.0dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.0dB 23dBm +3.0 / - 7.5dB 23dBm +3.0 / - 14.5dB 23dBm +3.0 / - 16.0dB 23dBm +3.0 / - 17.5dB |  |
| 6.2.3A.1_1 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 2dB MPR = 3dB 3.0GHz < f ≤ 4.2GHz MPR = 2dB MPR = 3dB |  |  |  | 0.7 dB 0.7 dB 1.0 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 6.2dB 23dBm +3.0 / - 5.0dB 23dBm +3.0 / - 6.5dB |  |
| 6.2.3A.1_2 Maximum Power Reduction (MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 5dB 3.0GHz < f ≤ 4.2GHz MPR = 5dB |  |  |  | 0.7 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 9.7dB 23dBm +3.0 / - 10.0dB |  |
| 6.2.3A.2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB 4.2GHz < f ≤ 6GHz MPR = 0dB MPR = 1dB MPR = 2dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 1.5 dB 1.5 dB 1.5 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +3.5 / - 4.5 dB 23dBm +3.5 / - 5.5 dB 23dBm +3.5 / - 6.5 dB |  |
| 6.2.3A.2_1 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 2dB MPR = 3dB |  |  |  | 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 5.7 dB 23dBm +2.7 / - 6.7 dB |  |
| 6.2.3A.2_2 Maximum Power Reduction (MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 5dB 3.0GHz < f ≤ 4.2GHz MPR = 5dB |  |  |  | 0.7 dB 1.0 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 7.7 dB 23dBm +2.7 / - 9.4 dB 23dBm +3.0 / - 8.0 dB 23dBm +3.0 / - 9.7 dB |  |
| 6.2.3A.3 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·TC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.3_1 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·TC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.3_2 Maximum Power Reduction (MPR) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | Power class 3 f ≤ 3.0GHz MPR = 7 dB MPR = 12 dB MPR = 18 dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR -·DTC dBm) Power class 3: 23dBm +2.7 / - 12.7 dB 23dBm +2.7 / - 18.7 dB 23dBm +2.7 / - 25.7 dB |  |
| 6.2.3A.4 Maximum Power Reduction (MPR) for CA (3UL CA) |  | Power class 3 f ≤ 3.0GHz for intra-band MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 3.5dB 3.0GHz < f ≤ 4.2GHz for intra-band MPR = 0dB MPR = 1dB MPR = 2dB MPR = 3dB MPR = 3.5dB f ≤ 3.0GHz for inter-band MPR = 0dB MPR = 1dB MPR = 1dB MPR = 2dB MPR = 2dB 3.0GHz < f ≤ 4.2GHz for intra-band MPR = 0dB MPR = 1dB MPR = 1dB MPR = 2dB MPR = 2dB |  |  |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1 dB 1 dB 1 dB 1 dB 1 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1 dB 1 dB 1 dB 1 dB 1 dB | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm +2.7 / - 2.7dB 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +2.7 / - 5.2dB 23dBm +2.7 / - 6.2dB 23dBm +3 / -3 dB 23dBm +3 / -4 dB 23dBm +3 / -5 dB 23dBm +3 / -5.5 dB 23dBm +3 / -6.5 dB Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = MIN(linear sum of PPowerClass – MPR c -·TC,c, 23 dBm) Power class 3: 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 5.7 dB 23dBm +2.7 / - 3.7 dB 23dBm +2.7 / - 5.7 dB 23dBm +3 / - 4 dB 23dBm +3 / - 4 dB 23dBm +3 / - 6 dB 23dBm +3 / - 4 dB 23dBm +3 / - 6 dB |  |
| 6.2.3B Maximum Power Reduction (MPR) for UL-MIMO |  | Same as 6.2.3 |  | Same as 6.2.3 |  |  | Same as 6.2.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.3E Maximum Power Reduction for UE category 0 |  | Same as 6.2.3E |  | Same as 6.2.3E |  |  | Same as 6.2.3E |  |
| 6.2.3EA Maximum Power Reduction for UE category M1 |  | Power class 3 f ≤ 3.0GHz MPR ≤ 1dB MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz MPR ≤ 1dB MPR ≤ 2dB Power class 5 f ≤ 3.0GHz MPR ≤ 1dB MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz MPR ≤ 1dB MPR ≤ 2dB Power class 6 f ≤ 3.0GHz MPR = 0dB MPR = 1dB MPR = 2dB 3.0GHz < f ≤ 4.2GHz MPR = 0dB MPR = 1dB MPR = 2dB |  | 0.7 dB 0.7 dB 1.0 dB 1.0 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – MPR – TT 23dBm +2.7 / - 3.7dB 23dBm +2.7 / - 4.7dB 23dBm +3.0 / - 4.0dB 23dBm +3.0 / - 5.0dB 20dBm +2.7 / - 3.7dB 20dBm +2.7 / - 4.7dB 20dBm +3.0 / - 4.0dB 20dBm +3.0 / - 5.0dB 14dBm +3.2 / - 3.2dB 14dBm +3.2 / - 4.2dB 14dBm +3.2 / - 6.2dB 14dBm +3.5 / - 3.5dB 14dBm +3.5 / - 4.5dB 14dBm +3.5 / - 6.5dB |  |
| 6.2.3EB Maximum Power Reduction for UE category 1bis |  | Same as 6.2.3 |  | Same as 6.2.3 |  |  | Same as 6.2.3 |  |
| 6.2.3F Maximum Power Reduction (MPR) for UE category NB1 and NB2 |  | Power class 3: f ≤ 3.0GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB Power class 5: f ≤ 3.0GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB 3.0GHz < f ≤ 4.2GHz BPSK: MPR = 0dB QPSK: MPR = 0dB QPSK: MPR ≤ 0.5dB QPSK: MPR ≤ 1dB QPSK: MPR ≤ 2dB |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: BPSK: 23dBm +2.7 / - 2.7dB QPSK: 23dBm +2.7 / - 2.7dB QPSK: 23dBm +2.7 / - 3.2dB QPSK: 23dBm +2.7 / - 3.7dB QPSK: 23dBm +2.7 / - 4.7dB BPSK: 23dBm +3.0 / - 3.0dB QPSK: 23dBm +3.0 / - 3.0dB QPSK: 23dBm +3.0 / - 3.5dB QPSK: 23dBm +3.0 / - 4.0dB QPSK: 23dBm +3.0 / - 5.0dB Power class 5: BPSK: 20dBm +2.7 / - 3.2dB QPSK: 20dBm +2.7 / - 3.2dB QPSK: 20dBm +2.7 / - 4.7dB QPSK: 20dBm +2.7 / - 5.2dB QPSK: 20dBm +2.7 / - 6.7dB BPSK: 20dBm +3.0 / - 3.5dB QPSK: 20dBm +3.0 / - 3.5dB QPSK: 20dBm +3.0 / - 5.0dB QPSK: 20dBm +3.0 / - 5.5dB QPSK: 20dBm +3.0 / - 7.0dB |  |
| 6.2.3G.1.1 Maximum Power Reduction (MPR) for V2X Commincation / Non-concurrent with E-UTRA uplink transmissions |  | Power class 3: 4.2GHz < f ≤ 6GHz QPSK: MPR ≤ 1.5dB 16QAM: MPR ≤ 2dB |  | 1.3 dB 1.3 dB |  |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 3: QPSK: 23dBm +3.3 / - 4.8dB 16QAM: 23dBm +3.3 / - 5.3dB |  |
| 6.2.3G.1.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 3: 5 RBs/10MHz: MPR ≤4.5dB 25 RBs/10MHz: MPR ≤3.0dB 40 RBs/10MHz: MPR ≤2.0dB 10 RBs/20MHz: MPR ≤4.5dB 50 RBs/20MHz: MPR ≤3.0dB 80 RBs/20MHz: MPR ≤2.0dB |  | 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 3: 23dBm + 3.3 / - 9.8dB 23dBm + 3.3 / - 6.8dB 23dBm + 3.3 / - 5.3dB 23dBm + 3.3 / - 9.8dB 23dBm + 3.3 / - 6.8dB 23dBm + 3.3 / - 5.3dB |  |
| 6.2.3G.1.3 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | Power class 3 MPR ≤ 1.5dB MPR ≤ 2dB |  | 1.3 dB 1.3 dB |  |  | Formula: Upper limit + THIGH (PCMAX_H(p, q)) + TT, Lower limit – TLOW (PCMAX_L(p, q)) – MPR – ΔTIB,c – TT Power class 3: 23dBm +3.3 / -7.8dB 23dBm +3.3 / -8.3dB |  |
| 6.2.3G.1.4 Maximum Power Reduction (MPR) for V2X Communication / Power class 3 / Intra-band contiguous multi-carrier operation |  | Power class 3 MPR ≤ 3dB |  | 1.3 dB |  |  | Formula: Upper limit + THIGH (PCMAX_H(p, q)) + TT, Lower limit – TLOW (PCMAX_L(p, q)) – MPR – ΔTIB,c – TT Power class 3: 23dBm +3.3 / -6.8dB |  |
| 6.2.3G.2.1 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 2: QPSK : MPR ≤ 2dB 16QAM : MPR ≤ 2.5dB |  | 1.3 dB 1.3 dB |  |  | Formula: Upper limit + TT, Lower limit – MPR – TT Power class 2: QPSK : 26dBm +3.3 / -5.3dB 16QAM : 26dBm +3.3 / -5.8dB |  |
| 6.2.3_s Maximum Power Reduction (MPR) for subslot/slot TTI |  | Same as 6.2.3 |  | Same as 6.2.3 |  |  | Same as 6.2.3 |  |
| 6.2.3G.2.2 Maximum Power Reduction (MPR) for V2X Communication / Power class 2 / Non-contiguous allocation of PSCCH and PSSCH / Non-concurrent with E-UTRA uplink transmissions |  | Power class 2 4.2GHz < f ≤ 6GHz For 10MHz BW: NRB_alloc 10: MPR = 4.5dB NRB_alloc 12: MPR = 4dB NRB_alloc 42: MPR = 2.5dB For 20MHz BW: NRB_alloc 10: MPR = 9dB NRB_alloc 22: MPR = 5.5dB NRB_alloc 82: MPR = 3dB |  | 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB 1.3 dB |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-MPR Power class 2: For 10MHz BW: NRB_alloc 10: 26dBm +3.3 / -7.8dB NRB_alloc 12: 26dBm +3.3 / -7.3dB NRB_alloc 42: 26dBm +3.3 / -5.8dB For 20MHz BW: NRB_alloc 10: 26dBm +3.3 / -15.3dB NRB_alloc 22: 26dBm +3.3 / -9.3dB NRB_alloc 82: 26dBm +3.3 / -6.3dB |  |
| 6.2.4 UE Maximum Output Power with additional requirements |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 3: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB For network signalled value NS_03 , NS_04 (5MHz only), NS_05, to NS_06: A-MPR ≤ 1dB For network signalled value NS-04; Depending on the RB_start and RB allocation (10MHz, 15MHz and 20MHz): For 10MHz Region A with RB_start=0 – 12: A-MPR ≤ 3dB. Region B with RB_start=13 – 36 : A-MPR ≤ 2dB. Region C with RB_start=37 – 49 : A-MPR ≤ 3dB. For 15MHz Region A with RB_start=0 – 18: A-MPR ≤ 3dB. Region B with RB_start=19 – 55 : A-MPR ≤ 2dB. Region C with RB_start=56 – 74 : A-MPR ≤ 3dB. For 20MHz Region A with RB_start=0 – 24: A-MPR ≤ 3dB. Region B with RB_start=25 – 74 : A-MPR ≤ 2dB. Region C with RB_start=75 – 99 : A-MPR ≤ 3dB. For network signalled value NS-11; Depending on the RB_start and RB allocation (15MHz and 20MHz): For 15MHz Fc < 2012.5 Region A with RB_start=0 – 4: A-MPR ≤ 15dB. Region B with RB_start=5 – 21: A-MPR ≤ 7dB (LCRB=7-50) or ≤ 10dB (LCRB=0-6 & ≥50). Region C with RB_start=22 – 56: A-MPR ≤ 0dB (LCRB=≤25) or ≤ 6dB (LCRB>25). Region D with RB_start=57 – 74: A-MPR ≤ 15dB. For 15MHz Fc = 2012.5 Region A with RB_start=0 – 12: A-MPR ≤ 10dB. Region B with RB_start=13 – 39: A-MPR ≤ 6dB (LCRB=≥ 30) or ≤ 0dB (LCRB=< 30). Region C with RB_start=40 – 65: A-MPR ≤ 2dB. Region D with RB_start=66 – 74: A-MPR ≤ 6.5dB. For 20MHz Region A with RB_start=0 – 12: A-MPR ≤ 15dB. Region B with RB_start=13 – 29: A-MPR ≤ 7dB (LCRB=10-60) or ≤ 10dB (LCRB=1-9 & > 60). Region C with RB_start=30 – 68: A-MPR ≤ 0dB (LCRB=1-24) or ≤ 7dB (LCRB= ≥ 25). Region D with RB_start=69 – 99: A-MPR ≤ 15dB. For network signalled value NS-20; Depending on the RB_start and RB allocation (5MHZ, 10MHz, 15MHz and 20MHz): For 5MHz Region A with RB_start=≤24: A-MPR ≤ 17dB. Region B with RB_start=0 – 3: A-MPR ≤ 1dB (LCRB=15-19) or ≤ 4dB (LCRB= ≥ 20). Region C with RB_start=4 – 6: A-MPR ≤ 2dB. Region D with RB_start=≤24: A-MPR ≤ 0dB. For 10MHz Fc = 2005 Region A with RB_start=0 – 25: A-MPR ≤ 16dB. Region B with RB_start=26 – 34: A-MPR ≤ 2dB (LCRB=8-15) or ≤ 5dB (LCRB= > 15). Region C with RB_start=35 – 49: A-MPR ≤ 6dB. For 10MHz Fc = 2015 Region A with RB_start=0 – 5: A-MPR ≤ 4dB. Region B with RB_start=6 – 10: A-MPR ≤ 2dB. For 15MHz Region A with RB_start=0 – 14: A-MPR ≤ 11dB (LCRB=1-9 & 40-5) or ≤ 6dB (LCRB= 10-39). Region B with RB_start=15 – 24: A-MPR ≤ 1dB (LCRB=24-29) or ≤ 7dB (LCRB=≥30). Region C with RB_start=25 – 39: A-MPR ≤ 5dB. Region D with RB_start=61 – 74: A-MPR ≤ 6dB. For 20MHz Region A with RB_start=0 – 21: A-MPR ≤ 17dB. Region B with RB_start=22 – 31: A-MPR ≤ 12dB (LCRB=1-9 & 31-75) or ≤ 6dB (LCRB=10-30). Region C with RB_start=32 – 38: A-MPR ≤ 9dB. Region D with RB_start=39 – 49: A-MPR ≤ 7dB. Region E with RB_start=50 – 69: A-MPR ≤ 5dB. Region F with RB_start=70 – 99: A-MPR ≤ 16dB. For network signalled value NS-27; Depending on the RB_start and RB allocation (15MHz and 20MHz): For 15MHz With LCRB ≤15, LRB_start=0 – 6 or RB_end=68-74: A-MPR ≤ 4dB. With LCRB ≥ 60, LRB_start≥0: A-MPR ≤ 2dB. For 20MHz With LCRB ≤20, LRB_start=0 – 12 or RB_end=87-99: A-MPR ≤ 4dB. With LCRB ≤3, LRB_start=13 – 15 or RB_end=84-86: A-MPR ≤ 1dB. With LCRB ≥ 60, LRB_start≥0: A-MPR ≤ 3dB. For network signalled value NS_56; Depending on the RB_start and RB allocation (5MHz and 10MHz): For 5MHz Region A with RB_start≤8 and LCRB ≤8: A-MPR ≤ 8dB. Region B with RB_start≤8 and LCRB >8: A-MPR ≤ 2dB. For 10MHz Region A with RB_start≤5 and LCRB ≤5: A-MPR ≤ 7dB. Region B with RB_start≤18 and LCRB ≤12: A-MPR ≤ 5dB. Region C with RB_start≤18 and LCRB >12: A-MPR ≤ 7dB. Region D with RB_start≥35 and LCRB ≤7: A-MPR ≤ 4dB. Region E with RB_start≥35 and LCRB >7: A-MPR ≤ 2dB. Region F with RB_start≥40 and LCRB ≤7: A-MPR ≤ 5dB. Region G with RB_start≥40 and LCRB >7: A-MPR ≤ 3dB. |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  |  | Formula: Upper limit + TT, A: Lower limit – TT, B: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – MPR – TT, C: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – TT, D: (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT E: (UE Maximum Output Power from 6.2.2) – TL – A-MPR – TT, F: (UE Maximum Output Power from 6.2.2) – TL – A-MPR – MPR – TT Power class 3: Test Requirement Configuration ID versus Formula Above Network signalled value NS_03: [A]: 2, 5, 10, 15, 20, 25 [B]: 1, 3, 7 [C]: 9, 14, 19, 24 [D]: 4, 6, 8, 11, 12, 13, 16, 17, 18, 21, 22, 23, 26, 27 Network signalled value NS_04 (5, 10, 15, 20MHz): [A] 3 [B] 10, 11, 19, 20, 28, 29 [C] 2, 6, 7, 13, 14, 15, 16, 23, 24, 25, 32 [D] 1, 4, 5, 8, 9, 12, 17, 18, 21, 22, 26, 27, 30, 31 Network signalled value NS_05: [A] 1, 3, 4, 8, 9, 14, 15, [B] 2, 5, 10, 11, 16, 17 [C] None [D] 6, 7, 12, 13, 18, 19 Network signalled value NS_06: [A]: 2, 5, 8, 11, 14, 17 [B]: 1, 3, 4, 6, 7, 9, 10, 12, 13, 15, 16, 18 [C]: None [D]: None Network signalled value NS_07: [A]:3, 8, 12 [B]: 7, 9 [C]: 1, 2, 5, 13, 15 [D]: 4, 6, 10, 11, 14, 16 Network signalled value NS_08: [A]:1, 2, 4, 5, 12 [B]: 3, 6, 11, 13 [C]: None [D]: 7, 8, 9, 10, 14, 15, 16, 17 Network signalled value NS_11: [A]: 5c, 6b, 10c, 11c, 25 [B]: 8b, 12c, 13c, 14c, 17a, 17b, 21a, 21b, 28 [C]: 5a, 5b, 6a, 6c, 10a, 10b, 11, 11b, 15a, 15b, 16a, 16b, 24 [D]: 1a, 1b, 2a, 2b, 3a, 3b, 4a, 4b ,, 7a,7b,7c, 8a, 8c, 9a, 9b,9c, 12a, 12b, 13a, 13b, 14a, 14b, 18a, 18b, 19a, 19b, 20a, 20b, 22a, 22b, 23a, 23b, 26, 27, 29, 30 Network signalled value NS_12: [A]: 3, 8, 13 [B]: None [C]: 1, 4, 6, 11 [D]: 2, 5, 7, 9, 10, 12, 14, 15 Network signalled value NS_13: [A]: None [B]: 3, 4 [C]: 1 [D]: 2, 5 Network signalled value NS_14: [A]: None [B]: 2, 4, 7, 9 [C]: 1, 6 [D]: 3, 5, 8, 10 Network signalled value NS_15: [A]: 1, 7, 14 [B]: 3, 6, 10, 15, 19, 21, 22, 25 [C]: 12, 13 [D]: 2, 4, 5, 8, 9, 11, 16, 17, 18, 20, 23, 24, 26 Network signalled value NS_20: [A]: 1b, 1c, 1d, 6b, 7b [B]: 2d, 3d, 4d, 5d, 9b [C]: 1a, 6a, 7a, 11, 18, 19 [D]: 2a, 2b, 2c, 3a, 3b, 3c, 4a, 4b, 4c, 5a, 5b, 5c, 8a, 8b, 9a, 10a, 11b, 12, 13, 14, 15, 16, 17, 20, 21, 22, 23, 24, 25, 26 Network signalled value NS_27: TBD Network signalled value NS_56 A: None B: None C: 1, 4, 5, 8, 9, 10, 11, D: 3, 6, 7, 14 E: 13 F: 2, 12 Network signalled value NS_62 [B]: 1, 2, 3, 4, 5, 6, 7, 8, 9 |  |
| 6.2.4_1 Additional Maximum Power Reduction (A-MPR) for HPUE |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 1: QPSK: MPR ≤ 1dB 16QAM: Depending on the number RB allocated: 16QAM: MPR ≤ 1dB 16QAM: MPR ≤ 2dB Power class 1: For network signalled value NS_06: A-MPR 0dB |  | 0.7dB |  |  | Formula: Upper limit + TT, (UE Maximum Output Power from 6.2.2) - T(PCMAX) – A-MPR – MPR – TT |  |
| 6.2.4_2 Additional Maximum Power Reduction (A-MPR) for UL 64QAM |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.4_3 Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.4_4 Additional Maximum Power Reduction (A-MPR) for UL 256QAM |  | For the UE maximum output power modified by MPR and A-MPR, the power limits specified in TS 36.101 [2] clause 6.2.5 apply. For transmission bandwidths (Figure 5.4.2-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Power class 3: 256QAM: MPR ≤ 5dB A-MPR values as given by tables in section 6.2.4.3. |  | f ≤ 3.0GHz 0.7 dB 3.0GHz < f ≤ 4.2GHz 1.0 dB |  |  | Formula: Upper limit + TT, A: (UE Maximum Output Power from 6.2.2) - T(PCMAX) - MPR - TT, B: (UE Maximum Output Power from 6.2.2) - T(PCMAX) - A-MPR - MPR - TT Power class 3: Test Requirement Configuration ID versus Formula Above Network signalled value NS_03 [A]: 1, 3 [B]: 2, 4, 5, 6, 7, 8, 9, 10, 11 Network signalled value NS_04 [A]: 1, (2, 3, 4, 5) if not low range [B]: (2, 3, 4, 5) for low range Network signalled value NS_05 [A]: 1, 5, 6 [B]: 2, 3, 4, 7 Network signalled value NS_06 [A]: 1, 2, 3 [B]: None Network signalled value NS_07 [A]: 2 [B]: 1, 3, 4 Network signalled value NS_08 [A]: 1 [B]: 2, 3 Network signalled value NS_09 [A]: 1 [B]: 2, 3 Network signalled value NS_11 [A]: 3b, 5c, 6c, 8a with high range RB allocation [B]: 1a, 1b, 2a, 2b, 3a, 3c, 4a, 4b, 4c, 5a, 5b, 6a, 6b, 7a, 7b, 8a with low range RB allocation, 8b, 9a, 9b, 10a, 10b, 11, 12, 13 Network signalled value NS_12 [A]: None [B]: 1, 2, 3, 4 Network signalled value NS_13 [A]: None [B]: 1 Network signalled value NS_14 [A]: None [B]: 1, 2 Network signalled value NS_15 [A]: None [B]: 1, 2, 3, 4, 5 Network signalled value NS_16 [A]: 1, 2 for FC = 810.0 MHz, 3 for FC = 814.5 MHz [B]: 2 for FC < 810.0 MHz, 3 for FC < 814.5 MHz, 4 Network signalled value NS_17 [A]: 1, 2 [B]: None Network signalled value NS_18 [A]: None [B]: 1, 2, 3, 4 Network signalled value NS_19 [A]: 1, (2, 3, 4) with high range RB allocation [B]: (2, 3, 4) with low range RB allocation Network signalled value NS_20 [A]: 1b with high range RB allocation, 1c, 1d, 2c, 2d, 3a with high range RB allocation, 3b, 5a with high range RB allocation [B]: 1a, 1b with low range RB allocation, 2a, 2b, 3a with low range RB allocation, 4a, 4b, 5a with low range RB allocation, 6a, 7a, 8a, 9a, 10a, 11a Network signalled value NS_21 [A]: None [B]: 1 Network signalled value NS_22 [A]: 1, 2 [B]: 3, 4, 5, 6, 7, 8 Network signalled value NS_23 [A]: 1, 2 [B]: None Network signalled value NS_24 [A]: 1, 3, 5 [B]: 2, 4, 6 Network signalled value NS_25 [A]: 6 [B]: 1, 2, 3, 4, 5 Network signalled value NS_27 [A]: 1, 2 [B]: 3, 4 Network signalled value NS_35 [A]: 1, 2, 3, 4 [B]: None Network signalled value NS_36 [A]: 2, 4, 6 [B]: 1, 3, 5 Network signalled value NS_56 [A]: None [B]: 1, 2, 3, 4 Network signalled value NS_62 [A]: 1, 2, 3, 4, 5, 6 |  |
| 6.2.4A.1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) |  | A-MPR values as detailed in core requirement |  | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4A.1_1 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.2.4A.1 |  | Same as 6.2.4A.1 |  |  | Same as 6.2.4A.1 |  |
| 6.2.4A.1_2 Additional Maximum Power Reduction (A-MPR) for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.2.4A.1 |  | Same as 6.2.4A.1 |  |  | Same as 6.2.4A.1 |  |
| 6.2.4A.2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.2_1 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.2_2 Additional Maximum Power Reduction (A-MPR) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4A.4 Additional Maximum Power Reduction (A-MPR) for CA (3UL CA) |  | A-MPR values as detailed in core requirement |  | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4B Additional Maximum Power Reduction (A-MPR) for UL-MIMO |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Same as 6.2.4 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.4E Additional Maximum Power Reduction (A-MPR) for UE category 0 |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.4EA Additional Maximum Power Reduction (A-MPR) for UE category M1 |  | A-MPR values as detailed in core requirement |  | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L )– A-MPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass-A-MPR |  |
| 6.2.4EB Additional Maximum Power Reduction (A-MPR) for UE category 1bis |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.4EC Additional Maximum Power Reduction (A-MPR) for UE category M2 |  | Same as 6.2.4EA |  | Same as 6.2.4EA |  |  | Same as 6.2.4EA |  |
| 6.2.4G.1 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | A-MPR values as detailed in core requirement |  | Same as 6.2.4 |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4G.2 Additional Maximum Power Reduction (A-MPR) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | A-MPR values as detailed in core requirement |  | 1.3dB |  |  | Formula: Upper limit: +T(PCMAX_H) + TT, Lower limit: -T(PCMAX_L ) – MPR – AMPR – TT PCMAX_H = PPowerClass PCMAX_L = PPowerClass – MPR – AMPR -TC |  |
| 6.2.4_s Additional Maximum Power Reduction (A-MPR) for subslot/slot TTI |  | Same as 6.2.4 |  | Same as 6.2.4 |  |  | Same as 6.2.4 |  |
| 6.2.5 Configured UE transmitted Output Power |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |  |
| 6.2.5_s Configured UE Transmitted Output Power for subslot/slot TTI |  | Same as 6.2.5 |  | Same as 6.2.5 |  |  | Same as 6.2.5 |  |
| 6.2.5_1 Configured UE transmitted Output Power for HPUE |  | f ≤ 3.0GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 23 ≤ PCMAX < 33 ± 2.7 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 23 ≤ PCMAX < 33 +3/-4 13 ≤ PCMAX < 18 ± 6 8 ≤ PCMAX < 13 ± 7 -40 ≤ PCMAX < 8 ± 8 |  |
| 6.2.5A.1 Configured UE transmitted Output Power for CA (intra-band contiguous DL CA and UL CA) |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 13 ≤ PCMAX < 18 ± 6.0 8 ≤ PCMAX < 13 ± 7.0 -40 ≤ PCMAX < 8 ± 8.0 |  |
| 6.2.5A.1_3 Configured UE transmitted Output Power for CA and HPUE (intra-band contiguous DL CA and UL CA) for HPUE |  | f ≤ 3.0GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 3.0GHz < f ≤ 4.2GHz 23 ≤ PCMAX < 33 ± 2.0 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB 0.7 dB 1.0 dB 1.0 dB 1.0 dB 1.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 23 ≤ PCMAX < 33 ± 2.7 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 23 ≤ PCMAX < 33 ± 3 13 ≤ PCMAX < 18 ± 6 8 ≤ PCMAX < 13 ± 7 -40 ≤ PCMAX < 8 ± 8 |  |
| 6.2.5A.3 Configured UE transmitted Output Power for CA (inter-band DL CA and UL CA) |  | f ≤ 3.0GHz 16 ≤ PCMAX < 20 ± 5.0 11≤ PCMAX < 16 ± 6.0 -40 ≤ PCMAX < 11 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 16 ≤ PCMAX < 20 ± 5.7 11 ≤ PCMAX < 16 ± 6.7 -40 ≤ PCMAX < 11 ± 7.7 |  |
| 6.2.5A.4 Configured UE transmitted Output Power for CA (intra-band non-contiguous DL CA and UL CA) |  | f ≤ 3.0GHz 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 0.7 dB 0.7 dB 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT 13 ≤ PCMAX < 18 ± 5.7 8 ≤ PCMAX < 13 ± 6.7 -40 ≤ PCMAX < 8 ± 7.7 |  |
| 6.2.5B Configured UE transmitted output power for UL-MIMO |  | f ≤ 3.0GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] 3.0GHz < f ≤ 4.2GHz [16] ≤ PCMAX < [20] ± [5.0] [11] ≤ PCMAX < [16] ± [6.0] [-40] ≤ PCMAX < [11] ± [7.0] |  | Same as 6.2.5 |  |  | Formula: Upper limit + TT, Lower limit – TT [16] ≤ PCMAX < [20] ± [5.7] [11] ≤ PCMAX < [16] ± [6.7] [-40] ≤ PCMAX < [11] ± [7.7] [16] ≤ PCMAX < [20] ± [6.0] [11] ≤ PCMAX < [16] ± [7.0] [-40] ≤ PCMAX < [11] ± [8.0] Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.2.5E Configurated Transmitted Power for UE category 0 |  | Same as 6.2.5 |  | Same as 6.2.5 |  |  | Same as 6.2.5 |  |
| 6.2.5EA Configured UE transmitted Power for UE category M1 |  | Same as 6.2.5, with addition of power class 6: f ≤ 3.0GHz 5 ≤ PCMAX < 8 ± 6.0 -40 ≤ PCMAX < 5 ± 7.0 3.0GHz < f ≤ 4.2GHz 5 ≤ PCMAX < 8 ± 6.0 -40 ≤ PCMAX < 5 ± 7.0 |  | Same as 6.2.5, with addition of power class 6: 0.7 dB 0.7 dB 1.0 dB 1.0 dB |  |  | Same as 6.2.5, with addition of power class 6: Formula: Upper limit + TT, Lower limit – TT f ≤ 3.0GHz 5 ≤ PCMAX < 8 ± 6.7 -40 ≤ PCMAX < 5 ± 7.7 3.0GHz < f ≤ 4.2GHz 5 ≤ PCMAX < 8 ± 7.0 -40 ≤ PCMAX < 5 ± 8.0 |  |
| 6.2.5EB Configured UE transmitted Power for UE category 1bis |  | Same as 6.2.5 |  | Same as 6.2.5 |  |  | Same as 6.2.5 |  |
| 6.2.5F Configured UE transmitted Output Power for UE category NB1 and NB2 |  | Tolerances as described in core requirements |  | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT |  |
| 6.2.5G.1 Configured UE transmitted Output Power for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 1.3 dB 1.3 dB 1.5 dB |  |  | Formula: Upper limit + TT, Lower limit - TT 13 ≤ PCMAX < 18 ± 6.3 8 ≤ PCMAX < 13 ± 7.3 -40 ≤ PCMAX < 8 ± 8.5 |  |
| 6.2.5G.2 Configured UE transmitted Output Power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | 16 ≤ PCMAX < 20 ± 5.0 11 ≤ PCMAX < 16 ± 6.0 -40 ≤ PCMAX < 11 ± 7.0 |  | 1.3 dB 1.3 dB 1.5 dB |  |  | Formula: Upper limit + TT, Lower limit - TT 16 ≤ PCMAX < 20 ± 6.3 11 ≤ PCMAX < 16 ± 7.3 -40 ≤ PCMAX < 11 ± 8.5 |  |
| 6.2.5G.3 Configured UE transmitted Output Power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 13 ≤ PCMAX < 18 ± 5.0 8 ≤ PCMAX < 13 ± 6.0 -40 ≤ PCMAX < 8 ± 7.0 |  | 1.3 dB 1.3 dB 1.5 dB |  |  | Formula: Upper limit + TT, Lower limit - TT 13 ≤ PCMAX < 18 ± 6.3 8 ≤ PCMAX < 13 ± 7.3 -40 ≤ PCMAX < 8 ± 8.5 |  |
| 6.3.2 Minimum Output Power |  | f ≤ 3.0GHz -40 dBm 3.0GHz < f ≤ 4.2GHz -40 dBm 4.2GHz < f ≤ 6GHz -40 dBm |  | 1.0 dB 1.3 dB 1.5 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm UE min. output power =–38.7 dBm UE min. output power =–38.5 dBm |  |
| 6.3.2A.1 Minimum Output Power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.2 |  | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2A.4 Minimum Output Power for CA (3UL CA) |  | Same as 6.3.2 |  | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2B Minimum Output Power for UL-MIMO |  | Same as 6.3.2 |  | Same as 6.3.2 |  |  | Same as 6.3.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.2E Minimum Output Power for UE category 0 |  | Same as 6.3.2 |  | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2EA Minimum Output Power for UE category M1 |  | Same as 6.3.2 |  | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2EB Minimum Output Power for UE category 1bis |  | Same as 6.3.2 |  | Same as 6.3.2 |  |  | Same as 6.3.2 |  |
| 6.3.2F Minimum Output Power for UE category NB1 and NB2 |  | -40 dBm |  | 1.0 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm |  |
| 6.3.2G.1 Minimum output power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz -30 dBm |  | 1.5 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–28.5 dBm |  |
| 6.3.2G.2 Minimum output power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz -40 dBm 4.2GHz < f ≤ 6GHz -30 dBm |  | 1.0 dB 1.5 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–39 dBm UE min. output power =–28.5 dBm |  |
| 6.3.2G.3 Minimum output power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz -3 0 dBm |  | 1.5 dB |  |  | Formula: Minimum Requirement + TT UE min. output power =–28.5 dBm |  |
| 6.3.3 Transmission ON/OFF Power |  | f ≤ 3.0GHz -50 dBm 3.0GHz < f ≤ 4.2GHz -50 dBm 4.2GHz < f ≤ 6GHz -50 dBm |  | 1.5 dB 1.8 dB 2.0 dB |  |  | Formula: Minimum Requirement + TT UE OFF Power -48.5 dBm UE OFF Power -48.2 dBm UE OFF Power -48.0 dBm |  |
| 6.3.3A.1 UE Transmit OFF power for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3A.2 Transmit OFF Power for CA (inter-band DL CA and UL CA) |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3A.3 UE Transmit OFF power for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3A.4 UE Transmit OFF power for CA (3UL CA) |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3B UE Transmit OFF power for UL-MIMO |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 Uplink power measurement applies to each Tx antenna connector |  |
| 6.3.3D UE Transmit OFF power for ProSe |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3E UE Transmit OFF Power for UE category 0 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3EA UE Transmit OFF power for UE category M1 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3EB UE Transmit OFF power for UE category 1bis |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3F Transmit OFF power for category NB1 and NB2 |  | Same as 6.3.3 |  | Same as 6.3.3 |  |  | Same as 6.3.3 |  |
| 6.3.3G.1 UE Transmit OFF power for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz -50 dBm |  | 2.0 dB |  |  | Formula: Minimum Requirement + TT UE OFF Power -48 dBm |  |
| 6.3.3G.2 UE Transmit OFF power for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz -50 dBm 3.0GHz < f ≤ 4.2GHz -50 dBm 4.2GHz < f ≤ 6GHz -50 dBm |  | 1.5 dB 1.8 dB 2.0 dB |  |  | Formula: Minimum Requirement + TT UE OFF Power -48.5 dBm UE OFF Power -48.2 dBm UE OFF Power -48.0 dBm |  |
| 6.3.3G.3 UE Transmit OFF power for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz -50 dBm |  | 2.0 dB |  |  | Formula: Minimum Requirement + TT UE OFF Power -48 dBm |  |
| 6.3.4.1 General ON/OFF time mask |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB |  | 1.5 dB 1.5 dB 1.8 dB 1.8 dB |  |  | Formula: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |  |
| 6.3.4.1_s General ON/OFF time mask for subslot/slot TTI |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4.2 PRACH and SRS time mask |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB |  | 1.5 dB 1.5 dB 1.8 dB 1.8 dB |  |  | Formula: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB |  |
| 6.3.4A.1.1 General ON/OFF time mask for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.4.1 for each CC |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.2 General ON/OFF time mask for CA (inter-band DL CA and UL CA) |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4A.1.3 General ON/OFF time mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.4.1 for each CC |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.4 General ON/OFF time mask for CA (3UL CA) |  | Same as 6.3.4.1 for each CC |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4A.1.5 General ON/OFF time mask for CA (34UL CA) |  | Same as 6.3.4.1 for each CC |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 for each CC |  |
| 6.3.4B.1 General ON/OFF time mask for UL-MIMO |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 Uplink OFF power measurement applies to each Tx antenna connector Uplink ON power measurement applies to [FFS] |  |
| 6.3.4C.1 General ON/OFF time mask for Dual Connectivity |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4E.1 General ON/OFF time mask for UE category 0 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4E.2 PRACH and SRS time mask for UE category 0 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4EA.1 General ON/OFF time mask for UE category M1 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4EA.2 PRACH and SRS ON/OFF time mask for UE category M1 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4EB.1 General ON/OFF time mask for UE category 1bis |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4EB.2 PRACH and SRS ON/OFF time mask for UE category 1bis |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4EC.2 PRACH and SRS ON/OFF time mask for UE category M2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4F.1 General ON/OFF time mask for category NB1 and NB2 |  | Same as 6.3.4.1 |  | Same as 6.3.4.1 |  |  | Same as 6.3.4.1 |  |
| 6.3.4F.2 NPRACH time mask for category NB1 and NB2 |  | Same as 6.3.4.2 |  | Same as 6.3.4.2 |  |  | Same as 6.3.4.2 |  |
| 6.3.4G.1 General ON/OFF time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB |  | 2.0dB 2.0dB |  |  | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48 dBm UE ON Power: Test value ± 8.0 dB |  |
| 6.3.4G.2 General ON/OFF time mask for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz OFF Power -50 dBm ON Power BW dependent 3.0GHz < f ≤ 4.2GHz OFF Power -50 dBm ON Power BW dependent 4.2GHz < f ≤ 6.0GHz OFF Power -50 dBm ON Power BW dependent Transmission ON Power value depends on the test parameters. In the particular test case parameters the ON power measurement has minimum requirements of ±6.0 dB |  | 1.5 dB 1.5 dB 1.8 dB 1.8 dB 2.0 dB 2.0 dB |  |  | Formulae: OFF Power Minimum Req’t + TT ON Power Upper limit + TT, Lower limit – TT UE OFF Power -48.5 dBm UE ON Power: Test value ± 7.5 dB UE OFF Power -48.2 dBm UE ON Power: Test value ± 7.8 dB UE OFF Power -48.0 dBm UE ON Power: Test value ± 8.0 dB |  |
| 6.3.4G.3 General ON/OFF time mask for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.3.4G.1 for each CC |  | Same as 6.3.4G.1 |  |  | Same as 6.3.4G.1 for each CC |  |
| 6.3.4G.4 PSSS/SSSS time mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.3.4G.1 |  | Same as 6.3.4G.1 |  |  | Same as 6.3.4G.1 |  |
| 6.3.5.1 Power Control Absolute power tolerance |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 4.2GHz < f ≤ 6GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB |  | 1.0 dB 1.0 dB 1.4 dB 1.4 dB 2.0 dB 2.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB Normal conditions ± 11.0 dB Extreme conditions ± 14.0 dB |  |
| 6.3.5.2 Power Control Relative power tolerance |  | TS 36.101 [2] clause 6.3.5.1 All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±2.5 dB 2 ≤ ΔP < 3; ±3.0 dB 3 ≤ ΔP < 4; ±3.5 dB 4 ≤ ΔP ≤ 10; ±4.0 dB 10 ≤ ΔP < 15; ±5.0 dB 15 ≤ ΔP; ±6.0 dB |  | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±3.2 dB 2 ≤ ΔP < 3; ±3.7 dB 3 ≤ ΔP < 4; ±4.2 dB 4 ≤ ΔP < 10; ±4.7 dB 10 ≤ ΔP < 15; ±5.7 dB 15 ≤ ΔP; ±6.7 dB |  |
| 6.3.5.3 Aggregate power control tolerance |  | Aggregate power control tolerance within 21 ms: PUCCH = ±2.5 dB PUSCH = ±3.5 dB |  | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit - TT PUCCH = ±3.2 dB PUSCH = ±4.2 dB |  |
| 6.3.5_1.1 Power Control Absolute power tolerance for HPUE |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB |  | 1.0 dB 1.0 dB 1.4 dB 1.4 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB |  |
| 6.3.5_1.2 Power Control Relative power tolerance for HPUE |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  |  | Same as 6.3.5.2 |  |
| 6.3.5_1.3 Aggregate power control tolerance for HPUE |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.1.1 Power Control Absolute power tolerance for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.2 Power Control Absolute power tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.3 Power Control Absolute power tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.4 Power Control Absolute power tolerance for CA(3UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.5 Power Control Absolute power tolerance for CA(SRS carrier based switching) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.1.6 Power Control Absolute power tolerance for CA(4UL CA) |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5A.2.1 Power Control Relative power tolerance for CA (intra-band contiguous DL CA and UL CA) |  | TS 36.101 [2] clause 6.3.5A.2 refers to Table 6.3.5.2.1-1. All combinations of PUSCH and PUCCH transitions: SCC power increase PCC 0dB (ΔP ≤ 1dB): Normal 0 ±1.0dB Exceptions: 0 ±6.0dB SCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB SCC power decrease PCC 0dB (ΔP ≤ 1dB): Normal 0 ±1.0dB Exceptions: 0 ±6.0dB SCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB PCC and SCC power increase PCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB SCC +9dB (4 ≤ ΔP ≤ 10): Normal +9 ±4.0dB Exceptions: +9 ±6.0dB PCC and SCC power decrease PCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB SCC -9dB (4 ≤ ΔP ≤ 10): Normal -9 ±4.0dB Exceptions: -9 ±6.0dB Each CC can have exceptions |  | -0.7, +0.8dB -0.7, +0.8dB -0.8, +0.7dB -0.8, +0.8dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.8dB -0.8, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.7, +0.8dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.7dB -0.8, +0.7dB |  |  | Relative power measurement uncertainty comprises two quantities: 1. Uplink Relative level measurement uncertainty of Test system 2. IQ image Leakage onto weaker CC at -25dBc is allowed for the UE, so is added arithmetically. It can only increase the measured power of the weaker CC. UL relative meas’t uncert ±0.7dB Leakage +0.17dB max for 11dB CC power difference, based on -25dBc. All limits rounded to nearest 0.1dB. SCC power increase PCC: Normal -1.7, +1.8dB Exceptions: -6.7, +6.8dB SCC: Normal +4.2, +13.7dB Exceptions: +2.2, +15.8dB SCC power decrease PCC: Normal -1.8, +1.7dB Exceptions: -6.8, +6.7dB SCC: Normal -13.8, -4.2dB Exceptions: -15.8, -2.2dB PCC and SCC power increase PCC: Normal +4.3, +13.8dB Exceptions: +2.3, +15.8dB SCC: Normal +4.3, +13.8dB Exceptions: +2.3, +15.8dB PCC and SCC power decrease PCC: Normal -13.8, -4.3dB Exceptions: -15.8, -2.3dB SCC: Normal -13.8, -4.3dB Exceptions: -15.8, -2.3dB The analysis is recorded in 3GPP TR 36.904 [17] When setting SCC power for the next step, three factors are relevant: 1. Uplink Relative level measurement uncertainty 2. IQ image Leakage onto weaker CC at -25dBc is allowed for the UE 3) TPC step 1dB ±1dB is allowed for the UE These are added arithmetically, giving an extra margin of (1+0.7+0.17) = 1.87dB (2dB is allowed) to avoid the same power step exception being counted twice. |  |
| 6.3.5A.2.2 Power Control Relative power tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5A.2.3 Power Control Relative power tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5A.2.1 |  | Same as 6.3.5A.2.1 |  |  | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.1 Power Control Relative power tolerance for CA (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.1 |  | Same as 6.3.5A.2.1 |  |  | Same as 6.3.5A.2.1 |  |
| 6.3.5A.2.4.2 Power Control Relative power tolerance for CA (inter-band 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.2 |  | Same as 6.3.5A.2.2 |  |  | Same as 6.3.5A.2.2 |  |
| 6.3.5A.2.4.3 Power Control Relative power tolerance for CA (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.3.5A.2.3 |  | Same as 6.3.5A.2.3 |  |  | Same as 6.3.5A.2.3 |  |
| 6.3.5A.3.1 Aggregate power control tolerance for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.2 Aggregate power control tolerance for CA (inter-band DL CA and UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.3 Aggregate power control tolerance for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.4 Aggregate Power Control Tolerance for CA (3UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5A.3.5 Aggregate Power Control Tolerance for CA (4UL CA) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5B.1 Power Control Absolute Power Tolerance for UL- MIMO |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5B.2 Power Control Relative power tolerance for UL-MIMO |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  |  | Same as 6.3.5.2 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5B.3 Aggregate power control tolerance for UL-MIMO |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 Uplink power measurement applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.3.5C.2 Power Control Relative power tolerance for Dual Connectivity |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5C.2_1 Power Control Relative power tolerance for asynchronous Dual Connectivity |  | Same as 6.3.5.2 on both PCC and SCC separately. |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |  | Same as 6.3.5.2 on both PCC and SCC separately. |  |
| 6.3.5E.1 Power Control Absolute power tolerance for UE category 0 |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5E.2 Power Control Relative power tolerance for UE category 0 |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  |  | Same as 6.3.5.2 |  |
| 6.3.5E.3 Aggregate power control tolerance for UE category 0 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5EA.1 Power Control Absolute power tolerance for UE category M1 |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB |  | 1.0 dB 1.0 dB 1.4 dB 1.4 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB |  |
| 6.3.5EA.2 Power Control Relative power tolerance for UE category M1 |  | TS 36.101 [2] clause 6.3.5.1 All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±2.5 dB 2 ≤ ΔP < 3; ±3.0 dB 3 ≤ ΔP < 4; ±3.5 dB 4 ≤ ΔP ≤ 10; ±4.0 dB 10 ≤ ΔP < 15; ±5.0 dB 15 ≤ ΔP; ±6.0 dB |  | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT All combinations of PUSCH and PUCCH transitions: ΔP < 2; ±3.2 dB 2 ≤ ΔP < 3; ±3.7 dB 3 ≤ ΔP < 4; ±4.2 dB 4 ≤ ΔP < 10; ±4.7 dB 10 ≤ ΔP < 15; ±5.7 dB 15 ≤ ΔP; ±6.7 dB |  |
| 6.3.5EA.3 Aggregate power control tolerance for UE category M1 |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5EA.3_1 Aggregate power control tolerance for UE category M1 (CE Mode B) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5EB.1 Power Control Absolute power tolerance for UE category 1bis |  | Same as 6.3.5.1 |  | Same as 6.3.5.1 |  |  | Same as 6.3.5.1 |  |
| 6.3.5EB.2 Power Control Relative power tolerance for UE category 1bis |  | Same as 6.3.5.2 |  | Same as 6.3.5.2 |  |  | Same as 6.3.5.2 |  |
| 6.3.5EB.3 Aggregate power control tolerance for UE category 1bis |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5EC.3_1 Aggregate power control tolerance for UE category M2 (CE Mode B) |  | Same as 6.3.5.3 |  | Same as 6.3.5.3 |  |  | Same as 6.3.5.3 |  |
| 6.3.5F.1 Power Control Absolute power tolerance for category NB1 and NB2 |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 3.0GHz < f ≤ 4.2GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB |  | 1.0 dB 1.0 dB 1.4 dB 1.4 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 10.4 dB Extreme conditions ± 13.4 dB |  |
| 6.3.5F.2 Power Control Relative power tolerance for UE category NB1 and NB2 |  | TS 36.101 [2] clause 6.3.5F.2 NPRACH transitions: ΔP = 0; ±1.5 dB ΔP = 2; ±2.0 dB ΔP = 4; ±3.5 dB ΔP = 6; ±4.0 dB |  | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit – TT NPRACH transitions: ΔP = 0; ±2.2 dB ΔP = 2; ±2.7 dB ΔP = 4; ±4.2 dB ΔP = 6; ±4.7 dB |  |
| 6.3.5F.3 Aggregate power control tolerance for category NB1 and NB2 |  | Aggregate power control tolerance within TBD ms: PUSCH = ±3.5 dB |  | 0.7 dB |  |  | Formula: Upper limit + TT, Lower limit - TT PUSCH = ±4.2 dB |  |
| 6.3.5G.1 Power Control Absolute power tolerance for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB |  | 2.0 dB 2.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.3.5G.2 Power Control Absolute power tolerance for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | f ≤ 3.0GHz Normal conditions ± 9.0 dB Extreme conditions ± 12.0 dB 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB |  | 1.0 dB 1.0 dB 2.0 dB 2.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 10.0 dB Extreme conditions ± 13.0 dB Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.3.5G.3 Power Control Absolute power tolerance for V2X Communication / Intra-band contiguous multi-carrier operation |  | 4.2GHz < f ≤ 6GHz Normal conditions ± 3.0 dB Extreme conditions ± 6.0 dB |  | 2.0 dB 2.0 dB |  |  | Formula: Upper limit + TT, Lower limit – TT Normal conditions ± 5.0 dB Extreme conditions ± 8.0 dB |  |
| 6.5.1 Frequency Error |  | Modulated carrier, f ≤ 4.2GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens 3.0GHz < f ≤ 4.2GHz DL power: Refsens |  | 15 Hz 0.7 dB 1.0 dB |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB Refsens +1.0dB |  |
| 6.5.1A.1 Frequency error for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.1 |  | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1A.2 Frequency Error for CA (inter-band DL CA and UL CA) |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  |  | Same as 6.5.1 per CC |  |
| 6.5.1A.3 Frequency Error for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  |  | Same as 6.5.1 per CC |  |
| 6.5.1A.5 Frequency error for CA (4UL CA) |  | Same as 6.5.1 per CC |  | Same as 6.5.1 per CC |  |  | Same as 6.5.1 per CC |  |
| 6.5.1B Frequency Error for UL-MIMO |  | Same as 6.5.1 |  | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1D.1 Frequency Error for ProSe Direct Discovery |  | Modulated carrier, f ≤ 4.2GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz Sidelink power: Refsens |  | 15 Hz 0.7 dB |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT Sidelink power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB |  |
| 6.5.1D.2 Frequency Error for ProSe Direct Communication |  | Same as 6.5.1D.1 |  | Same as 6.5.1D.1 |  |  | Same as 6.5.1D.1 |  |
| 6.5.1E Frequency Error for UE category 0 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1EA Frequency Error for UE category M1 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1EB Frequency Error for UE category 1bis |  | Same as 6.5.1 |  | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1EC Frequency Error for UE category M2 |  | Same as 6.5.1 |  | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1EC_1 Frequency Error for UE category M2 (CE Mode B) |  | Same as 6.5.1 |  | Same as 6.5.1 |  |  | Same as 6.5.1 |  |
| 6.5.1F Frequency Error for category NB1 and NB2 |  | Modulated carrier, f ≤ 3.0GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens 3.0GHz < f ≤ 4.2GHz DL power: Refsens |  | 15 Hz 0.7 dB 1.0 dB |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7dB Refsens +1.0dB |  |
| 6.5.1G.1 Frequency Error for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | Modulated carrier, f ≤ 6.0GHz Within 0.1 ppm compared to the received carrier frequency |  | 36 Hz |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 36 Hz) |  |
| 6.5.1G.2 Frequency error for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Modulated carrier, f ≤ 3.0GHz Within 0.1 ppm compared to the received carrier frequency f ≤ 3.0GHz DL power: Refsens Modulated carrier, 4.2GHz <f ≤ 6.0GHz Within 0.1 ppm compared to the received carrier frequency 4.2GHz <f ≤ 6.0GHz DL power: Refsens |  | 15 Hz 0.7 dB 36 Hz 1.5 dB |  |  | Formula: Modulated carrier frequency: Upper limit + TT, Lower limit – TT DL power: Refsens + TT Modulated carrier frequency error = (0.1 ppm + 15 Hz) Refsens +0.7 dB Modulated carrier frequency error = (0.1 ppm + 36 Hz) Refsens +1.5 dB |  |
| 6.5.2.1 Error Vector Magnitude |  | EVM limit: BPSK :17.5 % QPSK: 17.5 % 16QAM: 12.5 % |  | 0% |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1_1 Error Vector Magnitude (EVM) for UL 64QAM |  | EVM limit: 64QAM: 8.0 % |  | 0% |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1_2 Error Vector Magnitude (EVM) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % |  | 0.3%, 15dBm < PUL 0.8%, -25dBm < PUL ≤ 15dBm, 1.1%, -30dBm ≤ PUL ≤ -25dBm |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.1A PUSCH-EVM with exclusion period |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % |  | 0% |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.1 Error Vector Magnitude (EVM) for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.1_1 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1_1 |  | Same as 6.5.2.1_1 |  |  | Same as 6.5.2.1_1 |  |
| 6.5.2A.1.1_2 EVM for CA (intra-band contiguous DL CA and UL CA) with UL 256QAM |  | EVM limit: 256QAM: 3.5 % |  | Same as 6.5.2.1_2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_1 EVM for CA (inter-band DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.2_2 Error Vector Magnitude (EVM) for CA (inter-band DL CA and UL CA) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % |  | Same as 6.5.2.1_2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.3 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_1 EVM for CA (intra-band non-contiguous DL CA and UL CA) with UL 64QAM |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.3_2 Error Vector Magnitude (EVM) for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | EVM limit: 256QAM: 3.5 % |  | Same as 6.5.2.1_2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.1.4 Error Vector Magnitude (EVM) for CA (3UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2A.1.5 Error Vector Magnitude (EVM) for CA (4UL CA) |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2B.1 Error Vector Magnitude (EVM) for UL- MIMO |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2.1E.1 Error Vector Magnitude for UE category 0 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1E.2 PUSCH-EVM with exclusion period for UE category 0 |  | Same as 6.5.2.1A |  | Same as 6.5.2.1A |  |  |  |  |
| 6.5.2.1EA.1 Error Vector Magnitude for UE category M1 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1EA.2 PUSCH-EVM with exclusion period for UE category M1 |  | Same as 6.5.2.1A |  | Same as 6.5.2.1A |  |  |  |  |
| 6.5.2.1EB Error Vector Magnitude (EVM) for UE category 1bis |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1EC.1 Error Vector Magnitude for UE category M2 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1EC.2 PUSCH-EVM with exclusion period for UE category M2 |  | Same as 6.5.2.1A |  | Same as 6.5.2.1A |  |  | Same as 6.5.2.1A |  |
| 6.5.2.1F.1 Error Vector Magnitude (EVM) for UE category NB1 and NB2 |  | Same as 6.5.2.1 |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.1 Error Vector Magnitude (EVM) for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.2 Error Vector Magnitude (EVM) for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1G.3 Error Vector Magnitude (EVM) for V2X Communication / Intra-band contiguous multi-carrier operation |  | EVM limit: QPSK: 17.5 % 16QAM: 12.5 % |  | Same as 6.5.2.1 |  |  | Same as 6.5.2.1 |  |
| 6.5.2.1_s Error Vector Magnitude (EVM) for subslot/slot TTI |  | Same as 6.5.2 |  | Same as 6.5.2 |  |  | Same as 6.5.2 |  |
| 6.5.2.2 Carrier leakage |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc |  | 0.8dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.2F Carrier leakage for UE category NB1 and NB2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.1 Carrier leakage for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.2 Carrier leakage for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmission |  | For E-UTRA carrier, same as 6.5.2.2 For V2X carrier, same as 6.5.2.2G.1 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2G.3 Carrier leakage for V2X Communication / Intra-band contiguous MCC operation |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.3 In-band emissions for non allocated RB |  | For general emissions: within the subPRB allocation -18-5∙¦∆SubG-1)/LSCG For general emissions for any non-allocated RB: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc -40 dBm ≤ Output power < -30 dBm -10dBc For each evaluated RB, the test requirement is calculated as the higher of PRB – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) |  | 0.8dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2.4 EVM equalizer Spectrum flatness |  | Normal conditions. If (F-FUL_low ≥ [3MHz])&(FUL_high-F≥ [3MHz]) 4 dB else 8 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 5 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 7 dB Extreme conditions: If (F-FUL_low ≥ [5MHz])&(FUL_high-F≥ [5MHz]) 4 dB else 12 dB maximum coefficient in Range 1 - the minimum coefficient in Range 2 6 dB the maximum coefficient in Range 2 - the minimum coefficient in Range 10 dB |  | 1.4dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.1 Carrier leakage for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2A.2.2 Carrier leakage for CA (inter-band DL CA and UL CA) |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc |  | 0.8dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.3 Carrier leakage for CA (intra-band non-contiguous DL CA and UL CA) |  | For Output power >0 dBm -25dBc For -30 dBm ≤ Output power ≤0 dBm -20dBc For -40 dBm ≤ Output power < -30 dBm -10dBc |  | 0.8dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.5.2A.2.4 Carrier leakage for CA (3UL CA) |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2A.2.5 Carrier leakage for CA (4UL CA) |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2A.3.1 In-band emissions for non allocated RB for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.2 In-band emissions for non allocated RB for CA (inter-band DL CA and UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.3 In-band emissions for non allocated RB for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.4 In-band emissions for non allocated RB for CA (3UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2A.3.5 In-band emissions for non allocated RB for CA (4UL CA) |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2B.1 Error vector magnitude (EVM) for UL-MIMO |  | TBD |  | TBD |  |  | TBD |  |
| 6.5.2B.2 Carrier leakage for UL-MIMO |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2B.3 In-band emissions for non allocated RB for UL-MIMO |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 Uplink power measurement window applies to overall UL power, which is the linear sum of the output powers over all Tx antenna connectors |  |
| 6.5.2B.4 EVM equalizer spectrum flatness for UL-MIMO |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.2E Carrier Leakage for UE category 0 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2EA Carrier Leakage for UE category M1 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2EB Carrier leakage for UE category 1bis |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.2EC Carrier leakage for UE category M2 |  | Same as 6.5.2.2 |  | Same as 6.5.2.2 |  |  | Same as 6.5.2.2 |  |
| 6.5.2.3E In-band emissions for non allocated RB for UE category 0 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3EA In-band emissions for non allocated RB for UE category M1 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3EB In-band emissions for non allocated RB for UE category 1bis |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3EC In-band emissions for non allocated RB for UE category M2 |  | Same as 6.5.2.3 |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3F In-band emissions for non allocated RB for UE category NB1 and NB2 |  | For general emissions: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc -40 dBm ≤ Output power < -30 dBm -10dBc For each evaluated tone, the test requirement is calculated as the higher of Ptone – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) |  | Same as 6.5.2.3 |  |  | Same as 6.5.2.3 |  |
| 6.5.2.3G.1 In-band emissions for non-allocated RB for V2X Communication |  | For general emissions: For IQ image: -25dB For Carrier leakage: Output power >0 dBm -25dBc -30 dBm ≤ Output power ≤0 dBm -20dBc For each evaluated RB, the test requirement is calculated as the higher of PRB – 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) Sidelink Power Window >= 0dBm, (-30 dBm, 0 dBm) E-UTRA Uplink Power Window >= 0dBm, (-30 dBm, 0 dBm) |  | Emission: 0.8dB Sidelink Power: Absolute Transmitted Power tolerance: ±3dB referting to the TS 36.101 [2] clasue 6.3.5G.1 Wanted signal uncertainty: ±1.3 dB 4.2GHz < f ≤ 6.0GHz, for UE powers >= 0dBm ±1.5 dB, 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm Uplink power Relative Transmitted Power tolerance: ±2.5dB referting to the TS 36.101 [2] clasue 6.3.5.2 Test system power measurement uncertainty: ±0.7 dB, f ≤ 3.0GHz ±1.0 dB, 3.0GHz < f ≤ 4.2GHz ±1.3 dB, 4.2GHz < f ≤ 6GHz |  |  | Formula for emission: Minimum Requirement + TT Sidelink power measurement window: Bottom of the window: the Configured sidelink power - TT of sidelink power Top of the window: the Configured sidelink power + TT of sidelink power TT of sidelink power comprises 2 quantities: 1. Absolute Transmitted Power tolerance :±3dB 2. Test system power measurement uncertainty: ±1.3 dB 4.2GHz < f ≤ 6.0GHz for UE powers>= 0dBm. ±1.5 dB 4.2GHz < f ≤ 6.0GHz, for UE powers >= -40dBm Items 1 and 2 are added arithmetically: TT of sidelink power: (3dB+1.3dB) =4.3dB, 4.2GHz < f ≤ 6.0GH for UE powers >= 0 dBm: TT of sidelink power: (3dB+1.5dB) =4.5dB, 4.2GHz < f ≤ 6.0GH for powers >= -40 dBm: Ensure the bottom of the sidelink power measurement window is aligned with the bottom of the sidelink power window of the corresponding IBE requirement, and the top of the sidelink power measurement window is below the top of sidelink power window. Uplink power measurement window: Bottom of the window: the Configured sidelink power - TT of sidelink power Top of the window: the Configured sidelink power + TT of sidelink power TT of uplink power comprises 2 quantities: 1. Relative Transmitted Power tolerance :±2.5dB 2. Test system power measurement uncertainty: ±0.7 dB for f ≤ 3.0GHz. ±1 dB for 3.0GHz < f ≤ 4.2GHz Items 1 and 2 are added arithmetically: For f ≤ 3.0GHz TT of uplink power: (2.5dB+0.7dB) =3.2dB For 3.0GHz < f ≤ 4.2GHz TT of uplink power: (2.5dB+1dB) =3.5dB, Ensure the bottom of the uplink power measurement window is aligned with the bottom of the uplink power window of the corresponding IBE requirement, and the top of the uplink power measurement window is below the top of uplink power window. |  |
| 6.5.2.3G.2 In-band emissions for non-allocated RB for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  |  | same as 6.5.2.3 for E-UTRA uplink same as 6.5.2.3G.1 for E-UTRA V2X sidelink |  |
| 6.5.2.3G.3 In-band emissions for non-allocated RB for V2X Communication / Intra-band contiguous MCC operation |  | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink |  | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink |  |  | same as 6.5.2.3G.1 for each cc of E-UTRA V2X sidelink |  |
| 6.5.2.4E EVM equalizer spectrum flatness for UE category 0 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4EA EVM equalizer spectrum flatness for UE category M1 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4EB EVM equalizer spectrum flatness for UE category 1bis |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4EC EVM equalizer spectrum flatness for UE category M2 |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4G.1 EVM equalizer spectrum flatness for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.5.2.4G.3 EVM equalizer spectrum flatness for V2X Communication / Intra-band contiguous multi-carrier operation uplink transmissions |  | Same as 6.5.2.4 |  | Same as 6.5.2.4 |  |  | Same as 6.5.2.4 |  |
| 6.6.1 Occupied bandwidth |  | For 1.4 MHz channel bandwidth: Occupied channel bandwidth = 1.4 MHz For 3.0 MHz channel bandwidth: Occupied channel bandwidth = 3.0 MHz For 5 MHz channel bandwidth: Occupied channel bandwidth = 5 MHz For 10 MHz channel bandwidth: Occupied channel bandwidth = 10 MHz For 15 MHz channel bandwidth: Occupied channel bandwidth = 15 MHz For 20 MHz channel bandwidth: Occupied channel bandwidth = 20 MHz |  | 0kHz |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.1A.1 Occupied bandwidth for CA (intra-band contiguous DL CA and UL CA) |  | For 25(20+5) MHz channel bandwidth: Occupied channel bandwidth = 24.95 MHz For 30(20+10) MHz channel bandwidth: Occupied channel bandwidth = 29.90 MHz For 30(15+15) MHz channel bandwidth: Occupied channel bandwidth = 30.00 MHz For 35(20+15)MHz channel bandwidth: Occupied channel bandwidth = 34.85 MHz For 40(20+20) MHz channel bandwidth: Occupied channel bandwidth = 39.80 MHz |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1A.2 Occupied bandwidth for CA (inter-band DL CA and UL CA) |  | Same as 6.6.1 per CC |  | Same as 6.6.1 per CC |  |  | Same as 6.6.1 per CC |  |
| 6.6.1A.3 |  | Same as 6.6.1 for each CC |  | Same as 6.6.1 |  |  | Same as 6.6.1 for each CC |  |
| 6.6.1A.5 Occupied bandwidth for CA (4UL CA) |  | Same as 6.6.1 for each CC |  | Same as 6.6.1 |  |  | Same as 6.6.1 for each CC |  |
| 6.6.1B Occupied bandwidth for UL-MIMO |  | Same as 6.6.1 |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1E Occupied bandwidth for UE category 0 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1EA Occupied bandwidth for UE category M1 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1EB Occupied bandwidth for UE category 1bis |  | Same as 6.6.1 |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1EC Occupied bandwidth for UE category M2 |  | Same as 6.6.1 |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1F Occupied bandwidth for UE category NB1 |  | Occupied channel bandwidth = 200KHz |  | 0kHz |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.1G.1 Occupied bandwidth for V2X Communication / Non-concurrent with E-UTRA uplink transmission |  | Same as 6.6.1 |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1G.2 Occupied bandwidth for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.6.1 |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.1G.3 Occupied bandwidth for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.6.1 |  | Same as 6.6.1 |  |  | Same as 6.6.1 |  |
| 6.6.2.1 Spectrum Emission Mask |  | For 1.4 MHz BW: -10 dBm / 30kHz -25dBm to -10dBm / 1MHz For 3 MHz BW: -13 dBm / 30kHz -25dBm to -10dBm / 1MHz For 5 MHz BW: -15dBm / 30kHz -25dBm to -10dBm / 1MHz For 10 MHz BW: -18dBm / 30kHz -25dBm to -10dBm / 1MHz For 15 MHz BW: -20dBm / 30kHz -25dBm to -10dBm / 1MHz For 20 MHz BW: -21dBm / 30kHz -25dBm to -10dBm / 1MHz |  | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB |  |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.1_1 Spectrum Emission Mask for Multi-Cluster PUSCH |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1A.1 Spectrum emission mask for CA (intra-band contiguous DL CA and UL CA) |  | For 9.8MHz BW: -18 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 14.95MHz BW: -20 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 19.8MHz BW: -21 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 19.9MHz: -21 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 24.75MHz BW: -22 dBm / 30kHz -25 dBm to -10 dBm / 1MHz For 24.95 MHz BW: -22 dBm / 30kHz -25 dBm to - 10 dBm / 1 MHz For 29.9 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 30 MHz BW: -22.5 dBm / 30kHz -25dBm to -10dBm / 1MHz For 34.85 MHz BW: -23.5dBm / 30kHz -25dBm to -10dBm / 1MHz For 39.8 MHz BW: -24dBm / 30kHz -25dBm to -10dBm / 1MHz |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1A.2 Spectrum emission mask for CA (inter-band DL CA and UL CA) |  | Same as 6.6.2.1 per CC |  | Same as 6.6.2.1 per CC |  |  | Same as 6.6.2.1 per CC |  |
| 6.6.2.1A.3 Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.1 for each CC |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 for each CC |  |
| 6.6.2.1A.5 Additional Spectrum Emission Mask for CA (4UL) |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1B Spectrum Emission Mask for UL-MIMO |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1E Spectrum Emission Mask for UE category 0 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1EA Spectrum Emission Mask for UE category M1 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1EB Spectrum Emission Mask for UE category 1bis |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1EC Spectrum Emission Mask for UE category M2 |  | Same as 6.6.2.1 |  | Same as 6.6.2.1 |  |  | Same as 6.6.2.1 |  |
| 6.6.2.1F Spectrum Emission Mask for UE category NB1 |  | For all frequencies -35dBm to 26dBm / 30kHz For frequencies that are Foffset away from edge of NB1 channel edge as defined in Table 6.6.2.1F.3-2, same as 6.6.2.1 |  | All cases: f ≤ 3.0GHz 1.5dB |  |  | For all frequencies interpolated Minimum Requirement + TT |  |
| 6.6.2.1G.1 Spectrum Emission Mask for V2X Communication Non-concurrent with E-UTRA uplink transmissions |  | Bandwidth: 10MHz -20dBm /30kHz -25 ~ -10 / 1 MHz Bandwidth: 20MHz -21dBm /30kHz -25 ~ -10 / 1 MHz |  | ±2.0 dB, 4.2GHz < f ≤ 6GHz |  |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.1G.2 Spectrum Emission Mask for V2X Communication /Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.2.1G.1 for sidelink Same as 6.6.2.1G.1 for E-UTRA uplink |  | Same as 6.6.2.1G.1 |  |  | Same as 6.6.2.1G.1 |  |
| 6.6.2.1G.3 Spectrum Emission Mask for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.2.1G.1 |  | Same as 6.6.2.1G.1 |  |  | Same as 6.6.2.1G.1 |  |
| 6.6.2.2 Additional Spectrum Emission Mask |  | For 1.4 MHz BW: NS_03, NS_04 -10 dBm / 30 kHz -25 dBm to -13 dBm / 1MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1MHz For 3 MHz BW: NS_03, NS_04 -13 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -13 dBm / 30 kHz -13 dBm / 100kHz -25 dBm to -13 dBm / 1 MHz For 5 MHz BW: NS_03, NS_04 -15 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz NS_06 or NS_07 -15 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to -13 dBm / 1 MHz For 10 MHz BW: NS_03, NS_04, -18 dBm / 30 kHz -25 dBm to - 13dBm / 1 MHz NS_06 or NS_07 -18 dBm / 30 kHz -13 dBm / 100 kHz -25 dBm to - 13dBm / 1 MHz For 15 MHz BW: NS_03, NS_04 -20 dBm / 30kHz -25 dBm to -13 dBm / 1 MHz For 20 MHz BW: NS_03, NS_04 -21 dBm / 30 kHz -25 dBm to -13 dBm / 1 MHz |  | All cases: f ≤ 3.0GHz 1.5dB 3.0GHz < f ≤ 4.2GHz 1.8dB 4.2GHz < f ≤ 6.0GHz ±2.0 dB |  |  | Formula: Minimum Requirement + TT Note: The Test Tolerance would be 0dB for ΔfOOB ≥ 2 x Channel Bandwidth, but taking into account the filter position, the Test requirements specified all have ΔfOOB < 2 x Channel Bandwidth |  |
| 6.6.2.2_1 Additional Spectrum Emission Mask for UL 64QAM |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2_2 Additional Spectrum Emission Mask for UL 256QAM |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A-1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) |  | For network signalled value CA_NS_04: -21dBm/30kHz to -22.5dBm/kHz, BW-dependent, for 0-1MHz -13dBm/1MHz for 1MHz-5.5MHz -25dBm/1MHz beyond 5.5MHz |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.1_1 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.2A.1 |  | Same as 6.6.2.2A.1 |  |  | Same as 6.6.2.2A.1 |  |
| 6.6.2.2A.1_2 Additional Spectrum Emission Mask for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.2A.1 |  | Same as 6.6.2.2A.1 |  |  | Same as 6.6.2.2A.1 |  |
| 6.6.2.2A.2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_1 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.2_2 Additional Spectrum Emission Mask for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.3 Additional Spectrum Emission Mask for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2A.5 Additional Spectrum Emission Mask for CA (4UL CA) |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2B Additional Spectrum Emission Mask for UL-MIMO |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2E Additional Spectrum Emission Mask for UE category 0 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2EA Additional Spectrum Emission Mask for UE category M1 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2EB Additional Spectrum Emission Mask for UE category 1bis |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2EC Additional Spectrum Emission Mask for UE category M2 |  | Same as 6.6.2.2 |  | Same as 6.6.2.2 |  |  | Same as 6.6.2.2 |  |
| 6.6.2.2G.1 Additional Spectrum Emission Mask for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | ΔfOOB(MHz) 0-0.5 [-13-12∆fOOBMHz] ΔfOOB(MHz) 0.5-5 [-19-169∆fOOBMHz-0.5] ΔfOOB(MHz) 5-10 [-27-2∆fOOBMHz-5.0{ QUOTE -13-12∆fOOBMHz}] |  | Same as 6.6.2.2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.2.2G.2 Additional Spectrum Emission Mask for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | same as 6.6.2.2 for E-UTRA uplink same as 6.6.2.2G.1 for sidelink |  | same as 6.6.2.2 for E-UTRA uplink same as 6.6.2.2G.1 for sidelink |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.2.3 Adjacent Channel Leakage power Ratio |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 30 dB UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 |  | 0 dB 0.8 dB 0.8 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 29.2 dB UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 |  |
| 6.6.2.3_1 Adjacent Channel Leakage power Ratio for HPUE |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR for Power Class 1: 37 dB E-UTRA ACLR for Power Class 2: 31 dB |  | 0dB 0.8dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR for Power Class 1: 36.2 dB E-UTRA ACLR for Power Class 2: 30.2 dB |  |
| 6.6.2.3_2 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3_3 Adjacent Channel Leakage power Ratio for UL 64QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3_4 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 64QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3_5 Adjacent Channel Leakage power Ratio for UL 256QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3_6 Adjacent Channel Leakage power Ratio for Multi-Cluster PUSCH with UL 256QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. UTRA ACLR: 33 dB for UTRA ACLR 1 36 dB for UTRA ACLR 2 E-UTRA ACLR: 30 dB |  | 0 dB 0.8 dB 0.8 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT UTRA ACLR: 32.2 dB for UTRA ACLR 1 35.2 dB for UTRA ACLR 2 E-UTRA ACLR: 29.2 dB |  |
| 6.6.2.3A.1_1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3A.1 |  | Same as 6.6.2.3A.1 |  |  | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.1_2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3A.1 |  | Same as 6.6.2.3A.1 |  |  | Same as 6.6.2.3A.1 |  |
| 6.6.2.3A.2 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.6.2.3 per component carrier E-UTRA ACLR2: 40dB |  | Same as 6.6.2.3 per component carrier 1.3dB |  |  | Same as 6.6.2.3 per component carrier E-UTRA ACLR: 38.7 dB |  |
| 6.6.2.3A.2_1 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3 per component carrier |  | Same as 6.6.2.3 per component carrier |  |  | Same as 6.6.2.3 per component carrier |  |
| 6.6.2.3A.2_2 Adjacent Channel Leakage power Ratio for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3A.2 |  | Same as 6.6.2.3A.2 |  |  | Same as 6.6.2.3A.2 |  |
| 6.6.2.3A.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_1 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.3_2 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.1 Adjacent Channel Leakage power Ratio for CA (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.2 Adjacent Channel Leakage power Ratio for CA (inter-band 3DL CA and 3UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.4.3 Adjacent Channel Leakage power Ratio for CA (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3A.5 Adjacent Channel Leakage power Ratio for CA (4UL CA) |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3B Adjacent Channel Leakage power Ratio for UL-MIMO |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3E Adjacent Channel Leakage power Ratio for UE category 0 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3EA Adjacent Channel Leakage power Ratio for UE category M1 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3EB Adjacent Channel Leakage power Ratio for UE category 1bis |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3EC Adjacent Channel Leakage power Ratio for UE category M2 |  | Same as 6.6.2.3 |  | Same as 6.6.2.3 |  |  | Same as 6.6.2.3 |  |
| 6.6.2.3F Adjacent Channel Leakage power Ratio for UE category NB1 |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. GSM ACLR: 20 dB UTRA ACLR: 37 dB for UTRA ACLR |  | 0 dB 0.8 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT GSM ACLR: 19.2 dB UTRA ACLR: 36.2 dB for UTRA ACLR |  |
| 6.6.2.3G.1 Adjacent Channel Leakage power Ratio for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 30dB for power class 3 UE 31dB for power class 2 UE |  | 0.8 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement - TT E-UTRA ACLR: 29.2dB for power class 3 UE 30.2dB for power class 2 UE |  |
| 6.6.2.3G.2 Adjacent Channel Leakage power Ratio for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.6.2.3G.1 |  | Same as 6.6.2.3G.1 |  |  | Same as 6.6.2.3G.1 |  |
| 6.6.2.3G.3 Adjacent Channel Leakage power Ratio for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.6.2.3G.1 |  | Same as 6.6.2.3G.1 |  |  | Same as 6.6.2.3G.1 |  |
| 6.6.2.4 Additional ACLR requirements |  | If the adjacent channel power is greater than –50 dBm then the ACLR shall be higher than the values specified below. E-UTRA ACLR: 43 dB for UTRA ACLR 2 |  | 0 dB 0.8 dB |  |  | Formula: ACLR Minimum Requirement + TT Formula: ACLR Minimum Requirement – TT E-UTRA ACLR: 42.2 dB for UTRA ACLR 2 |  |
| 6.6.3.1 Transmitter Spurious emissions |  | 9 kHz f < 150 kHz: -36dBm / 1kHz 150 kHz f < 30 MHz: -36dBm / 10kHz 30 MHz f < 1 GHz: -36dBm / 100kHz 1 GHz f < 12.75 GHz: -30dBm / 1MHz 12.75 GHz ≤ f < 19 GHz: -30dBm / 1MHz |  | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.1_1Transmitter Spurious emissions for Multi-Cluster PUSCH |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.1A.1 Transmitter Spurious emissions for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3.1A.2 Transmitter Spurious emissions for CA (inter-band DL CA and UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.3 Transmitter Spurious emissions for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.1 Transmitter Spurious emissions for CA (3UL CA) (intra-band contiguous 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.2 Transmitter Spurious emissions for CA (3UL CA) (inter-band 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.4.3 Transmitter Spurious emissions for CA (3UL CA) (intra-band non-contiguous 3DL CA and 3UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3.1A.5 Transmitter Spurious emissions for CA (4UL CA) |  | Same as 6.6.3.1 per CC |  | Same as 6.6.3.1 per CC |  |  | Same as 6.6.3.1 per CC |  |
| 6.6.3B.1Transmitter Spurious emissions for UL-MIMO |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3E.1 Transmitter Spurious emissions for UE category 0 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3E.2 Spurious emission band UE co-existence for UE category 0 |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  |  |  |
| 6.6.3E.3 Additional spurious emissions for UE category 0 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3EA.1 Transmitter Spurious emissions for UE category M1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3EA.2 Spurious emission band UE co-existence for UE category M1 |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  |  |  |
| 6.6.3EA.3 Additional spurious emissions for UE category M1 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3EB.1 Transmitter Spurious emissions for UE category 1bis |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3EB.2 Spurious emission band UE co-existence for UE category 1bis |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  | Same as 6.6.3.2 |  |
| 6.6.3EB.3 Additional spurious emissions for UE category 1bis |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3EC.1 Transmitter Spurious emissions for UE category M2 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3EC.2 Spurious emission band UE co-existence for UE category M2 |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  | Same as 6.6.3.2 |  |
| 6.6.3EC.3 Additional spurious emissions for UE category M2 |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3F.1 Transmitter Spurious emissions for UE category NB1 |  | Same as 6.6.3.1 |  | Same as 6.6.3.1 |  |  | Same as 6.6.3.1 |  |
| 6.6.3G.1 Transmitter Spurious emissions for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | 9 kHz f < 150 kHz: -36dBm / 1kHz 150 kHz f < 30 MHz: -36dBm / 10kHz 30 MHz f < 1 GHz: -36dBm / 100kHz 1 GHz f < 12.75 GHz: -30dBm / 1MHz 12.75 GHz ≤ f < 26 GHz: -30dBm / 1MHz |  | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.1_1 Spurious emission band UE co-existence for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.2 Spurious emission for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.2_1 Spurious emission band UE co-existence for V2X Communication / Sidelink simultaneous with E-UTRA uplink transmissions |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3G.3 Spurious emission for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  |  |  |  |
| 6.6.3G.3_1 Spurious emission for V2X Communication / Intra-band contiguous MCC operation |  | Same as 6.6.3G.1 |  | Same as 6.6.3G.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2 Spurious emission band UE co-existence |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz Frequencies as detailed in core requirement |  | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.1 Spurious emission band UE co-existence for CA (intra-band contiguous DL CA and UL CA) |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz -15.5 dBm / 5MHz +1.6 dBm /5MHz Frequencies as detailed in core requirement |  | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.3 Spurious emission band UE co-existence for CA (intra-band non-contiguous DL CA and UL CA) |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.4 Spurious emission band UE co-existence for CA (3UL CA) |  | -35 dBm / 6.25kHz -36 dBm / 100kHz -41 dBm / 300kHz -37 dBm / 1MHz -40 dBm / 1MHz -50 dBm / 1MHz -15.5 dBm / 5MHz +1.6 dBm /5MHz Frequencies as detailed in core requirement |  | 0 dB |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.2A.5 Spurious emission band UE co-existence for CA (4UL CA) |  | Same as 6.6.3.2A.1 |  | Same as 6.6.3.2A.1 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3.3 Additional spurious emissions |  | NS_05 1884.5MHz f 1915.7MHz: -41dBm / 300kHz NS_07 769MHz ≤ f ≤ 775MHz -57dBm / 6.25kHz NS_08 860MHz ≤ f ≤ 895MHz -40dBm / 1MHz NS_09 1475.9MHz ≤ f ≤ 1510.9MHz -35dBm / 1MHz NS_44 2620MHz ≤ f ≤ 2645MHz -15.5dBm / 5MHz 2645MHz ≤ f ≤ 2690MHz -40dBm / 1MHz |  | 0 dB 1.5dB 0 dB 0 dB 0 dB |  |  | Formula: Minimum Requirement + TT -41dBm / 300kHz -55.5 dBm / 6.25kHz -40dBm / 1MHz -35dBm / 1MHz -15.5dBm / 5MHz -40dBm / 1MHz |  |
| 6.6.3.3_1 Additional spurious emissions for UL 64QAM |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3_2 Additional spurious emissions for UL 256QAM |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) |  | CA_NS_01 E-UTRA band 34: -50dBm / 1MHz 1884.5MHz f 1915.7MHz: -41dBm / 300kHz CA_NS_02 E-UTRA band 33: -50dBm / 1MHz E-UTRA band 34: -50dBm / 1MHz CA_NS_03 E-UTRA band 34: -50dBm / 1MHz E-UTRA band 39: -50dBm / 1MHz |  | 0 dB 0 dB 0 dB 0 dB 0 dB 0 dB |  |  | Formula: Minimum Requirement + TT -50 MHz / 1MHz -41 dBm / 300kHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz -50 MHz / 1MHz |  |
| 6.6.3.3A.1_1 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 64QAM |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  |  | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.1_2 Additional spurious emissions for CA (intra-band contiguous DL CA and UL CA) for UL 256QAM |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  |  | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.2 Additional spurious emissions for CA (inter-band DL CA and UL CA) |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_1 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 64QAM |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.2_2 Additional spurious emissions for CA (inter-band DL CA and UL CA) for UL 256QAM |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Same as 6.6.3.3 |  |
| 6.6.3.3A.4 Additional spurious emissions for CA (3UL CA) |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  |  | Same as 6.6.3.3A.1 |  |
| 6.6.3.3A.5 Additional spurious emissions for CA (3UL CA) |  | Same as 6.6.3.3A.1 |  | Same as 6.6.3.3A.1 |  |  | Same as 6.6.3.3A.1 |  |
| 6.6.3B.2 Spurious emission band UE co-existence for UL-MIMO |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3B.3 Additional spurious emissions for UL-MIMO |  | Same as 6.6.3.3 |  | Same as 6.6.3.3 |  |  | Formula: Minimum Requirement + TT |  |
| 6.6.3F.2 Spurious emission band UE co-existence for category NB1 UE |  | Same as 6.6.3.2 |  | Same as 6.6.3.2 |  |  | Formula: Minimum Requirement + TT |  |
| 6.7 Transmit intermodulation |  | Intermodulation Product 5MHz -29 dBc 10MHz -35 dBc CW Interferer level = -40 dBc |  | 0 dB |  |  | Formula: CW interferer Minimum Requirement– TT Intermod Products limits remain unchanged. CW interferer level = -40 dBc |  |
| 6.7A.1 Transmit intermodulation for CA (intra-band contiguous DL CA and UL CA) |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7A.2 Transmit intermodulation for CA (inter-band DL CA and UL CA) |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7A.5 Transmit intermodulation for CA (4UL CA) |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7B Transmit intermodulation for UL-MIMO |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7E Transmit intermodulation for UE category 0 |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7EA Transmit intermodulation for UE category M1 |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7EB Transmit intermodulation for UE category 1bis |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7EC Transmit intermodulation for UE category M2 |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7F Transmit intermodulation for UE category NB1 and NB2 |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7G.1 Transmit intermodulation for V2X Communication / Non-concurrent with E-UTRA uplink transmissions |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7G.2 Transmit intermodulation for V2X Communication / Simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.7G.3 Transmit intermodulation for V2X Communication / Intra-band contiguous multi-carrier operation |  | Same as 6.7 |  | Same as 6.7 |  |  | Same as 6.7 |  |
| 6.8B Time alignment error for UL-MIMO |  | The Time Alignment Error (TAE) shall not exceed 130 ns |  | 25 ns |  |  | Formula: Minimum Requirement+ TT |  |

## <<< END OF CHANGES >>>