3GPP TSG-RAN WG5 Meeting # 101	R5-236934

# Chicago, USA, November 13 – 17, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2538 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to 6.2I on transmitter power for RedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The table title for test requirement of UE maximum output power for RedCap is not correct. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the table title for UE MOP test requirement for RedCap. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The table title for UE MOP test requirement for RedCap will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2I.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2I	Transmitter power for RedCap

6.2I.1	UE maximum output power for RedCap

6.2I.1.1	Test purpose

To verify that the error of the UE maximum output power does not exceed the range prescribed by the specified nominal maximum output power and tolerance.

An excess maximum output power has the possibility to interfere to other channels or other systems. A small maximum output power decreases the coverage area.

6.2I.1.2	Test applicability

This test case applies to all types of NR RedCap UE release 17 and forward.

6.2I.1.3	Minimum conformance requirements

For Redcap UE, the requirements for power class 3 specified in clause 6.2.1 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2I.1.

6.2I.1.4	Test description

6.2I.1.4.1	Initial conditions

Same initial conditions as in 6.2.1 with following exception:

-	The test channel bandwidth are specified in TS 38.508-1 [5] subclause 4.3.1 for RedCap.

6.2I.1.4.2	Test procedure

Same test procedure as steps 1~3 of clause 6.2.1.4.2.

6.2I.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions.

Table 6.2I.1.4.3-1: PUSCH-Config

Derivation Path: TS 38.508-1 [5], Table 4.6.3-118 with condition TRANSFORM_PRECODER_ENABLED

6.2I.1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2I.1.5-1 for Power Class 7.

Table 6.2I.1.5-1: Maximum Output Power test requirement for Power Class 7

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 7 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n14 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: TT for each frequency and channel bandwidth is specified in Table 6.2I.1.5-2. |  |  |  |  |  |  |  |  |

Table 6.2I.1.5-2: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |

For the UE which supports SUL configuration, ΔTIB,c as specified in clause 6.2C.2 for SUL applies. Unless otherwise stated, ΔTIB,c is set to zero. In case the UE supports more than one of band combinations, and an operating band belongs to more than one band combinations then

a)	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔTIB,c shall be the average value for all band combinations defined in clause 6.2C.2 in this specification, truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔTIB,c among the different supported band combinations involving such band shall be applied

b)	When the operating band frequency range is > 1 GHz, the applicable additional ΔTIB,c shall be the maximum value for all band combinations defined in clause 6.2C.2 in this specification for the applicable operating bands.

==============================================================

<< End of changes >>