3GPP TSG-RAN5 Meeting #101	R5-236976

# Chicago , United States, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0567 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR IMS TC 11.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the core Spec, the precondition of bullet “2) if the UE is not able to provide an updated MSD” includes "datatype" attribute set to "eCall.MSD". “2) if the UE is not able to provide an updated MSD” means UE works with "datatype" attribute set to "eCall.MSD" and it can not get the updated MSD from uplayer, and then it needs to send an INFO. There is no core Spec for unsupported datatype in TP3 and the subsequent TP4. As for TP1 and TP2, they are already coverd by some other testcases. During an emergency session established for eCall type of emergency service as described in subclause 5.1.6.11.2, if the UE receives an INFO request with: 1) an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244]; 2) a multipart/mixed body including: a) an "application/EmergencyCallData.Control+xml" MIME body part as defined in RFC 8147 [244] containing a "request" element with an "action" attribute set to "send-data" and a "datatype" attribute set to "eCall.MSD"; and … the UE shall proceed as follows: 1) if the UE is able to provide an updated MSD, the UE shall send an INFO request containing: a) an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244]; b) … 2) if the UE is not able to provide an updated MSD, the UE shall send an INFO request containing: a) an Info-Package header field set to "EmergencyCallData.eCall.MSD" as defined in RFC 8147 [244]; … |  |  |  |  |  |  |  |  |
| Summary of change: |  | Void this testcase |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

11.6	Void

<End of Changes>