3GPP TSG-RAN5 Meeting #101	R5-237006

Chicago, USA, November 13 - 17 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2801 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR SA FR1-FR1 Event triggered reporting without DRX and with SSB time index for 2Rx UE test case 16.6.2.6 including Test Tolerance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_plus_ARCH-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | NR SA FR1-FR1 Event triggered reporting test case 16.6.2.6 is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case has been corrected. Following has been added/corrected; - Test Tolerance was added, - editor’s note removed, - Applicability corrected in 38.522 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 16.6.2.6 will remain incomplete in and WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependant: R4-2320133 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

#### 16.6.2.6	NR SA FR1-FR1 Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used for 2 Rx UE

16.6.2.6.1	Test purpose

To verify that the UE makes correct reporting of an event. This test will partly verify the inter-frequency cell search requirements in 38.133 [6] clause 9.3B.4.

16.6.2.6.2	Test applicability

This test applies to all types of NR RedCap UE with 1 Rx from Release 17 onwards.

16.6.2.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 16.6.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.16.6.2.6.

16.6.2.6.4	Test description

Two cells are deployed in the test, which are FR1 PCell (NR Cell 1) and a FR1 neighbour cell (NR Cell 2) on the different frequency as the PCell. The general and cell specific test parameters for PCell and neighbour cell are given in Table 16.6.2.6.4.1-3 and Table 16.6.2.6.5-1, respectively. In the measurement configuration a measurement object is configured for the frequency of the neighbour cell, and it indicates to the UE that event-triggered reporting with Event A3 is used.

16.6.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 16.6.2.6.4.1-1.

Table 16.6.2.6.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 16.6.2.6-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 16.6.2.6-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 16.6.2.6-3 | 30 kHz SSB SCS, 20 MHz bandwidth, TDD duplex mode |
| 16.6.2.6-4 | 15 kHz SSB SCS, 10 MHz bandwidth, HD-FDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: target NR cell has the same SCS, BW and duplex mode as NR serving cell |  |

Configure the test requirement and the DUT according to the parameters in Table 16.6.2.6.4.1-2.

Table 16.6.2.6.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.14-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 16.6.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.2.1 |  |
| Exceptions to connection diagram |  |  |  |

Table 16.6.2.6.4.1-3: General test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3,4 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3,4 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3,4 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1,2,3,4 | 0 | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1,2,3,4 | 9 |  |
| SMTC-SSB parameters |  | Config 1,2,4 | SSB.1 FR1 | As specified in clause A.3.10.1 |
|  |  | Config 3 | SSB.1 RedCap FR1 | As specified in clause A.3.10.1 |
| A3-Offset | dB | Config 1,2,3,4 | -6 |  |
| Hysteresis | dB | Config 1,2,3,4 | 0 |  |
| CP length |  | Config 1,2,3,4 | Normal |  |
| TimeToTrigger | s | Config 1,2,3,4 | 0 |  |
| Filter coefficient |  | Config 1,2,3,4 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3,4 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1,4 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3,4 | 5 |  |
| T2 | s | Config 1,2,3,4 | 1 |  |

16.6.2.6.4.2	Test procedure

Same as the test procedure given in clause 6.6.2.2.4.2 with following exceptions:

-	Step 6 in test procedure in clause 6.6.2.2.4.2 is replaced by following steps.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 920 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

16.6.2.6.4.3	Message contents

Same as the message contents given in clause 6.6.2.1.4.3 with following exceptions:

Table 16.6.2.6.4.3-1: Common Exception messages SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Default Message Contents |  |
| --- | --- |
| Default RRC messages and information elements contents exceptions | Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 RedCap |

16.6.2.6.5	Test requirement

Table 16.6.2.6.4.1-3 and Table 16.6.2.6.5-1 define the primary level settings including test tolerances.

Table 16.6.2.6.5-1: Cell specific test parameters for SA inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3,4 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
|  |  |  | Config 4 | HD-FDD |  |  |  |  |
| TDD configuration |  |  | Config 1,4 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2,4 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 20: NRB,c = 51 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3, 4 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1,4 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.2 |  |  | Config 1,2,3,4 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 | CCR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CCR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CCR.2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1,2,4 | SSB.1 FR1 |  |  | SSB.1 FR1 |  |
|  |  |  | Config 3 | SSB.1 RedCap FR1 |  |  | SSB.1 RedCap FR1 |  |
| SMTC configuration defined in A.4A |  |  | Config 1,4 | SMTC.5 RedCap |  |  | SMTC.5 RedCap |  |
|  |  |  | Config 2, 3 | SMTC.5 RedCap |  |  | SMTC.5 RedCap |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2,4 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3,4 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98 |  | -98 |  |  |
| Note2 |  | dBm/SCS | Config 1,2,4 | -98 |  | -98 |  |  |
|  |  |  | Config 3 | -95 |  | -95 |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2,4 | -94 | -94 |  | -Infinity | -91 |
|  |  |  | Config 3 | -91 | -91 |  | -Infinity | -88 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1,2,3,4 | 4 | 4 |  | -Infinity | 7 |
| IoNote3 |  | dBm/9.36MHz | Config 1,2,4 | -64.59 | -64.59 |  | -70.05 | -62.26 |
|  |  | dBm/18.72 MHz | Config 3 | -61.68 | -61.68 |  | -67.13 | -59.34 |
| Propagation Condition |  |  | Config 1,2,3,4 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 920 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

UE is not required to report SSB time index.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

///End of changes