3GPP TSG-RAN5 Meeting #101	R5-237017

Chicago, USA, November 13 - 17 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2811 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of SA event triggered reporting tests for FR1 without gap when DRX is not used 6.6.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM test case 6.6.2.11 is incomplete in TS 38.533 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RRM test case 6.6.2.11 has been updated. Following has beed added/corrected: Message content has been added. Cell mapping has been corrected. Applicability has been corrected in 38.522. Editor’s note updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test case 6.6.2.11 will remain incomplete in TS 38.533 and WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

#### 6.6.2.11	NR SA FR1-FR1 event triggered reporting tests for FR1 without gap when DRX is not used

Editor's Note: This test case is incomplete in following aspects:

-	TT analysis is missing.

6.6.2.11.1	Test purpose

To verify that the UE makes correct reporting of an event when no gap is configured and DRX is not used within inter-frequency NR cell search requirements in TS 38.133 [6] clause 9.3.9.

6.6.2.11.2	Test applicability

This test applies to all types of NR UE release 16 which supports interFrequencyMeas-Nogap-r16.

6.6.2.11.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.6.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.2.11.

6.6.2.11.4	Test description

6.6.2.11.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.2.11.4.1-1.

Table 6.6.2.11.4.1-1: SA event triggered reporting tests for FR1 without gap when DRX is not used supported test configurations

| Configuration | Description |
| --- | --- |
| 6.6.2.11-1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD – TDD duplex mode |
| 6.6.2.11-2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD – TDD duplex mode |
| 6.6.2.11-3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD – TDD duplex mode |
| NOTE.1: The UE is only required to be tested in one of the supported test configurations. |  |

Configure the test equipment and the DUT according to the parameters in Table 6.6.2.11.4.1-2.

Table 6.6.2.11.4.1-2: Test Environment parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.4-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2.. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.2.11.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - Without LTE link - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.1 for DUT part and A.3.1.8.4 for TE part. |  |  |

1.	The general test parameter settings are set up according to Table 6.6.2.10.4.1-3.

2.	Message contents are defined in clause 6.6.2.10.4.3.

3.	There are two NR cells on two carriers specified in the test. Cell 1 is the cell used for connection setup and Cell 2 is a target cell on a different carrier than Cell 1. The power levels and settings for Cell 2 are set according to Annex C.1.2.

Table 6.6.2.11.4.1-3: General test parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1,2,3 | 1, 2 | Two FR1 NR carrier frequencies is used. |
| Active cell |  | Config 1,2,3 | NR cell 1 (Pcell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1,2,3 | NR cell2 | NR cell 2 is on NR RF channel number 2. |
| A3-Offset | dB | Config 1,2,3 | -6 |  |
| Hysteresis | dB | Config 1,2,3 | 0 |  |
| CP length |  | Config 1,2,3 | Normal |  |
| TimeToTrigger | s | Config 1,2,3 | 0 |  |
| Filter coefficient |  | Config 1,2,3 | 0 | L3 filtering is not used |
| DRX |  | Config 1,2,3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
|  |  | Config 2,3 | 3s | Synchronous cells. |
| T1 | s | Config 1,2,3 | 5 |  |
| T2 | s | Config 1,2,3 | 1 |  |

6.6.2.11.4.2	Test procedure

In this test, there are two cells: NR cell 1 as PCell in FR1 on NR RF channel 1 and NR cell 2 as neighbour cell in FR1 on NR RF channel 2.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used. The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of NR cell 2.

1. Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2. Set the parameters according to T1 in Table 6.6.2.11.4.1-2.

3. The SS shall transmit an RRCReconfiguration message.

4. The UE shall transmit RRCReconfigurationComplete message. T1 starts.

5. When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.6.2.11.4.1-2. T2 Starts.

6. UE shall transmit a MeasurementReport message triggered by Event A3. If the overall delays measured from the beginning of time period T2 is less than 800 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement, then the number of failure tests is increased by one.

7. After the SS receives the MeasurementReport message in step 6 or when T2 expires, the SS shall:

-	transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources

OR

-	switch the UE off.

8. Set Cell 2 physical cell identity = ((current cell 2 physical cell identity + 1) mod 14 + 2) for next iteration of the test procedure loop.

9. Depending on the choice in Step 7, the SS:

-	if the RRC Connection Release has been sent, transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On according to TS 38.508-1 [14] clause 4.5.),
OR:

-	if the device has been switched off, switches on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On  according to TS 38.508-1 [14] clause 4.5.

10. Repeat step 2-9 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

11. Repeat step 1-10 for each sub-test in Table 6.6.2.11.4.1-2 as appropriate.

6.6.2.11.4.3	Message contents

Table 6.6.2.11.4.3-1: Common Exception messages SA inter frequency event triggered reporting without SSB time index detection in non-DRX

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.1-1 Table H.3.1-2 with Condition INTER-FREQ Table H.3.1-4 with A3-offset = -6dB Table H.3.1-5 Table H.3.1-7 with Condition INTER-FREQ |
| Specific message contents exceptions for Test Configuration 6.6.2.1-1 | Table H.3.1-3 with Conditions INTER-FREQ MO Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.5 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-2 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |
| Specific message contents exceptions for Test Configuration 6.6.2.1-3 | Table H.3.1-3 with Conditions INTER-FREQ MO and Synchronous cells Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.4 |

6.6.2.11.5	Test requirement

Table 6.6.2.11.5-1 defines the primary level settings including test tolerances for SA event triggered tests for FR1 without gap when DRX is not used.

Table 6.6.2.11.5-1: Cell specific test parameters for SA event triggered reporting tests for FR1 without gap when DRX is not used

| Parameter |  | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 |  | T1 | T2 |
| NR RF Channel Number |  |  | Config 1,2,3 | 1 |  |  | 2 |  |
| Duplex mode |  |  | Config 1 | FDD |  |  |  |  |
|  |  |  | Config 2,3 | TDD |  |  |  |  |
| TDD configuration |  |  | Config 1 | Not Applicable |  |  |  |  |
|  |  |  | Config 2 | TDDConf.1.1 |  |  |  |  |
|  |  |  | Config 3 | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP BW |  | MHz | Config 1,2 | 10: NRB,c = 52 |  |  |  |  |
|  |  |  | Config 3 | 40: NRB,c = 106 |  |  |  |  |
| BWP configuration | Initial DL BWP |  | Config 1, 2, 3 | DLBWP.0.1 |  |  | NA |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  |  | NA |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  |  | NA |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  |  | NA |  |
| TRS configuration |  |  | Config 1 | TRS.1.1 FDD |  |  | NA |  |
|  |  |  | Config 2 | TRS.1.1 TDD |  |  | NA |  |
|  |  |  | Config 3 | TRS.1.2 TDD |  |  | NA |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1,2,3 | OP.1 |  |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | SR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | SR2.1 TDD |  |  |  |  |
| CORESET Reference Channel |  |  | Config 1 | CR.1.1 FDD |  |  |  |  |
|  |  |  | Config 2 | CR.1.1 TDD |  |  |  |  |
|  |  |  | Config 3 | CR2.1 TDD |  |  |  |  |
| SSB parameters |  |  | Config 1 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 2 | SSB.1 FR1 |  |  | SSB.5 FR1 |  |
|  |  |  | Config 3 | SSB.2 FR1 |  |  | SSB.6 FR1 |  |
| SMTC configuration defined in A.3.11 |  |  | Config 1 | SMTC.2 |  |  | SMTC.5 |  |
|  |  |  | Config 2, 3 | SMTC.1 |  |  | SMTC.4 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1,2 | 15 |  |  |  |  |
|  |  |  | Config 3 | 30 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | Config 1,2,3 | 0 |  |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz |  | -98+TT |  | -98+TT |  |  |
| Note2 |  | dBm/SCS | Config 1,2 | -98+TT |  | -98+TT |  |  |
|  |  |  | Config 3 | -95+TT |  | -95+TT |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | Config 1,2 | -94+TT | -94+TT |  | -Infinity | -91+TT |
|  |  |  | Config 3 | -91+TT | -91+TT |  | -Infinity | -88+TT |
|  |  | dB | Config 1,2,3,4,5,6 | 4 | 4 |  | -Infinity | 7+TT |
|  |  | dB | Config 1,2,3 | 4 | 4 |  | -Infinity | 7+TT |
| IoNote3 |  | dBm/9.36MHz | Config 1,2 | -64.59+TT | -64.59+TT |  | -70.05+TT | -62.26+TT |
|  |  | dBm/38.16MHz | Config 3 | -58.49+TT | -58.49+TT |  | -63.94+TT | -56.15+TT |
| Propagation Condition |  |  | Config 1,2,3 | AWGN |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 800 ms from the beginning of time period T2. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The UE is not required to read the neighbour cell SSB index in this test.

The rate of correct events observed during repeated tests shall be at least 90% with the confidence level of 95%.

///End of changes