3GPP TSG-RAN5 Meeting #101	R5-237035

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2989 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating SearchSpace content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some PEI settings are missing from SearchSpace. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PEI settings are missing to SearchSpace. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PEI configuration will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 4.6.3	Radio resource control information elements

[SECTION SKIPPED]

#### –	SearchSpace

Table 4.6.3-162: SearchSpace

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition CSS |  | CSS |
|  | SearchSpaceId with condition USS |  | USS |
|  | SearchSpaceId with condition SISS |  | SISS |
|  | SearchSpaceId with condition MSS |  | MSS |
|  | SearchSpaceId with condition SLSS |  | SLSS |
|  | SearchSpaceId with condition DCI_2_6 |  | DCI_2_6 |
|  | SearchSpaceId with condition PEI |  | PEI |
| controlResourceSetId | ControlResourceSetId |  |  |
|  | ControlResourceSetId with condition Common0 |  | CSS, SISS, PEI |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| sl5 | 0 |  | (CSS OR USS) AND HD_FDD |
| sl10 | 5 |  | SISS |
| } |  |  |  |
| duration | Not present | 1 slot per default |  |
|  | 2 |  | SISS |
|  | 4 |  | (CSS OR USS) AND HD_FDD |
| monitoringSymbolsWithinSlot | 10000000000000 |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel1 | n0 |  |  |
| aggregationLevel2 | n4 |  | FR1 |
|  | n3 |  | FR2 |
| aggregationLevel4 | n2 |  |  |
| aggregationLevel8 | n1 |  |  |
| aggregationLevel16 | n0 |  |  |
| } |  |  |  |
| searchSpaceType | Not present |  | MSS |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS, DCI_2_6, PEI |
| dci-Format0-0-AndFormat1-0 SEQUENCE { |  |  |  |
| } |  |  |  |
| dci-Format2-0 | Not present |  |  |
| dci-Format2-1 | Not present |  |  |
| dci-Format2-2 | Not present |  |  |
| dci-Format2-3 | Not present |  |  |
| } |  |  |  |
| ue-Specific SEQUENCE { |  |  | USS, SLSS |
| dci-Formats | formats0-0-And-1-0 |  | Short_DCI |
| dci-Formats | formats0-1-And-1-1 |  |  |
| dci-Formats-MT-r16 | Not present |  |  |
| dci-FormatsSL-r16 | formats3-0 |  | SLSS |
| dci-FormatsExt-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| CSS | Common SearchSpace |
| USS | UE-Specific SearchSpace |
| Short_DCI | Used in test scenarios requiring DCI formats 0-0 and 1-0 on USS |
| SISS | SearchSpace for SI |
| MSS | SearchSpace for MBS Multicast reception |
| PEI | Paging Early Indication is configured in the cell. |
| SLSS | SearchSpace for SL mode 1 transmission |
| DCI_2_6 | Transmission of DCI_2_6 is required |
| HD_FDD | SIG AND pc_halfDuplexFDD_TypeA_RedCap_r17 (i.e HD_FDD UE are performing signalling test on FDD band) |