3GPP TSG-RAN5 Meeting #101	R5-237037

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2550 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PC2 configuration n1 and n3 into MPR UL MIMO TC 6.2D.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_bands_UL_MIMO_R18-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MPR of n1 and n3 PC2 configuration for UL MIMO is missing in TC 6.2D.2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of PC2 configuration n1 and n3 into MPR UL MIMO TC 6.2D.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PC2 UL MIMO bands can’t be test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

Table 6.2D.2.5-2: UE Power Class test requirements (for Bands n1, n3, n41, n77, n78, n79) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO without indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 2 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 3 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 4 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 5 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 6 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 7 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 8 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 9 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 10 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 11 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 12 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 13 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 14 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2a: UE Power Class test requirements (for Bands n1, n3, n41, n77, n78, n79) for Power Class 2 UEs supporting 2-layer codebook based UL MIMO and indicating ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 | （3.02） | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 4 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.6 - TT2） |
| 5 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.6 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 9 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 13 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 14 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2D.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2b: UE Power Class test requirements (for Bands n1, n3, n41, n77, n78, n79) for Power Class 2 UEs supporting ULFPTx and ul-FullPwrMode1-TPMIGroup-r1 and indicating TxD

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 4 | 26 | 0 | 1.0 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 5 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 9 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 15 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 16 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 17 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 18 | 26 | 0 | 5.5 | 0 | （1.52） | 20.5 | （19.02） | 6.0 | （5.02） | 3 | 28.0 + TT | 14.5 - TT | （14.0 - TT2） |
| 19 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 20 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 21 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 22 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 23 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 24 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 25 | 26 | 0 | 4.0 | 0 | （1.52） | 22.0 | （20.52） | 5.0 | （6.02） | 3 | 28.0 + TT | 17.0 - TT | （14.5 - TT2） |
| 26 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 27 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 28 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 29 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 30 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 31 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| 32 | 26 | 0 | 8.0 | 0 | （1.52） | 18.0 | （16.52） | 5.0 |  | 3 | 28.0 + TT | 13.0 - TT | （11.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2D.2.5-2c: UE Power Class test requirements (for Bands n1, n3, n41, n77, n78, n79) for Power Class 2 UEs supporting ULFPTx and, (supporting ul-FullPwrMode1-TPMIGroup-r1 with indicating TxD) or (supporting ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 3.0 |  | 3 | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 3.0 |  | 3 | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 3.0 |  | 3 | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.5 - TT | （17.0 - TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 5.0 | （6.02） | 3 | 28.0 + TT | 16.5 - TT | （14.0 - TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 3.0 |  | 3 | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 21.0 - TT | （17.5 - TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 3.0 | （5.02） | 3 | 28.0 + TT | 20.0 - TT | （16.5 - TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 5.0 |  | 3 | 28.0 + TT | 17.5 - TT | （16.0 - TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 5.0 |  | 3 | 28.0 + TT | 14.5 - TT | （13.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |

<<< END OF CHANGES >>>