Source:        Samsung

Title:           TP analysis on FR1 6.4A.1 frequency error for CA

Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC frequency error, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for frequency error of 2UL CA case.

Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA is defined as table below.

Table 6.5.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |
| Network signalling value |  |  |  | NS_01 Unless given by Table 7.3.3-3 for the band with active uplink carrier |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |
| 6 | 25 | NA | QPSK | 31 | P_6@0 | S_25@0 |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 |
| 15 | 25 |  | QPSK | 40 | P_15@0 | S_25@0 |
| 25 | 6 |  | QPSK | 31 | S_25@0 | P_6@0 |
| 15 | 25 |  | QPSK | 40 | P_15@0 | S_25@0 |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 |
| 25 | 75 |  | QPSK | 100 | S_25@0 | P_75@0 |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 |
| 75 | 100 |  | QPSK | 175 | P_75@0 | S_100@0 |
| 100 | 50 |  | QPSK | 100 | P_100@0 | S_50@0 |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |

The test configuration for UL and DL intra-band contiguous CA in LTE:

Table 6.5.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Low range, High range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 3) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | UL Allocation |  |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation | CC MOD | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 | N/A | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 75 | P_50@0 | S_25@0 | - | - |
| 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 75 | 25 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 129 | P_75@0 | S_54@0 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 100 | 25 |  | QPSK | 125 | P_100@0 | S_25@0 | - | - |
| 100 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| 100 | 100 |  | QPSK | 130 | P_100@0 | S_30@0 | - | - |
| 100 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3A.0-1 for UE supporting two uplink carriers is tested per Test CA configuration. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 4: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |  |  |

The test configuration for UL and DL intra-band non-contiguous CA in LTE:

Table 6.5.1A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC, TL/VL, TL/VH, TH/VL, TH/VH |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: N/A (Range is not relevant), Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | PCC & SCC RB allocation |  | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | NA |  | QPSK | P_25@0 | QPSK | S_25@0 |
| 2 | 50 | 50 | 25 |  |  | QPSK | P_50@0 | QPSK | S_50@0 |
| 3 | 100 | 100 | 5 |  |  | QPSK | P_100@0 | QPSK | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: Depending on CA configurations, only the appropriate Uplink RB allocation value according to table 7.3A.0-4 for UE supporting two uplink carriers is tested per Test CA configuration. Note 3: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 4: For the DL signal one sided dynamic OCNG Pattern OP.1 FDD/TDD is used. |  |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for frequency error for single carrier mode is defined as table below.

Table 6.4.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Smallest, Mid and highest supported SCS per Channel Bandwidth |  |
| Test Parameters |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |
| 1 | CP-OFDM QPSK | Full RB (NOTE 1) | DFT-s-OFDM QPSK | REFSENS (NOTE 2) |
| NOTE 1: Full RB allocation shall be used per each SCS and channel BW as specified in Table 7.3.2.4.1-2 NOTE 2: REFSENS refers to Table 7.3.2.4.1-3 which defines uplink RB configuration and start RB location for each SCS, channel BW and NR band. |  |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Test Environment

Based on the baseline agreed in [1], the same test environment as LTE can be reused for FR1. In LTE test case for Frequency Error, NC, TL/VL, TL/VH, TH/VL and TH/VH are defined.

Proposal 1: Define Test Environment as NC, TL/VL, TL/VH, TH/VL and TH/VH for Frequency Error of 2UL CA in FR1.

Test Frequencies

As mentioned in [3] testing on Mid frequency range for frequency error would be good balance of test accuracy and time since the test frequency condition has small impact on the conversion of RF frequency. Hence it is proposed to test Mid frequency range for inter-band CA and intra-band contiguous frequency error. As for intra-band non-contiguous CA, only low range and high range are defined. So it’s proposed to test low range or high range frequency arbitrarily.

Proposal 2: Select Mid range as test frequency for NR Frequency Error of 2UL inter-band CA and 2UL intra-band CA for FR1. Choose one frequency chosen from Low range or High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA

Test Channel Bandwidth

Same as suggested in [3], testing on highest channel bandwidth for both PCC and SCC would be enough for test coverage at most changeling condition. Hence it is proposed to use highest channel bandwidth of each band for inter-band UL CA frequency error testing. For intra-band CA, highest NRB_agg should be selected.

Proposal 3: Select the highest channel bandwidth to measure Frequency Error for FR1 2UL inter-band CA. Select highest NRB_agg for intra-band CA.

Test Subcarrier Spacing

Same as single carrier case discussed in [3], it is proposed to select lowest SCS case to verify the highest spectrum utilization case for CA in test of frequency error.

Proposal 4: Select the lowest Subcarrier Spacing to test.

DL and UL configuration

Same as single carrier case discussed in [3], the frequency error shall be tested with the DL at REFSENS level [6]. In order to avoid tighter test requirements as in the reference sensitivity test case the same UL and DL RB allocation and waveform shall be applied. TS 38.101-1 requires that REFSENS is tested with DFT-s-OFDM as UL waveform.

Proposal 5: Apply DFT-s-OFDM as UL waveform for Frequency Error in FR1 for 2UL CA.

Proposal 6: Follow the UL and DL RB allocation of REFSENS TC for Frequency Error in FR1 for inter-band 2UL CA and intra-band 2UL CA.

For the downlink full allocation is proposed as the UE’s ability to synchronize its frequency to the downlink is indirectly tested in the REFSENS test case.

Proposal 7: Apply full allocation for the downlink configurations for FR1 inter-band 2UL CA.

Number of test points for Frequency Error

|  | Environmental conditions |  | Maximum Number of Frequencies |  | Maximum Number of ChBW |  | Number SCS |  | Number of steps (mod and RB) |  | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA | 5 |  | 1 |  | 1 |  | 1 |  | 1 |  | 5 |
| Intra-band contiguous CA | 5 |  | 1 |  | 1 |  | 1 |  | 1 |  | 5 |
| Intra-band non-contiguous CA | 5 | 1 | 1 | 1 | 1 | 5 |

Conclusion

As the inter-band UL CA frequency error requirement is defined the same as single carrier requirement with the update of condition of 2UL and 2DL CA active. Hence the test condition is proposed in the same way of NR single

Proposal 1: Define Test Environment as NC, TL/VL, TL/VH, TH/VL and TH/VH for Frequency Error of 2UL inter-band CA in FR1.

Proposal 2: Select Mid range as test frequency for NR Frequency Error of 2UL inter-band CA for FR1.

Proposal 3: Select the highest channel bandwidth to measure Frequency Error for FR1 2UL inter-band CA

Proposal 4: Select the lowest Subcarrier Spacing to test for inter-band 2UL CA.

Proposal 5: Apply DFT-s-OFDM as UL waveform for Frequency Error in FR1 for inter-band 2UL CA.

Proposal 6: Follow the UL and DL RB allocation of REFSENS TC for Frequency Error in FR1 for inter-band 2UL CA and intra-band 2UL CA.

Proposal 7: Apply full allocation for the downlink configurations for FR1 inter-band 2UL CA.

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-185316, TP for updating TR 38.905 with FR1 Frequency Error test point analysis

[4] 3GPP TS 36.521-1 v15.4.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v15.1.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 15).

[6] 3GPP TS 38.101-1 v15.4.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15).