Source:       Samsung, Huawei

Title:         TP analysis on FR1 6.5A.3.1.1 Spurious emission for CA (2UL CA)

Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for NR Spurious emission of 2UL CA case.

Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA is defined as table below.

Table 6.6.3.1A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 25 |  | QPSK | 2 | P_1@5 | S_1@24 | - | - |
| 6 | 25 |  | QPSK | 31 | P_6@0 | S_25@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 6 | 50 |  | QPSK | 2 | P_1@5 | S_1@49 | - | - |
| 6 | 50 |  | QPSK | 56 | P_6@0 | S_50@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 15 |  | QPSK | 2 | P_1@24 | S_1@14 | - | - |
| 25 | 15 |  | QPSK | 40 | P_25@0 | S_15@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@24 | S_1@24 | - | - |
| 25 | 25 |  | QPSK | 50 | P_25@0 | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@24 | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 | S_50@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 50 |  | QPSK | 2 | P_1@49 | S_1@49 | - | - |
| 50 | 50 |  | QPSK | 100 | P_50@0 | S_50@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 50 | 100 |  | QPSK | 2 | P_1@49 | S_1@99 | - | - |
| 50 | 100 |  | QPSK | 150 | P_50@0 | S_100@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 25 |  | QPSK | 2 | P_1@74 | S_1@24 | - | - |
| 75 | 25 |  | QPSK | 100 | P_75@0 | S_25@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 75 | 75 |  | QPSK | 2 | P_1@74 | S_1@74 | - | - |
| 75 | 75 |  | QPSK | 150 | P_75@0 | S_75@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 75 |  | QPSK | 2 | P_1@99 | S_1@74 | - | - |
| 100 | 75 |  | QPSK | 175 | P_100@0 | S_75@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 50 |  | QPSK | 2 | P_1@99 | S_1@49 | - | - |
| 100 | 50 |  | QPSK | 150 | P_100@0 | S_50@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@0 | S_1@0 | - | - |
| 100 | 100 |  | QPSK | 2 | P_1@99 | S_1@99 | - | - |
| 100 | 100 |  | QPSK | 200 | P_100@0 | S_100@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

In LTE specification [4], the test configuration for UL and DL intra-band contiguous CA is defined as table below.

Table 6.6.3.1A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  |  | NC |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  |  | Low range, High range |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  |  | Lowest NRB_agg, Highest NRB_agg (Note 2) |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |  |
| 25 | 25 |  | QPSK | 25 | P_25@0 |  | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 1 | P_1@0 |  | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 1 | P_1@24 |  | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 2 | P_1@0 |  | S_1@24 | - | - |
| 25 | 25 | N/A for Spurious Emissions testing | QPSK | 50 | P_25@0 |  | S_25@0 | - | - |
| 25 | 50 |  | QPSK | 25 | P_25@0 |  | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 1 | P_1@0 |  | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 1 | P_1@24 |  | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 2 | P_1@0 |  | S_1@49 | - | - |
| 25 | 50 |  | QPSK | 75 | P_25@0 |  | S_50@0 | - | - |
| < Some Lines Omitted > |  |  |  |  |  |  |  |  |  |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

In LTE, the test configuration table for intra-band non-contiguous UL CA is extracted as below:

Table 6.6.3.1A.3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: N/A (Range is not relevant), Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A |  | QPSK | P_1@0 | QPSK | S_1@0 |
| 2 | 25 | 25 | 35 |  |  | QPSK | P_1@24 | QPSK | S_1@24 |
| 3 | 25 | 25 | 35 |  |  | QPSK | P_25@0 | QPSK | S_25@0 |
| 4 | 50 | 50 | 25 |  |  | QPSK | P_1@0 | QPSK | S_1@0 |
| 5 | 50 | 50 | 25 |  |  | QPSK | P_1@49 | QPSK | S_1@49 |
| 6 | 50 | 50 | 25 |  |  | QPSK | P_50@0 | QPSK | S_50@0 |
| 7 | 100 | 100 | 5 |  |  | QPSK | P_1@0 | QPSK | S_1@0 |
| 8 | 100 | 100 | 5 |  |  | QPSK | P_1@99 | QPSK | S_1@99 |
| 9 | 100 | 100 | 5 |  |  | QPSK | P_100@0 | QPSK | S_100@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The measurements need to be executed for in-gap interferer positions when the sub-block gap size Wgap allows. |  |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for spurious emission for single carrier mode is defined as table below.

Table 6.5.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1. |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1. |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for Spurious Emissions testing | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | OuterFull |
| 2 |  | CP-OFDM QPSK | Edge_1RB_Left |
| 3 |  | CP-OFDM QPSK | Edge_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 Common UL configuration |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Test Environment

In LTE, Spurious emission for CA is tested under NC only. According to baseline endorsed in [1], it’s proposed to reuse NC for Spurious emission of inter-band UL CA and intra-band contiguous UL CA.

Proposal 1: Define test environment as NC for spurious emission in FR1 for UL inter-band CA and intra-band  UL CA.

Test Subcarrier Spacing

In [5] the smallest SCS are selected for Single Carrier Spurious emission verification. It is proposed to reuse the same way for inter-band UL CA and intra-band contiguous UL CA.

Proposal 2: Select smallest SCS for Spurious emission in FR1 for inter-band UL CA and intra-band UL CA.

Test Frequencies

In LTE, Spurious emission for inter-band CA and intra-band contiguous CA is tested for both Low range and High range. And intra-band non-contiguous CA is tested at max Wgap. The same test coverage as for LTE would be enough to ensure that the spurious emssion obeys the test requirement at low range and high range. In NR, the test frequency range low and high are defined for intra-band non-contiguous UL CA. So low range and high range should be tested in NR for non-contiguous UL CA.

Proposal 3: Test Spurious emission for Low range for PCC and SCC, High range for PCC and SCC for inter-band UL CA; Low range and High range for intra-band contiguous CA. Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA.

Test Channel Bandwidth

In LTE, Spurious emission for inter-band CA and intra-band contiguous CA is tested for Lowest and Highest NRB_agg. Hence it proposed to reuse the LTE method for spurious emission test for inter-band UL CA and intra-band CA.

Proposal 4: Test spurious emission for Lowest and Highest NRB_agg in FR1 for UL inter-band CA and intra-band CA.

Test Modulations and waveforms

For single carrier Spurious emission TC peresented in [5] only CP-OFDM with QPSK is selected for testing. For CA case it’s proposed to reuse this configuration.

Proposal 5: Test Spurious emission for CA with CP-OFDM QPSK.

RB allocation

Considering RB allocation defined in LTE CA and NR SC for Spurious emission, the OuterFull, Edge_1RB_Left and Edge_1RB_Right would be enough to ensure NR CA spurious test coverage.

For inter-band UL CA configuration, the RB allocation on 2 UL cells would be:

Table 2.6-1:	RB allocation for inter-band UL CA

| PCell | SCell |
| --- | --- |
| Outer_Full | Outer_Full |
| Edge_1RB_Left | Edge_1RB_Left |
| Edge_1RB_Right | Edge_1RB_Right |

For intra-band contiguous UL CA configuration, the RB allocation on 2 UL cells would be:

Table 2.6-2:	RB allocation for intra-band contiguous UL CA

| PCell | SCell |
| --- | --- |
| Outer_Full | 0 |
| Edge_1RB_Left | 0 |
| Edge_1RB_Right | 0 |
| Edge_1RB_Left | Edge_1RB_Right |
| Outer_Full | Outer_Full |

For intra-band non-contiguous UL CA configuration, the RB allocation on 2 UL cells would be:

Table 2.6-3:	RB allocation for intra-band non-contiguous UL CA

| PCell | SCell |
| --- | --- |
| Edge_1RB_Left | Edge_1RB_Left |
| Edge_1RB_Right | Edge_1RB_Right |
| Outer_Full | Outer_Full |

Proposal 6: Test Spurious emission for CA with the RB allocation listed in Table 2.6-1 and Table 2.6-2 and Table 2.6-3.

Number of test points for CA spurious emission

|  | Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-band | 1 | 2 | 2 | 1 | 3 | 12 |
| Intra-band contiguous | 1 | 2 | 2 | 1 | 5 | 20 |
| Intra-band non-contiguous | 1 | 2 | 2 | 1 | 3 | 12 |

Conclusion

For inter-band UL CA NR ACLR requirement, the test condition is proposed as below:

Proposal 1: Define test environment as NC for spurious emission in FR1 for UL inter-band CA and intra-band UL CA.

Proposal 2: Select smallest SCS for Spurious emission in FR1 for inter-band UL CA and intra-band UL CA.

Proposal 3: Test Spurious emission for Low range for PCC and SCC, High range for PCC and SCC for inter-band UL CA; Low range and High range for intra-band contiguous CA. Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA.

Proposal 4: Test spurious emission for Lowest and Highest NRB_agg in FR1 for UL inter-band CA and intra-band CA.

Proposal 5: Test Spurious emission for CA with CP-OFDM QPSK.

Proposal 6: Test Spurious emission for CA with the RB allocation listed in Table 2.6-1 and Table 2.6-2 and Table 2.6-3.

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-182019, "Discussion of NR FR1 Test Point for TX Spurious Emission test cases "

[4] 3GPP TS 36.521-1 v16.7.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 v16.6.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone (Release 16).

[6] 3GPP TS 38.101-1 v16.6.0 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 16).