Source:        Huawei, Samsung

Title:           TP analysis on FR1 6.5A.4.1 Transmit intermodulation for CA

Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC TXIM, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for Transmit intermodulation of 2UL CA case.

Discussion

In LTE specification [4], the test configuration for UL and DL inter-band and intra-band CA is defined as table below.

Table 6.7A.2.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Mid range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2, 3) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 | N/A for this test | QPSK | 16 | P_8@0 | S_8@0 | - | - |
| 25 | 50 |  | QPSK | 20 | P_8@0 | S_12@0 | - | - |
| 50 | 25 |  | QPSK | 20 | P_12@0 | S_8@0 | - | - |
| 50 | 50 |  | QPSK | 24 | P_12@0 | S_12@0 | - | - |
| 50 | 75 |  | QPSK | 28 | P_12@0 | S_16@0 | - | - |
| 50 | 100 |  | QPSK | 30 | P_12@0 | S_18@0 | - | - |
| 75 | 25 |  | QPSK | 24 | P_16@0 | S_8@0 | - | - |
| 75 | 50 |  | QPSK | 28 | P_16@0 | S_12@0 | - | - |
| 75 | 75 |  | QPSK | 32 | P_16@0 | S_16@0 | - | - |
| 75 | 100 |  | QPSK | 34 | P_16@0 | S_18@0 | - | - |
| 100 | 50 |  | QPSK | 30 | P_18@0 | S_12@0 | - | - |
| 100 | 75 |  | QPSK | 34 | P_18@0 | S_16@0 | - | - |
| 100 | 100 |  | QPSK | 36 | P_18@0 | S_18@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2 Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. Note 3: If in the CA Configuration UE supports CC Combinations with channel BW below 5MHz on at least one CC, these CC Combinations are disregarded for Lowest NRB_agg. Only CC Combinations which have channel BW 5MHz or above on both component carriers are considered for Lowest NRB_agg. |  |  |  |  |  |  |  |  |

Table 6.7A.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  | Mid range |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 25 | 25 | N/A | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1 Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for TX IM for single carrier mode is defined as table below.

Table 6.5.4.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Lowest, Highest |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A for transmit intermodulation test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM PI/2 BPSK | Inner Full |
| 2 |  | DFT-s-OFDM QPSK | Inner Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Test Environment

In LTE test case for TX IM, Normal condition is used for test. Based on the baseline agreed in [1], the same test environment as LTE can be reused for FR1.

Proposal 1: Define Test Environment as NC for TX IM of 2UL CA in FR1.

Test Frequencies

In LTE test case for TX IM, Mid range is used for test. Based on the baseline agreed in [1], the same frequency range as LTE can be reused for FR1.

Proposal 2: Select Mid range as test frequency for NR TX IM of 2UL CA for FR1.

Test Channel Bandwidth

In LTE test case for CA TX IM, both Lowest and highest NRB_agg are used for testing. Based on the baseline agreed in [1], the same frequency range as LTE can be reused for FR1 CA case.

Proposal 3: Select both Lowest and highest NRB_agg to measure TX IM for FR1 2UL CA

Test Subcarrier Spacing

Same as single carrier case discussed in [3], it is proposed to select lowest and highest SCS case to verify TX IM performance for CA

Proposal 4: Select the lowest and highest Subcarrier Spacing to test for inter-band 2UL CA.

DL and UL configuration

Same as single carrier case discussed in [3], DFT-s-OFDM PI/2 BPSK with Inner Full allocation and DFT-s-OFDM QPSK with Inner Full allocation are suggested to be used as UL configuration. For intra-band UL CA, DFT-s-OFDM PI/2 BPSK + Inner Full and DFT-s-OFDM QPSK + Inner Full have the minimum MPR requirement. It’s reasonable to use the same UL configuration.

Proposal 5: Select DFT-s-OFDM PI/2 BPSK with Inner Full allocation and DFT-s-OFDM QPSK with Inner Full allocation

Number of test points

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test Steps |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 2 | 2 | 16 |

Conclusion

For the UL CA TX IM requirement, the test condition is proposed as below

Proposal 1: Define Test Environment as NC for TX IM of 2UL CA in FR1.

Proposal 2: Select Mid range as test frequency for NR TX IM of 2UL CA for FR1.

Proposal 3: Select both Lowest and highest NRB_agg to measure TX IM for FR1 2UL CA

Proposal 4: Select the lowest and highest Subcarrier Spacing to test for 2UL CA.

Proposal 5: Select DFT-s-OFDM PI/2 BPSK with Inner Full allocation and DFT-s-OFDM QPSK with Inner Full allocation

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-185423, “Discussion on Uplink configuration for NR Transmit Intermodulation in FR1”

[4] 3GPP TS 36.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone

[6] 3GPP TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (Release 15).