3GPP TSG-RAN5 Meeting #101	R5-237060

# Chicago , United States, 13th – 17th Nov. 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2566 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating EVM testing for intra-band UL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test points for intra-band contiguous UL CA and non-contiguous UL CA are absent. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding test configuration table for intra-band UL CA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EVM for CA test case can’t be tested for intra-band UL CA |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TR 38.905 CR 0844 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TP in R5-237061 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

6.4A.2	Transmit modulation quality for CA

The requirements in this clause apply with PCC and SCC in the UL configured and activated: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

For intra-band contiguous CA:

The Carrier leakage frequency is optionally indicated by the UE via IE UplinkTxDirectCurrentList , IE UplinkTxDirectCurrentTwoCarrierList-r16 for CA with two component carriers configured for uplink or IE UplinkTxDirectCurrentMoreCarrierList-r17 for CA of any configuration.

If the UE does not indicate DC location parameters, the carrier leakage measurement requirement in clauses 6.4A.2.2 and 6.4A.2.3 shall be waived and the UE’s UL signal left uncorrected for carrier leakage. Any requirement relaxation to accommodate the IQ image shall be omitted.

If the UE indicates carrier leakage frequency as 3300 or 3301 with IE UplinkTxDirectCurrentList or UplinkTxDirectCurrentTwoCarrierList-r16, or if the carrier leakage frequency is outside the activated UL component carriers, the carrier leakage measurement requirement in clauses 6.4A.2.2 and 6.4A.2.3 shall be waived and the UE’s UL signal left uncorrected for carrier leakage. Any requirement relaxation to accommodate the IQ image shall be omitted.

For intra-band non-contiguous CA:

For intra-band non-contiguous carrier aggregation, the requirements in subclauses 6.4A.2.1, 6.4A.2.2 applies.

The requirements in this clause apply with PCC and SCC in the UL configured and activated: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

Carrier leakage frequency is indicated by the UE with IE UplinkTxDirectCurrentMoreCarrierList-r17 or UplinkTxDirectCurrentTwoCarrierList-r16 or UplinkTxDirectCurrentList.

The carrier leakage measurement requirement in clause 6.4A.2.2.2 shall be waived and the UE’s UL signal left uncorrected for carrier leakage when one of the following qualifying conditions apply:

1.	UE reports the parameter 3300 or 3301

2.	UE doesn’t indicate the DC location parameters

Any requirement relaxation to accommodate the IQ image shall be omitted if the qualifying conditions above are present or if the IQ image frequency is outside the activated UL component carriers.

6.4A.2.1	Error Vector Magnitude for CA

6.4A.2.1.0	Minimum conformance requirements

For Inter-band carrier aggregation, EVM measurements are evaluated for each component carrier, and for the different modulations schemes, the EVM requirements shall not exceed the values specified in Table 6.4.2.1.3-1 for the parameters defined in Table 6.4.2.1.3-2, if CA is configured in uplink.

For the intra-band non-contiguous carrier aggregation, EVM measurements are evaluated for each component carrier,

and for the different modulations schemes, the EVM requirements shall not exceed the values specified in Table

6.4A.2.1.0-1, if CA is configured in uplink.

For the intra-band contiguous carrier aggregation, EVM measurements are evaluated for each component carrier, and for the different modulations schemes, the EVM requirements shall not exceed the values specified in Table 6.4A.2.1.0-1, if CA is configured in uplink.

When a single component carrier is configured Table 6.4.2.1.3-1 apply.

The EVM requirements are according to Table 6.4A.2.1.0-1 if CA is configured in uplink with the parameters defined in Table 6.4.2.1.3-2.

Table 6.4A.2.1.0-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level per CC |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

The normative reference for this requirement is TS 38.101 [2] clause 6.4A.2.1

6.4A.2.1.1	Error Vector Magnitude for CA (2UL CA)

6.4A.2.1.1.1	Test Purpose

For 2UL carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirement applies for the allocated component carrier, when all other component carriers are activated, but not allocated.

Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in clause 6.4.2.1.

6.4A.2.1.1.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports 2UL CA.

6.4A.2.1.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.4A.2.1.0.

6.4A.2.1.1.4	Test description

6.4A.2.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR CA configuration specified in clause 5.5A. All of these configurations shall be tested with applicable test parameters for each CA configuration, and are shown in Table 6.4A.2.1.1.4.1-1 and 6.4A.2.1.1.4.1-2. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4A.2.1.1.4.1-1: Inter band CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  | Low range for PCC and SCC High range for PCC and SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg for both PCC and SCC Highest NRB_agg for both PCC and SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest and biggest supported SCS per Channel Bandwidth |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 13 | N/A | DFT-s-OFDM PI/2 BPSK | Inner Full | 0 |
| 23 |  | DFT-s-OFDM PI/2 BPSK | Outer Full | 0 |
| 3 |  | DFT-s-OFDM QPSK | Inner Full | 0 |
| 4 |  | DFT-s-OFDM QPSK | Outer Full | 0 |
| 5 |  | DFT-s-OFDM 16 QAM | Inner Full | 0 |
| 6 |  | DFT-s-OFDM 16 QAM | Outer Full | 0 |
| 7 |  | DFT-s-OFDM 64 QAM | Outer Full | 0 |
| 8 |  | DFT-s-OFDM 256 QAM | Outer Full | 0 |
| 9 |  | CP-OFDM QPSK | Inner Full | 0 |
| 10 |  | CP-OFDM QPSK | Outer Full | 0 |
| 11 |  | CP-OFDM 16 QAM | Inner Full | 0 |
| 12 |  | CP-OFDM 16 QAM | Outer Full | 0 |
| 13 |  | CP-OFDM 64 QAM | Outer Full | 0 |
| 14 |  | CP-OFDM 256 QAM | Outer Full | 0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.3.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |

Table 6.4A.2.1.1.4.1-2: Intra band contiguous CA and non-contiguous CA Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Contiguous CA: Low range, High range Non-contiguosu CA: Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest NRB_agg, Highest NRB_agg |  |  |
| Test SCS as specified in Table 5.3.5-1 |  | lowest and highest supported SCS |  |  |
| Test Parameters |  |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Modulation (NOTE 3) | RB allocation (NOTE 1) |  |
|  |  |  | PCC | SCC |
| 13 | N/A | DFT-s-OFDM PI/2 BPSK | Inner Full | 0 |
| 23 |  | DFT-s-OFDM PI/2 BPSK | Outer Full | 0 |
| 3 |  | DFT-s-OFDM QPSK | Inner Full | 0 |
| 4 |  | DFT-s-OFDM QPSK | Outer Full | 0 |
| 5 |  | DFT-s-OFDM 16 QAM | Inner Full | 0 |
| 6 |  | DFT-s-OFDM 16 QAM | Outer Full | 0 |
| 7 |  | DFT-s-OFDM 64 QAM | Inner Full | 0 |
| 8 |  | DFT-s-OFDM 64 QAM | Outer Full | 0 |
| 9 |  | DFT-s-OFDM 256 QAM | Inner Full | 0 |
| 10 |  | DFT-s-OFDM 256 QAM | Outer Full | 0 |
| 11 |  | CP-OFDM QPSK | Inner Full | 0 |
| 12 |  | CP-OFDM QPSK | Outer Full | 0 |
| 13 |  | CP-OFDM 16 QAM | Inner Full | 0 |
| 14 |  | CP-OFDM 16 QAM | Outer Full | 0 |
| 15 |  | CP-OFDM 64 QAM | Inner Full | 0 |
| 16 |  | CP-OFDM 64 QAM | Outer Full | 0 |
| 17 |  | CP-OFDM 256 QAM | Inner Full | 0 |
| 18 |  | CP-OFDM 256 QAM | Outer Full | 0 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths and Test SCS are checked separately for each NR CA band combination, which applicable channel bandwidths is specified in Table 5.5A.1-1 and SCS is specified in Table 5.3.5-1 for each NR band. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, in Figure A.3.1.1.3 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause4.4.3.

3.	Downlink signals for PCC are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.4A.2.1.1.4.1-1 and 6.4A.2.1.1.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.4A.2.1.1.4.3

6.4A.2.1.1.4.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1, C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [5] clause 5.5.1. Message contents are defined in clause 6.4A.2.1.1.4.3.

3.	SS activates SCC by sending the activation MAC CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 2 seconds (Refer TS 38.133[19], clause9.3).

4.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.4A.2.1.1.4.1-1 and 6.4A.2.1.1.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

5.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level, allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

6.	Measure the EVM and on PCC using Global In-Channel Tx-Test (Annex E).

7.	For modulations except 256QAM, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Pmin, where:

-	Pmin is the minimum output power according to Table 6.3.1.3-1.

-	MU is the test system uplink power measurement uncertainty and is specified in Table F.1.2-1 for the carrier frequency f and the channel bandwidth BW.

-	Uplink power control window size = 1dB (UE power step size) + 0.7dB (UE power step tolerance) + (Test system relative power measurement uncertainty), where, the UE power step tolerance is specified in TS 38.101-1 [2], Table 6.3.4.3-1 and is 0.7dB for 1dB power step size, and the Test system relative power measurement uncertainty is specified in Table F.1.2-1.

For 256 QAM, send uplink power control commands to the UE using 1dB power step size to ensure that the UE output power measured by the test system is within the Uplink power control window, defined as +MU to +(MU + Uplink power control window size) dB of the target power level Pmin + 10 dB, where Pmin, MU and Uplink power control window size are defined above.

8.	Measure the EVM and on PCC using Global In-Channel Tx-Test (Annex E).

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration table 6.5.2.2.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

NOTE2:	The purpose of the Uplink power control window is to ensure that the actual UE output power is no less than the target power level, and as close as possible to the target power level. The relationship between the Uplink power control window, the target power level and the corresponding possible actual UE Uplink power window is illustrated in Annex F.4.2.

6.4A.2.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.

6.4A.2.1.1.5	Test requirement

The PUSCH EVM, derived in Annex E.4.2, shall not exceed the values in Table 6.4A.2.1.1.5-1 for inter-band CA and table 6.4A.2.1.1.5-2 for intra-band CA.

The PUSCH,derived in Annex E.4.6.2, shall not exceed the values in Table 6.4A.2.1.1.5-1 for inter-band CA and table 6.4A.2.1.1.5-2 for intra-band CA when embedded with data symbols of the respective modulation scheme.

Table 6.4A.2.1.1.5-1: Test requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level |
| --- | --- | --- |
| Pi/2-BPSK | % | 30+TT |
| QPSK | % | 17.5+TT |
| 16QAM | % | 12.5+TT |
| 64QAM | % | 8+TT |
| 256 QAM | % | 3.5+TT |

Table 6.4A.2.1.1.5-2: Test Tolerance for Error Vector Magnitude

| Parameter | f ≤ 6.0GHz, BW ≤ 100MHz |  |  |
| --- | --- | --- | --- |
|  | 15dBm < PUL | -25dBm < PUL ≤ 15dBm | -40dBm ≤ PUL ≤ -25dBm |
| Pi/2-BPSK | 0% | 0% | 0% |
| QPSK | 0% | 0% | 0% |
| 16QAM | 0% | 0% | 0% |
| 64QAM | 0% | 0% | 0% |
| 256 QAM | 0.3% | 0.8% | 1.1% |

<End of Changes>