Source:        Huawei, Samsung

Title:           TP analysis on FR1 6.4A.2.1.1 EVM for CA

Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC EVM, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for EVM of 2UL CA case.

Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA, intra-band contiguous CA and non-contiguous CA are defined as table below.

Table 6.5.2A.1.2.4.1-1: Test Configuration Table for inter-band CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 6 | 25 | N.A. | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 2 | 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 3 | 6 | 25 |  | QPSK | 6 | P_6@0 | S_0@0 | - | - |
| 4 | 6 | 25 |  | 16QAM | 1 | P_1@0 | S_0@0 | - | - |
| 5 | 6 | 25 |  | 16QAM | 1 | P_1@5 | S_0@0 | - | - |
| 6 | 6 | 25 |  | 16QAM | 6 | P_6@0 | S_0@0 | - | - |
| 7 | 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 8 | 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 9 | 6 | 50 |  | QPSK | 6 | P_6@0 | S_0@0 | - | - |
| 10 | 6 | 50 |  | 16QAM | 1 | P_1@0 | S_0@0 | - | - |
| 11 | 6 | 50 |  | 16QAM | 1 | P_1@5 | S_0@0 | - | - |
| 12 | 6 | 50 |  | 16QAM | 6 | P_6@0 | S_0@0 | - | - |
| 13 | 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 14 | 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 15 | 15 | 25 |  | QPSK | 15 | P_15@0 | S_0@0 | - | - |
| 16 | 15 | 25 |  | 16QAM | 4 | P_4@0 | S_0@0 | - | - |
| 17 | 15 | 25 |  | 16QAM | 4 | P_4@11 | S_0@0 | - | - |
| 18 | 15 | 25 |  | 16QAM | 15 | P_15@0 | S_0@0 | - | - |
| 19 | 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 20 | 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 21 | 25 | 6 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 22 | 25 | 6 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 23 | 25 | 6 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 24 | 25 | 6 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 25 | 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 26 | 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 27 | 25 | 15 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 28 | 25 | 15 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 29 | 25 | 15 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 30 | 25 | 15 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 31 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 32 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 33 | 25 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 34 | 25 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 35 | 25 | 25 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 36 | 25 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 37 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 38 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 39 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 40 | 25 | 50 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 41 | 25 | 50 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 42 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 43 | 50 | 6 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 44 | 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 45 | 50 | 6 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 46 | 50 | 6 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 47 | 50 | 6 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 48 | 50 | 6 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 49 | 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 51 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 50 | 25 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 53 | 50 | 25 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 54 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 55 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 56 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 57 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 58 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 59 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 60 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 61 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 62 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 63 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 64 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 65 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 66 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 67 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 68 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 69 | 75 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 70 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 71 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 72 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 74 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 77 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 78 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 79 | 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 80 | 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 81 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 82 | 100 | 50 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 83 | 100 | 50 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 84 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 85 | 100 | 75 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 86 | 100 | 75 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 87 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 88 | 100 | 75 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 89 | 100 | 75 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 90 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 91 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 92 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 93 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 94 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 95 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| 96 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 97 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 |  |  |
| 98 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 |  |  |
| 99 | 25 | 75 |  | QPSK | 25 | P_25@0 | S_0@0 |  |  |
| 100 | 25 | 75 |  | 16QAM | 8 | P_8@0 | S_0@0 |  |  |
| 101 | 25 | 75 |  | 16QfAM | 8 | P_8@17 | S_0@0 |  |  |
| 102 | 25 | 75 |  | 16QAM | 25 | P_25@0 | S_0@0 |  |  |
| X+1 | 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| X+2 | 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| X+3 | 75 | 25 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| X+4 | 75 | 25 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| X+5 | 75 | 25 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| X+6 | 75 | 25 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.1.1.4.1-1: Test Configuration Table for intra-band contiguous CA

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB Note 1 | SCCs NRB Note 1 | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 | 50 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 2 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 3 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 4 | 50 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 5 | 50 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 6 | 50 | 50 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 7 | 50 | 25 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 8 | 50 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 9 | 50 | 25 |  | QPSK | 25 | P_25@25 | S_0@0 | - | - |
| 10 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 11 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | - | - |
| 12 | 50 | 25 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 13 | 50 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 14 | 50 | 25 |  | 16QAM | 25 | P_25@25 | S_0@0 | - | - |
| 15 | 50 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 16 | 50 | 25 |  | 16QAM | 8 | P_8@42 | S_0@0 | - | - |
| 17 | 25 | 50 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 18 | 25 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 19 | 25 | 50 |  | QPSK | 12 | P_12@13 | S_0@0 | - | - |
| 20 | 25 | 50 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 21 | 25 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 22 | 25 | 50 |  | 16QAM | 12 | P_12@13 | S_0@0 | - | - |
| 23 | 75 | 75 | N.A. | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 24 | 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 25 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 26 | 75 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 27 | 75 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 28 | 75 | 75 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 29 | 100 | 25 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 30 | 100 | 25 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 31 | 100 | 25 |  | QPSK | 25 | P_25@75 | S_0@0 | - | - |
| 320 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 33 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 34 | 100 | 25 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 35 | 100 | 25 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 36 | 100 | 25 |  | 16QAM | 25 | P_25@75 | S_0@0 | - | - |
| 37 | 100 | 25 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 38 | 100 | 25 |  | 16QAM | 8 | P_8@92 | S_0@0 | - | - |
| 39 | 25 | 100 |  | QPSK | 25 | P_25@0 | S_0@0 | - | - |
| 40 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 41 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 42 | 25 | 100 |  | 16QAM | 25 | P_25@0 | S_0@0 | - | - |
| 43 | 25 | 100 |  | 16QAM | 8 | P_8@0 | S_0@0 | - | - |
| 44 | 25 | 100 |  | 16QAM | 8 | P_8@17 | S_0@0 | - | - |
| 453 | 100 | 50 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 46 | 100 | 50 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 47 | 100 | 50 |  | QPSK | 50 | P_50@50 | S_0@0 | - | - |
| 48 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 49 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 50 | 100 | 50 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 51 | 100 | 50 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 52 | 100 | 50 |  | 16QAM | 50 | P_50@50 | S_0@0 | - | - |
| 53 | 100 | 50 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 54 | 100 | 50 |  | 16QAM | 12 | P_12@88 | S_0@0 | - | - |
| 55 | 50 | 100 |  | QPSK | 50 | P_50@0 | S_0@0 | - | - |
| 56 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 57 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 58 | 50 | 100 |  | 16QAM | 50 | P_50@0 | S_0@0 | - | - |
| 59 | 50 | 100 |  | 16QAM | 12 | P_12@0 | S_0@0 | - | - |
| 60 | 50 | 100 |  | 16QAM | 12 | P_12@38 | S_0@0 | - | - |
| 61 | 100 | 75 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 62 | 100 | 75 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 63 | 100 | 75 |  | QPSK | 75 | P_75@25 | S_0@0 | - | - |
| 64 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 65 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 66 | 100 | 75 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 67 | 100 | 75 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 68 | 100 | 75 |  | 16QAM | 75 | P_75@25 | S_0@0 | - | - |
| 69 | 100 | 75 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 70 | 100 | 75 |  | 16QAM | 16 | P_16@84 | S_0@0 | - | - |
| 71 | 75 | 100 |  | QPSK | 75 | P_75@0 | S_0@0 | - | - |
| 72 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 73 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 74 | 75 | 100 |  | 16QAM | 75 | P_75@0 | S_0@0 | - | - |
| 75 | 75 | 100 |  | 16QAM | 16 | P_16@0 | S_0@0 | - | - |
| 76 | 75 | 100 |  | 16QAM | 16 | P_16@59 | S_0@0 | - | - |
| 77 | 100 | 100 |  | QPSK | 100 | P_100@0 | S_0@0 | - | - |
| 78 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 79 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 80 | 100 | 100 |  | 16QAM | 100 | P_100@0 | S_0@0 | - | - |
| 81 | 100 | 100 |  | 16QAM | 18 | P_18@0 | S_0@0 | - | - |
| 82 | 100 | 100 |  | 16QAM | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.1.3.4.1-1: Test Configuration for intra-band non-contiguous CA

| Table Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Low Range for CC1; High Range for CC2 PCC-SCC: CC1-CC2 and CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | QPSK | N/A | QPSK | P_8@0 | QPSK | S_0@0 |
| 2 | 25 | 25 | 35 | QPSK |  | QPSK | P_8@17 | QPSK | S_0@0 |
| 3 | 25 | 25 | 35 | QPSK |  | QPSK | P_25@0 | QPSK | S_0@0 |
| 4 | 25 | 25 | 35 | 16QAM |  | 16QAM | P_8@0 | 16QAM | S_0@0 |
| 5 | 25 | 25 | 35 | 16QAM |  | 16QAM | P_8@17 | 16QAM | S_0@0 |
| 6 | 25 | 25 | 35 | 16QAM |  | 16QAM | P_25@0 | 16QAM | S_0@0 |
| 7 | 50 | 50 | 25 | QPSK |  | QPSK | P_12@0 | QPSK | S_0@0 |
| 8 | 50 | 50 | 25 | QPSK |  | QPSK | P_12@38 | QPSK | S_0@0 |
| 9 | 50 | 50 | 25 | QPSK |  | QPSK | P_50@0 | QPSK | S_0@0 |
| 10 | 50 | 50 | 25 | 16QAM |  | 16QAM | P_12@0 | 16QAM | S_0@0 |
| 11 | 50 | 50 | 25 | 16QAM |  | 16QAM | P_12@38 | 16QAM | S_0@0 |
| 12 | 50 | 50 | 25 | 16QAM |  | 16QAM | P_50@0 | 16QAM | S_0@0 |
| 13 | 75 | 75 | 15 | QPSK |  | QPSK | P_16@0 | QPSK | S_0@0 |
| 14 | 75 | 75 | 15 | QPSK |  | QPSK | P_16@59 | QPSK | S_0@0 |
| 15 | 75 | 75 | 15 | QPSK |  | QPSK | P_75@0 | QPSK | S_0@0 |
| 16 | 75 | 75 | 15 | 16QAM |  | 16QAM | P_16@0 | 16QAM | S_0@0 |
| 17 | 75 | 75 | 15 | 16QAM |  | 16QAM | P_16@59 | 16QAM | S_0@0 |
| 18 | 75 | 75 | 15 | 16QAM |  | 16QAM | P_75@0 | 16QAM | S_0@0 |
| 19 | 100 | 100 | 5 | QPSK |  | QPSK | P_18@0 | QPSK | S_0@0 |
| 20 | 100 | 100 | 5 | QPSK |  | QPSK | P_18@82 | QPSK | S_0@0 |
| 21 | 100 | 100 | 5 | QPSK |  | QPSK | P_100@0 | QPSK | S_0@0 |
| 22 | 100 | 100 | 5 | 16QAM |  | 16QAM | P_18@0 | 16QAM | S_0@0 |
| 23 | 100 | 100 | 5 | 16QAM |  | 16QAM | P_18@82 | 16QAM | S_0@0 |
| 24 | 100 | 100 | 5 | 16QAM |  | 16QAM | P_100@0 | 16QAM | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

In [3] for NR single carrier EVM requirement, TP analysis provides with several modifications compared with LTE. In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline. It should be noted that in CA EVM test only PUSCH will be verified. Hence following discussion refer to PUSCH discussion in [3] mainly.

Test Environment

In LTE, EVM is tested applying normal condition as test environment. Since the EVM test methodology for NR FR1 is very similar to LTE, using the same test environment is a reasonable approach.

Proposal 1: Define test environment as Normal for CA EVM in FR1.

Test Subcarrier Spacing

In [3] lowest and highest SCS for EVM are selected for single carrier EVM testing. This could be applicable for 2UL CA to ensure test coverage.

Proposal 2: Select lowest and highest SCS for CA EVM in FR1.

Test Frequencies

In LTE, inter-band UL CA EVM is tested for Low range and High range to balance the test coverage and test time. It is proposed to reuse the same low and high frequency range for NR CA EVM testing.

Proposal 3: Test EVM for Low range and High range frequencies for CA EVM in FR1.

Test Channel Bandwidth

In LTE, CA EVM is tested for Lowest and Highest channel bandwidths. NR CA EVM can follow this approach as well.

Proposal 4: Test EVM for Lowest and Highest channel bandwidth for inter-band CA EVM in FR1. Test lowest NRB_agg and highest NRB_agg for intra-band contiguous UL CA and non-contiguous UL CA.

Test Modulations and waveforms for EVM PUSCH

The minimum requirements are specified for Pi/2-BPSK, QPSK, 16 QAM, 64 QAM, and 256 QAM. In order to ensure that the UE meets all these requirements the test has to cover these. Due to the differences in the modulation scheme (with and without precoding) and due to the different CREST factors for dft-s-OFDM and CP-OFDM both waveforms should be tested.

Proposal 5: Test EVM PUSCH for the modulation waveform combinations stated in Table 2.5-1.

Table 2.5-1: Modulation and waveform combinations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

DFT-s-OFDM 256 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

CP-OFDM 256 QAM

RB allocation for EVM PUSCH

In LTE, CA EVM is measured with full allocation and the highest partial allocation where no MPR occurs for PCC and 0PRB allocation for SCC. For inter-band CA,since for 64 QAM and 256 QAM always the same MPR is applied, it is sufficient to test these modulations with outer_full only. The lower modulations should be tested with outer_full and inner_full allocation to consider the different MPR since EVM depends on the power level.

Proposal 6: Test EVM PUSCH with the modulation, waveform and RB allocation combinations comprised in Table 2.6-1 for inter-band CA and table 2.6-2 for intra-band CA.

Table 2.6-1: Modulation, waveform and RB combinations for inter-band CA

| Test ID | Uplink Configuration |  |  |
| --- | --- | --- | --- |
|  | Modulation | PCC RB allocation | SCC RB allocation |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full | 0 |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full | 0 |
| 3 | DFT-s-OFDM QPSK | Inner Full | 0 |
| 4 | DFT-s-OFDM QPSK | Outer Full | 0 |
| 5 | DFT-s-OFDM 16 QAM | Inner Full | 0 |
| 6 | DFT-s-OFDM 16 QAM | Outer Full | 0 |
| 7 | DFT-s-OFDM 64 QAM | Outer Full | 0 |
| 8 | DFT-s-OFDM 256 QAM | Outer Full | 0 |
| 9 | CP-OFDM QPSK | Inner Full | 0 |
| 10 | CP-OFDM QPSK | Outer Full | 0 |
| 11 | CP-OFDM 16 QAM | Inner Full | 0 |
| 12 | CP-OFDM 16 QAM | Outer Full | 0 |
| 13 | CP-OFDM 64 QAM | Outer Full | 0 |
| 14 | CP-OFDM 256 QAM | Outer Full | 0 |

Table 2.6-2: Modulation, waveform and RB combinations for intra-band CA

| Test ID | Uplink Configuration |  |  |
| --- | --- | --- | --- |
|  | Modulation | PCC RB allocation | SCC RB allocation |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full | 0 |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full | 0 |
| 3 | DFT-s-OFDM QPSK | Inner Full | 0 |
| 4 | DFT-s-OFDM QPSK | Outer Full | 0 |
| 5 | DFT-s-OFDM 16 QAM | Inner Full | 0 |
| 6 | DFT-s-OFDM 16 QAM | Outer Full | 0 |
| 7 | DFT-s-OFDM 64 QAM | Inner Full | 0 |
| 8 | DFT-s-OFDM 64 QAM | Outer Full | 0 |
| 9 | DFT-s-OFDM 256 QAM | Inner Full | 0 |
| 10 | DFT-s-OFDM 256 QAM | Outer Full | 0 |
| 11 | CP-OFDM QPSK | Inner Full | 0 |
| 12 | CP-OFDM QPSK | Outer Full | 0 |
| 13 | CP-OFDM 16 QAM | Inner Full | 0 |
| 14 | CP-OFDM 16 QAM | Outer Full | 0 |
| 15 | CP-OFDM 64 QAM | Inner Full | 0 |
| 16 | CP-OFDM 64 QAM | Outer Full | 0 |
| 17 | CP-OFDM 256 QAM | Outer Full | 0 |
| 18 | CP-OFDM 256 QAM | Inner Full | 0 |

Test point

In this document several proposals have been stated for the configuration parameters. In all of cases the analysis has been focused on reducing testing time without losing coverage in UE testing for NR FR1.

EVM PUSCH:

|  | Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA | 1 | 2 | 2 | 2 | 14 | 112 |
| Intra-band CA | 1 | 2 | 2 | 2 | 18 | 144 |

Conclusion

For inter-band UL CA EVM requirement the test condition is proposed as below:

Proposal 1: Define test environment as Normal for CA EVM in FR1.

Proposal 2: Select lowest and highest SCS for CA EVM in FR1.

Proposal 3: Test EVM for Low range and High range frequencies for CA EVM in FR1.

Proposal 4: Test EVM for Lowest and Highest channel bandwidth for inter-band CA EVM in FR1. Test lowest NRB_agg and highest NRB_agg for intra-band contiguous UL CA and non-contiguous UL CA.

Proposal 5: Test EVM PUSCH for the modulation waveform combinations stated in Table 1.

Table 1: Modulation and waveform combinations

DFT-s-OFDM PI/2 BPSK

DFT-s-OFDM QPSK

DFT-s-OFDM 16 QAM

DFT-s-OFDM 64 QAM

DFT-s-OFDM 256 QAM

CP-OFDM QPSK

CP-OFDM 16 QAM

CP-OFDM 64 QAM

CP-OFDM 256 QAM

Proposal 6: Test EVM PUSCH with the modulation, waveform and RB allocation combinations comprised in Table 2 for inter-band CA and table 3 for intra-band CA.

Table 2: Modulation, waveform and RB combinations for inter-band CA

| Test ID | Uplink Configuration |  |  |
| --- | --- | --- | --- |
|  | Modulation | PCC RB allocation | SCC RB allocation |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full | 0 |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full | 0 |
| 3 | DFT-s-OFDM QPSK | Inner Full | 0 |
| 4 | DFT-s-OFDM QPSK | Outer Full | 0 |
| 5 | DFT-s-OFDM 16 QAM | Inner Full | 0 |
| 6 | DFT-s-OFDM 16 QAM | Outer Full | 0 |
| 7 | DFT-s-OFDM 64 QAM | Outer Full | 0 |
| 8 | DFT-s-OFDM 256 QAM | Outer Full | 0 |
| 9 | CP-OFDM QPSK | Inner Full | 0 |
| 10 | CP-OFDM QPSK | Outer Full | 0 |
| 11 | CP-OFDM 16 QAM | Inner Full | 0 |
| 12 | CP-OFDM 16 QAM | Outer Full | 0 |
| 13 | CP-OFDM 64 QAM | Outer Full | 0 |
| 14 | CP-OFDM 256 QAM | Outer Full | 0 |

Table 3: Modulation, waveform and RB combinations for intra-band CA

| Test ID | Uplink Configuration |  |  |
| --- | --- | --- | --- |
|  | Modulation | PCC RB allocation | SCC RB allocation |
| 1 | DFT-s-OFDM PI/2 BPSK | Inner Full | 0 |
| 2 | DFT-s-OFDM PI/2 BPSK | Outer Full | 0 |
| 3 | DFT-s-OFDM QPSK | Inner Full | 0 |
| 4 | DFT-s-OFDM QPSK | Outer Full | 0 |
| 5 | DFT-s-OFDM 16 QAM | Inner Full | 0 |
| 6 | DFT-s-OFDM 16 QAM | Outer Full | 0 |
| 7 | DFT-s-OFDM 64 QAM | Inner Full | 0 |
| 8 | DFT-s-OFDM 64 QAM | Outer Full | 0 |
| 9 | DFT-s-OFDM 256 QAM | Inner Full | 0 |
| 10 | DFT-s-OFDM 256 QAM | Outer Full | 0 |
| 11 | CP-OFDM QPSK | Inner Full | 0 |
| 12 | CP-OFDM QPSK | Outer Full | 0 |
| 13 | CP-OFDM 16 QAM | Inner Full | 0 |
| 14 | CP-OFDM 16 QAM | Outer Full | 0 |
| 15 | CP-OFDM 64 QAM | Inner Full | 0 |
| 16 | CP-OFDM 64 QAM | Outer Full | 0 |
| 17 | CP-OFDM 256 QAM | Outer Full | 0 |
| 18 | CP-OFDM 256 QAM | Inner Full | 0 |

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-185412, “TP for updating TR 38.905 with EVM test point analysis”

[4] 3GPP TS 36.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone.

[6] 3GPP TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone