Source:        Huawei, Samsung

Title:           TP analysis on FR1 6.4A.2.3.1 In-band emissions for CA (2UL CA)

Introduction

According to baseline agreement endorsed in [1][2] regarding testing parameters selection and agreement in [3] for FR1 SC IBE, this contribution provides corresponding analysis for selection of frequency, channel BW, SCS, RB allocation and waveform for IBE of 2UL inter-band CA case.

Discussion

In LTE specification [4], the test configuration for UL and DL inter-band CA, intra-band contiguous CA and Intra-band non-contiguous CA are defined as table below.

Table 6.5.2A.3.2.4.1-1: Test Configuration Table (Inter-band CA)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes. |  |  |  | Low range for PCC and SCC High range for PCC and SCC |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  | Lowest NRB_agg Highest NRB_agg (Note 2) |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |
| CA Configuration / NRB_agg |  | DL Allocation | CC MOD | UL Allocation |  |  |  |  |
| PCC NRB | SCCs NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 6 | 25 | N/A for this test | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 6 | 25 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 6 | 50 |  | QPSK | 1 | P_1@0 | S_0@0 | - | - |
| 6 | 50 |  | QPSK | 1 | P_1@5 | S_0@0 | - | - |
| 15 | 25 |  | QPSK | 4 | P_4@0 | S_0@0 | - | - |
| 15 | 25 |  | QPSK | 4 | P_4@11 | S_0@0 | - | - |
| 25 | 6 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 6 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 15 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 15 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 50 | 6 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 25 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 50 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 100 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-2. Note 2: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg, only the combination with the highest NRB_PCC is tested. Note 3: The frequencies of PCC and SCC shall be switched and tested in each configuration. |  |  |  |  |  |  |  |  |

Table 6.5.2A.3.1.4.1-1: Test Configuration Table (Intra-band contiguous )

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | NC |  |  |  |  |
| Test Frequencies as specified in TS36.508 [7] subclause 4.3.1 for different CA bandwidth classes. PCC and SCC mapped onto component carriers CC according to the notation: PCC-SCC: CCi-CCj, which means PCC on CCi and SCC on CCj, with CCi/j frequencies defined in the corresponding intra-band contiguous CA band in TS36.508 [7] as above. |  |  |  |  | See Table 6.5.1A.1.4.1-1: PCC-SCC: CC1-CC2, CC2-CC1 |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration |  |  |  |  | See Table 6.5.1A.1.4.1-1: |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| Config ID | CA Configuration / NRB_agg |  | DL Allocation (PDCCH on PCC) | CC MOD | UL Allocation |  |  |  |  |
|  | PCC NRB | SCC NRB | PCC & SCC RB allocation |  | NRB_alloc | PCC & SCC RB allocations (LCRB @ RBstart) |  |  |  |
| 1 Note 2 | 25 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 2 Note 3 | 25 | 25 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 3 Note 2 | 25 | 75 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 4 Note 3 | 25 | 75 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 5 Note 2 | 75 | 25 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 6 Note 3 | 75 | 25 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 7 Note 2 | 75 | 75 | N.A. | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 8 Note 3 | 75 | 75 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 9 Note 2 | 100 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 10 Note 3 | 100 | 25 |  | QPSK | 8 | P_8@92 | S_0@0 | - | - |
| 11 Note 2 | 25 | 100 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 12 Note 3 | 25 | 100 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 13 Note 2 | 100 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 14 Note 3 | 100 | 50 |  | QPSK | 12 | P_12@88 | S_0@0 | - | - |
| 15 Note 2 | 50 | 100 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 16 Note 3 | 50 | 100 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 17 Note 2 | 100 | 75 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 18 Note 3 | 100 | 75 |  | QPSK | 16 | P_16@84 | S_0@0 | - | - |
| 19 Note 2 | 75 | 100 |  | QPSK | 16 | P_16@0 | S_0@0 | - | - |
| 20 Note 3 | 75 | 100 |  | QPSK | 16 | P_16@59 | S_0@0 | - | - |
| 21 Note 2 | 100 | 100 |  | QPSK | 18 | P_18@0 | S_0@0 | - | - |
| 22 Note 3 | 100 | 100 |  | QPSK | 18 | P_18@82 | S_0@0 | - | - |
| 23 Note 2 | 50 | 50 |  | QPSK | 12 | P_12@0 | S_0@0 | - | - |
| 24 Note 3 | 50 | 50 |  | QPSK | 12 | P_12@38 | S_0@0 | - | - |
| 25 Note 2 | 25 | 50 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 26 Note 3 | 25 | 50 |  | QPSK | 8 | P_8@17 | S_0@0 | - | - |
| 27 Note 2 | 50 | 25 |  | QPSK | 8 | P_8@0 | S_0@0 | - | - |
| 28 Note 3 | 50 | 25 |  | QPSK | 8 | P_8@42 | S_0@0 | - | - |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-1. Note 2: Applicable only for PCC-SCC: CC1-CC2. Note 3: Applicable only for PCC-SCC: CC2-CC1. Note 4: If the UE supports multiple CC Combinations in the CA Configuration with the same NRB_agg , only the combination with the highest NRB_PCC is tested. |  |  |  |  |  |  |  |  |  |

Table 6.5.2A.3.3.4.1-1: Test Configuration Table (Intra-band non-contiguous CA)

| Initial Conditions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508[7] subclause 4.1 |  |  |  |  | Normal |  |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 for different CA bandwidth classes, and PCC and SCCs are mapped onto physical frequencies according to Table 6.1-2. |  |  |  |  | Refer to test points A: N/A (Range is not relevant), Maximum Wgap |  |  |  |  |
| Test CC Combination setting (NRB_agg) as specified in subclause 5.4.2A.1 for the CA Configuration across bandwidth combination sets supported by the UE. |  |  |  |  | Refer to test point Test only test points with Lowest NRB for PCC and SCC Highest NRB for PCC and SCC |  |  |  |  |
| Test Parameters for CA Configurations |  |  |  |  |  |  |  |  |  |
| ID | CA Configuration / NRB_agg |  | Wgap [MHz] | DL Allocation |  | UL Allocation |  |  |  |
|  | PCC NRB | SCCs NRB |  | CC MOD | PCC & SCC RB allocation | CC MOD | PCC NRB_alloc (LCRB @ RBstart) | CC MOD | SCC NRB_alloc (LCRB @ RBstart) |
| Test Parameters for CA_4A-4A Configurations |  |  |  |  |  |  |  |  |  |
| 1 | 25 | 25 | 35 | N/A |  | QPSK | P_8@0 | QPSK | S_0@0 |
| 2 | 25 | 25 | 35 |  |  | QPSK | P_8@17 | QPSK | S_0@0 |
| 3 | 50 | 50 | 25 |  |  | QPSK | P_12@0 | QPSK | S_0@0 |
| 4 | 50 | 50 | 25 |  |  | QPSK | P_12@38 | QPSK | S_0@0 |
| 5 | 100 | 100 | 5 |  |  | QPSK | P_18@0 | QPSK | S_0@0 |
| 6 | 100 | 100 | 5 |  |  | QPSK | P_18@82 | QPSK | S_0@0 |
| Note 1: CA Configuration Test CC Combination settings are checked separately for each CA Configuration, which applicable aggregated channel bandwidths are specified in Table 5.4.2A.1-3. Note 2: The carrier centre frequency of PCC in the UL operating band is configured closer to the DL operating band. |  |  |  |  |  |  |  |  |  |

In NR specification [5], the Test configuration for IBE for single carrier mode is defined as table below.

Table 6.4.2.3.4.1-1: Test Configuration Table for PUSCH

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  | Smallest supported SCS per Channel Bandwidth |  |
| Test Parameters |  |  |  |
| Test ID | Downlink Configuration | Uplink Configuration |  |
|  | N/A | Modulation | RB allocation (NOTE 1) |
| 1 |  | DFT-s-OFDM QPSK | Inner_1RB_Left |
| 2 |  | DFT-s-OFDM QPSK | Inner_1RB_Right |
| 3 |  | CP-OFDM QPSK | Inner_1RB_Left |
| 4 |  | CP-OFDM QPSK | Inner_1RB_Right |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: Test Channel Bandwidths are checked separately for each NR band, which applicable channel bandwidths are specified in Table 5.3.5-1. |  |  |  |

In following sub-clauses, study for test environment, frequencies, bandwidth and subcarrier spacing and DL and UL allocation is provided based on above baseline.

Test Environment

In LTE, IBE for CA is tested under Normal condition. Hence it is proposed to apply normal condition as test environment to align with LTE case.

Proposal 1: Define test environment as Normal for IBE in FR1 for UL CA.

Test Subcarrier Spacing

As the IBE is a sinusoidal unmodulated signal it is not expected that its power level depends on the choice of subcarrier spacing. Smallest SCS is selected as it is the most common one.

Proposal 2: Select smallest SCS for IBE in FR1 for UL CA.

Test Frequencies

In LTE, IBE for Inter-band CA is tested for Low range and High range. The same test coverage as for LTE would be enough to ensure that the IBE obeys the test requirement at low range and high range.

Proposal 3: Test IBE for Low range and High range frequencies in FR1 for inter-band and Intra-band contiguous UL CA. Test Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA

Test Channel Bandwidth

In LTE, IBE for inter-band CA is tested for Lowest and Highest NRB_agg. Hence it proposed to reuse the LTE method for IBE test for inter-band UL CA. For Intra-band contiguous UL CA and non-contiguous UL CA, it’s reasonable to test lowest and highest NRB_agg as well.

Proposal 4: Test IBE for Lowest and Highest NRB_agg in FR1 for UL CA.

Test Modulations and waveforms

For single carrier TP analysis presented in [3] both DFT-s-OFDM QPSK and CP-OFDM are selected for testing. For CA case it’s proposed to reuse this configuration.

Proposal 5: Test In-band Emissions for CA with DFT-s-OFDM QPSK and CP-OFDM QPSK.

RB allocation

In LTE, In-band Emissions for CA are tested with highest allocation without MPR where the partial allocation is located on the lower and upper channel edge similar as single carrier case. For NR single carrier case, Inner_1RB_Left or Inner_1RB_Right are selected in [3] as they keep the allocation to a minimum while having no MPR. It’s proposed to apply this for inter-band CA test on IBE as well.

For Intra-band contiguous CA, the minimum requirement applies when one single contiguous PRB allocation of bandwidth  at the edge of the aggregated transmission bandwidth configuration. So Edge_Full_Left on PCC when PCC being the carrier with lower center frequency, and Edge_Full_Right on PCC when PCC being the carrier with higher center frequency should be selected.

For Intra-band non-contiguous CA, the in-band emission requirement is defined for each component carrier. The same RB allocation as Inter-band CA could be applied.

Proposal 6: Test IBE with Inner_1RB_Left and use Inner_1RB_Right for PCC with 0PRB allocation for SCC for Inter-band CA.

Proposal 7: Test IBE with Edge_Full_Left on PCC and 0 RB on SCC when PCC being the carrier with lower center frequency, and Edge_Full_Right on PCC and 0 RB on SCC when PCC being the carrier with higher center frequency.

Proposal 8: Test IBE with Inner_1RB_Left and use Inner_1RB_Right for PCC with 0PRB allocation for SCC for Intra-band non-contiguous CA.

Number of test points for CA IBE

| Environmental conditions | Maximum Number of Frequencies | Maximum Number of ChBW | Number SCS | Number of steps (mod and RB) | Maximum Number of Test points |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 2 | 1 | 4 | 16 |

Conclusion

For inter-band UL CA IBE requirement the test condition is proposed as below:

Proposal 1: Define test environment as Normal for IBE in FR1 for UL CA.

Proposal 2: Select smallest SCS for IBE in FR1 for UL CA.

Proposal 3: Test IBE for Low range and High range frequencies in FR1 for inter-band and Intra-band contiguous UL CA. Test Low range and High range test frequencies as specified in tables for non-contiguous CA configuration with UL CA

Proposal 4: Test IBE for Lowest and Highest NRB_agg in FR1 for UL CA.

Proposal 5: Test In-band Emissions for CA with DFT-s-OFDM QPSK and CP-OFDM QPSK.

Proposal 6: Test IBE with Inner_1RB_Left and use Inner_1RB_Right for PCC with 0PRB allocation for SCC for Inter-band CA.

Proposal 7: Test IBE with Edge_Full_Left on PCC and 0 RB on SCC when PCC being the carrier with lower center frequency, and Edge_Full_Right on PCC and 0 RB on SCC when PCC being the carrier with higher center frequency.

Proposal 8: Test IBE with Inner_1RB_Left and use Inner_1RB_Right for PCC with 0PRB allocation for SCC for Intra-band non-contiguous CA.

Reference

[1] R5-180065, Discussion about Test parameters for NR TRx TCs

[2] R5-180111, Discussion of test channel BW and sub-carrier spacing (SCS) approach for NR TRx test cases

[3] R5-185319, “TP for updating TR 38.905 with FR1 In-band Emissions test point analysis”

[4] 3GPP TS 36.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Conformance Testing

[5] 3GPP TS 38.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone.

[6] 3GPP TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.