Title: 	Discussion on test point selection for NS_17/NS_47 A-MPR for CA in FR1

Source: 	Huawei

- Introduction

As agreed in RAN5#78, each test case would require specific analysis to conform its test configuration table based on the study for Frequency, Channel BW, SCS, RB allocation (including modulations).

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for NS_17/NS_47 combination in Additional Maximum Power Reduction for CA test case in FR1.

- Discussion

UE additional maximum output power reduction for CA FR1 is defined in Transmitter power for CA section in [1] where its scope is described as follows:

Unless otherwise stated, for inter-band carrier aggregation with one uplink carrier assigned to one NR band, the requirements in subclause 6.2.3 apply.

Unless specified in Table 6.2A.3.1.3-1, for inter-band carrier aggregation with uplink assigned to two NR bands, the requirements in clause 6.2.3 apply only to the indicated carrier. The requirements in Table 6.2A.3.1.3-1 are specified in terms of an additional spectrum emission requirement with their associated network signalling values and the allowed A-MPR. Unless otherwise stated, the combined requirements and allowed A-MPR are applicable on both bands when both component carriers are active. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet the additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

In table 6.2A.3.1.3-1, the AMPR requirement for CA_n28A-n41A is extracted as below:

Table 1: Additional Requirements for uplink inter-band carrier aggregation (two-bands)

| NR CA combination | Band | Applied NS | Requirements (clause) | A-MPR (table/clause) | Note |
| --- | --- | --- | --- | --- | --- |
| … | … | … | … | … | … |
| CA_n28-n41 | n28 | 17 | 6.5.3.3.2 | N/A | 2 |
|  | n41 | 47 | 6.5.3.3.15 | Table 6.2.3.18-2 |  |
| … | … | … | … | … | … |
| NOTE 1: NS_05U, NS_43U and NS_100 can be signalled for NR bands that have UTRA services deployed and the requirements in clause 6.5.2.4.2 are only applicable to the signalling carrier. NOTE 2: Applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |  |  |

No AMPR defined for NS_17. The AMPR for NS_47 is defined as below:

Table 2: A-MPR regions and types for NS_47

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.18-2. |  |  |  |  |

Table 3: A-MPR for modulation and waveform type

| Modulation/Waveform | A1(dB) |  | A2(dB) |  | A3(dB) |  | A4(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 |  | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 |  | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  |  |  | ≤ 7 |

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

  - Test Environment

Based on the baseline agreed in [3], test environment for UE additional maximum output power reduction for CA can be defined as in LTE regardless of number of CCs.

Proposal 1: Select Test Environment as Normal Conditions (NC) for NS_17/NS_47 for A-MPR for CA in FR1.

  - Test Frequencies

For single carrier test case 6.2.3 A-MPR for NS_56, low range is considered. For inter-band CA testing, it’s reasonable to select the same test frequencies for NS_56, which is PCC in this test case.

In table 1, NS_17 for band n28 is applicable when the assigned NR carrier is confined within 718 MHz and 748 MHz and CBW= 5MHz and 10MHz. Therefore it’s reasonable to select mid range for PCC for NS_17, which is PCC is this test case.

For single carrier test case 6.2.3 A-MPR for NS_47, the test frequencies are defined in table 6.2.3.4.1-18a and 6.2.3.4.1-18b. Therefor in CA test cases, the test frequencies same as single-carrier AMPR test case for NS_47 should be selected.

Proposal 2: Select mid range as test frequency for NS_17 for PCC.

Proposal 3: Select same test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for NS_47 for SCC.

  - Test Channel Bandwidth

As specified above, 5MHz and 10MHz for NS_17 should be selected. 30MHz for NS_47 should be selected.

Proposal 4: Select 5MHz and 10MHz for PCC and 30MHz for SCC for NS_17/NS_47 A-MPR for CA in FRI.

  - Test Subcarrier Spacing

The selection of test subcarrier spacing can be based on the agreed proposals for SEM, i.e., Lowest and Highest supported subcarrier spacing.

Proposal 5: Select Lowest and Highest SCS for PCC and SCC for NS_17/NS_47 A-MPR for CA testing.

  - Uplink Waveform, Modulation and RB allocation

According to the clause 4.1.1.2 in TR 38.905, three configurations should be tested in A-MPR for CA testing, which are:

-	Test the minimum Total power backoff value

-	Test the maximum Total power backoff value

-	Test the maximum unbalanced Total power backoff among CCs (max Pcmaxc difference).

Where the Total power backoff value means: MAX[MPR, A-MPR]

No AMPR requirements are specified for NS_17. Based on the AMPR requirement for NS_47 and MPR requirement, following configurations are derived for each CC:

- For PC3 UL CA ( or PC2 UL CA with both bands supporting PC3) tesing:

The minimum Total power backoff configuration:

PCC:	DFT-s-OFDM + QPSK + Inner_Full RB allocation (Backoff = 0dB)

SCC:	DFT-s-OFDM + QPSK + Edge_1RB_Right RB allocation (Backoff = 2dB)

The maximum Total power backoff configuration:

PCC: 	CP-OFDM + 256QAM + Outer_Full RB allocation (Backoff = 6.5dB)

SCC: 	DFT-s-OFDM +QPSK + Edge_1RB_Left (Backoff = 7)

The maximum unbalanced Total power backoff configuration:

PCC: 	DFT-s-OFDM + QPSK + Inner_Full RB allocation (Backoff = 0dB)

SCC: 	DFT-s-OFDM +QPSK + Edge_1RB_Left (Backoff = 7dB)

- For PC2 UL CA with PCC supporting PC3 and SCC supporting PC2 tesing:

The minimum Total power backoff configuration:

PCC:	DFT-s-OFDM + QPSK + Inner_Full RB allocation (Backoff = 0dB)

SCC:	DFT-s-OFDM + PI/2 BPSK + Edge_1RB_Right (Backoff = 5dB)

The maximum Total power backoff configuration:

PCC: 	CP-OFDM + 256QAM + Outer_Full RB allocation (Backoff = 6.5dB)

SCC: 	DFT-s-OFDM + PI/2 BPSK + Edge_1RB_Left (Backoff = 10dB)

The maximum unbalanced Total power backoff configuration:

PCC: 	DFT-s-OFDM + QPSK + Inner_Full RB allocation (Backoff = 0dB)

SCC: 	DFT-s-OFDM + PI/2 BPSK + Edge_1RB_Left (Backoff = 10dB)

Based on the above configuration analysis, the Uplink configurations specified in Table 4 and 5 should be selected.

Table 4 Test Parameters for PC3 NS_17/NS_47 AMPR

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Mid range for PCC Test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz and 10MHz for PCC 30 MHz for SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest as specified in test parameters |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | DFT-s-OFDM 16QAM |  | Inner Full | Inner Full |
| 2 |  | CP-OFDM 256QAM | DFT-s-OFDM QPSK |  | Outer Full | Edge_1RB_Left |
| 3 |  | DFT-s-OFDM QPSK | DFT-s-OFDM QPSK |  | Inner Full | Edge_1RB_Left |

Table 5 Test Parameters for PC2 NS_17/NS_47 AMPR

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause4.3.1.1.3 for inter band CA in FR1 |  |  |  | Mid range for PCC Test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for SCC |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz and 10MHz for PCC 30 MHz for SCC |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | Lowest, Highest as specified in test parameters |  |  |
| Test Parameters |  |  |  |  |  |  |
|  | DL | UL configuration |  |  |  |  |
| Test ID | configur | Modulation |  |  | RB allocation |  |
|  | ation | PCC | SCC |  | PCC | SCC |
| 1 | N/A | DFT-s-OFDM QPSK | CP-OFDM 256QAM |  | Inner Full | Edge_1RB_Right |
| 2 |  | CP-OFDM 256QAM | DFT-s-OFDM PI/2 BPSK |  | Outer Full | Edge_1RB_Left |
| 3 |  | DFT-s-OFDM QPSK | DFT-s-OFDM PI/2 BPSK |  | Inner Full | Edge_1RB_Left |

Proposal 6: Select Uplink Waveform, Modulation and RB allocation as shown in Table 4 for PC3 UL CA and table 5 for PC2 UL CA.

  - Number of Test Points

Based on above proposals, the total test points of NS_17/NS_47 A-MPR for CA are tabulated below.

Table 2 Total test points of NS_56/NS_04 A-MPR for CA

| Environment conditions | Test Frequencies | Test ChBW | Test SCS | Test Configs | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | 2 | 3 | 12 |

- Conclusion

Proposal 1: Select Test Environment as Normal Conditions (NC) for NS_17/NS_47 for A-MPR for CA in FR1.

Proposal 2: Select mid range as test frequency for NS_17 for PCC.

Proposal 3: Select same test frequencies as specified in table 6.2.3.4.1-18a and 6.2.3.4.1-18b for NS_47 for SCC.

Proposal 4: Select 5MHz and 10MHz for PCC and 30MHz for SCC for NS_17/NS_47 A-MPR for CA in FRI.

Proposal 5: Select Lowest and Highest SCS for PCC and SCC for NS_17/NS_47 A-MPR for CA testing.

Proposal 6: Select Uplink Waveform, Modulation and RB allocation as shown in Table 4 for PC3 UL CA and table 5 for PC2 UL CA.

- References

[1] 3GPP TS 38.521-1 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone;

[2] 	3GPP TS 38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone

[3]	3GPP TR 38.905 Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases