3GPP TSG-RAN5 Meeting #101	R5-237090

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5272 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of legacy NB-IoT TC for NGSO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update the pre-test condition of applicable legacy NB-IoT cases for NGSO scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The pre-test conditions were updated for NGSO scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases will be inaplicable to NGSO. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.1.3.1, 22.2.4.3.1, 22.4.1.3.1, 22.4.4.3.1, 22.4.5.3.1, 22.4.8.3.1, 22.4.9.3.1, 22.4.14.3.1, 22.5.1.3.1, 22.5.3.3.1, 22.5.4.3.1, 22.5.6.3.1, 22.5.9.3.1, 22.5.17.3.1, 22.5.18.3.1 and 22.6.1a.3.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

22.1.1.3.1	Pre-test conditions

System Simulator:

-	Ncell 50, default system information combination.

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test of M3 for NTN NGSO scenario.

UE:

-	UE is in test mode with predication of satellite trajectory deactivated during the test of M3 for NTN NGSO scenario.

Preamble:

-	UE is in State 1-NB switched off according to TS 36.508 [18].

[Skip unchanged text]

22.2.4.3.1	Pre-test conditions

System Simulator:

-	Ncell 1 and Ncell 2 on frequency f1 but have different tracking areas according to TS 36.508 Table 8.1.4.2-6. Ncell 3 on frequency f2 according to TS 36.508 Table 8.1.4.2-6.

-	System information combination 2 as defined in TS 36.508 [18] clause 8.1.4.3.1 is used.

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), System information    combination 10 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used and same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in state Switched OFF (state 1) ac-NB cording to [18 TS 36.508].

[Skip unchanged text]

22.4.1.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is configured to request the use of eDRX (in the ATTACH REQUEST and TRACKING AREA UPDATE REQUEST messages).

-    The UE sends “extended DRX parameters IE” in ATTACH REQUEST message and receives “extended DRX parameters IE” in ATTACH ACCEPT message.

-     The eDRX value in ATTACH ACCEPT is set to a value such that the eDRX cycle is longer than the System Information Modification period.

-    The UE is in state Registered, Idle mode (State 3-NB) according to [18].

[Skip unchanged text]

22.4.4.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in TS 36.508 clause 8.1.4.1.2 are configured broadcasting PLMNs as indicated in Table 22.4.4.3.1–1.

-	The PLMNs are identified in the test by the identifiers in Table 22.4.4.3.1–1.

Table 22.4.4.3.1–1: PLMN identifiers

| Ncell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

-	System information combination 4 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in NB-IoT cells;

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), System information    combination 13 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used and same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 22.4.4.3.1–2.

Table 22.4.4.3.1–2: USIM configuration

| USIM field | Value | Access Technology Identifier |
| --- | --- | --- |
| EFIMSI | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | PLMN2 Remaining mandatory entries use default values | All specified E-UTRAN |
| EFOPLMNwACT | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | PLMN4 | E-UTRAN |
| EFACC | Type “A” as defined in TS 34.108 clause 8.3.2.15 |  |

-	The UE belong to access class 0.

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode G on Ncell 13 according to [18]

[Skip unchanged text]

22.4.5.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in TS 36.508 clause 8.1.4.1.2 are configured broadcasting PLMNs as indicated in Table 22.4.5.3.1–1.

-	The PLMNs are identified in the test by the identifiers in Table 22.4.5.3.1–1.

Table 22.4.5.3.1–1: PLMN identifiers

| Ncell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

-	System information combination 4 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in NB-IoT cells;

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), System information    combination 13 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used and same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 22.4.5.3.1–2.

Table 22.4.5.3.1–2: USIM configuration

| USIM field | Value | Access Technology Identifier |
| --- | --- | --- |
| EFIMSI | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | PLMN2 Remaining mandatory entries use default values | All specified E-UTRAN |
| EFOPLMNwACT | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | PLMN4 | E-UTRAN |
| EFACC | Type “C” as defined in TS 34.108 clause 8.3.2.15 |  |

-	The UE belong to access class 0 and special access class 11 and 15.

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode G on Ncell 13 according to [18]

[Skip unchanged text]

22.4.8.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in TS 36.508 clause 8.1.4.1.2 are configured broadcasting PLMNs as indicated in Table 22.4.8.3.1–1.

-	The PLMNs are identified in the test by the identifiers in Table 22.4.8.3.1–1.

Table 22.4.8.3.1–1: PLMN identifiers

| Ncell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

-	System information combination 4 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in NB-IoT cells;

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), System information    combination 13 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used and same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 22.4.8.3.1–2.

Table 22.4.8.3.1–2: USIM configuration

| USIM field | Value | Access Technology Identifier |
| --- | --- | --- |
| EFIMSI | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | PLMN2 Remaining mandatory entries use default values | All specified E-UTRAN |
| EFOPLMNwACT | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | PLMN4 | E-UTRAN |
| EFACC | Type “A” as defined in TS 34.108 clause 8.3.2.15 |  |
| EFNASCONFIG | “Exception Data Reporting Allowed is set to 01” |  |

-	The UE belong to access class 0.

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in state Registered, Idle Mode (State 3-NB) on Ncell 13 according to [18].

[Skip unchanged text]

22.4.9.3.1	Pre-test conditions

System Simulator:

-	Three inter-frequency multi-PLMN cells as specified in TS 36.508 clause 8.1.4.1.2 are configured broadcasting PLMNs as indicated in Table 22.4.9.3.1–1.

-	The PLMNs are identified in the test by the identifiers in Table 22.4.9.3.1–1.

Table 22.4.9.3.1-1: PLMN identifiers

| Ncell | PLMN name | MCC | MNC |
| --- | --- | --- | --- |
| 1 | PLMN4 | 001 | 01 |
| 12 | PLMN1 | 001 | 11 |
| 13 | PLMN2 | 001 | 21 |

-	System information combination 4 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in NB-IoT cells;

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), System information    combination 13 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used and same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in Automatic PLMN selection mode.

-	The UE is equipped with a USIM containing default values (as per TS 36.508) except for those listed in Table 22.4.9.3.1–2.

Table 22.4.9.3.1-2: USIM configuration

| USIM field | Value | Access Technology Identifier |
| --- | --- | --- |
| EFIMSI | The HPLMN (MCC+MNC) of the IMSI is set to PLMN4. |  |
| EFPLMNwAcT | PLMN2 Remaining mandatory entries use default values | All specified E-UTRAN |
| EFOPLMNwACT | PLMN1 Remaining defined entries use default values | All specified |
| EFHPLMNwAcT | PLMN4 | E-UTRAN |
| EFACC | Type “C” as defined in TS 34.108 clause 8.3.2.15 |  |
| EFNASCONFIG | “Exception Data Reporting Allowed is set to 01” |  |

-	The UE belongs to access class 0 and special access class 11 and 15.

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in state Registered, Idle Mode (State 3-NB) on Ncell 13 according to [18].

[Skip unchanged text]

22.4.14.3.1	Pre-test conditions

System Simulator:

-	Ncell 1.

NB-IoT Operation Mode set to standalone

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	UE is in State 1-NB switched off.

[Skip unchanged text]

22.5.1.3.1	Pre-test conditions

System Simulator:

-	2 NB-IoT cells, Ncell 1 and Ncell 11, default system information

-	Ncell 1, set to "Serving cell"

-	Ncell 2, set to "Non-Suitable cell"

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in NB-IoT UE Attach, Connected mode (State 11-NB) on Ncell 1 according to TS 36.508 [18].

[Skip unchanged text]

22.5.3.3.1	Pre-test conditions

System Simulator:

-	Ncell 1, default system information.

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in NB-IoT UE Attach, Connected mode (State 2-NB) on Ncell 1 according to TS 36.508 [18].

[Skip unchanged text]

22.5.4.3.1	Pre-test conditions

System Simulator:

-	2 NB-IoT cells, default system information

-	Ncell 1 "Serving cell", PLMN1, TAI-1

-	Ncell 12 "Non-Suitable cell", PLMN2, TAI-3

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is made register on NB-IoT Ncell 1 and enter State 3-NB "Registered, Idle Mode" according to TS 36.508 [18]; during the attach GUTI-1 is assigned, PLMN2 is not included in the equivalent PLMN list , nor TAI-3 in the TAI list assigned.

-	After registration the UE is switched off in order to enter State 1-NB "Switched OFF" according to TS 36.508 [18].

[Skip unchanged text]

22.5.6.3.1	Pre-test conditions

System Simulator:

-	Ncell 50, Ncell 51 are defined in clause 8.1.4.2 in TS 36.508[18]

-	NB-IOT system information combination 2 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in all cells

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), System information combination 10 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used and same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

-	Ncell 50 and Ncell 51 belong to PLMN1.

Table 22.5.6.3-1: Time instances of cell power level and parameter changes

|  | Parameter | Unit | Ncell 50 | Ncell 51 |
| --- | --- | --- | --- | --- |
| T1 | Cell-specific NRS EPRE | dBm/ 15kHz | -85 | -120 |
| T2 | Cell-specific NRS EPRE | dBm/ 15kHz | -120 | -85 |
| T3 | Cell-specific NRS EPRE | dBm/ 15kHz | -85 | -85 |
| T4 | Cell-specific NRS EPRE | dBm/ 15kHz | -85 | -120 |
| T5 | Cell-specific NRS EPRE | dBm/ 15kHz | -85 | -97 |
| T6 | Cell-specific NRS EPRE | dBm/ 15kHz | -97 | -85 |
| T7 | Cell-specific NRS EPRE | dBm/ 15kHz | -85 | Off |

UE:

-	The Test UICC shall be inserted. This shall contain a USIM application on UICC.

-	the UE is previously registered on NB-IOT, and when on NB-IOT, the UE is last authenticated and registered on Ncell 50 using default message contents according to TS 36.508 [18].

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	the UE is in state Switched OFF (State 1-NB) according to TS 36.508 [18].

- 	The NAS integrity algorithm shall be set to a different value than 'EPS integrity algorithm' EIA0 throughout the whole duration of the test.

[Skip unchanged text]

22.5.9.1.3.1	Pre-test conditions

System Simulator:

-	NB-IoT Ncell 1, set to "Serving cell";

-	NB-IoT Ncell 2, set to Non-Suitable cell".

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in state Registered, Idle mode (State 3-NB) on Ncell 1, according to TS 36.508 [18] Table 8.1.5.1-1.

[Skip unchanged text]

22.5.17.3.1	Pre-test conditions

System Simulator:

-	NB-IoT Ncell 1, default parameters.

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on the NB-IoT cell using default message contents according to TS 36.508 [18].

-	The UE is configured to use Power Saving Mode.

-	The UE is configured to use the timer T3324 set to two minutes.

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	UE is in State 1-NB switched off.

[Skip unchanged text]

22.5.18.3.1	Pre-test conditions

System Simulator:

-	NB-IoT Ncell 1, set to "Serving cell";

-	NB-IoT Ncell 11, set to Non-Suitable cell".

-	System information combination 2 as defined in TS 36.508[18] clause 8.1.4.3.1.1 is used in NB-IoT cells.

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), System information combination 10 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used and same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is previously registered on E-UTRAN, and when on E-UTRAN, the UE is last authenticated and registered on the NB-IoT cell using default message contents according to TS 36.508 [18].

- 	the UE is configured to request the use of eDRX (in the ATTACH REQUEST and TRACKING AREA UPDATE messages).

-	The UE is configured to use Power Saving Mode.

-	The UE is configured to set T3324 to two minutes.

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	UE is in State 1-NB switched off.

[Skip unchanged text]

22.6.1a.1.3	Test description

22.6.1a.1.3.1	Pre-test conditions

System Simulator:

-	NB-IoT Ncell 1, default parameter;

-	If UE supports NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), same ephemeris information in SystemInformationBlockType31-NB is broadcasted during the test for NTN NGSO scenario.

UE:

-	The UE is in test mode with predication of satellite trajectory deactivated during the test for NTN NGSO scenario.

Preamble:

-	The UE is in state Switched OFF (State 1).

[End of change]