3GPP TSG-RAN WG5 Meeting #101	R5-237111

Chicago, USA, 13th- 17th November 2023

Title:	On ephemeris definition

Source:	Keysight Technologies UK Ltd, Thales, MediaTek

Agenda item:	5.3.29.9

Document for:	Discussion and endorsement

# 1.	Introduction

RAN5 has been working on the definition of NTN test cases for IoT NTN ([1]) and NR NTN ([2]).

RAN5 required some clarifications to RAN4 to understand how ephemeris information should be defined for each type of test cases (RF TRX, demod and RRM tests) as per LS in R5-233672 [3].

RAN4 provided feedback to LS in R5-233672 [3] as per R4-2314001 [4], R4-2317006 [5] and R4-2316967 [6].

Taking such feedback into account, this document pretends to define a way forward on ephemeris definition required in test specifications [7-10].

# 2. 	Discussion

## 2.1 Types of satellites

According to the feedback provided by RAN4 in [4-6], all RF TRx, demod and RRM test cases might require ephemeris information for both GSO and NGSO satellites, as test cases execution will depend on the type of satellite supported by the UE (refer to [11-12] for further details).

In the case of RRM UL timing accuracy test, to enable the whole range of one-way delay coverage from UE to satellite in case of NGSO satellites (refer to [6]), both LEO-600 and LEO-1200 are required.

In the case of GSO satellites, considering the feedback of satellite industry indicating that pure GEO satellites do not exist in reality and, considering that inclination different from zero will be a worse case, GSO satellites with inclination different from zero is proposed for the testing. An inclination equal to 7 degrees is proposed considering actual deployment intentions from satellites operators.

Proposal 1: Define ephemeris files for GSO (7 degrees inclination), LEO-600 and LEO-1200 satellites.

## 2.2 UE location

Ephemeris information shall be defined such satellite is visible for the UE while performing the tests.

In the case of IoT NTN, 36.508 [13] already defined UE location in a common section for all type of test cases. This location can be used for ephemeris definition.

Proposal 2: Use UE location in section 4.13 in TS 36.508 ([13]) to calculate ephemeris files for IoT NTN RF/RRM test cases.

For NR NTN, section 4.5C defined UE location in a common section for all type of test cases. However, 5.6 included a placeholder to define UE location different to the one used for signalling test cases. Hence 2 options are possible for the UE location for NR NTN RF test cases:

Option 1: Leverage the location defined in section 4.5C (same as for IoT NTN)

Option 2: Define and test a different UE location compared to IoT NTN RF.

Proposal 3: Leverage UE location defined in section 4.5C in TS 38.508-1 ([14]) to calculate ephemeris files for IoT NTN RF/RRM test cases.

## 2.2 Ephemeris granularity

The definition of ServingSatelliteInfo-r17 in IoT NTN ([15]) and of NTN-Config-r17 in NR NTN ([16]) tie the signalling of the ephemeris information to the epochTime-r17 value. EpochTime-r17 is defined through 2 parameters:

For GSO satellites:

If ephemeris granularity is defined as a submultiple of the possible values of System Frame Numbers (1024) multiplied by 10ms, the epoch time for each ephemeris information in the full orbit will be exactly the same independently on the absolute time, i.e 2n*10ms, where n can be any positive integer between 0 and 8. The higher the n value, the simpler the ephemeris file for GSO satellite will be. On the other hand, it is important that one of the inputs in the GSO ephemeris file is close to zero Doppler conditions.

The criterion for the zero Doppler conditions proposed is to select the highest granularity that allows the definition of an input to the ephemeris file with an absolute Doppler lower than 50 Hz for a 2 GHz carrier frequency.

For NGSO satellites, as only the satellite visibility window is to be defined in the ephemeris file, even a submultiple of possible values of subframe multiplied by 1ms can be used (1ms, 2ms, 5ms)

Proposal 4: Use 2.56s as ephemeris granularity for GSO satellites.

Proposal 5: Use 0.64ms as ephemeris granularity for NGSO satellites.

## 2.3 Ephemeris file content

To enable the configuration/testing of both ephemeris formats, ephemeris file shall contain at least the parameters associated to both possible ephemeris formats (state vector and orbital parameters):

- EphemerisInfo-positionX

- EphemerisInfo-positionY

- EphemerisInfo-positionZ

- EphemerisInfo-velocityVX

- EphemerisInfo-velocityVY

- EphemerisInfo-velocityVZ

- EphemerisInfo-semiMajorAxis

- EphemerisInfo-eccentricity

- EphemerisInfo-periapsis

- EphemerisInfo-longitude

- EphemerisInfo-inclination

- EphemerisInfo-meanAnomaly

If the ephemeris files also include the following additional information (even when it will not be signalled to the UE explicitly):

- Elevation angle

- One-way delay

- Doppler effect [ppm]

And they also contain:

- In the case of GSO satellites, the full orbit.

- In the case of NGSO satellite, the whole satellite visibility window.

ephemeris configuration per test case can be done easily according to the appropriate elevation angle, Doppler and delay required.

Proposal 6: Ephemeris files shall contain ephemeris information in both ephemeris formats (StateVector and Orbital parameters).

Proposal 7: Ephemeris files shall include the full orbit for GSO satellites and the satellite visibility window for NGSO satellite including all possible elevation angles.

Proposal 8: Even when not explicitly signalled, ephemeris files shall include additional information as elevation angle, one-way delay and Doppler effect in [ppm] to enable the selection of required ephemeris configuration per test case.

# 3. 	Conclusion

This document pretends to define the way forward on ephemeris definition required in [7-10].

The following proposals are made:

Proposal 1: Define ephemeris files for GSO (7 degrees inclination), LEO-600 and LEO-1200 satellites.

Proposal 2: Use UE location in section 4.13 in TS 36.508 ([13]) to calculate ephemeris files for IoT NTN RF/RRM test cases.

Proposal 3: Leverage UE location defined in section 4.5C in TS 38.508-1 ([14]) to calculate ephemeris files for IoT NTN RF/RRM test cases

Proposal 4: Use 2.56s as ephemeris granularity for GSO satellites.

Proposal 5: Use 0.64ms as ephemeris granularity for NGSO satellites.

Proposal 6: Ephemeris files shall contain ephemeris information in both ephemeris formats (StateVector and Orbital parameters).

Proposal 7: Ephemeris files shall include the full orbit for GSO satellites and the satellite visibility window for NGSO satellite including all possible elevation angles.

Proposal 8: Even when not explicitly signalled, ephemeris files shall include additional information as elevation angle, one-way delay and Doppler effect in [ppm] to enable the selection of required ephemeris configuration per test case.

A CR to TS 36.508 [13] implementing proposals in this document for IoT NTN has been submitted as per R5-237112[14]. Similarly for TS 38.508-1 [15], CR implementing these proposals can be found in R5-237113 [16]

# 4. 	References

- RP-223437, “WID on NB-IoT/eMTC core & performance requirements for NTN”, MediaTek Inc., RAN#98-e

- RP-221241, “WID on UE Conformance – Solutions for NR to support non-terrestrial networks (NTN) plus CT aspects”, Qualcomm Incorporated, RAN#96

- R5-233672, “LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, RAN5#99

- R4-2314001, “Reply LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, Thales, RAN4#108

- R4-2317006, “Further Reply LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, RAN4#108bis

- R4-2316967, “WF on NTN RAN task”, MediaTek Inc., RAN4#108bis.

- 3GPP TS 36.521-4, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Satellite access Radio Frequency (RF) and performance Conformance Testing”.

- 3GPP TS 36.521-3, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Radio Resource Management (RRM) conformance testing”.

- 3GPP TS 38.521-5, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance”

- 3GPP TS 38.533, “NR; User Equipment (UE) conformance specification; Radio Resource Management (RRM)”

- R5-237109, “IoT NTN discussion on SAN type coverage in testing”, Keysight Technologies UK Ltd, RAN5#101

- R5-237110, “NR NTN discussion on SAN type coverage in testing”, Keysight Technologies UK Ltd, RAN5#101

- 3GPP TS 36.508, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing”.

- R5-237112, “Common test environment for IoT NTN RF/RRM testing”, Keysight Technologies UK Ltd, Thales, [MediaTek], RAN5#101

- 3GPP TS 38.508-1, “5GS; User Equipment (UE) conformance specification; Part 1: Common test environment”

- R5-237113, “Common test environment for NR NTN RF/RRM testing”, Keysight Technologies UK Ltd, Thales, [MediaTek], RAN5#101