3GPP TSG-RAN5 Meeting # 101	R5-237113

# Chicago, USA,  13th November 2023 - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 2993 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Common test environment for NR NTN RF/RRM testing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd, Thales |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Common test environment for NR NTN RF/RRM testing requires some updates to align with R5-237110 and R5-237111. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added common test environment for IoT NTN RF/RRM testing as per discussion papers R5-237110 and R5-237111. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the work plan will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6 (New), 7.6 (New), 8.2.6 (New), 8.4.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on R5-237110 and R5-237111. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1>>>

## 5.6	Test environment for NTN testing

This section defines the test environment which applies to all RF and DEMOD test cases executed for NR NTN UEs, unless otherwise specified.

### 5.6.1	UE location

UE shall determine its location during the test using any of the following means.

1.	UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test.

2.	Other options such as providing UE location via AT command are not precluded.

UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows:

Longitude: 121.56076999

Latitude: 25.08439333

Altitude: 0

### 5.6.2	Ephemeris Information

The ephemeris information to be used in NR NTN tests defined in TS 38.521-5 [57] are contained in archives listed in Table 5.6.2-1 which accompany the present document:

Table 5.6.2-1. NR NTN Ephemeris files

| Type of satellite | Archive |
| --- | --- |
| GSO (7º inclination) | Ephemeris_GSO.zip |
| NGSO 1 (LEO-600 km) | Ephemeris_LEO600.zip |
| NGSO 2 (LEO-1200 km) | Ephemeris_LEO1200.zip |

All NR NTN test cases defined in sections 6, 7 and 8 in TS 38.521-5 [57], unless otherwise stated, shall use the input in the ephemeris file corresponding to zero Doppler and constant delay for the type of stellite under test, keeping such information constant throughout the duration of each measurement in the test. In the case of NGSO testing, Ephemeris_LEO1200.zip shall be used for the ephemeris values.

#### 5.6.2.1	Assumptions for Ephemeris generation

This section provides the assumptions and steps used to generate Ephemeris files for GSO and NGSO orbits.

Assumptions:

1.	Ephemeris files are generated for 3 types of satellites: GSO (inclination equal to 7º), LEO-600 and LEO-1200.

2.	Ephemeris files include the full orbit in case of GSO satellite and only satellite visibility window for NGSO satellites.

3.	Each input in the ephemeris files include:

- Information for both ephemeris formats according to 38.331 [6] (stateVectors and OrbitalParameters).

- Resulting elevation angle, one way delay and Doppler effect (in [ppm]) so each test case can select the appropriate Doppler and delay conditions to be used.

4.	UE location is as defined in section 5.6.1.

5.	All possible elevation angles of the SAN as seen by the UE are included in the ephemeris files. Minimum elevation angle restriction will be defined per test case.

6.	Ephemeris granularity used shall comply with the following conditions:

- Enable leveraging of the same ephemeris information values in each period comprising the whole range of System Frame Number within epochTime in servingSatelliteInfo-r17 information element [17].

- For GSO, minimize ephemeris file size.

- For GSO, include an input corresponding to zero Doppler conditions with an absolute Doppler error lower than 50 Hz for a carrier frequency below 2GHz.

#### 5.6.2.2	Ephemeris Generation

This section provides the steps to generate Ephemeris files for both GSO and NGSO orbits.

The software tool used to generate GSO ephemeris file has been GMAT (from NASA), which offers free licenses.

The steps to generate ephemeris files are as follows:

- Configure satellite initial vector, time and keplerian representation.

- Configure the ground station simulating the UE with relevant UE position.

- Configure the propagators and the force models.

- Configure UE topocentric coordinate systems.

- Configure subscribers to represent plots, ground-track view or orbit view and setting the GMAT report file with variables for storage of results.

- Prepare generation of parameters such as satellite position, speed, range, Delay and Doppler with respect to given elevation angles.

- Set the mission preparation using the GUI or the script.

- Generate data.

### 5.6.3	NR NTN message contents for RF Tx/Rx, Demodulation tests

#### 5.6.3.1	ntn-config-r17 values in SIB19

The ntn-config-r17 values including the EphemerisInfo-r17 values in SIB19 shall be periodically signalled to the UE during test based on the Ephemeris files generated in clause 5.6.2

### 5.6.4	FFS

## <<< END OF CHANGES 1>>>

## <<< START OF CHANGES 2>>>

## 7.5	Common procedures for RRM testing

### 7.5.1	Procedure to configure SCC(s) for NR RRM CA testing

Same procedure as described in clause 5.5.1.

### 7.5.2	Procedure to configure SCC(s) for EN-DC RRM CA testing

Same procedure as described in clause 5.5.1.

## 7.6	Test environment for NTN testing

This section defines the test environment which applies to all RRM test cases executed for NR NTN UEs, unless otherwise specified.

### 7.6.1	UE location

Same as in section 5.6.1.

### 7.6.2	Ephemeris Generation

This section provides the means to generate Ephemeris files for GSO and NGSO orbits.

#### 7.6.2.0	Ephemeris files

The ephemeris information to be used in NR NTN tests defined in TS 38.533 [18] are contained in archives listed in Table 5.6.2-1 which accompany the present document.

All test cases defined in sections 14 in TS 38.533 [18], unless otherwise stated, shall use the input in the ephemeris file corresponding to constant non-zero Doppler and constant delay for the type of stellite under test for an elevation angle of 30º, keeping such information constant throughout the duration of each measurement in the test.

#### 7.6.2.1	Assumptions for Ephemeris generation

Same as in section 5.6.2.1.

#### 7.6.2.2	Ephemeris Generation

Same as in section 5.6.2.2.

### 7.6.3	IoT NTN message contents for RRM tests

#### 7.6.3.1	ServingSatelliteInfo-r17 values in SIB31

Same as in section 5.6.3.

## <<< END OF CHANGES 2>>>

Add attached .zip files to TS 38.508-1 “Ephemeris_GSO.zip”, “Ephemeris_LEO600.zip” and “Ephemeris_LEO1200.zip”