3GPP TSG-RAN WG5 Meeting #101	R5-237115

Chicago, USA, 13th- 17th November 2023

Title:	Selection of ephemeris information for NTN RRM test cases

Source:	Keysight Technologies UK Ltd, MediaTek

Agenda item:	5.3.29.9

Document for:	Discussion and endorsement

# 1.	Introduction

RAN5 has been working on a work item on   UE Conformance - NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN) ([1]) and TS 36.521-3 is being drafted ([2]).

RAN5 required some clarifications to RAN4 to understand how ephemeris information should be defined for each type of test cases (RF TRX, demod and RRM tests) as per LS in R5-233672 [3].

RAN4 provided feedback to LS in R5-233672 [3] as per R4-2314001 [4], R4-2317006 [5] and R4-2316967 [6]. As a very high-level summary, these documents clarified that all RF/demod/RRM conformance tests will be carried out under static conditions of delay and Doppler during the duration of the measurements/test.

Additionally, the following contributions have been submitted to RAN5#101:

- Two discussion papers to discuss and align on the type of satellites to be covered in NTN RF and demod test case for IoT NTN (R5-237109 [7]) as well as NR NTN (R5-237110 [8]).

- One discussion paper (R5-237111 [9]) and 2 CRs, one for 36.508 (R5-237112 [10]) and another one for 38.508-1 (R5-237113 [11]), to propose which ephemeris information should be included in the test specifications and how to use such information in the test cases.

This document proposes how to select concrete ephemeris information for RRM test cases for NB-IoT NTN and NR NTN.

# 2. 	Discussion

## 2.1 RAN4 guidance for different types of test cases

Considering RAN4’s feedback in [4-6], only one set of conditions required for RF and demod NTN test cases:

- Set 1: Constant non-zero Doppler and associated constant non-zero delay, to be used in all RRM test cases.

According to R4-2316967 ([6]), in the case of UL timing test cases, at least the worst-case delay value shall be tested: 6.67ms (highest for LEO orbit 1200 km) and 128.79 ms for GSO satellites. For other RRM NTN test cases, the only condition is that minimum elevation angle is 30º.

### 2.2 Further RAN5 decisions required

For Set 1 described above:

- Maximum delay case is supposed to provide an acceptable coverage of the RRM UL timing NTN test cases. Hence, there is no need to test different delay values.

- As for other RRM NTN test cases, the only condition to select the constant non-zero Doppler and associated delay is to run test cases with a minimum elevation angle of 30º, which matches with the maximum delay case for RRM UL timing NTN test case and it is supposed to be one of the most challenges scenarios, it is proposed to use exactly the same conditions as per RRM NTN UL timing test cases (elevation angle =30º).

- In case of UEs supporting either NGSO only or both GSO and NGSO satellites, as satellite at 1200 km altitude provide worse case delays, testing LEO1200 satellite will suffice for RRM tests.

Proposal 1: When testing RRM NTN test cases, ephemeris values shall correspond to an elevation angle of 30º (maximum delay: 128.79ms for GSO and 6.67 ms for NGSO – LEO1200 satellites).

Proposal 2: When testing RRM NTN test cases for UEs which support only NGSO or both GSO and NGSO satellites, ephemeris values shall correspond to a 1200 km satellite altitude.

# 3. 	Conclusion

This document proposes how to select concrete ephemeris information for RF and demod test cases for NB-IoT NTN and NR NTN.

The following proposals are made:

Proposal 1: When testing RRM NTN test cases, ephemeris values shall correspond to an elevation angle of 30º (maximum delay: 128.79ms for GSO and 6.67 ms for NGSO – LEO1200 satellites).

Proposal 2: When testing RRM NTN test cases for UEs which support only NGSO or both GSO and NGSO satellites, ephemeris values shall correspond to a 1200 km satellite altitude.

Proposals 1 and 2 are implemented in R5-237112 ([10]) for IoT NTN and in R5-237113 ([11]) for NR NTN.

# 4. 	References

- RP-231308, “UE Conformance – NB-IoT (Narrowband IoT)/eMTC (enhanced Machine Type Communication) core & performance requirements for Non-Terrestrial Networks (NTN)”, China Mobile, MediaTek, RAN#98

- 3GPP TS 36.521-3, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Radio Resource Management (RRM) conformance testing”.

- R5-233672, “LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, RAN5#99

- R4-2314001, “Reply LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, Thales, RAN4#108

- R4-2317006, “Further Reply LS on clarifications for Non-Terrestrial Networks”, Keysight Technologies UK Ltd, RAN4#108bis

- R4-2316967, “WF on NTN RAN task”, MediaTek Inc., RAN4#108bis.

- R5-237109, “IoT NTN discussion on SAN type coverage in testing”, Keysight Technologies UK Ltd, RAN5#101

- R5-237110, “NR NTN discussion on SAN type coverage in testing”, Keysight Technologies UK Ltd, RAN5#101

- R5-237111, “On ephemeris definition”, Keysight Technologies UK Ltd, Thales, MediaTek, RAN5#101

- R5-237112, “Common test environment for IoT NTN RF/RRM testing”, Keysight Technologies UK Ltd, Thales, MediaTek, RAN5#101

- R5-237113, “Common test environment update for NR NTN RF/RRM testing”, Keysight Technologies UK Ltd, Thales, RAN5#101