3GPP TSG-RAN5 Meeting # 101	R5-237116

# Chicago, USA,  13th November 2023 - 17th November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Text Proposal |  |  |  |  |  |  |  |  |
|  | 36.521-4 | CR | N/A | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core requirements alignment for IoT NTN test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-3 |
| Category: | N/A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Verification guidelines in core specifications are being updated as per R4-2318399 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Align test specification with changes in core requirements as per R4-2318399. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will be misaligned with RAN4 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 7.1, 8.1, Annex A.6 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR … CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR depends on RAN4 CR R4-2318399. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES 1>>>

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single transmit antenna. For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

The transient periods due to power steps, OFF/ON and ON/OFF transitions could occur at slot or symbol boundary with transients, on one or both sides of the boundary. The measurement period and whether to exclude the transient periods are specified in the respective sections below.

For testing of category NB1 in all operation bands, standalone is used as default operation mode unless otherwise stated by the test case.

<< END OF CHANGES 1>>

<<< START OF CHANGES 2>>>

## 7.1	General

The requirements in clause 7.1 of TS 36.101 [7] shall apply.

All requirements in this section are applicable to devices supporting GSO and/or NGSO satellites.

<< END OF CHANGES 2>>

<<< START OF CHANGES 3>>>

## 8.1	General

.

<< END OF CHANGES 3>>

<<< START OF CHANGES 4>>>

## A.5.3	OCNG Patterns for Narrowband IoT

The following OCNG patterns are used for modelling allocations to virtual narrowband IoT UEs (which are not under test). The OCNG pattern for each sub frame specifies the allocations that shall be filled with OCNG, and furthermore, the relative power level of each such allocation.

In each test case the OCNG is expressed by parameters OCNG_RA and OCNG_RB which together with a relative power level () specifies the NPDSCH EPRE-to-NRS EPRE ratios in OFDM symbols with and without Narrowband reference symbols, respectively. The relative power, which is used for modelling boosting per virtual UE allocation, is expressed by:

where  denotes the relative power level of the i:th virtual UE. The parameter settings of OCNG_RA, OCNG_RB, and the set of relative power levels are chosen such that when also taking allocations to the UE under test into account, as given by a NPDSCH or NPDCCH reference channel, a transmitted power spectral density that is constant on an OFDM symbol basis is targeted.

### A.5.3.1	Narrowband IoT OCNG pattern 1

Table A.5.3.1-1: NB.OP.1 FDD: OCNG FDD Pattern 1

| Bandwidth | Relative power level [dB] | NPDCCH and corresponding NPDSCH Data |
| --- | --- | --- |
|  | Subframe |  |
|  | Unused subframes |  |
| 200KHz | 0 | Note 2 |
| Note 1: These subframes are assigned to an arbitrary number of virtual UEs with one NPDSCH per virtual UE with corresponding NPDCCH; the data transmitted over the OCNG NPDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. The parameteris used to scale the power of NPDSCH and NPDCCH. Note 2: Subframes and/or REs available for narrowband IOT DL transmission depend on the in-band, guard band or standalone mode indicated in MIB, and scheduling delay between NPDCCH, NPDSCH, NPUSCH format 2 and NPDCCH specified in test cases. |  |  |

# A.6	Testing related to Satellite Access

## A.6.1	General

The following test conditions should be maintained for Satellite Access

-  The same ephemeris info will be maintained during each test iteration.

-  A set of ephemeris information are pre-defined for each satellite corresponding to respective epoch times in TS 36.508 [12].

-  The range of the selected constant delay shift is as follows:

-  For NGSO an altitude of 600km and 1200km on a circular orbit are considered. The range of the one-way delay between UE and satellite is from 2ms (lowest value for LEO orbit 600km) to 6.67ms (highest value for LEO orbit 1200km).

-  Constant delay value is derived from ephemeris info (SIB31) and UE location associated to zero Doppler or non-zero Doppler value under test.

## A.6.2	Test condition for transmitter characteristics

All requriements in section 6 for transmitter characteristics, other than frequency error in clauses 6.4A.1 and 6.4B.1 shall be verified when Doppler conditions are set to zero and delay conditions are set to constant for all types of satellites.

Frequency error requirement in clauses 6.4A.1 and 6.4B.1 shall be verified for at least two cases: one with zero Doppler condition and the other one with constant Doppler (different from zero) up to 0.993 ppm for GSO satellites and up to 24 ppm for NGSO satellites.

## A.6.3	Test condition for receiver characteristics

All requirements in section 7 for receiver characteristics shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of satellites.

## A.6.4	Test condition for performance requirements

All requirements in section 8 for performance requirements shall be verified when Doppler conditions related to satellite motion for DL in service link are set to zero and delay conditions are set to constant for all types of satellites.

Annex B (normative):
Propagation Conditions

<< END OF CHANGES 4>>