3GPP TSG-RAN5 Meeting #101	R5-237126

# Chicago, United States, 13th Nov 2023 - 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.508 | CR | 1452 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to IoT NTN test environment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_plus_EPS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. Tables 6.3.5.1-1 & 6.3.5.2-1 mention about Cell 1/2/3, but the meaning is different from the rest of TS 36.508 clauses, this is confusing. 2. Various unclear/incorrect terms are used, causing unclarity to the NTN test environment description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Cell 1/2/3 has been replaced by First Cell / Second Cell / Third Cell. 2. Various corrections have been made in the NTN test environment clauses to bring clarify and harmomisation of terms used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IoT NTN test environment will remain prone to confusion for the reader. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13, 4.14, 6.3.5.1, 6.3.5.2, 8.1.5B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.13	UE position requirements for NTN testing

Clauses 4.14 and 8.1.5B define the test environment which applies to all test cases executed for eMTC and NB-IoT NTN UEs, unless otherwise specified.

-	UE's positioning engine (e.g., standalone GNSS receiver) shall be enabled to allow it to acquire the position. UE position during the test is stationary. Below position shall be used to pre-configure UE position:

-	Latitude: 25.08439333

-	Longitude: 121.56076999

-	Altitude: 0

## 4.14	Common test environment for Vertical UEs

### 4.14.1	eMTC NTN

This clause defines the test environment which applies to all test cases executed for eMTC NTN UEs, unless otherwise specified.

-	Test frequencies specified in Table 6.2.3.1-1d are used.

-	Test configures following IEs with condition NTN of MasterInformationBlock in Table 4.4.3.2-1, SystemInformationBlockType1-BR in Table 4.4.3.3-3A, SystemInformationBlockType31 in Table 4.4.3.3-21 and RadioResourceConfigCommonSIB-DEFAULT in Table 4.6.3-14.

-	Test uses Combination 32 for single cell scenario and Combination 33 for intra-frequency multi cell scenario.

### <SECTIONS SKIPPED>

### 6.3.5	Satellite ephemeris information for NTN

#### 6.3.5.1	GSO scenario

The satellite ephemeris information of GSO is defined in Table 4.4.3.3-21 for eMTC NTN and Table 8.1.4.3.3-10 for NB-IoT NTN with below assumption:

-	Inclination: 0

-	Eccentricity: 0

-	Latitude: 0

-	Longitude: 121.56076999

-	Altitude: 35786 km

-	Speed: 0 km/s

In case of multi-cell NTN GSO scenario, a maximum of 3 NTN cells can be used. The satellite ephemeris information is defined in Table 6.3.5.1-1 unless otherwise specified in a test case.

Table 6.3.5.1-1: Satellite ephemeris information in multi-cell NTN GSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX/Y/Z | nta-Common | Koffset |
| --- | --- | --- | --- | --- | --- | --- |
| First | -16976014 | 27636499 | 0 | 0 | 8243100 | 550 |
| Second | -23550424 | 22301093 | 0 | 0 | 8248322 | 550 |
| Third | -14502736 | 29010891 | 0 | 0 | 8248322 | 550 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. |  |  |  |  |  |  |

For example, in TC 22.5.23 runing in GSO scenario, Ncell 50 is the serving cell in the preamble so it uses the satellite ephemeris information of First Cell, while Ncell 62 uses the satellite ephemeris information of Second Cell.

#### 6.3.5.2	NGSO scenario

The satellite ephemeris information of NGSO is defined in Table 4.4.3.3-21 for eMTC NTN and Table 8.1.4.3.3-10 for NB-IoT NTN with below assumption:

-	Inclination: 90

-	Eccentricity: 0

-	Longitude: 121.56076999

-	Altitude: 600 km

-	Speed: 7.56 km/s

The UE position defined in clause 4.13 is the projection position from one of the satellite information to the ground.

As the NGSO satellite is moving and to avoid varying nta-Common, nta-CommonDrift and nta-CommonDriftVariation, the Reference Point (RP) is defined as the satellite itself.

Considering the mininum elevation angle for both NTN Gateway and UE is 10°, the initial latitude of the satellite is: 25.0843933 (UE latitude) - 15.8394129 (the max intersection angle between UE and the satellite in ECEF coordinates) = 9.24498034°.

The table of the satellite ephemeris information of NGSO NTN lasts 510 seconds as the NGSO satellite can serve the UE for about 8.5 minutes with the periodicity set to the si-WindowLength-r13 = 160 ms. The satellite ephemeris information in systemInformationBlockType31 or systemInformationBlockType31-NB is defined and where appropriate is given in Excel format in .xlsx files for each time instance in the appropriate data file specified in Annex B.1.

In case of multi-cell NTN NGSO scenario, a maximum of 3 NTN cells can be used. The satellite ephemeris information of each cell has the same value as the default, but the epoch time is shifted with respect to the previous cell by 20 seconds.

Table 6.3.5.2-1: Satellite ephemeris information in multi-cell NTN NGSO scenario

| Cell type (Note 1) | Time offset (second) | positionY | positionZ | velocityVX/Y/Z | nta-Common | Koffset |
| --- | --- | --- | --- | --- | --- | --- |
| First | 0 | See Annex B.1 |  |  |  |  |
| Second | +20 | See Annex B.1 |  |  |  |  |
| Third | +40 | See Annex B.1 |  |  |  |  |
| Note 1: First Cell is the dell defined as default serving cell in the test case at the end of the preamble. Second Cell and Third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cellbeing defined as the 2nd neighbour cell. |  |  |  |  |  |  |

For example, in TC 22.5.23 runing in NGSO scenario, Ncell 50 is the serving cell in the preamble so it uses the satellite ephemeris information of First Cell, while Ncell 62 uses the satellite ephemeris information of Second Cell.

### <SECTIONS SKIPPED>

### 8.1.5B	Common test environment for Vertical UEs

#### 8.1.5B.1	NB-IoT NTN

This clause defines the test environment which applies to all test cases executed for NB-IoT NTN UEs, unless otherwise specified.

-	Test frequencies specified in Table 8.3.2.3.1-1 are used.

-	Test configures following IEs with condition NTN of MasterInformationBlock-NB in Table 8.1.4.3.2-1, SystemInformationBlockType1-NB in Table 8.1.4.3.2-3, SystemInformationBlockType31-NB in Table 8.1.4.3.3-10 and RadioResourceConfigCommonSIB-NB-DEFAULT in Table 8.1.6.3-9.

-	Test uses Combinations 8 to 13.

-	For Rel-13 to Rel-15 NB-IoT test cases applicable to a UE supporting NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC), UE is pre-configured with the UE position defined in clause 4.13.