3GPP TSG-RAN5 Meeting #101	R5-237132

Chicago, US, 13th Nov 2023 – 17th Nov 2023

Agenda item:	5.4.17

Source:	Keysight Technologies, CAICT

Title:	PC3 Measurement Grids based on Alternate 6x2 Antenna Array Assumption

Document for:	Discussion and Endorsement

# Introduction

In RAN4#108, initial discussions were held to augment the PC3 measurement grids with another option based on the 6x2 (worst case) antenna configuration [1]. No decisions were made that meeting but continued discussions were held on this topic in RAN4#108bis [2][3]. Eventually, consensus was reached to send the LS to RAN5 [3] and task RAN5 with defining the new measurement grids and to incorporating them in TS 38.521-2 and TS 38.508-2. This contribution is providing an overview of the proposed measurement grids.

# Measurement Grids for PC3 Based on 6x2 Array Config

The single-element array patterns for the 6x2 array configuration are based on Table 5.2.3.3-1 of [4] and summarized in Table 1.

Table 1: Single-Element Antenna Array Assumptions

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,V(θ″)=-min12θ″-90°θ3dB2,SLAV,θ3dB=90°,SLAV=25dB |
| Antenna element horizontal radiation pattern (dB) | AE,H(ϕ″)=-min12ϕ″ϕ3dB2,Am,ϕ3dB=90°,Am=25dB |
| Combining method for 3D antenna element pattern (dB) | A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| M x N array | 6 x 2 |
| (dv, dh) | (0.5λ, 0.5λ) |

The resulting antenna patterns are highlighted in the 3D pattern plots of the 6x2 antenna array in Figure 1. The pattern has an HPBW of ~50o and ~16.9o in each of the principal planes.

Figure 1: 6x2 Antenna Pattern.

# Beam Peak Search Measurement Grid for 6x2 Antenna Array Assumption

The simulation assumptions including the antenna patterns for the TRP measurement grids are the same as [5] except a 6x2 antenna array assumption instead of the 4x2 array assumption.

For the simulations, the relative orientation of the simulated antenna array and the measurement grid was altered randomly. The statistical results from simulations using 50,000 random orientations are then used to determine mean error, standard deviation and percentile analysis on CDF curve of all maximum EIRPs for each measurement grid. The EIRPs are normalized by the known 6x2 antenna peak antenna gain.

Sample histograms and CDF distributions for the beam peak error for constant step-size measurement grids are shown in Figure 2 and for the constant density measurement grid (based on the charged particle implementation) in Figure 3. The histograms show a half-normal distribution.

Given the half-normal distribution, the MU term should be based on the determination of the offset from the beam peak that contains 95% of the distribution (alternatively, the value at which the CDF is 5%). This offset shall be considered a systematic error in the MU budget. The various statistical metrics are illustrated in Figure 4.

Figure 2: Histogram of maximum beam peak errors for sample constant-step size meausurement grids (left: 5o, right: 9o step size)

Figure 3: Histogram of maximum beam peak errors for sample constant density measurement grids (left: 2000, right: 600 grid points)

Figure 4: Statistical metrics for a sample half-normal distribution

The offset at which the CDF is 5% are tabulated in Table 2 for the constant step size grids and in Table 3 for the constant density grids.

Table 2: Statistical Analyses of the 50k simulations for the constant step size grids

| Dq=Df [°] | Number of unique grid points | Offset5%CDF [dB] |
| --- | --- | --- |
| 5 | 2522 | 0.15 |
| 6 | 1742 | 0.22 |
| 8 | 1106 | 0.34 |
| 9 | 762 | 0.49 |
| 10 | 614 | 0.60 |
| 11.25 | 482 | 0.77 |
| 15 | 266 | 1.35 |

Table 3: Statistical Analyses of the 50k simulations for the constant-density grids

| Number of unique grid points | Offset5%CDF [dB] |
| --- | --- |
| 2000 | 0.14 |
| 1500 | 0.18 |
| 750 | 0.36 |
| 650 | 0.42 |
| 625 | 0.44 |
| 600 | 0.46 |
| 575 | 0.48 |
| 550 | 0.50 |
| 525 | 0.52 |
| 500 | 0.55 |

For PC3, it was agreed to limit the Offset5%CDF to a maximum value of 0.5 dB for the default (8x2) and optional grids (based on 4x2 antenna configuration); the same limit should be selected for this additional grid (based on 6x2 antenna configuration) to keep the MUs/MTSUs the same. The proposed min. number of grid points for beam peak searches with the 6x2 antenna configuration is summarized in Table 4.

Table 4: Minimum number of unique grid points for beam peak search with 6x2 antenna configuration

| Systematic Error of ‘Beam Peak Search’: Offset from Beam Peak at which CDF is 5% | Minimum Number of Unique Grid Points for Constant Step Size Grid | Minimum Number of Unique Grid Points for Constant Density Grid |
| --- | --- | --- |
| 0.5dB | 762 (9ostep size) | 575 |

Proposal 1: Adopt the measurement grids highlighted in Table 4 for Beam Peak Search for PC3 UEs based on the alternate 6x2 antenna assumption with a systematic error of ‘Beam Peak Search’ of 0.5dB.

In Table 5, the different beam peak search grids for PC3 UEs are summarized.

Table 5: Minimum number of unique grid points for different PC3 UE antenna configurations

| Antenna Configuration | Minimum Number of Unique Grid Points for Constant Step Size Grid | Minimum Number of Unique Grid Points for Constant Density Grid |
| --- | --- | --- |
| 8x2 (default) | 1106 (7.5° step size) | 800 |
| 6x2 (declared) | 762 (9° step size) | 575 |
| 4x2 (declared) | 422 (12° step size) | 310 |

# TRP Measurement Grids for 6x2 Antenna Array Assumption

The simulation assumptions including the antenna patterns for the TRP measurement grids the same as those in [5] except a 6x2 antenna array assumption instead of the 4x2 array assumption.

The results tabulated in this section outline the results of statistical analyses with the positioning concept taken into account, i.e., the analyses were performed with and without the assumption that the beam peak direction is oriented away from the hemisphere towards the pole at = 180o. Additionally, the standard deviations are presented when ranges of pattern values are disregarded (zeroed out). For the constant-step size measurement grids, separate investigations were performed to determine how many latitudes (grid points with constant q) towards the back pole/positioner can be skipped/disregarded. As the positioner/mast is commonly blocking the measurement direction towards q=180°, at least this one grid point must be skipped for TRP. The results with the re-positioning concept applied are summarized in Table 6 for the sin(theta) and the Clenshaw-Curtis quadratures while the results without the re-positioning concept applied are summarized in Table 7.

For the constant density measurement grids, a similar investigation was performed using the Charged Particle implementation. Here, the effect of disregarding grids points within specific angular regions near the back pole/positioner was also studied. The results without the re-positioning concept applied are summarized in Table 8 for the Charged Particle implementation while the results with the re-positioning concept applied are summarized in Table 9.

Similar to default PC3 TRP grids, it is assumed that the standard deviation shall not exceed 0.25dB. Thus, the following number of points shall be included in the alternate TRP measurement grids for PC3 UEs (based on the 6x2 antenna array assumption).

If the re-positioning concept is not applied to TRP test cases:

-	100 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.13 dB

-	10 latitudes and 18 longitudes (Dq=Df=20°, 146 unique grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.23dB with the allowance to skip and interpolate measurements at the pole at =180o, see Annex M.4.4 [6]

-	10 latitudes and 16 longitudes (Dq=20°, Df=22.5°, 130 unique grid points) for constant step size grid – Clenshaw Curtis weights integration approach, with standard deviation of 0.23dB with the allowance to skip and interpolate measurements at the pole at =180o, see Annex M.4.4 [6]

If the re-positioning concept is applied to TRP test cases:

-	90 measurement grid points for constant density grid – Charged Particle implementation, with standard deviation of 0.21 dB with the allowance to skip and interpolate measurements for ≥150o, see Annex M.4.4 [6]

-	10 latitudes and 18 longitudes (Dq=Df=20°, 146 unique grid points) for constant step size grid – sin (theta) weights integration approach, with standard deviation of 0.23dB with the allowance to skip and interpolate measurements ≥140o, see Annex M.4.4 [6]

-	10 latitudes and 16 longitudes (Dq=20°, Df=22.5°, 122 unique grid points) for constant step size grid – Clenshaw-Curtis weights integration approach, with standard deviation of 0.18dB with the allowance to skip and interpolate measurements for ≥140o, see Annex M.4.4 [6]

Proposal 2: Adopt the proposed alternate measurement grids listed above for TRP for PC3 UEs based on the alternate 6x2 antenna assumption.

Table 6: Statistics of quadrature approaches for constant step size measurement grids for the 6x2 alternate antenna array with the re-positioning concept applied.

| Number of |  | Step Size Dq [°] | Step Size Df [°] | Number of Unique Grid Points (without latitudes disregarded) | Number of Unique Grid Points (with latitudes disregarded) | Number of Latitudes disregarded | Mean Error [dB] | Std. Dev [dB] | Min TRP Error [dB] | Max TRP Error [dB] | Quadrature | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 24 | 15 | 15 | 266 | 265 | 1 | -0.03 | 0.09 | -0.67 | 0.02 | Sin(theta) | yes |
| 13 | 24 | 15 | 15 |  | 265 | 1 | 0.00 | 0.01 | -0.03 | 0.03 | Clenshaw-Curtis | yes |
| 13 | 24 | 15 | 15 |  | 241 | 2 | -0.03 | 0.09 | -0.68 | 0.01 | Sin(theta) | yes |
| 13 | 24 | 15 | 15 |  | 241 | 2 | 0.00 | 0.01 | -0.04 | 0.02 | Clenshaw-Curtis | yes |
| 13 | 24 | 15 | 15 |  | 217 | 3 | -0.03 | 0.09 | -0.68 | 0.01 | Sin(theta) | yes |
| 13 | 24 | 15 | 15 |  | 217 | 3 | 0.00 | 0.01 | -0.04 | 0.02 | Clenshaw-Curtis | yes |
| 10 | 18 | 20 | 20 | 146 | 145 | 1 | -0.05 | 0.23 | -1.57 | 0.41 | Sin(theta) | yes |
| 10 | 18 | 20 | 20 |  | 145 | 1 | 0.00 | 0.13 | -0.46 | 0.41 | Clenshaw-Curtis | yes |
| 10 | 18 | 20 | 20 |  | 127 | 2 | -0.05 | 0.23 | -1.58 | 0.41 | Sin(theta) | yes |
| 10 | 18 | 20 | 20 |  | 127 | 2 | 0.00 | 0.13 | -0.46 | 0.41 | Clenshaw-Curtis | yes |
| 10 | 18 | 20 | 20 |  | 109 | 3 | -0.06 | 0.23 | -1.57 | 0.38 | Sin(theta) | yes |
| 10 | 18 | 20 | 20 |  | 109 | 3 | -0.01 | 0.13 | -0.50 | 0.38 | Clenshaw-Curtis | yes |
| 10 | 16 | 20 | 22.5 | 130 | 129 | 1 | -0.05 | 0.26 | -1.57 | 0.72 | Sin(theta) | yes |
| 10 | 16 | 20 | 22.5 |  | 129 | 1 | 0.00 | 0.18 | -0.87 | 0.73 | Clenshaw-Curtis | yes |
| 10 | 16 | 20 | 22.5 |  | 113 | 2 | -0.06 | 0.27 | -1.59 | 0.72 | Sin(theta) | yes |
| 10 | 16 | 20 | 22.5 |  | 113 | 2 | -0.01 | 0.18 | -0.87 | 0.73 | Clenshaw-Curtis | yes |
| 10 | 16 | 20 | 22.5 |  | 97 | 3 | -0.06 | 0.27 | -1.58 | 0.68 | Sin(theta) | yes |
| 10 | 16 | 20 | 22.5 |  | 97 | 3 | -0.01 | 0.18 | -0.97 | 0.68 | Clenshaw-Curtis | yes |
| 10 | 15 | 20 | 24 | 122 | 121 | 1 | -0.06 | 0.31 | -1.57 | 1.06 | Sin(theta) | yes |
| 10 | 15 | 20 | 24 |  | 121 | 1 | -0.01 | 0.25 | -1.41 | 1.06 | Clenshaw-Curtis | yes |
| 10 | 15 | 20 | 24 |  | 106 | 2 | -0.06 | 0.32 | -1.58 | 1.06 | Sin(theta) | yes |
| 10 | 15 | 20 | 24 |  | 106 | 2 | -0.01 | 0.25 | -1.40 | 1.06 | Clenshaw-Curtis | yes |
| 10 | 15 | 20 | 24 |  | 91 | 3 | -0.07 | 0.31 | -1.57 | 1.01 | Sin(theta) | yes |
| 10 | 15 | 20 | 24 |  | 91 | 3 | -0.02 | 0.25 | -1.52 | 1.01 | Clenshaw-Curtis | yes |

Table 7: Statistics of quadrature approaches for constant step size measurement grids for the 6x2 alternate antenna array without the re-positioning concept applied.

| Number of |  | Step Size Dq [°] | Step Size Df [°] | Number of Unique Grid Points (without latitudes disregarded) | Number of Unique Grid Points (with latitudes disregarded) | Number of Latitudes disregarded | Mean Error [dB] | Std. Dev [dB] | Min TRP Error [dB] | Max TRP Error [dB] | Quadrature | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Latitudes | Longitudes |  |  |  |  |  |  |  |  |  |  |  |
| 13 | 24 | 15 | 15 | 266 | 265 | 1 | -0.03 | 0.09 | -0.67 | 0.02 | Sin(theta) | no |
| 13 | 24 | 15 | 15 |  | 265 | 1 | -0.02 | 0.07 | -0.73 | 0.03 | Clenshaw-Curtis | no |
| 13 | 24 | 15 | 15 |  | 241 | 2 | -0.22 | 0.67 | -4.56 | 0.01 | Sin(theta) | no |
| 13 | 24 | 15 | 15 |  | 241 | 2 | -0.20 | 0.67 | -4.54 | 0.02 | Clenshaw-Curtis | no |
| 13 | 24 | 15 | 15 |  | 217 | 3 | -0.38 | 0.80 | -4.92 | 0.01 | Sin(theta) | no |
| 13 | 24 | 15 | 15 |  | 217 | 3 | -0.38 | 0.83 | -4.95 | 0.02 | Clenshaw-Curtis | no |
| 10 | 18 | 20 | 20 | 146 | 145 | 1 | -0.05 | 0.23 | -1.57 | 0.41 | Sin(theta) | no |
| 10 | 18 | 20 | 20 |  | 145 | 1 | -0.03 | 0.19 | -1.64 | 0.41 | Clenshaw-Curtis | no |
| 10 | 18 | 20 | 20 |  | 127 | 2 | -0.44 | 1.22 | -7.28 | 0.41 | Sin(theta) | no |
| 10 | 18 | 20 | 20 |  | 127 | 2 | -0.42 | 1.21 | -7.25 | 0.41 | Clenshaw-Curtis | no |
| 10 | 18 | 20 | 20 |  | 109 | 3 | -0.74 | 1.38 | -9.00 | 0.41 | Sin(theta) | no |
| 10 | 18 | 20 | 20 |  | 109 | 3 | -0.71 | 1.38 | -9.02 | 0.41 | Clenshaw-Curtis | no |
| 10 | 16 | 20 | 22.5 | 130 | 129 | 1 | -0.05 | 0.27 | -1.57 | 0.73 | Sin(theta) | no |
| 10 | 16 | 20 | 22.5 |  | 129 | 1 | -0.03 | 0.23 | -1.64 | 0.73 | Clenshaw-Curtis | no |
| 10 | 16 | 20 | 22.5 |  | 113 | 2 | -0.45 | 1.24 | -7.30 | 0.73 | Sin(theta) | no |
| 10 | 16 | 20 | 22.5 |  | 113 | 2 | -0.42 | 1.22 | -7.27 | 0.72 | Clenshaw-Curtis | no |
| 10 | 16 | 20 | 22.5 |  | 97 | 3 | -0.74 | 1.38 | -9.00 | 0.67 | Sin(theta) | no |
| 10 | 16 | 20 | 22.5 |  | 97 | 3 | -0.72 | 1.41 | -9.03 | 0.67 | Clenshaw-Curtis | no |
| 10 | 15 | 20 | 24 | 122 | 121 | 1 | -0.06 | 0.32 | -1.57 | 1.06 | Sin(theta) | no |
| 10 | 15 | 20 | 24 |  | 121 | 1 | -0.04 | 0.29 | -1.64 | 1.06 | Clenshaw-Curtis | no |
| 10 | 15 | 20 | 24 |  | 106 | 2 | -0.44 | 1.23 | -7.29 | 1.06 | Sin(theta) | no |
| 10 | 15 | 20 | 24 |  | 106 | 2 | -0.42 | 1.23 | -7.25 | 1.06 | Clenshaw-Curtis | no |
| 10 | 15 | 20 | 24 |  | 91 | 3 | -0.74 | 1.40 | -9.00 | 1.01 | Sin(theta) | no |
| 10 | 15 | 20 | 24 |  | 91 | 3 | -0.73 | 1.41 | -9.03 | 1.01 | Clenshaw-Curtis | no |

Table 8: Statistics for constant density measurement grid types for the 6x2 alternate antenna array with the re-positioning concept applied (charged particle implementation only)

| Number of Grid Points | Range of Angles disregarded | Mean Error [dB] | Std. Dev [dB] | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- |
| 200 | none | 0.00 | 0.02 | yes |
| 180 |  | 0.00 | 0.02 | yes |
| 160 |  | 0.00 | 0.03 | yes |
| 140 |  | 0.00 | 0.04 | yes |
| 120 |  | 0.00 | 0.07 | yes |
| 100 |  | 0.00 | 0.14 | yes |
| 90 |  | -0.01 | 0.21 | yes |
| 80 |  | -0.01 | 0.33 | yes |
| 70 |  | -0.03 | 0.53 | yes |
| 200 | 165o-180o | 0.00 | 0.02 | yes |
| 180 |  | 0.00 | 0.02 | yes |
| 160 |  | 0.00 | 0.03 | yes |
| 140 |  | 0.00 | 0.04 | yes |
| 120 |  | 0.00 | 0.07 | yes |
| 100 |  | -0.01 | 0.14 | yes |
| 90 |  | -0.01 | 0.21 | yes |
| 80 |  | -0.01 | 0.33 | yes |
| 70 |  | -0.04 | 0.53 | yes |
| 200 | 150o-180o | 0.00 | 0.02 | yes |
| 180 |  | 0.00 | 0.02 | yes |
| 160 |  | 0.00 | 0.03 | yes |
| 140 |  | 0.00 | 0.04 | yes |
| 120 |  | 0.00 | 0.07 | yes |
| 100 |  | -0.01 | 0.14 | yes |
| 90 |  | -0.01 | 0.21 | yes |
| 80 |  | -0.02 | 0.33 | yes |
| 70 |  | -0.04 | 0.53 | yes |

Table 9: Statistics for constant density measurement grid types for the 6x2 alternate antenna array without the re-positioning concept applied (charged particle implementation only)

| Number of Grid Points | Range of Angles disregarded | Mean Error [dB] | Std. Dev [dB] | Re-Positioning Concept Applied |
| --- | --- | --- | --- | --- |
| 200 | none | 0.00 | 0.02 | no |
| 180 |  | 0.00 | 0.02 | no |
| 160 |  | 0.00 | 0.03 | no |
| 140 |  | 0.00 | 0.04 | no |
| 120 |  | 0.00 | 0.07 | no |
| 100 |  | 0.00 | 0.13 | no |
| 80 |  | -0.01 | 0.33 | no |
| 60 |  | -0.08 | 0.81 | no |
| 40 |  | -0.29 | 1.70 | no |

# Spherical Coverage Measurement Grid for 6x2 Antenna Array Assumption

In the absence of beam steering assumptions provided in [3] for the 6x2 array configuration, the same assumptions as for 4x2 (and 8x2) were assumed. The simulation assumptions including the antenna patterns for the spherical coverage measurement grids are the same as those presented in [5] except the 6x2 antenna array assumption instead of 4x2 assumptions.

The reference CDF curve, which utilized scaling the PDFs by Clenshaw-Curtis weights, was determined with a very fine constant step size measurement grid using a 1o step size in  and f.

At the 50%-tile CDF, i.e., the target CDF for Power Class 3, statistical analyses of all 10000 EIRPs, EIRP50%CDF, is performed. For the example of the 15o constant step size grid, the histogram is shown in Figure 5.

Figure 5: Sample Histogram of the 10000 EIRPs at the 50%-tile CDF for a 15º constant step size measurement grid

The simulations in this contribution were only for the case where the beam peak is oriented in completely random orientations, i.e., the beam peak is not necessarily aligned to a grid point. The results for various constant-step size measurement grids are tabulated in Table 10.

Table 10: Statistical results of EIRP50%CDF for the 6x2 antenna array for constant step size measurement grids and the beam peak oriented in completely random orientations.

| Step Size [o] | Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- | --- |
| 9.0 | 762 | 0.06 | 0.01 |
| 10.0 | 614 | 0.09 | 0.01 |
| 12.0 | 422 | 0.12 | 0.02 |
| 15.0 | 266 | 0.15 | 0.03 |
| 18.0 | 182 | 0.18 | 0.04 |
| 20.0 | 146 | 0.28 | 0.06 |

Similar results for the constant-density measurement grids are tabulated in Table 11.

Table 11: Statistical results of EIRP50%CDF for the 6x2 antenna array for constant density measurement grids and the beam peak oriented in completely random orientations.

| Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| --- | --- | --- |
| 150 | 0.19 | 0.04 |
| 175 | 0.16 | 0.04 |
| 200 | 0.14 | 0.04 |
| 225 | 0.14 | 0.03 |
| 250 | 0.14 | 0.03 |
| 500 | 0.08 | 0.02 |
| 575 | 0.07 | 0.01 |

Highlighted in yellow are the measurement grids previously adopted for both 8x2 and 4x2 (with a maximum uncertainty value of 0.12dB used to determine the MTSU of the spherical coverage test cases). For this 6x2 antenna configuration, the MUs are slightly larger than those previously obtained for the 8x2 antenna configuration, e.g., for the 15° step size grids [6], which seems counterintuitive; however, it should be highlighted that the PC3 8x2 antenna configuration measurement grids were based on the 260°/130° HPBW assumptions while they have been revised later to 90°/90° for subsequent PC3 assumptions [8]. Table 12 compares the statistical results for various antenna configurations (including HPBW assumptions for the 8x2 configuration) which confirms the observation above, i.e., the MU for spherical coverage is lower for the 260°x130° HPBW assumption (when compared to the 90°x90° assumption)

Table 12: Statistical results of EIRP50%CDF for the various antenna array configurations for constant step size measurement grids.

| Antenna Config |  | 4x2 (90°x90°) |  | 6x2 (90°x90°) |  | 8x2 (260°x130°) |  | 8x2 (90°x90°) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step Size [o] | Number of unique grid points | Std. Dev [dB] | ¦Mean Error¦ [dB] | Std. Dev [dB] | ¦Mean Error¦ [dB] | Std. Dev [dB] | ¦Mean Error¦ [dB] | Std. Dev [dB] | ¦Mean Error¦ [dB] |
| 10.0 | 614 | 0.06 | 0.01 | 0.09 | 0.01 | 0.06 | 0.00 | 0.12 | 0.02 |
| 12.0 | 422 | 0.08 | 0.02 | 0.12 | 0.02 | 0.09 | 0.01 | 0.15 | 0.02 |
| 15.0 | 266 | 0.11 | 0.02 | 0.15 | 0.03 | 0.11 | 0.01 | 0.18 | 0.04 |
| 18.0 | 182 | 0.13 | 0.04 | 0.18 | 0.04 | 0.15 | 0.02 | 0.24 | 0.05 |

In order to keep the same spherical coverage grids for the 6x2 array assumption as those of the 8x2 default array assumption, it is recommended to use the following recommendation in terms of min. number of grid points and standard deviation for spherical coverage grids:

- constant density grid (using the charged particle implementation) with at least 200 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.14dB

- constant step size grid with at least 266 grid points: standard deviation (MU element ‘Influence of spherical coverage grid’) of 0.15dB

- the MU element ‘Systematic error related to EIS spherical coverage’ is the DL step size, i.e., 0.2dB.

Proposal 3: Adopt the proposed measurement grids and MUs for spherical coverage measurement grids for PC3 UEs with the alternate 6x2 antenna array assumption.

The slight increase in MU for the 6x2 spherical coverage grids is not applicable if the spherical coverage analyses are performed based on the fine beam peak search grids (highlighted in orange in Table 10 and Table 11).

# Applicability

The applicability of the PC3 measurement grids is rather unique now since there is one default worst-case 8x2 antenna configuration and two optional worst-case configurations based on a vendor declaration, i.e., 4x2 and 6x2. The decision tree for the appropriate selection of applicable measurement grids based on the presence or absence of a vendor declaration is shown in Figure 6.

Figure 6: Decision tree for PC3 measurement grid selection.

# Conclusion

The following observations and proposals were made:

Proposal 1: Adopt the measurement grids highlighted in Table 4 for Beam Peak Search for PC3 UEs based on the alternate 6x2 antenna assumption with a systematic error of ‘Beam Peak Search’ of 0.5dB.

Proposal 2: Adopt the proposed alternate measurement grids listed above for TRP for PC3 UEs based on the alternate 6x2 antenna assumption.

Proposal 3: Adopt the proposed measurement grids and MUs for spherical coverage measurement grids for PC3 UEs with the alternate 6x2 antenna array assumption.

# References

- R4-2311231, Measurement Grids for Optional 6x2 PC3 Antenna Array Configuration, Apple, 3GPP RAN WG4 Meeting #108, August 2023

- R4-2316322, Motivation for LS to RAN5 regarding PC3 6x2 Antenna Configuration Declaration, Keysight Technologies, Apple, 3GPP TSG-RAN4 Meeting #108-bis, October 2023

- R5-235994, LS to RAN5 for measurement grid based on the assumption of 6x2 for PC3, TSG WG RAN4, 3GPP TSG-RAN WG5 Meeting #101, November 2023

- TR 38.803, Study on new radio access technology: Radio Frequency (RF) and co-existence aspects

- R5-213812, Measurement Grids for Optional 4x2 PC3 Antenna Array Configuration, Keysight Technologies, 3GPP TSG-RAN5 Meeting #91-e, May 2021

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases

- R5-206618, PC1 Assumptions for CDF Curve for MOP EIRP Spherical Coverage, Qualcomm Incorporated, 3GPP TSG-RAN5 Meeting #89-e, November 2020