3GPP TSG-RAN5 Meeting #101	R5-237136

Chicago, US, 13th Nov 2023 – 17th Nov 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TEXT PROPOSAL |  |  |  |  |  |  |  |  |
|  | 38.561 | CR | N/A | rev | - | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR on Alternate TRS Linearization |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_TRP_TRS-UEConTest |  |  |  |  |  | Date: |  |  | 2023-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Alternate EIS linearization is introduced that does not require correction of TRS results |  |  |  |  |  |  |  |  |
| Summary of change: |  | Two-step procedure of alternate EIS linearization is introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TRS testing without correcting TRS results is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.3.3.3, A.4.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion Paper in R5-237135 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

#### A.3.3.3	TRS Test procedure

The TRS of the DUT is measured by sampling effective isotropic sensitivity (EIS) of the DUT with three-dimensional scan at various locations surrounding the device. The measurement is performed with a constant sampling step of 30 degrees in both theta () and phi () axes for TRS measurement.

EIS, or receiver sensitivity measurements, is defined as the minimum downlink signal power received at the UE antenna input required to provide a data throughput rate greater than or equal to 95% of the maximum throughput of the reference measurement channel (RMC) (the maximum throughput is per Appendix A of TS 38.521-1 [TBD]).

For TRS measurement, the evaluations shall be performed at maximum transmit power.

The measurement procedure includes the following steps:

1)	Place the DUT inside the QZ following the positioning guideline defined in Clause 6.

2)	Connect the SS with the DUT through the measurement antenna.

3)	Follow steps 1 through 4 in section 7.3.2.4.2 of TS 38.521-1 [TBD], with the following exception: determine each EIS, i.e., by adjusting the downlink signal level until the minimum power level at which the throughput exceeds or equal to 95% of the maximum throughput of the specified RMC, at each sampling point. The downlink power step size shall be no more than 0.5 dB when the RF power level is near the NR sensitivity level.

The TRS value is calculated using the equation outlined in Clause 5.2 in [TBD]. The reported TRS value shall be corrected by -0.5·output level step resolution of the final power step search and note the correction in the test report. If the alternate EIS search procedure, detailed in A.3.3.3.1 is applied to assess the EIS at each grid point, no correction of TRS shall be applied and no additional MU shall be considered.

This TRS test procedure is common to both SA and EN-DC measurements. The detailed UE configurations for TRS test in SA and EN-DC mode are specified in Clause A.2.

##### A.3.3.3.1	Alternate EIS Search Procedure with Linearization

The alternate EIS search procedure is based on two steps. Step 1 is performed once on a set of UEs; the results from this evaluation are recorded and can be reused for every UE TRS measurement, i.e., Step 1 does not need to be performed prior to each device measurement.

Step 1: Polynomial characterization performed using reference measurements inside the anechoic chamber (needs to be performed just once)

a) Record radiated TP as a function of DL power with fine search step size (0.1 dB or less) for a few DUTs (reference measurements)

b) Normalize the TP curves and determine the polynomial coefficients of the averaged, normalized TP curves

c) Record the polynomial coefficients from the reference measurements

Step 2: Apply polynomial interpolation from the reference measurements during EIS search for each grid point

a) Apply polynomial interpolation using the polynomial coefficients (from Step 1c) to the two EIS search measurements (one passing, one failing) with 0.5dB step size near the sensitivity threshold

b) Record the interpolated EIS for each grid point

c) Determine TRS from the interpolated EIS measurements

<<< Skip Unchanged Sections >>>

#### A.4.2.6	Sensitivity measurement: output level step resolution

Editor’s Note: An alternate EIS search procedure with interpolation is not precluded.

When output power of the communication tester is swept to reach the throughput target that defines the sensitivity threshold, the final power step resolution represents an asymmetric uncertainty contribution that can be corrected since this uncertainty is device and test system independent. The lab shall correct the TRS results by -0.5·output level step resolution of the final power step search and note the correction in the test report. If the alternate EIS search procedure with linearization, detailed in A.3.3.3.1 is applied to assess the EIS at each grid point, no correction of TRS shall be applied and no additional MU shall be considered.

<<< END OF CHANGES >>>