3GPP TSG RAN WG5 Meeting #101		R5-237151

Chicago, USA, November 13 – 17th 2023

Agenda item:	5.3.22.2

Source:	Apple, Vodafone, AT&T, ETS Lindgren, Telecom Italia

Title:	On TRP test time reduction adopting reduced grids

WI/SI:	NR_FR1_TRP_TRS-UEConTest

Release:	Rel-18

Document for:	Approval

# 1	Introduction

- Test lab procedures

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - Newly defined Coarser measurement grid in TR 38.870 can be used for TRP TRS measurement

In previous RAN4 meeting it was defined on [1] that the working procedure for REL-18 Measurement campaign would be conducted adopting coarser TRP and TRS measurement grid.

The proposed coarser grid is defined on [2] as follows and was introduced in RAN5 test specification via [3] and [4]:

# 2	Measured data collection and post-processing

In this contribution additional analysis based on TRP measurement results is offered. Thirty four test-points were taken in Free Space (FS) and Besides the Head with Right Hand (BHHR), with different frequencies and different DUTs.

To test the robustness of the reduced grid proposals on [2], all test-points were selected based on antenna radiation pattern with directivity below 5dBi and higher than 6.0 dBi. Radiation pattern directivity higher than 6 dBi is not uncommon in higher frequency bands, and can also be present at lower bands (sub-1GHz) while testing with phantoms.

# 3	Reference Grids

In this contribution TRP measurement results were postprocessed based on 11 grids that have its visual representation on Figures 1 ~ 11.

The Figure 1, 15 (theta) x 15 (phi) is the baseline set of EiRP measurements.

The elevation 180 degrees is not part of this post-processing, since most Anechoic Chambers implementation have this probe position restriction due turn-table physical interference.

Figure 1, 15 degree grid (baseline)

Figure 2, 15/30 Hybrid Grid

Figure 3, 15/30 Theta Phi grid

Figure 4, 30 Degrees grid

Figure 5,  30 degrees Theta Phi grid

Figure 6, 30/60 degrees square grid

Figure 7, 30/60 degrees hybrid

Figure 8, 30/60 Theta Dependent Phi grid

Figure 9, 45 degrees grid average last cut

Figure 10,  45 degrees grid average 2 points

Figure 11, 60 degrees hybrid grid

# 4	Results and discussions

The proposed alternative grids were validated using measured TRP data which was down-sampled from a 15 degree data set.  TRP measurement results were collected on thirty four devices.

The results are presented based on the delta in (dB) from the baseline TRP measurements taken with elevation and azimuth with 15º grid resolution.

The post-processed TRP measurement results are presented on Table 1 and Figure 12.

The results on Table 1 are sorted from lower antenna directivity (blue, threshold < 5dBi) to higher antenna directivity (red, threshold > 6dBi).

Table 1, TRP deltas vs. grid dependency (≤0.04dB, >0.04≥0.1dB, >0.1dB)

Figure 12, Grid dependent delta vs. frequency

Considering a threshold on TRP delta > 0.1dB to be unfit to validate the reduced grid method. The analysis of such patterns indicates that the grid alternatives on 15/30 Hybrid and 15/30 Theta Phi are the reduced grid alternatives with lower overall delta (<0.04dB) in all radiation pattern directivities from 3.52~8.63 dBi, therefore these are the  reduced grids alternatives with best implementation feasibility based on this limited study.

We propose to RAN5 re-evaluate the guidance in TS 38.561 Annex A.4.2.12 that was introduced via [3] and [4], with a decision based on commercial devices measurement TRP measurement results, most importantly taking into consideration the difference in radiation pattern directivities. The analysis should stress the proposed reduced grid adopting devices with high antenna radiation pattern directivity, e.g.: >6dBi.

Reduced grid proposals for TRS shall be based on TRS measurements in commercial devices, TRS has higher measurement uncertainty budget, and different MU contributors than TRP.

Observation 1:	Reduced grid results postprocessed from radiation patterns with lower directivity, considered in this study < 5dBi, might lead to an incorrect conclusion that larger grid reduction can be achieved with acceptable MU. Previous analysis based on simulated radiation patterns also demonstrated optimistic results when compared with measured data, therefore reduced grid analysis based on simulated radiation patterns shall be precluded.

Observation 2:	Radiation patterns with higher directivity, not necessarily will have its higher directivity at horizon (90º) elevation.

Observation 3:	Handsets due its dimension being close to ¼ wavelength at sub-1GHz frequencies, will produce more uniform radiation patterns with higher directivity (~3 dB) at 90º elevation in Free Space, and nulls at the poles. However, same devices can produce patterns with higher directivity at upper of lower hemisphere when tested with head and hand phantoms.

Observation 4:	As shown on Figure 13, the EiRP elevation weighting based on Clenshaw-Curtis approach is biased towards devices with antenna radiation patterns with higher directivity at the 90º elevation, i.e.: EiRP weight = 0.262 at 90º elevation, while at elevations 30º/150º the weight = 0.1315.

Figure 13, Clenshaw-Curtis elevation weighting distribution

Observation 5:	The observations on this study are only valid for TRP measurements. TRS reduced grid analysis shall be performance based on refined grid EiS measurements, e.g.: 15º theta/phi resolution. TRS measurements have higher uncertainty, besides Tx radiation pattern might not be the best indicator for Rx antenna directivity in many cases.

# 5	Observations and Proposals

Observation 1:	Reduced grid results postprocessed from radiation patterns with lower directivity, considered in this study < 5dBi, might lead to an incorrect conclusion that larger grid reduction can be achieved with acceptable MU. Previous analysis based on simulated radiation patterns also demonstrated optimistic results when compared with measured data, therefore reduced grid analysis based on simulated radiation patterns shall be precluded.

Observation 2:	Radiation patterns with higher directivity, not necessarily will have its higher directivity at horizon (90º) elevation.

Observation 3:	Handsets due its dimension being close to ¼ wavelength at sub-1GHz frequencies, will produce more uniform radiation patterns with higher directivity (≈3 dB) at 90º elevation in Free Space, and nulls at the poles. However, same devices can produce patterns with higher directivity at upper of lower hemisphere when tested with head and hand phantoms.

Observation 4:	As shown on Figure 13, the EiRP elevation weighting based on Clenshaw-Curtis approach is biased towards devices with antenna radiation patterns with higher directivity at the 90º elevation, i.e.: EiRP weight = 0.262 at 90º elevation, while at elevations 30º/150º the weight = 0.1315.

Observation 5:	The observations on this study are only valid for TRP measurements. TRS reduced grid analysis shall be performance based on refined grid EiS measurements, e.g.: 15º theta/phi resolution. TRS measurements have higher uncertainty. Tx radiation pattern might not be the best indicator for Rx directivity in many cases.

Proposal 1:	RAN 5 to consider a large device data collection based on pre-defined radiation pattern directivity goals, frequency, FS vs. user case. Post-processing the results  to make a final decision on reduced grid approach.

Proposal 2:	RAN 5 to not consider reduced grids beyond legacy 15º  Theta/Phi for handsets TRP and 30º  Theta/Phi for handsets TRS measurements, until more measurement data-points are analyzed.

Proposal 3:	Conclusions related to reduced grid based on EiRP should be restricted to TRP measurements. TRS reduced grid analysis shall be done based on baseline EiS/TRS measurements.

Proposal 4:	Moving forward RAN 5 shall consider reduced grid analysis based only on measurement results, simulated radiation patterns analysis shall be precluded, the reduced grid analysis shall also be done considering  radiation patterns with high directivity, i.e.: >6 dBi.

Proposal 5:	Revert current references to coarse grids in TRP/TRS test procedures in TS 38.561 [3,4] and move any associated MU values into square brackets.

# 6	References

[1]		R4-2316995, Working procedure for Rel-18 lab alignment and harmonization campaign, vivo, RAN4 #108-bis

[2]		TR 38.870, Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS), RAN4 #108-bis

[3]	R5-235940, pCR with TRP/TRS MU Updates, Keysight Technologies UK Ltd, RAN5 #100

[4]	R5-235194, General TRP/TRS MU Views, Keysight Technologies UK Ltd, RAN5 #100