3GPP TSG-RAN5 Meeting #101	R5-237190

# Chicago, USA, Nov 13th - 17th 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2853 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time test case 5.5.6.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2023-11-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RRM test case 5.5.6.4.1 is missing in TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add RRM test case 5.5.6.4.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM test case 5.5.6.4.1 will remain missing in TS 38.533 and WI cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.6.4.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

///Start of changes

##### 5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time

Editor's Note: This test case is incomplete in following aspects:

-	Test procedure is missing

-	Message contents are missing

-	TT analysis is missing

5.5.6.4.1.1	Test purpose

The purpose of this test is to verify

1) the interruption due to RRM and CSI measurement during SCell dormancy on spCell is within the limits specified in clause 8.2.1.2.15.2 and 8.2.1.2.15.3 in TS 38.133 [6] for NR victim cell, and

2) the SCell dormancy switch delay is within the requirement defined in clause 8.6.2 in TS 38.133 [6], and the SCell dormancy switch interruption is within the limits defined in clause 8.2.1.2.15.1 in TS 38.133 [6] for NR victim cell.

5.5.6.4.1.2	Test applicability

This test applies to all types of EN-DC UE Rel-16 and forward supporting EN-DC with PSCell in FR2.

5.5.6.4.1.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 5.5.6.4.0.

The normative reference for this requirement is TS 38.133 [6] clause A.5.5.6.4.1.

5.5.6.4.1.4	Test description

The test scenario comprises of one E-UTRA PCell (Cell 1), one NR PSCell (Cell 2) and one NR SCell (Cell 3)

The tests consist of three consecutive time periods T1, T2, and T3, respectively. All cells have constant signal levels throughout the test. The UE is continuously scheduled in PCell and PSCell throughout the test.

5.5.6.4.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 5.5.6.4.1.4.1-1.

Table 5.5.6.4.1.4.1-1: Dormancy switch supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations Note 2: A UE which fulfils the requirements in test case in clause 5.5.6.4.2 can skip the test cases in current clause 5.5.6.4.1. Note 3: NR configuration is the same for PSCell and SCells. |  |

Configure the test equipment and the DUT according to the parameters in Table 5.5.6.4.1.4.1-2.

Table 5.5.6.4.1.4.1-2: Initial conditions for EN-DC PSCell FR2

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.3-1 and TS 38.508-1 [14] clause 4.3.1 for E-UTRA and 7.2.3 for NR. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 5.5.6.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.2. |
| Connection Diagram | TE Part | Figure A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | Figure A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |  |

- The general test parameter settings are set up according to Table 5.5.6.4.1.4.1-3. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 in TS 38.133 [6], cell-specific parameters of NR PSCell are specified in Table 5.5.6.4.1.5-1.

- Message contents are defined in clause 5.5.6.4.1.4.3.

- There are one E-UTRAN cell and two NR cells specified in the test. E-UTRAN Cell 1 is the cell used for connection setup with the power level set according to clause C.1.1 and C.1.2 for this test.

Table 5.5.6.4.1.4.1-3: General test parameters for Dormancy switch in synchronous EN-DC

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Subtest 1 | Subtest 2 |  |
| E-UTRA RF Channel Number |  | 1 |  | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2, 3 |  | Two NR radio channel is used for this test |
| Active PCell |  | Cell 1 |  | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 |  | PSCell on RF channel number 2. |
| Active SCell |  | Cell 3 |  | SCell on RF channel number 3. |
| CP length |  | Normal |  |  |
| DRX |  | OFF |  |  |
| Measurement gap pattern Id |  | OFF |  |  |
| bwp-InactivityTimer | ms | 500 |  |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 |  | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 |  | Individual offset for cells on PSCC. |
| Cell-individual offset for cells on RF channel number 3 | dB | 0 |  | Individual offset for cells on SCC. |
| Cell2 timing offset to cell1 | s | 3 |  | Synchronous EN-DC |
| Cell3 timing offset to cell2 | s | 0 |  | Synchronous cells |
| Triggering DCI format |  | DCI 1_1 |  | Triggering DCI format for triggering during active time |
| OFDM symbol range in slot for transmission of DCI with dormancy indication |  | 0 – 2 | 3 – 11 | Test1 is based on that triggering DCI is received within the first three OFDM symbols of a slot. Test2 is based on that the triggering DCI is received later than within the first three OFDM symbols of a slot. |
| T1 | s | 0.2 |  |  |
| T2 | s | 5 |  |  |
| T3 | s | 0.2 |  |  |

5.5.6.4.1.4.2	Test procedure

TBD

5.5.6.4.1.4.3	Message contents

TBD

5.5.6.4.1.5	Test requirement

Table 5.5.6.4.1.5-1 defines the NR cell specific test parameters. Table 5.5.6.4.1.5-2 defines OTA related test parameters, not including test tolerances yet [will update after TT analysis].

5.5.6.4.1.5-1: NR Cell specific test parameters for DL BWP switch in synchronous EN-DC

| Parameter | Unit | Subtest 1 |  | Subtest 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| Frequency Range |  | FR2 |  | FR2 |  |
| Duplex mode |  | TDD |  | TDD |  |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | 100 MHz: NRB,c = 66 |  | 100 MHz: NRB,c = 66 |  |
| Active BWP ID |  | 0 | 0 | 0 | 0 |
| Initial DL BWP Configuration |  | DLBWP.0.2 | DLBWP.0.2 | DLBWP.0.2 | DLBWP.0.2 |
| Active DL BWP-0 Configuration |  | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 | DLBWP.1.1 |
| Active DL BWP-1 Configuration |  | NA | DLBWP.1.2 | NA | DLBWP.1.2 |
| Initial UL BWP Configuration |  | ULBWP.0.2 | ULBWP.0.2 | ULBWP.0.2 | ULBWP.0.2 |
| Active UL BWP-0 Configuration |  | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 | ULBWP.1.1 |
| Active UL BWP-1 Configuration |  | NA | ULBWP.1.2 | NA | ULBWP.1.2 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET parameters |  | CR.3.1 TDD |  | CR.3.1 TDD |  |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |  | CCR.3.2 TDD | CCR.3.1 TDD |
| OCNG Patterns |  | OP.1 |  | OP.1 |  |
| SSB Configuration |  | SSB.1 FR2 |  | SSB.1 FR2 |  |
| SMTC Configuration |  | SMTC.1 |  | SMTC.1 |  |
| TCI State |  | TCI.State.0 |  | TCI.State.0 |  |
| TRS Configuration |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| CSI-RS for CSI reporting |  | CSI-RS.3.1 TDD |  | CSI-RS.3.1 TDD |  |
| CSI reporting periodicity | slots | 640 |  | 640 |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  | 640 |  |
| Antenna Configuration |  | 1x2 |  | 1x2 |  |
| Propagation Condition |  | AWGN |  | AWGN |  |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: For unpaired spectrum, a DL BWP is linked with an UL BWP. DLBWP.0.2 is linked with ULBWP.0.2; DLBWP.1.1 is linked with ULBWP.1.1; DLBWP.1.3 is linked with ULBWP.1.3. |  |  |  |  |  |

Table 5.5.6.4.1.5-2: OTA related test parameters for Dormancy switch in synchronous EN-DC

| Parameter | Unit | Cell 2 | Cell 3 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 according to clause A.3.15 in TS 38.133 [6] |  |
| NocNote 1 | dBm/15 kHz | -112 + TT | -112 + TT |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -85 + TT | -85 + TT |
| Ês/Iot | dB | 18 + TT | 18 + TT |
| IoNote2 | dBm/95.04 MHz Note4 | -56 + TT | -56 + TT |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone. |  |  |  |

During T1, any interruption on PSCell due to dormancy switching of SCell shall be within the requirement specified in in clause 5.5.6.4.0.1for NR victim cell.

During T2, interruptions on PSCell due to CSI and RRM measurements on dormant SCell shall be within the interruption rate requirements specified in 5.5.6.4.0.1for NR victim cell.

During T3, any interruption on PSCell due to dormancy switching of SCell shall be within the requirement specified in in clause 5.5.6.4.0.1for NR victim cell. Monitoring of PDCCH for SCell in PSCell shall be resumed within the dormancy switching time specified in TS 38.133 [6] clause 8.6.2A.

For an event to be considered to be correct, all requirements above have to be fulfilled.

The rate of correct events observed during repeated tests shall be at least 90%.

///End of changes