3GPP TSG-RAN WG5 Meeting #101	R5-237223

Chicago, USA, November 13th – November 17th, 2023

Agenda Item:	5.3.29.9

Source:	THALES

Title: 	Summary of SIB19/SIB31 parameters for NGSO and GSO NTN UE/NTN IoT testing

Document for:	Information

# Introduction

The main objective of this technical discussion document is first to recall the transfer function and secondly to provide an application example with SIB19/SIB31 values. The data required by the Test Equipment to implement SIB19 (NTN dedicated SIB from 5G NR Rel-17) or respectively SIB31/SIB32 (for IoT NTN) is based on two potential information: ECEF information and/or Orbital information. Effectively, it has been agreed in RAN1 that UEs are provided with satellite ephemeris in ECI of the date (namely ECEF) in either state vector (position velocity) or orbital information (Kepler elements).

This document presents examples of ephemeris generation. Please also refer to R5-237216 (THALES, “NGSO satellite Ephemeris file generation methodology for NTN NR UE testing”) and R5-237213 (THALES, “GSO satellite Ephemeris file generation methodology for NTN NR UE testing”) for a more complete set of explanations and results.

# Recall of RRC Specification Protocol (TS 38.331) in Rel-17

The specification TS 38.331 "Technical Specification Group Radio Access Network - NR - Radio Resource Control (RRC) protocol specification" indicates the required satellite ephemeris information to be included in SIB and transmitted/broadcasted by the satellite. The UE uses this information (namely SIB19 for NTN 5G NR) to perform UL Doppler and timing advance pre-compensation.

-- ASN1START

-- TAG-EPHEMERISINFO-START

EphemerisInfo-r17 ::=          CHOICE {

positionVelocity-r17           PositionVelocity-r17,

orbital-r17                    Orbital-r17

}

PositionVelocity-r17 ::=       SEQUENCE {

positionX-r17                  PositionStateVector-r17,

positionY-r17                  PositionStateVector-r17,

positionZ-r17                  PositionStateVector-r17,

velocityVX-r17                 VelocityStateVector-r17,

velocityVY-r17                 VelocityStateVector-r17,

velocityVZ-r17                 VelocityStateVector-r17

}

Orbital-r17 ::=                SEQUENCE {

semiMajorAxis-r17              INTEGER (0..8589934591),

eccentricity-r17               INTEGER (0..1048575),

periapsis-r17                  INTEGER (0..268435455),

longitude-r17                  INTEGER (0..268435455),

inclination-r17                INTEGER (-67108864..67108863),

meanAnomaly-r17                INTEGER (0..268435455)

}

PositionStateVector-r17 ::= INTEGER (-33554432..33554431)

VelocityStateVector-r17 ::= INTEGER (-131072..131071)

-- TAG-EPHEMERISINFO-STOP

-- ASN1STOP

With the following description:

EphemerisInfo field descriptions

Eccentricity Satellite orbital parameter: eccentricity e, see NIMA TR 8350.2 [71]. Unit is radian. Step of 1.431 * 10-8. Actual value = field value * (1.431 * 10-8).

Inclination Satellite orbital parameter: inclination i, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

Longitude Satellite orbital parameter: longitude of ascending node , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

meanAnomaly Satellite orbital parameter: Mean anomaly M at epoch time, see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

Periapsis Satellite orbital parameter: argument of periapsis , see NIMA TR 8350.2 [71]. Unit is radian. Step of 2.341* 10-8 rad. Actual value = field value * (2.341* 10-8).

positionX, positionY, positionZ X, Y, Z coordinate of satellite position state vector in ECEF. Unit is meter. Step of 1.3 m. Actual value = field value * 1.3.

semiMajorAxis Satellite orbital parameter: semi major axis , see NIMA TR 8350.2 [71]. Unit is meter. Step of 4.249 * 10-3 m. Actual value = 6500000 + field value * (4.249 * 10-3).

velocityVX, velocityVY, velocityVZ X, Y, Z coordinate of satellite velocity state vector in ECEF. Unit is meter/second. Step of 0.06 m/s. Actual value = field value * 0.06.

# Application examples

The transfer function as defined in TS 38.331 Release-17 allows the conversion of the Datafield to its corresponding INTEGER value as shown in below:

For NTN UE, the SIB19 requires to provide either “positionVelocity-r17” or “orbital-r17” Datafield as INTEGER, meaning that ephemeris given to Network Operating Center are to be coded to INTEGER prior to any transmission to a UE

Coded parameter= Physical Value-ba

Two cases of generation of SIB19 “positionVelocity-r17” and “orbital-r17” for LEO (600 km) and GSO are proposed based upon available ephemeris generated by, in this case GMAT (used for ephemeris generation). Results are represented in the table below:

Further, one can represent the following few “selected” scenarios (as examples, to give some idea about the ranges) computed with a min elevation angle target of 30°:

| Frequency | Constellation | Elevation | Doppler shift (kHz) | OneWayDelay_ms | Xsat_ECEF_Coded_m | Ysat_ECEF_Coded_m | Zsat_ECEF_Coded_m | VXsat_ECEF_Coded_m | VYsat_ECEF_Coded_m | VZsat_ECEF_Coded_m | sma_ECEF_Coded | Excentricity_ECEF_Coded | Inc_ECEF_Coded | RAAN_ECEF_Coded | AOP_ECEF_Coded | MeanAnom_ECEF_Coded |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,00 GHz | LEO@600 km | 30,0 | 39,734 | 3,6 | -2654979,0 | 4388008,0 | 1590654,0 | 14542,0 | -34415,0 | 120208,0 | 127433469,0 | 625589,0 | 65357030,0 | 89802029,0 | 197030161,0 | 11181850,0 |
| 2,00 GHz | LEO@600 km | 149,9 | -39,899 | 3,7 | -2401021 | 3830217 | 2907041 | 28879 | -61410 | 105774 | 126505185 | 585942 | 64768697 | 89517532 | 197339614 | 22986454 |
| 2,00 GHz | LEO@600 km | 89,6 | 0,026 | 2,0 | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 127026053 | 607046 | 65099618 | 89701469 | 196621435 | 16735395 |
| 2,00 GHz | LEO@1200 km | 29,9 | 35,127 | 6,7 | -2935802 | 4896147 | 1184965 | 8471 | -23252 | 118268 | 273627907 | 769347 | 65474780 | 89834882 | 198836074 | 7798966 |
| 2,00 GHz | LEO@1200 km | 149,9 | -35,273 | 6,8 | -2471684 | 3860360 | 3621839 | 31517 | -67464 | 94643 | 272191987 | 715100 | 64360117 | 89258597 | 199900686 | 27917535 |
| 2,00 GHz | LEO@1200 km | 89,5 | 0,007 | 4,0 | -2771542 | 4509243 | 2455572 | 20837 | -46176 | 109528 | 273057241 | 744972 | 65034798 | 89682433 | 197978669 | 17169725 |
| 30,00 GHz | LEO@600 km | 30,0 | 596,011 | 3,6 | -2654979 | 4388008 | 1590654 | 14542 | -34415 | 120208 | 127433469 | 625589 | 65357030 | 89802029 | 197030161 | 11181850 |
| 30,00 GHz | LEO@600 km | 149,9 | -598,486 | 3,7 | -2401021 | 3830217 | 2907041 | 28879 | -61410 | 105774 | 126505185 | 585942 | 64768697 | 89517532 | 197339614 | 22986454 |
| 30,00 GHz | LEO@600 km | 89,6 | 0,384 | 2,0 | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 127026053 | 607046 | 65099618 | 89701469 | 196621435 | 16735395 |
| 30,00 GHz | LEO@1200 km | 29,9 | 526,908 | 6,7 | -2935802 | 4896147 | 1184965 | 8471 | -23252 | 118268 | 273627907 | 769347 | 65474780 | 89834882 | 198836074 | 7798966 |
| 30,00 GHz | LEO@1200 km | 149,9 | -529,095 | 6,8 | -2471684 | 3860360 | 3621839 | 31517 | -67464 | 94643 | 272191987 | 715100 | 64360117 | 89258597 | 199900686 | 27917535 |
| 30,00 GHz | LEO@1200 km | 89,5 | 0,110 | 4,0 | -2771542 | 4509243 | 2455572 | 20837 | -46176 | 109528 | 273057241 | 744972 | 65034798 | 89682433 | 197978669 | 17169725 |
| 2,00 GHz | GEO | 90,14 | 0,000 | 119,37 | -16975960 | 27636606 | 68083 | 0 | 0 | -48 | 8393577246 | 0 | 224827774 | 67241608 | 268307824 | 18691093 |
| 2,00 GHz | GSO | 58,85 | 0,19 | 121,99 | -16926361 | 27652344 | -903320 | 387 | 36 | -6130 | 8393577246 | 0 | 224829171 | 69524249 | 1191777 | 143241486 |
| 2,00 GHz | GSO | 58,85 | -0,19 | 121,99 | -17018430 | 27595775 | -903373 | 207 | 328 | 6130 | 8393577246 | 0 | 90637528 | 69524136 | 133006899 | 257895279 |
| 2,00 GHz | GSO | 68,87 | 0,000 | 120,59 | -16850867 | 27425327 | 3983665 | -330 | -203 | 0 | 8393577246 | 0 | 23571205 | 5259905 | 201262439 | 66369698 |
| 30,00 GHz | GEO | 90,14 | 0,000 | 119,37 | -16975960 | 27636606 | 68083 | 0 | 0 | -48 | 8393577246 | 0 | 224827774 | 67241608 | 268307824 | 18691093 |
| 30,00 GHz | GSO | 58,85 | 2,87 | 121,99 | -16926361 | 27652344 | -903320 | 387 | 36 | -6130 | 8393577246 | 0 | 224829171 | 69524249 | 1191777 | 143241486 |
| 30,00 GHz | GSO | 58,85 | -2,87 | 121,99 | -17018430 | 27595775 | -903373 | 207 | 328 | 6130 | 8393577246 | 0 | 90637528 | 69524136 | 133006899 | 257895279 |
| 30,00 GHz | GSO | 68,87 | 0,000 | 120,59 | -16850867 | 27425327 | 3983665 | -330 | -203 | 0 | 8393577246 | 0 | 23571205 | 5259905 | 201262439 | 66369698 |

Similarly, one can represent the following few “selected” scenarios (as examples, to give some idea about the ranges) computed with a min elevation angle target of 10°:

| Frequency | Constellation | Elevation | Doppler shift (kHz) | OneWayDelay_ms | Xsat_ECEF_Coded_m | Ysat_ECEF_Coded_m | Zsat_ECEF_Coded_m |  | VXsat_ECEF_Coded_m | VYsat_ECEF_Coded_m | VZsat_ECEF_Coded_m | sma_ECEF_Coded | Excentricity_ECEF_Coded | Inc_ECEF_Coded | RAAN_ECEF_Coded | AOP_ECEF_Coded | MeanAnom_ECEF_Coded |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2,00 GHz | LEO@600 km | 9,9 | 45,304 | 6,5 | -2717798,0 | 4550992,0 | 849128,0 |  | 6122,0 | -19350,0 | 124295,0 | 127718928,0 | 639459,0 | 65536691,0 | 89845489,0 | 198612804,0 | 5766116,0 |
| 2,00 GHz | LEO@600 km | 170,0 | -45,239 | 6,6 | -2199272 | 3404229 | 3535794 |  | 35394 | -74414 | 94682 | 125875956 | 565437 | 64365904 | 89211929 | 199233028 | 30409196 |
| 2,00 GHz | LEO@600 km | 89,6 | 0,026 | 2,0 | -2550081 | 4151097 | 2263034 |  | 21970 | -48164 | 114109 | 127026053 | 607046 | 65099618 | 89701469 | 196621435 | 16735395 |
| 2,00 GHz | LEO@1200 km | 10,0 | 40,062 | 10,4 | -2962861 | 5019276 | 97850 |  | -2615 | -3875 | 120784 | 273800787 | 777311 | 65607366 | 89853215 | 201096313 | 632648 |
| 2,00 GHz | LEO@1200 km | 170,0 | -39,918 | 10,7 | -2142675 | 3156398 | 4434942 |  | 38457 | -82453 | 78501 | 271404201 | 698334 | 63736334 | 88651622 | 203306610 | 37653139 |
| 2,00 GHz | LEO@1200 km | 89,5 | 0,007 | 4,0 | -2771542 | 4509243 | 2455572 |  | 20837 | -46176 | 109528 | 273057241 | 744972 | 65034798 | 89682433 | 197978669 | 17169725 |
| 30,00 GHz | LEO@600 km | 9,9 | 679,567 | 6,458 | -2717798 | 4550992 | 849128 |  | 6122 | -19350 | 124295 | 127718928 | 639459 | 65536691 | 89845489 | 198612804 | 5766116 |
| 30,00 GHz | LEO@600 km | 170,0 | -678,582 | 6,605 | -2199272 | 3404229 | 3535794 |  | 35394 | -74414 | 94682 | 125875956 | 565437 | 64365904 | 89211929 | 199233028 | 30409196 |
| 30,00 GHz | LEO@600 km | 89,6 | 0,384 | 2,031 | -2550081 | 4151097 | 2263034 |  | 21970 | -48164 | 114109 | 127026053 | 607046 | 65099618 | 89701469 | 196621435 | 16735395 |
| 30,00 GHz | LEO@1200 km | 10,0 | 600,927 | 10,441 | -2962861 | 5019276 | 97850 |  | -2615 | -3875 | 120784 | 273800787 | 777311 | 65607366 | 89853215 | 201096313 | 632648 |
| 30,00 GHz | LEO@1200 km | 170,0 | -598,774 | 10,656 | -2142675 | 3156398 | 4434942 |  | 38457 | -82453 | 78501 | 271404201 | 698334 | 63736334 | 88651622 | 203306610 | 37653139 |
| 30,00 GHz | LEO@1200 km | 89,5 | 0,110 | 4,039 | -2771542 | 4509243 | 2455572 |  | 20837 | -46176 | 109528 | 273057241 | 744972 | 65034798 | 89682433 | 197978669 | 17169725 |
| 2,00 GHz | GEO | 90,14 | 0,000 | 119,37 | -16975960 | 27636606 | 68083 |  | 0 | 0 | -48 | 8393577246 | 0 | 224827774 | 67241608 | 268307824 | 18691093 |
| 2,00 GHz | GSO | 58,85 | 0,19 | 121,99 | -16926361 | 27652344 | -903320 |  | 387 | 36 | -6130 | 8393577246 | 0 | 224829171 | 69524249 | 1191777 | 143241486 |
| 2,00 GHz | GSO | 58,85 | -0,19 | 121,99 | -17018430 | 27595775 | -903373 |  | 207 | 328 | 6130 | 8393577246 | 0 | 90637528 | 69524136 | 133006899 | 257895279 |
| 2,00 GHz | GSO | 68,87 | 0,000 | 120,59 | -16850867 | 27425327 | 3983665 |  | -330 | -203 | 0 | 8393577246 | 0 | 23571205 | 5259905 | 201262439 | 66369698 |
| 30,00 GHz | GEO | 90,14 | 0,000 | 119,37 | -16975960 | 27636606 | 68083 |  | 0 | 0 | -48 | 8393577246 | 0 | 224827774 | 67241608 | 268307824 | 18691093 |
| 30,00 GHz | GSO | 58,85 | 2,87 | 121,99 | -16926361 | 27652344 | -903320 |  | 387 | 36 | -6130 | 8393577246 | 0 | 224829171 | 69524249 | 1191777 | 143241486 |
| 30,00 GHz | GSO | 58,85 | -2,87 | 121,99 | -17018430 | 27595775 | -903373 |  | 207 | 328 | 6130 | 8393577246 | 0 | 90637528 | 69524136 | 133006899 | 257895279 |
| 30,00 GHz | GSO | 68,87 | 0,000 | 120,59 | -16850867 | 27425327 | 3983665 |  | -330 | -203 | 0 | 8393577246 | 0 | 23571205 | 5259905 | 201262439 | 66369698 |

As also considered in TS 38.508-1, UE position during the test is considered as stationary and configured as shown below:

- Latitude: 25.08439333

- Longitude: 121.56076999

- Altitude: 0

# Conclusion

Observation 1: A summary of SIB19/SIB31 values have been provided in this contribution for NTN UE/IoT testing purpose.

Observation 2: A complete set of SIB19/SIB31 values for NTN UE/IoT testing purpose are provided in TS 38.508-1 and TS 36.508 for both NGSO and GSO cases.

Observation 3: Please also refer to R5-237216 and R5-237213 contributions.